3GPP TSG RAN WG1 Meeting #103-e	                	   	                R1-2007823

e-Meeting, October 26th – November 13th, 2020

Source:	CATT

Title:	Discussion on linear value of SRS Power split by UE

Agenda item:	7.2.8

Document for:	Discussion and Decision

Introduction

In RAN1#102-e meeting, the following agreements were achieved on remaining issues on Rel-16 NR positioning [1]. In this contribution, we discuss one TP on linear value of SRS Power split by UE.

Agreement: - TP#0-1 in R1-2007125 for TS38.211 is endorsed. Agreement: - TP#2-1 and TP#3-1 in R1-2007125 for TS38.214 is endorsed. Agreement: - Change names of the following higher layer parameters in the TS 38.214 clause “5.1.6.5 PRS reception procedure” - ‘nr-DL-PRS-RstdMeasurementInfoRequest-r16’ to ‘NR-DL-TDOA-SignalMeasurementInformation’ - ‘DL-PRS-UE-Rx-Tx-MeasurementInfo’ to ‘NR-Multi-RTT-SignalMeasurementInformation’ respectively. - Change names of the following higher layer parameter in the TS 38.211 to align with RAN2 specifications - dl-PRS-SequenceId-r16 to dl-PRS-SequenceID-r16 - Change names of the following higher layer parameters in the TS 38.211 and TS 38.214 to align with RAN2 specifications - dl-PRS-ReOffset-r16 to dl-PRS-CombSizeN-and-ReOffset-r16 - mutingOption1-r16 to dl-PRS-MutingOption1-r16 - mutingOption2-r16 to dl-PRS-MutingOption2-r16 - dl-PRS-MutingPatternList-r16 to dl-PRS-MutingOption1-r16 and dl-PRS-MutingOption2-r16 Agreement: - TP2.3B and TP2.4Abis in Section 3 of R1-2007104 for Clause 6.2.1 of TS 38.214 are endorsed. Agreement: - Text proposal 1 in Section 5 of R1-2007095 for TS 38.214 is endorsed. Agreement: - Text proposals 2 and 3 in Section 5 of R1-2007095 for TS 38.215 are endorsed.

Discussion

Linear value of SRS Power split by UE

In section 7.3 of TS 38.213 [2], it is mentioned that, for SRS, a UE splits a linear value  of the transmit power  equally across the configured antenna ports for SRS, as shown in below.

- 7.3 Sounding reference signals For SRS, a UE splits a linear value of the transmit power on active UL BWP of carrier of serving cell equally across the configured antenna ports for SRS.

However, for SRS-Pos, only one antenna port can be configured, therefore, this above sentence should be only applicable for SRS-MIMO, but not for SRS-Pos.

In order to solve the above issues, one text proposal is proposed for section 7.3 in TS 38.213 as follows.

Proposal 1: Adopt the following text proposal (TP-A) for linear value of SRS Power split by UE in section 7.3 of 38.213:

# Proposed TP-A -----------------------------------------------Start of Text Proposal for 38.213---------------------------------------------- 7.3 Sounding reference signals For SRS configured by the higher parameter SRS-Resource, a UE splits a linear value of the transmit power on active UL BWP of carrier of serving cell equally across the configured antenna ports for SRS. ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ --------------------------------------------------------End of Text Proposal --------------------------------------------------

Conclusions

In this contribution, we discuss one TP on linear value of SRS Power split by UE. Based on the discussion, we propose:

Proposal 1: Adopt the following text proposal (TP-A) for linear value of SRS Power split by UE in section 7.3 of 38.213:

# Proposed TP-A -----------------------------------------------Start of Text Proposal for 38.213---------------------------------------------- 7.3 Sounding reference signals For SRS configured by the higher parameter SRS-Resource, a UE splits a linear value of the transmit power on active UL BWP of carrier of serving cell equally across the configured antenna ports for SRS. ----------------------------------------------------- Unchanged part omitted ------------------------------------------------ --------------------------------------------------------End of Text Proposal --------------------------------------------------

References

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #102-e.

- 3GPP TS 38.213, “NR; Physical layer procedures for control”，v16.3.0，2020-09.