3GPP TSG RAN WG1 #103-e		                                    R1-2007841

e-Meeting, October 26th – November 13th, 2020

Source:                CATT

Title:		Discussion on efficient activation and de-activation mechanism for SCell in NR CA

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

Support efficient activation/de-activation mechanism for one SCG and SCells is approved as the objective of further Multi-RAT Dual-Connectivity enhancements.[1] During the last RAN1 meeting, the design of temporary RS was preliminarily discussed and the following agreements were achieved:

Working Assumption

At least for the case of known cell, temporary RS is supported to expedite the activation process during the SCell activation procedure for efficient SCell activation for both FR1 and FR2:

         The temporary RS should provide at least the functionalities of AGC settling and time/frequency tracking during SCell activation procedure.

         FFS potential functionalities of CSI measurement/acquisition and cell search

Agreements:

TRS is selected as temporary RS for Scell activation

         If more functionalities are confirmed to be supported by temporary RS, other RS candidates, e.g. aperiodic CSI-RS, P/SP-CSI RS, SRS and RS based on SSS/PSS, are not precluded.

         The TRS should be triggered by DCI or MAC-CE. FFS which exact triggering command.

Agreements:

UEs measure the triggered temporary RS during Scell activation procedure no earlier than a slot m:

         FFS timeline values m which may need coordination with RAN4.

         FFS if the triggered temporary RS can be associated with a BWP, then the measurement above is independent of the activation state of the BWP.

Agreements:

Companies are encouraged to provide design details of temporary RS next meeting, at least including:

- TRS structure, e.g. whether to fully reuse existing Rel-15/16 TRS structure and configuration restriction (refer to S5.1.6.1.1 of TS 38.214), or any modification

- QCL information, if any

- Triggering command: DCI format/fields or MAC-CE fields

- Triggering timeline/scheduling offset

In this contribution, we provide our views on the procedure of efficient activation and de-activation mechanism for SCell in NR CA.

Discussion

A de-activated SCell can be activated by the MAC CE carried by a PDSCH on an activated serving cell. Generally speaking, the SCell activation/de-activation delay consists of three components, i.e. THARQ, Tactivation_time and TCSI_Reporting, which is defined in TS38.133.  One example assuming SCell measurement cycle is smaller than 160ms and the SCell is known in FR1 is shown in Figure 1.

Figure 1: SCell activation delay for a known SCell in FR1

The timing for secondary cell activation/deactivation which is defined in the current specification quoted as below. The intention of the highlighted parts is to describe when a UE on the activating secondary cell applies the corresponding actions on an active SCell, which includes CSI reporting for the SCell, PDCCH monitoring on the SCell, PDCCH monitoring for the SCell and so on.  However, it only specifies a time window where the normal SCell operation may happen without any accurate timing. As shown in Figure 1, there are three time points which may result in different understandings on the SCell activation operation.

- Time point#1: If a SCell is regarded as activated right after 3 ms later than the ACK corresponding to the MAC CE carrying the activation indication, A-CSI reporting for the SCell and PDCCH monitoring for the SCell is available after time point#1. However, the time/frequency tracking cannot be properly handled before time point#2 if it can only be based on SSB.

- Time point#2: If SCell is regarded as activated right after AGC operation, A-CSI reporting for the SCell and PDCCH monitoring for the SCell is available as long as the UE finishes the time/frequency synchronization. An aperiodic CSI reporting can be triggered immediately and the SCell activation delay mainly comes from the alignment time to the first available SSB.

- Time point#3: If SCell is regarded as activated right after time point#3, A-CSI reporting for the SCell and PDCCH monitoring for the SCell is only available after the valid CSI reporting.  The CSI reporting is based on the periodic CSI-RS. Correspondingly, the SCell activation delay consists of alignment time to the first available SSB and the uncertain CSI reporting delay.

TS38.213, section 4.3 With reference to slots for PUCCH transmissions, when a UE receives in a PDSCH an activation command [11, TS 38.321] for a secondary cell ending in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than slot , except for the following: - the actions related to CSI reporting on a serving cell that is active in slot - the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies in slot - the actions related to CSI reporting on a serving cell which is not active in slot that the UE applies in the earliest slot after in which the serving cell is active. The value of is where is a number of slots for a PUCCH transmission with HARQ-ACK information for the PDSCH reception and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format scheduling the PDSCH reception as described in Subclause 9.2.3 and is a number of slots per subframe for the SCS configuration of the PUCCH transmission.

The precise time point for SCell activation was preliminarily discussed and the views from companies are diverse. Some companies argued that the timeline in the current specification is clear enough and no need to further discuss. However, as analyzed above, the exact time that a UE applies the actions relevant to an active SCell, i.e. PDCCH monitoring for the SCell, PDCCH monitoring on the SCell and CSI reporting, is totally UE implementation.  It should be kept in mind that all the efforts on reducing the activation time focus on the period when UE cannot apply the active actions. In the other words, the current mechanisms are sufficient if the actions on active cell are available, e.g. A-TRS can be triggered by a DCI for time/frequency tracking to reduce the latency comes from SMTC and A-CSI reporting can be triggered by a DCI for valid CSI reporting to reduce the latency comes from uncertain time of CSI reporting.  The impact on the design of efficient activation and de-activation mechanism for SCell related to different time points is summarized in Table 1.

Table 1: The impact comes from different time points during SCell activation procedure

| The time point of applying actions on active cell | The possible solutions to expedite SCell activation/de-activation |
| --- | --- |
| Time point#1 | PDCCH monitoring and CSI reporting for the SCell is allowed after time point#1. We don’t need further discuss the triggering command for A-TRS/CSI-RS as gNB can already trigger the aperiodic RS via DCI. |
| Time point#2 | PDCCH monitoring and CSI reporting for the SCell is allowed after time point #2. The dominated part of activation latency is the waiting time of first available SSB. Temporary RS is needed in order to reduce the alignment delay to first available SSB. |
| Time point#3 | PDCCH monitoring and CSI reporting for the SCell is allowed after time point #3. The delay comes from first available SSB and uncertain CSI reporting should be both take into consideration. Temporary RS is needed in order to reduce the overall delay. |

Proposal 1: The accurate timing for SCell activation should be clarified in order to facilitate the design of efficient SCell activation and de-activation.

If time point#2 or time point#3 is the common understanding from which the actions on active cell are applicable, temporary RS is needed to speed up the procedure of SCell activation/de-activation. As shown in Figure 2, the dominated factor of SCell delay is the alignment time to the first SSB and the delay caused by the uncertainty of the valid CSI reporting. The straightforward solution is to trigger a temporary RS after gNB receives an ACK to the MAC CE carrying an activation indication. Subsequently, UE handles the time/frequency tracking and CSI measurement based on the temporary RS. The temporary RS can be triggered by either DCI or MAC CE activating the SCell. One example is shown in Figure 2.

Figure 2: Efficient SCell activation by introducing temporary RS

Observation: The A-CSI-RS/A-TRS based time-frequency tracking and CSI measurement is beneficial for efficient SCell activation.

Based on the observation, we have the following proposal:

Proposal 2: The A-CSI-RS/A-TRS based time-frequency tracking and CSI measurement should be adopted for efficient SCell activation.

It is well-known that all the BWP configured on an inactive cell are inactive before the cell is activated. If a UE measures the triggered temporary RS during Scell activation procedure, the measurement on the target BWP should be allowed despite of the activation state of the BWP. The target BWP doesn’t have to be the one with firstActiveDownlinkBWP-Id as gNB should have the full flexibility to trigger A-TRS on the expected BWP. Generally speaking, TRS is sufficient for time/frequency tracking purpose. The restrictions on the TRS configuration focus on the time-domain locations, bandwidth, density and the periodicity of P-TRS, which should not be the flaw for the usage of time/frequency tracking. Considering the triggering timeline can be down to 0 slot, the delay would be significantly reduced compared to the alignment delay coming with first available SSB.  However, the restriction on the periodicity, i.e. the periodicity has to be larger than 10 ms, should be further considered if P-TRS is adopted.

Proposal 3: If new temporary RS is introduced to expedite the activation process during the SCell activation procedure for efficient SCell activation

- UE can measure the temporary RS on the target BWP despite of the activation state of the BWP

- At least reuse the existing TRS structure and restriction for A-TRS

Conclusion

This contribution discusses the issues corresponding to efficient SCell activation. We have the following observation and proposals:

Observation: The CSI-RS/TRS based time-frequency tracking and CQI measurement is beneficial for efficient SCell activation.

Proposal 1: The accurate timing for SCell activation should be clarified.

Proposal 2: The A-CSI-RS/A-TRS based time-frequency tracking and CSI measurement should be adopted for efficient SCell activation.

Proposal 3: If new temporary RS is introduced to expedite the activation process during the SCell activation procedure for efficient SCell activation

- UE can measure the temporary RS on the target BWP despite of the activation state of the BWP

- At least reuse the existing TRS structure and restriction for A-TRS

References

  - RP-201040, “Revised WID on Further Multi-RAT Dual-Connectivity enhancements”, Huawei, RAN#88e

  - TS38.213, V16.3.0