3GPP TSG RAN WG1 Meeting #103-e	R1-2007905

Oct 26th – Nov 13th, 2020

Agenda Item:	8.8.2.1

Source:	IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

Title:	PUSCH coverage enhancements

Document for:	Discussion and decision

# Motivation

In RAN#86, NR coverage enhancement was identified as one RAN work area for Rel-17 [1]. The SI requires to identify the performance targets for coverage enhancement, and study the potential solutions for coverage enhancements for the identified scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study additional enhanced solutions for FR2, if any

  - Evaluate the performance of the potential solutions based on link level simulations.

In our companion contribution [4], we provided baseline evaluations for the 5G NR coverage as per the agreements made in [2], [3]. In this contribution, we provide an overview of the solutions that should be considered for enhancing the coverage of 5G NR in Rel-17.

# Potential Solutions for Coverage Enhancement of 5G NR

Coverage expansion especially for rural use cases has been a major requirement arising from Indian IMs of 3GPP for several years. This requirement stems from a large-scale rural deployment envisaged for 5G applications. It is generally expected that Indian deployments will adopt 5G NR for both rural and urban use cases in the available 5G NR bands including the 3.3-3.6 GHz band. The 3GPP specifications should aim to maximize the coverage for 3.3-3.6 GHz TDD band as well sub GHz FDD (e.g., 700MHz) bands.

Low-Mobility-Large-Cell (LMLC) (Rural-C) has been specified by ITU as a mandatory requirement for IMT-2020 specifications. Though LMLC evaluations specify 6Km ISD at 700MHz carrier frequency, the actual deployment scenario need not be restricted to 6Km ISD. We expect Rel-17 specifications to support significantly higher ISD for rural uses cases both 3.3-3.6 GHz TDD band as well sub GHz FDD (e.g., 700MHz) bands.

Increased coverage fundamentally stems from an improvement in link budgets offered by the physical layer. In the context of Rel-17 of 5G NR, such improvements may be obtained due to an increase in UE transmission power, use of additional antennas at gNB, or an inherent ability of the system to operate at lower SINR compared to Rel-15 and Rel-16 of 5G NR. Enhancements such as an increase in base station height (that reduces path-loss) are deployment-related improvements and are generally not related to 3GPP specifications.

Large cells result in low SNR or SINR. Therefore, the SI should target physical layer enhancements that support VoIP and other eMBB services under much lower SINR conditions at cell edge than currently supported by 5G NR specifications. To support large cells, the link should support cell edge SINR that is [x] dB less than rel-16 of 5G NR. This implies that the system can support an additional [x] dB increase in MCL over rel-16 of 5G NR.

Observation: 5G NR coverage enhancement should support additional [x] dB increase in MCL over rel-16 of 5G NR.

Observation:  Coverage enhancement SI should support higher MCL which directly results in higher ISD compared to existing IMT-2020 evaluations.

Proposal: Identify [x] dB via system and link-level simulations.

## TBS related enhancements

The specification should support uplink narrowband operation with sustained UE scheduling over multiple slots under low SINR conditions. This could consider TBS size calculation enhancements to support transmissions over multiple UL slots i.e., TB processing over multiple slots. Below, we consider the case of a single TB sized over multiple slots. We show the gains obtained using this method for one such scenario described in Table-1.

Table 1: PUSCH settings for TB processing over multiple slots

| Parameter | Value |
| --- | --- |
| Channel model | Rural, TDL-D, 30ns |
| Waveform | DFT-s-OFDM |
| MCS | 3 |
| PRBs | 4 (for 100kbps), 1 (for 30kbps) |
| SCS | 30kHz |
| DMRS | 2 in a slot (2nd and 11th symbols) |
| Number of Transmit Chains | 1 |
| Number of Receiver Chains | 2 |
| Slot aggregation | 2, TBS calculated using the number of slots aggregated. |
| Data rate | 100kbps/ 30kbps |
| Frame structure | DDDSU |

Figure: BLER comparison with (slot agg = 2) and without (slot agg = 1) slot aggregation for 100 kbps (left) and 30 kbps (right)

The gains obtained with this multiple slot TB processing depend on the settings and the data rates being considered. The gains would be large when smaller payloads are considered due to reduction in the CRC overhead achieved via multi slot TBS calculations.

Proposal: Study enhanced TBS calculations to increase the MCL for Rel-17 by supporting transmissions over multiple UL slots.

## Waveform related enhancements

For large cells, with increasing distance, the link becomes dominated by the thermal noise. This is different from smaller cells where interference plays a significant role in determining cell capacity. Any technique that improves the link margin would result in a proportional increase in data rate independent of interference characteristics. DFT-S-OFDM waveform with pi/2 BPSK modulation offers low PAPR and allows maximum UE transmit power up to 26 dBm in Rel-15 and Rel-16. This feature should be considered for enhancing the cell coverage. Given that the system is noise limited, the increased UE power directly translates to an improvement in the link SINR. These results are shown in our associated contribution [4] where we show the gains in MCL/MIL/MPL when using the proposed waveforms. Clearly there is a gain and enhanced coverage improvements with the proposed solutions.

Observation: UE with 26 dbm max Tx power for UL duty cycle < 50% provides a substantial increase in cell edge data rates.

HPUE is designed to support 26 dBm transmission for QPSK DFT-s-OFDM. However, QPSK transmission does not reach the PCMax while pi/2 BPSK can operate close to PCMax. In Rel-17, the RAN4 should consider feasibility of increasing UE transmission power well above 26 dBm (up to 29 dBm) based on the UL duty cycle for pi/2 BPSK.

For more results on the same, please see our contribution [4] for the gains obtained using these proposed enhancements and the Appendix in this document. Considering these benefits, RAN1 must consider sending an LS to RAN4 to study the feasibility of power boosting based on UL duty cycle.

Proposal: Make pi/2 BPSK power boosting a function of the UL duty cycle.

Proposal: Send LS to RAN4 to study the feasibility of power boosting for pi/2 BPSK modulation beyond 26 dBm as a function of the UL duty cycle.

# References

[1] RP-193240, New SID on NR coverage enhancement

[2] RAN1 Chairmans Report, RAN1#101e, June 2020.

[3] RAN1 Chairmans Report, RAN1#102e, Aug 2020.

[4] R1-2007904, Baseline coverage performance for uplink, IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

# Appendix (Multi-cell System Simulation for Proposed Enhancements)

In this section, we show the system level evaluations in the form of SINR CDF and spectral efficiency/ data rate calculations for the rural setup. We specifically show the benefits of employing the pi/2 BPSK modulation with power boosting as per the frame structure.

Table 3: SLS Setup

| Parameter | Value |
| --- | --- |
| Carrier frequency | 4 GHz |
| Antennas at BS | 8×4×2= 64 antenna elements for fc=4 GHz |
| Tx RUs per sector | 8 |
| Inter-site Distance | 30 Km |
| Antenna at UE | 1 |
| Pathloss Model | RMa-LMLC [ITU M.2412] |
| Fast Fading Model | RMa [3GPP TR 38.901] |
| System Bandwidth | 10 MHz |
| User Density | 10 per sector |
| User Distribution | 40% indoor, 40% outdoor, 20% outdoor in car |
| Mechanical/Electrical Tilt | 90 in GCS/92 in LCS |
| BS/UE Height | 35 m/1.5 m |
| UE transmit power | 23 dBm |
| BS/UE antenna gain | 8 dBi/0 dBi |
| BS Noise Figure | 3 dB |
| Resource allocation | Equal allocation |
| Receiver Type | MMSE |
| Uplink Power Control | P0=-118 dBm, alpha=0.9 |
| Interference Modelling | Explicit |
| CQI Tables | For the current scheme, existing CQI tables in Rel-15/Rel-16 are used. For the proposed scheme, enhanced CQI tables with lower code rates are used. |

To support lower SNRs encountered in large cell scenarios, we propose a new CQI table as shown below

| CQI | Modulation | code-rate × 1024 | Efficiency |
| --- | --- | --- | --- |
| 1 | QPSK/BPSK | 11/q | 0.0107 |
| 2 | QPSK/BPSK | 14/q | 0.0136 |
| 3 | QPSK/BPSK | 21/q | 0.0205 |
| 4 | QPSK/BPSK | 36/q | 0.0351 |
| 5 | QPSK/BPSK | 63/q | 0.0615 |
| 6 | QPSK/BPSK | 106/q | 0.1036 |
| 7 | QPSK/BPSK | 156/q | 0.1523 |
| 8 | QPSK/BPSK | 240/q | 0.2344 |
| 9 | QPSK/BPSK | 386/q | 0.3770 |
| 10 | QPSK/BPSK | 716/q | 0.6016 |
| 11 | QPSK/BPSK | 898/q | 0.8770 |
| 12 | QPSK/BPSK | 1204/q | 1.1758 |
| 13 | 16QAM | 378 | 1.4766 |
| 14 | 16QAM | 490 | 1.9141 |
| 15 | 16QAM | 616 | 2.4063 |

The SLS results with the proposed enhancements using pi/2 BPSK modulation scheme are shown below.  In the results below, the current scheme refers to the evaluations done using the current CQI table in Rel-15/Rel-16 specifications. The results using the proposed scheme refers to the evaluations done using the proposed CQI table. Whenever pi/2 BPSK modulation is used, Pcmax is allowed to be boosted to 26 dBm and 29 dBm based on the UL duty cycle.

Figure: CDF of UL SINR and UL Throughput with ISD=30 Km and 40% duty cycle in the UL. For a 40% duty cycle case, proposed scheme uses Pcmax=26 dBm with pi/2 waveform

| Metric | fc=4GHz |  |
| --- | --- | --- |
|  | Current | Proposed |
| 5% UE throughput | 12.26 Kbps | 21.00 Kbps |
| 50% UE throughput | 74.47 Kbps | 142.2 Kbps |
| Mean UE throughput | 83.75 Kbps | 149.2 Kbps |
| Mean Cell throughput | 837.5 Kbps | 1.492 Mbps |

Table 4: Comparison with the current and proposed methods for coverage enhancement - 40% duty cycle

Below we show results when the UL occupancy is with 20% duty cycle. In this case, the Pcmax for pi/2 BPSK can be allowed to go to 29dBm while the average still remains at 23 dBm.

Figure: CDF of UL SINR and UL Throughput with ISD=30 Km and 20% duty cycle for UL. For a 20% duty cycle case, proposed scheme uses Ptx=29 dBm with pi/2 waveform.

| Metric | fc=4GHz |  |
| --- | --- | --- |
|  | Current | Proposed |
| 5% UE throughput | 6.130 Kbps | 21.79 Kbps |
| 50% UE throughput | 37.23 Kbps | 100.4 Kbps |
| Mean UE throughput | 41.87 Kbps | 104.9 Kbps |
| Mean Cell throughput | 418.7 Kbps | 1.049 Mbps |

Table 4: Comparison with the current and proposed methods for coverage enhancement - 20% duty cycle

## Observations

For ISD=30 Km, in case of 40% duty cycle, with Ptx=26 dBm, we observe 2x gains and in case of 20% duty cycle, with Ptx=29 dBm, we observe close to 4x gains. Clearly, pi/2 BPSK waveform can support the required data rates and significantly enhance the coverage requirements for 5G NR.

Note that, and ISD of 30km at 4 GHz carrier frequency in rural areas is a substantial improvement over the existing releases.