3GPP TSG RAN WG1 Meeting #103-e	R1-2007906

Oct 26th – Nov 13tth, 2020

Agenda Item:	8.8.2.3

Source:	IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

Title:	Coverage enhancement for Msg3

Document for:	Discussion and decision

# Motivation

In RAN#86, NR coverage enhancement was identified as one RAN work area for Rel-17. The SI requires to identify the performance targets for coverage enhancement, and study the potential solutions for coverage enhancements for the identified scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study additional enhanced solutions for FR2, if any

  - Evaluate the performance of the potential solutions based on link level simulations.

In our companion contribution [2], we provided baseline evaluations for the 5G NR coverage as per the agreements made in [3]. In this contribution, we provide an overview of the solutions that could be considered for enhancing the coverage of 5G NR in Rel-17, specifically for the Msg3 which is the first UL channel sent by the UE after PRACH transmission.

# Potential Solutions for Coverage Enhancement of Control Channels of 5G NR

RAN1 is discussing potential solutions for enhancing the coverage of the PUSCH and PUCCH channels in a variety of scenarios. Some discussions are also ongoing for the enhancement of Msg3 transmission. Msg3 is the first UL channel post preamble transmission in the UL. It is therefore crucial that this channel is reliably sent by the UE even at the cell edge. Following potential solution directions were agreed to be considered for Msg3

- Repetitions

- (FFS)Potential multiple antenna techniques

While repetitions can indeed improve the coverage, they will potentially impact the PRACH timeline which may delay the UE connection establishment timeline and impact the control plane latency. We propose to consider alternative solutions for Msg3 coverage enhancement. Specifically, we propose to consider solutions based on pi/2 BPSK using DFT-s-OFDM waveform.

## Proposed Solution

The physical channels that use CP-OFDM will not have the benefits of high transmission power as compared to DFT-s-OFDM. UL transmission can benefit from power boosting wherein the max transmission power can reach up to 26dBm or 29dBm based on the UL duty cycle when DFT-s-OFDM waveform with pi/2 BPSK modulation is used. Every dB gained in transmission power significantly enhances the coverage of the UE.

Therefore, we propose pi/2BPSK with spectrum shaping-based design for Msg3.  There are benefits of using the pi/2 BPSK waveform with DFT-s-OFDM transmission as shown in the link budgets in our contribution [2], [4].

The current 5G NR spec already supports configuring DFT-s-OFDM waveform for a UE via the RACH configuration. This configuration must include an additional support for configuring of pi/2 BPSK waveform with spectrum shaping and power boosting based on the Msg3 time resource allocation.

Proposal: pi/2 BPSK waveform with spectrum shaping should be used for Msg3 transmission.

# References

[1] RAN1 Chairman’s Report, RAN1#101e, June 2020.

[2] R1-2007904, Baseline coverage performance for uplink, IITH, CeWiT, IITM, Reliance Jio, Tejas Networks

[3] RP-193240, New SID on NR coverage enhancement

[4] R1-2007905, PUSCH Coverage Enhancements, IITH, CeWiT, IITM, Reliance Jio, Tejas Networks