3GPP TSG RAN WG1 Meeting #103-E															R1-2007934

e-Meeting, October 26th – November 13th, 2020

Source:	Intel Corporation

Title:	Remaining opens of sidelink physical structure for NR V2X design

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This contribution presents remaining corrections for PHY structure of Release 16 NR V2X. Our views on other NR-V2X design aspects are summarized in our companion contributions [5] and [6].

PSFCH OFDM symbols

In clausa 8.3.4.2.1 of [1] (38.211) the parameter l' is defined as “l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213];”. However, in [3] (38.213) this parameter is not defined. We thus suggest the following text additions based on past agreements.

  - Adopt the following TP to define the first OFDM symbol of the PSFCH transmission

| Title: | Corrections to define First OFDM symbol for PSFCH transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | First OFDM symbol for PSFCH transmission not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define first OFDM symbol for PSFCH transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Parameter for the generation of the PSFCH symbols undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------begin text proposal for 38.213 ---------------------------------------------------------

<<<unchanged text omitted>>>

A UE is provided by sl-PSFCH-RB-Set-r16 a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH.

The first OFDM symbol of the PSFCH transmission l' is defined as l' = startSLsymbols + lengthSLsymbols - 3. The parameters startSLsymbols and lengthSLsymbols are defined by clause 8.1.2.1 of [6, 38.214].

<<<unchanged text omitted>>>

----------------------------------------------------end text proposal for 38.213 -----------------------------------------------------------

Editorial Topic

In clausa 8.3.1.2 of [1] (38.211) the table “Table 6.3.1.2-1” is present. To comply with the naming conventions within the document the label of this table should be “Table 8.3.1.2-1”

  - Adjust the numbering of table 6.3.1.2-1 in clausa 8.3.1.2 of 38.211 to the clausa number based on the following TP

| Title: | Corrections to resolve multiple tables defined with the same name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Two tables with the name “Table 6.3.1.2-1: Supported modulation schemes.” are currently defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the numbering of reference and table definition for supported modulations for SL shared channel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | As the content of different tables defined with the same name is not the same, it is unclear which information should be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------begin text proposal for 38.211 ---------------------------------------------------------

<<<unchanged text omitted>>>

Modulation for Mbit,SCI2(q)≤i< Mbit(q)  shall be done as described in clause 5.1 using one of the modulation schemes in Table 86.3.1.2-1 where Msymb,2(q)=Mbit,data(q)Qm.

Table 86.3.1.2-1: Supported modulation schemes.

| Modulation scheme | Modulation order Qm |
| --- | --- |
| QPSK | 2 |
| 16QAM | 4 |
| 64QAM | 6 |
| 256QAM | 8 |

<<<unchanged text omitted>>>

----------------------------------------------------end text proposal for 38.211 -----------------------------------------------------------

Conclusions

In this contribution, we provided our views on main remaining issues of the physical structure for NR-V2X sidelink communication. Based on discussions, we have following proposals:

  - Adopt the following TP to define the first OFDM symbol of the PSFCH transmission

----------------------------------------------------begin text proposal for 38.213 ---------------------------------------------------------

<<<unchanged text omitted>>>

A UE is provided by sl-PSFCH-RB-Set-r16 a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH.

The first OFDM symbol of the PSFCH transmission l' is defined as l'= startSLsymbols + lengthSLsymbols - 3. The parameters startSLsymbols and lengthSLsymbols are defined by clause 8.1.2.1 of [6, 38.214].

<<<unchanged text omitted>>>

----------------------------------------------------end text proposal for 38.213 -----------------------------------------------------------

  - Adjust the numbering of table 6.3.1.2-1 in clausa 8.3.1.2 of 38.211 to the clausa number based on the following TP

----------------------------------------------------begin text proposal for 38.211 ---------------------------------------------------------

<<<unchanged text omitted>>>

Modulation for Mbit,SCI2(q)≤i< Mbit(q)  shall be done as described in clause 5.1 using one of the modulation schemes in Table 86.3.1.2-1 where Msymb,2(q)=Mbit,data(q)Qm.

Table 86.3.1.2-1: Supported modulation schemes.

| Modulation scheme | Modulation order Qm |
| --- | --- |
| QPSK | 2 |
| 16QAM | 4 |
| 64QAM | 6 |
| 256QAM | 8 |

<<<unchanged text omitted>>>

----------------------------------------------------end text proposal for 38.211 -----------------------------------------------------------

References

- 3GPP TS 38.211, “Physical channels and modulations”, version 16.3.0, September 2020.

- 3GPP TS 38.212, “Multiplexing and channel coding”, version 16.3.0, September 2020.

- 3GPP TS 38.213, “Physical layer procedures for control”, version 16.3.0, September 2020.

- 3GPP TS 38.214, “Physical layer procedures for data”, version 16.3.0, September 2020.

- R1-2007935 “Corrections related to Mode-2 resource allocation”, Intel Corporation, e-Meeting, October 2020.

- R1-2007936 “Corrections related to Mode-1 resource allocation”, Intel Corporation, e-Meeting, October 2020.