3GPP TSG RAN WG1 #103-e                                                           R1-2007944

e-Meeting, October 26th – November 13th, 2020

Source:	Intel Corporation

Title:	Remaining Details of Evaluation Methodology for NR Positioning

Agenda item:	8.5.1

Document for:	Discussion and Decision

Introduction

The RAN WG approved study item on NR Positioning Enhancements [1]. The study item includes the following objective:

Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT).

During August meeting, a good progress on the I-IoT use cases definition, positioning requirements, evaluation scenarios and assumptions has been achieved. The summary of RAN1 agreements and remaining details left for further study can be found in [2]. In this contribution, we discuss the opens and share our view on these cases.

Our views on other topics are provided in companion contributions [3] - [4].

# Remaining Open Items Discussion and Proposals

Open item #1 – accuracy and latency requirements for commercial and I-IoT use cases

Based on the feature lead proposal, the following requirements X1 – X7 should be defined.

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal positioning accuracy < 1 m for X1 of UEs

  - Vertical positioning accuracy < 3 m for X2 of UEs

  - End-to-end latency for position estimation of UE < 100 ms

  - Physical layer latency for position estimation of UE < X3

- In Rel-17 target positioning requirements for I-IoT use cases are defined as follows:

  - Horizontal positioning accuracy < X4 for 90 % of UEs

  - Vertical positioning accuracy < X5 for 90 % of UEs

  - End-to-end latency for position estimation of UE < X6

  - Physical layer latency for position estimation of UE < X7

  - For the accuracy and latency requirements, we suggest defining X1 = 80 %, X2 = 80 %, X3 = 100 ms, X4 = (0.2 m or 0.5 m), X5 = (0.2 m or 1 m), X6 = 10 ms, X7 = 10 ms

Open item #2 – parameters definition for UE/gNB TX/RX calibration timing errors

The following agreement was made by RAN WG1 with respect to DL PRS resource allocation at the RAN1#102E meeting:

Agreement: Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms.

Based on the feature lead proposal, parameters defining the range of timing error distribution should be defined, including X value for gNB and Y value for UE.

  - Define the tentative timing error parameter X = 5 ns at the gNB side and Y = 10 ns at the UE side

  - Send LS to RAN4 group and ask for feedback on the proposed tentative values for X and Y

Conclusions

In this contribution we have shared our view on the remaining opens in the NR positioning evaluation methodology. In summary, we have following list of proposals:

  - For the accuracy and latency requirements, we suggest defining X1 = 80 %, X2 = 80 %, X3 = 100 ms, X4 = (0.2 m or 0.5 m), X5 = (0.2 m or 1 m), X6 = 10 ms, X7 = 10 ms

  - Define the tentative timing error parameter X = 5 ns at the gNB side and Y = 10 ns at the UE side

  - Send LS to RAN4 group and ask for feedback on the proposed tentative values for X and Y

References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

- RP-200927, “SR for RAN 88e R17 Positioning”, June 29 – July 2, 2020.

- R1-2007946, “NR Positioning Design Enhancements”, Intel Corporation, October 26 – November 13, 2020.

- R1-2007945, “Evaluation Results for NR Positioning Performance in I-IoT Scenarios”, Intel Corporation, October 26 – November 13, 2020.