3GPP TSG RAN WG1 Meeting #103_E                                                                               R1-2007949

e-Meeting, Oct 26th – Nov 13th, 2020

Agenda Item:		8.6.3

Source: 	Intel Corporation

Title:	On coverage recovery for RedCap NR UEs

Document for: 	Discussion/Decision

# Introduction

At RAN plenary meeting #86, a study item (SI) for the support of reduced capability NR devices was agreed, the following objective was one of those identified for the SI [1].

Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including [RAN1]:

- Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction.

  - Note: For FR1, coverage analysis for wearables can include consideration of potential reduced antenna efficiency due to device size limitations as part of the antenna gains. The extent of additional recovery of coverage loss due to reduced antenna efficiency is to be limited to 3 dB

- The study includes evaluations of the impact to network capacity and spectral efficiency

Note2: Potential overlap with coverage enhancements study is discussed and resolved in RAN#87.

In past RAN1 meetings, the following agreements on coverage recovery were achieved.

Agreements:

- For FR1, study at least 20MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

- For FR2, study 50MHz and 100 MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

Agreements:

- Cost/complexity breakdowns can be separate for FR1 and FR2 if found beneficial.

- For FR1, study two antenna configurations for RedCap UEs, namely 1Rx/1Tx and 2Rx/1Tx.

- For FR2, study two antenna configurations for RedCap UEs, namely 1Rx/1Tx and 2Rx/1Tx.

- Study HD-FDD operation Type A and Type B (as defined in LTE) in RAN1, where study of Type A is prioritized.

Agreements Replace the agreement corresponding to Proposal 14 with the following: For evaluation of UE power saving, for wearables, use the traffic models FTP model 3 and VoIP from TR 38.840 to characterize the wearables service types including IM, VoIP, heartbeat, etc. with proper modification of at least packet size and mean inter-arrival time. Values are FFS.

Agreements Replace the agreement corresponding to Proposal 15 with the following: For evaluation of UE power saving, for industrial wireless sensor use cases, use a traffic model based on the service performance requirements for the process monitoring use case in TS 22.104 Table 5.2-2. At least 64 bytes UL message (plus headers, e.g. MAC, RLC, etc.) transmitted periodically with a periodicity [100 ms] should be considered (other values are not precluded encouraged).

Agreements:  If/when coverage evaluations outside the CE SI are needed,  The basic evaluation methodology is based on link-level simulation for FR1. Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements. Step 2: Obtain the baseline performance based on required SINR and link budget template. Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately  The evaluation methodology based on system-level simulation is optional for FR1. Note: The simulation assumptions for SLS are up to companies’ reports.  The evaluation methodology for FR2 is the same as FR1.

Agreements: The reference NR device for evaluation of cost/complexity reduction supports the following:

- All mandatory Rel-15 features (with or without capability signaling)

- Single RAT

- Operation in a single band at a time

- Band and duplex mode support:

  - FR1: Operation in a single FDD band or a single TDD band at a time

  - FR2: Operation in a single TDD band at a time

- Maximum bandwidth:

  - For FR1: 100 MHz for DL and UL

  - For FR2: 200 MHz for DL and UL

- Antennas:

  - For FR1 FDD: 2Rx/1Tx

  - For FR1 TDD: 4Rx/1Tx

  - For FR2: 2Rx/1Tx

- Power class: PC3

- Processing time: Capability 1

- Modulation:

  - For FR1: support 256QAM for DL and 64QAM for UL

  - For FR2: support 64QAM for DL and 64QAM for UL

- Access: Direct DL/UL access between UE and gNB

Note: For each complexity reduction technique, the study includes whether the complexity reduction accumulates across RF bands.

Note: The study will consider impacts on the cost/complexity reduction from support of multiple RF bands within FR1 or FR2.

Agreements:

   If/when link-level coverage evaluations outside the CE SI are needed,

   The CE SI link-level simulation assumptions can be used as a starting point.

   For calibration purposes, the following settings can be used:

| Parameters | FR1 values | FR2 values |
| --- | --- | --- |
| Scenario and frequency | Urban: 2.6 GHz (TDD) (primary choice) 4 GHz (TDD) (secondary choice) Rural: 700 MHz (FDD) | Indoor: 28 GHz (TDD) |
| Frame structure for TDD | For 2.6 GHz: DDDDDDDSUU (S: 6D:4G:4U) For 4 GHz: DDDSUDDSUU (S: 10D:2G:2U) | DDDSU (S: 10D:2G:2U) |
| Channel model | TDL-C | TDL-A |
| UE velocity | 3 km/h | 3 km/h |

Agreements:

The evaluation of performance impacts includes at least peak data rate and, latency and reliability (as needed for the use cases). Other performance metrics such as power consumption, spectral efficiency and PDCCH blocking probability may also be considered if appropriate for a specific technique.

Agreements

For the channel(s) affected by complexity reduction, the following methodology can be used to determine the target performance for coverage recovery

- Step 1: Obtain the link budget performance of the channel based on link budget evaluation

- Step 2: Obtain the target performance requirement for RedCap UEs within a deployment scenario

  - FFS on the target performance requirement

- Step 3: Find the coverage recovery value for the channel if the link budget performance is worse than the target performance requirement

Agreements:

- Link budget evaluation for RedCap should include at least PDCCH/PDSCH and PUCCH/PUSCH

Agreements:

- For initial access related channels, at least Msg2, Msg3, Msg4 and PDCCH scheduling Msg2/4 are included for link budget evaluation

  - Other initial access related channels are not precluded

Agreements:

- The impact of small form factor is considered for all the uplink and downlink channels

  - A 3dB loss of antenna gain is included in link budget calculation for FR1

    - FFS on the application to both FDD and TDD bands or only FDD bands

Agreements: Down-selection on the following options for the target performance requirement for RedCap UEs in RAN1#103-e (aim for early in the e-meeting):

- Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment

- Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario

  - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL

- The details for the target performance requirement are FFS

Agreements: For RedCap UE, adopt the following target data rates for link budget evaluation for FR1 Rural.

- 1 Mbps on DL and 100kbps in UL

Agreements: For RedCap UE, down-selection on adopt the following target data rates for link budget evaluation for FR1 Urban.

- 2 Mbps on DL and 1Mbps in UL

Note: The 2Mbps target data rate in downlink is the scaled value of the 10Mbps in the CE SI by a factor of 0.2

Agreements: For RedCap UEs, the target data rates for link budget evaluation for FR2 are as follows:

- 25Mbps for BW 50MHz/100MHz on DL and 5Mbps in UL

  - Optionally, 12.5Mbps for BW 50MHz as the target data rate for DL, assuming the same DL PSD as that of BW 100MHz

  - Note: in case of 50MHz BW, the maximum supported DL data rate is half that of the 100MHz BW in DL

Agreements:

- For link budget evaluation, the antenna gain loss due to the small form factor can be applied to all the FR1 bands

- For RedCap coverage analysis, the agreements in the Rel-17 CE SI regarding link budget template and antenna array gain are reused.

  - Continue to discuss and decide the performance metric in RAN1-103 e-meeting

Agreements:

- For RedCap coverage evaluation, the Rel-17 CE SI agreements on gNB antenna configuration, # gNB Tx/Rx chains, channel model and delay spread are reused with the following revision and/or addition

| Parameters | FR1 values | FR2 values |
| --- | --- | --- |
| Channel model | TDL-C | TDL-A CDL-A(optional) |
| Delay spread | 300ns | 30ns |
| UE velocity | 3 km/h | 3 km/h |
| Antenna correlation | Low | Low |
| # gNB Tx chains | 2 or 4 | 2 |
| # gNB Rx chains | 2 or 4 | 2 |

- For RedCap coverage evaluation, adopt the following table for the reference NR UE.

| Parameters | FR1 values | FR2 values |
| --- | --- | --- |
| # UE Tx chains | 1 | 1 |
| # UE Rx chains | Urban: 4 and Rural: 2 | 2 |
| UE BW | Urban: 100 MHz (273 PRBs) Rural: 20 MHz (106 PRBs) | 100 MHz (66 PRBs) |

- For RedCap coverage evaluation, adopt the following table for the RedCap UE.

  - Other UE BWs are not precluded

| Parameters | FR1 values | FR2 values |
| --- | --- | --- |
| # UE Tx chains | 1 | 1 |
| # UE Rx chains | 1 or 2 | 1 or 2 |
| UE BW | Urban: 20 MHz (51 PRBs) Rural: 20 MHz (106 PRBs) | 50 MHz (32 PRBs) or 100 MHz (66 PRBs) |

Agreements:

- For RedCap coverage evaluation, reuse the Rel-17 CE SI agreements on channel specific parameters with the following revision and/or addition

  - TBS/PRB/MCS of PDSCH (except for Msg2)/PUSCH for the RedCap UE are based on the agreed target data rates or message sizes and reported by companies

  - Adopt the following table for Msg2 evaluation

    - Note: the TBS scaling is not precluded in the table entry “PRBs/TBS/MCS”

| Parameters | Values |
| --- | --- |
| PRBs/TBS/MCS | MCS is fixed to zero. Companies to report the used number of PRBs and corresponding TBS value |
| PDSCH duration | 12 OS |
| DMRS configuration | Type I, 3 DMRS symbol, no multiplexing with data |
| Waveform | CP-OFDM |
| HARQ configuration | No retransmission |

In this contribution, we provide our LLS evaluation results for reference UE and RedCap UE, identify the bottleneck channel of reference UE and obtain the channel needs enhancement for RedCap UE. Potential techniques for coverage recovery, necessary for RedCap UEs, are also provided.

# Evaluations to identify channels needs enhancements for RedCap UEs

In addition to the agreed evaluation methodologies/assumptions, one remaining issue is how to determine the performance target for RedCap UE,

Agreements: Down-selection on the following options for the target performance requirement for RedCap UEs in RAN1#103-e (aim for early in the e-meeting):

- Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment

- Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario

  - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL

- The details for the target performance requirement are FFS

The different channels have different link budgets. The coverage of a reference UE is determined by a bottleneck channel among the channels. According to an objective of the SI, “Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction”, performance of the bottleneck channel of reference UE can be the target for RedCap UE. A channel of RedCap UE which less worse link budget than the bottleneck channel must be enhanced for the same coverage as reference UE. On the other hand, if a channel of RedCap UE is not worse than the bottleneck channel, no clear motivation to enhance it since such enhancement anyway does not impact the coverage. In the following link budget analysis, MIL based performance is presented. The detailed simulation assumptions and link curves are provided in ANNEX A.

Proposal 1: To adopt option 3, i.e. to determine the target MIL performance which corresponds to a bottleneck channel of reference UE as the target performance requirements for RedCap UE.

The antenna gain correction factors are obtained based on system level simulations. In the simulations, the antenna gain correction factor is determined based on the difference between theoretical antenna gain and 95th percentile antenna gain. For Rural 700MHz, the correction factors are 0 dB.

Table 1. Antenna gain correction factor for different scenarios

| Scenario | Theoretic antenna gain (dB) | 95th percentile antenna gain (dB) | Antenna gain correction factor (dB) |
| --- | --- | --- | --- |
| Urban at 4GHz: Broadcast channel | 16.81 | 12.10 | 4.71 |
| Urban at 4GHz: Unicast channel | 16.81 | 13.27 | 3.54 |
| Urban at 2.6GHz: Broadcast channel | 16.81 | 12.52 | 4.29 |
| Urban at 2.6GHz: Unicast channel | 16.81 | 13.78 | 3.03 |
| Indoor at 28GHz: Broadcast channel | 18.06 | 8.16 | 9.80 |
| Indoor at 28GHz: Unicast channel | 18.06 | 9.09 | 8.97 |

## Urban 2.6GHz

Figure 1 illustrates MIL for various channels under Urban scenario at 2.6GHz for RedCap UE with 2Rx. From the figure, the bottleneck channel for reference UE is PUSCH. PUSCH needs to be enhanced for RedCap UE. From the figure, the DL channels can achieve much better MIL. Consequently, it is expected that PUSCH will be the channel to be enhanced even for RedCap UE with 1Rx.

Figure 1: MIL for Urban 2.6GHz, RedCap UE w/ 2Rx

Observation 1: PUSCH is the bottleneck channel and the channel that needs enhancement for RedCap UE in Urban scenario at 2.6GHz.

## Urban 4GHz

Figure 2 illustrates MIL for various channels under Urban scenario at 4GHz for RedCap UE with 2Rx. From the figure, PUSCH is the bottleneck channel for reference UE and the channel to be enhanced for RedCap UE. The same observation may apply to RedCap UE with 1Rx too. However, the margin for PDSCH of RedCap UE with 1Rx is reduced.

Figure 2: MIL for Urban 4GHz, RedCap UE w/ 2Rx

Observation 2: PUSCH is the bottleneck channel and the channel that needs enhancement for RedCap UE in Urban scenario at 4GHz. The margin for PDSCH is about 5dB.

## Rural 700MHz

Figure 3 illustrates MIL for various channels under Rural scenario at 700MHz for RedCap UE with 1Rx. From the figure, PUSCH is the bottleneck channel for reference UE. Both PUSCH and msg3 of RedCap UE have a worse MIL than the bottleneck channel. Therefore, PUSCH and msg3 need to be enhanced for RedCap UE with 1Rx. The margin for PRACH, PUCCH with payload of 22 bits and PDSCH are small too.

Figure 3: MIL for Rural 700MHz, RedCap UE w/ 1Rx

Observation 3: PUSCH is the bottleneck channel and PUSCH and msg3 are the channels that need enhancement for RedCap UE in Rural scenario at 700MHz. The margin for PRACH, PUCCH with payload of 22 bits and PDSCH are less than 5dB.

## Indoor, 28GHz

Figure 4 illustrates MIL for various channels under Indoor scenario at 28GHz for RedCap UE with 100MHz BW, 1Rx and maximum Tx power 23dBm. Different from FR1, PDSCH is the bottleneck channel for reference UE. PDSCH for RedCap UE is necessary to be enhanced. Further, it can be observed that the margin for other DL channels including msg2, msg4, PDCCH USS and PDCCH CSS are not large too.

Figure 4: MIL for Indoor 28GHz, RedCap UE with 100MHz BW, 1Rx and 23dBm max power

On the other hand, as shown in Figure 5, for RedCap UE with 100MHz BW, 1Rx and maximum Tx power 12dBm, PDSCH is still the bottleneck channel since bottleneck channel is determined by reference UE. In addition to all identified DL channels that need enhancement, PUSCH has the worst MIL and should be enhanced too.

Figure 5: MIL for Indoor 28GHz, RedCap UE with 100MHz BW, 1Rx and 12dBm max power

The MIL performance for RedCap UE with 50MHz BW and 1Rx can be deduced from Figure 4 and 5. Assuming the DL target data rate can be scaled down by half for RedCap UE with 50MHz BW, the MIL performance for DL channels may reduce a bit due to the loss on frequency diversity within 50MHz BW. However, it is expected that the reduction is minimal. The MIL for PUSCH for RedCap UE with 100MHz BW and 50MHz BW are similar for the same target data rate. For other channels, they normally require a smaller number of PRBs for transmission, therefore the impacts of BW of RedCap UE are limited. In conclusion, it is expected similar observations apply to RedCap UE with 1Rx and 50MHz BW and 100MHz BW.

On the other hand, for RedCap UE with 50MHz BW and 2Rx, it is expected that the MIL for DL channels can be only slightly worse than reference UE due to the loss of frequency diversity. There is no impact on the MIL of UL channels. From Figure 4 for maximum Tx power 23dBm, PDSCH may be still the channel that is necessary for enhancement though the gap to reference UE is small. On the other hand, the margin for other DL channels are enlarged, which results in less motivation for enhancements for them. From Figure 5 for maximum Tx power 12dBm, PDSCH may still need to be enhanced to compensate the loss of frequency diversity. Further PUSCH is the channel that is necessary for enhancement.

Observation 4: In indoor scenario at 28GHz,

- For RedCap UE with 100MHz BW, 1Rx and maximum Tx power 23dBm, PDSCH is the bottleneck channel and the channel that need enhancement. Other DL channels including msg2, msg4, PDCCH USS and PDCCH CSS only have small margins.

- For RedCap UE with 100MHz BW, 1Rx and maximum Tx power 12dBm, PUSCH needs enhancement additionally compared to the UE with maximum Tx power 23dBm.

- For RedCap UE with 50MHz BW and 1Rx, it is expected that similar observations from RedCap UE with 100MHz BW and 2Rx are applicable, but with an additional 1.5 – 3 dB DL link budget loss.

- For RedCap UE with 50MHz BW, 2Rx and maximum Tx power 23dBm, it is expected that PDSCH is still the channel that needs enhancement. On other hand, there is less motivation to enhance other DL channels.

- For RedCap UE with 100MHz BW, 2Rx and maximum Tx power 12dBm, it is expected that PUSCH and PDSCH are the channels that need enhancement.

Proposal 2: It is proposed to at least enhance the identified channels in observation 1~4 for RedCap UE to achieve the same coverage as reference UE.

# Potential enhancements for RedCap NR UEs

In accordance to the evaluations and observations in section 2, the potential techniques to enhance the performance for RedCap UE are provided in this section.

A straightforward scheme to solve coverage issue is to do repetitions, which in general could be applied to all kinds of channels. In fact, time repetitions for PDSCH, PUSCH and PUCCH are already supported since NR Rel-15. Due to the even worse link performance of RedCap UE compared to a reference UE, it is expected that the maximum number of supported repetitions for some physical channels may need to be increased compared to the Rel-15 maximum of 8 repetitions, e.g. 16 or more. On the other hand, frequency domain repetition may be considered for some DL channels too.

Slot-level repetition for PDSCH and PUSCH can be configured by RRC signalling in Rel-15. That is, a same SLIV of PDSCH or PUSCH is mapped to multiple consecutive slots. In Rel-16 eURLLC WI, the consecutive repetitions of SLIV is also introduced for PUSCH. However, considering the combination of latency and reliability targets for RedCap use cases, such a mechanism may be overly complicated for RedCap UEs. For example, if a nominal repetition is crossing slot boundary, an exception handling is needed in the PUSCH rate matching and resource mapping. In fact, Rel-15 also support consecutive repetitions by scheduling a SLIV of full slot to the UE. If coverage is a problem, it could be a typical choice of gNB scheduler to allocate full slot transmission and repetitions for a TB.

One more point for consideration is semi-static configuration or dynamic indication of the number of repetitions. Rel-16 eURLLC and M-TRP already supports dynamic indication for the repetition of PUSCH and PDSCH. Thus, similar features should be considered as a starting point for indicating the numbers of repetitions dynamically to RedCap UEs.

Specifically, PDCCH is not identified as the channel that is necessary for enhancement as discussed in section 2. However, it is also observed that the margin for PDCCH in both USS and CSS can be limited. In this sense, PDCCH may become the limited channel in certain scenario too. In general, two schemes could be considered to improve link performance. A first scheme for enhancement is to rely on use of compact DCI formats. As the DCI size is decreased, the coverage can improve using same amount of time/frequency resources. A second scheme is to do repetition. The more repetitions, the more transmission power is assigned to a PDCCH, which then improves the link performance. Considering large PDCCH aggregation level, e.g. 16 or more is supported, the repetition must be in time domain practically. In fact, both two schemes are extensively studied in NR eURLLC in Rel-16, but only the first scheme was specified in Rel-16. PDCCH repetition can be reconsidered, however, it may be in low priority.

Repetition is the general way to improve coverage, however, it normally sacrifices the data rate of RedCap UE. In this sense, it is worth to consider solutions other than repetition.

Frequency hopping can be considered for better link performance for the diversity gain. It is especially useful for a channel with small number of PRBs. NR already support frequency hopping for some channels, but it is normally limited to 2 hops. In general, larger number of hops can provide better diversity gain. For PDSCH or PUSCH if almost full BW of RedCap UE are occupied for transmission, hopping the frequency location of the BW occupied by RedCap UE can be considered. This concept is similar to BWP switching in NR. Whether/how to handle BWP switching time needs to be considered. Hopping across multiple BWPs results in increased complexity too.

Another way to improve the link performance for a channel is to improve the channel estimation performance. For example, if the channel is repeated in consecutive slots, interpolation across slots can be considered for better channel estimation. On the other handle, the increase of the granularity of PRB bundling in channel estimation can be studied. For a flat channel, it is expected larger size of PRB bundling could be beneficial.

In it noted that the evaluations in sections normally adopt random precoder cycling as the spatial diversity scheme. Such open-loop scheme is simple and doesn’t rely on UE feedback. On the other hand, it is always true that the link performance can be improved for unicast channels if an accurate estimation on channel condition is available. However, taking downlink as example, it is necessary to reduce the amount of UE feedback which is beneficial for cost reduction. Therefore, it is critical to balance the DL performance and UE feedback.

Proposal 3: The following potential enhancements may be considered to solve the coverage issue of RedCap UE

- Repetitions in time domain and frequency domain can be considered. The number of repetitions can be increased.

- Semi-static or dynamic indication on the number of repetitions could be considered.

- If PDCCH is to be enhanced, compact DCI size and repetition can be further discussed.

- Frequency hopping can be considered, however, the impact of BWP switching delay and the increase of complexity need to be studied.

- Ways to improve channel estimation performance can be considered.

# Conclusion

In this contribution, we identify the channels for RedCap UE that need to be enhanced for different scenarios based on LLS evaluations and provide potential techniques that could be considered for the enhancement. we make the following proposals.

Proposal 1: To adopt option 3, i.e. to determine the target MIL performance which corresponds to a bottleneck channel of reference UE as the target performance requirements for RedCap UE.

Observation 1: PUSCH is the bottleneck channel and the channel that needs enhancement for RedCap UE in Urban scenario at 2.6GHz.

Observation 2: PUSCH is the bottleneck channel and the channel that needs enhancement for RedCap UE in Urban scenario at 4GHz. The margin for PDSCH is about 5dB.

Observation 3: PUSCH is the bottleneck channel and PUSCH and msg3 are the channels that need enhancement for RedCap UE in Rural scenario at 700MHz. The margin for PRACH, PUCCH with payload of 22 bits and PDSCH are less than 5dB.

Observation 4: In indoor scenario at 28GHz,

- For RedCap UE with 100MHz BW, 1Rx and maximum Tx power 23dBm, PDSCH is the bottleneck channel and the channel that need enhancement. Other DL channels including msg2, msg4, PDCCH USS and PDCCH CSS only have small margins.

- For RedCap UE with 100MHz BW, 1Rx and maximum Tx power 12dBm, PUSCH needs enhancement additionally compared to the UE with maximum Tx power 23dBm.

- For RedCap UE with 50MHz BW and 1Rx, it is expected that similar observations from RedCap UE with 100MHz BW and 2Rx are applicable, but with an additional 1.5 – 3 dB DL link budget loss.

- For RedCap UE with 50MHz BW, 2Rx and maximum Tx power 23dBm, it is expected that PDSCH is still the channel that needs enhancement. On other hand, there is less motivation to enhance other DL channels.

- For RedCap UE with 100MHz BW, 2Rx and maximum Tx power 12dBm, it is expected that PUSCH and PDSCH are the channels that need enhancement.

Proposal 2: It is proposed to at least enhance the identified channels in observation 1~4 for RedCap UE to achieve the same coverage as reference UE.

Proposal 3: The following potential enhancements may be considered to solve the coverage issue of RedCap UE

- Repetitions in time domain and frequency domain can be considered. The number of repetitions can be increased.

- Semi-static or dynamic indication on the number of repetitions could be considered.

- If PDCCH is to be enhanced, compact DCI size and repetition can be further discussed.

- Frequency hopping can be considered, however, the impact of BWP switching delay and the increase of complexity need to be studied.

- Ways to improve channel estimation performance can be considered.

# References

[1] RP-201386, Revised SID on Study on support of reduced capability NR devices, Ericsson

[2] Chairman notes for RAN1#101-e

[3] Chairman notes for RAN1#102-e

# ANNEX Evaluation Assumptions

Table A1-1: Common evaluation assumptions

| Parameters | FR1, Urban | FR1, Urban | FR1, Rural | FR2, Indoor |
| --- | --- | --- | --- | --- |
| Carrier Frequency | 2.6 GHz (TDD) | 4 GHz (TDD) | 0.7 GHz (FDD) | 28 GHz (TDD) |
| SCS | 30 kHz | 30 kHz | 15 kHz | 120 kHz |
| Frame structure for TDD | DDDDDDDSUU (S: 6D:4G:4U) | DDDSUDDSUU (S: 10D:2G:2U) | N/A | DDDSU(S: 10D:2G:2U) |
| # of gNB TX chains | 4 | 4 | 2 | 2 |
| # of gNB RX chains | 4 | 4 | 2 | 2 |
| Channel Model | TDL-C, NLOS | TDL-C, NLOS | TDL-C, NLOS | TDL-A, NLOS |
| UE antenna correlation | low | low | low | low |
| delay spread | 300 ns | 300 ns | 300 ns | 30 ns |
| UE velocity | 3 km/h | 3 km/h | 3 km/h | 3 km/h |

Table A1-2: Assumptions of reference NR UE

| Parameters | FR1, Urban | FR1, Urban | FR1, Rural | FR2, Indoor |
| --- | --- | --- | --- | --- |
| BW | 100 MHz (273 PRBs) | 100 MHz (273 PRBs) | 20 MHz (106 PRBs) | 100 MHz (66 PRBs) |
| # of UE TX chains | 1 | 1 | 1 | 1 |
| # of UE RX chains | 4 | 4 | 2 | 2 |

Table A1-3: Assumptions of RedCap UE

| Parameters | FR1, Urban | FR1, Urban | FR1, Rural | FR2, Indoor |
| --- | --- | --- | --- | --- |
| BW | 20 MHz (51 PRBs) | 20 MHz (51 PRBs) | 20 MHz (106 PRBs) | 50 (32 PRBs) or 100 MHz (66 PRBs) |
| # of UE TX chains | 1 | 1 | 1 | 1 |
| # of UE RX chains | 2 | 2 | 1 | 1 |

Table A2-1: Evaluation assumptions for PDSCH in FR1

| Parameters |  | Values |  |
| --- | --- | --- | --- |
|  |  | Reference NR UE | RedCap UE |
| waveform |  | CP-OFDM |  |
| Target data rate |  | 10 Mbps (Urban), 1 Mbps (Rural) | 2 Mbps (Urban), 1 Mbps (Rural) |
| Performance target |  | 10% iBLER |  |
| TDRA |  | 12 OFDM symbols |  |
| DMRS |  | Type I, 1 DMRS symbol, no multiplexing with data. |  |
| TBS | 4 GHz: | 11280 | 2280 |
|  | 2.6 GHz: | 7944 | 1608 |
|  | 700 MHz: | 1128 | 1128 |
| MCS | 4 GHz: | 2 | 2 |
|  | 2.6 GHz: | 0 | 0 |
|  | 700 MHz: | 0 | 0 |
| FDRA | 4 GHz: | 225 | 46 |
|  | 2.6 GHz: | 256 | 51 |
|  | 700 MHz: | 35 | 35 |
| Number of transmissions |  | 1 (no HARQ) |  |
| Rx combining |  | MRC |  |
| Diversity scheme |  | Precoder cycling |  |
| PRB bundling |  | 2 |  |

Table A2-2: Evaluation assumptions for PDSCH in FR2

| Parameters |  | Values |  |
| --- | --- | --- | --- |
|  |  | Reference NR UE | RedCap UE |
| waveform |  | CP-OFDM |  |
| Target data rate |  | 25 Mbps | 25 Mbps (100 MHz UE) 12.5 Mbps (50 MHz UE) |
| Performance target |  | 10% iBLER |  |
| TDRA |  | 12 OFDM symbols |  |
| DMRS |  | Type I, 1 DMRS symbol, no multiplexing with data. |  |
| TBS | 28 GHz: | 5888 | 5888 (100 MHz UE) |
| MCS | 28 GHz: | 5 | 5 (100 MHz UE) |
| FDRA | 28 GHz: | 60 | 60 (100 MHz UE) |
| Number of transmissions |  | 1 (no HARQ) |  |
| Rx combining |  | MRC |  |
| Diversity scheme |  | Precoder cycling |  |
| PRB bundling |  | 2 |  |

Table A3-1: Evaluation assumptions for PDCCH in FR1

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 1% BLER |
| CORESET size | 2 symbols, 48 PRBs |
| Frequency distributed | Yes |
| Payload | 40 bits |
| Number of transmissions | 1 |
| Rx combining | MRC |
| Diversity scheme | Normal PDCCH: Precoder cycling every 6 REGs PDCCH for Msg2/4: wideband RS |
| Aggregation level | 16 |

Table A3-2: Evaluation assumptions for PDCCH in FR2

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 1% BLER |
| CORESET size | 2 symbols, 48 PRBs (100MHz UE) |
| Frequency distributed | Yes |
| Payload | 40 bits |
| Number of transmissions | 1 |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling |
| Aggregation level | 16 |

Table A4: Evaluation assumptions for PBCH in FR1

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 1% rBLER |
| T/F resources | Per RAN1 spec |
| Payload | 32 bits payload, 24 bits CRC |
| Number of transmissions | 4 |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling |
| Periodicity | 20 ms |

Table A5-1: Evaluation assumptions for PUSCH in FR1

| Parameters |  | Values |  |
| --- | --- | --- | --- |
|  |  | Reference NR UE | RedCap UE |
| waveform |  | DFT-s-OFDM |  |
| Target data rate |  | 1 Mbps (Urban), 100 kbps (Rural) |  |
| Performance target |  | 10% iBLER |  |
| TDRA |  | 14 OFDM symbols |  |
| DMRS |  | Type I, 1+1 DMRS symbol, no multiplexing with data. |  |
| TBS | 4 GHz: | 2856 | 2856 |
|  | 2.6 GHz: | 2792 | 2792 |
|  | 700 MHz: | 136 | 136 |
| MCS | 4 GHz: | 5 | 5 |
|  | 2.6 GHz: | 1 | 1 |
|  | 700 MHz: | 0 | 0 |
| FDRA | 4 GHz: | 30 | 30 |
|  | 2.6 GHz: | 51 | 51 |
|  | 700 MHz: | 4 | 4 |
| Number of transmissions |  | 1 (no HARQ) |  |
| Rx combining |  | MRC |  |

Table A5-2: Evaluation assumptions for PUSCH in FR2

| Parameters |  | Values |  |
| --- | --- | --- | --- |
|  |  | Reference NR UE | RedCap UE |
| waveform |  | DFT-s-OFDM |  |
| Target data rate |  | 5 Mbps |  |
| Performance target |  | 10% iBLER |  |
| TDRA |  | 14 OFDM symbols |  |
| DMRS |  | Type I, 1+1 DMRS symbol, no multiplexing with data. |  |
| TBS | 28 GHz: | 3496 | 3496 |
| MCS | 28 GHz: | 2 | 2 (100MHz UE) |
| FDRA | 28 GHz: | 63 | 63 (100MHz UE) |
| Number of transmissions |  | 1 (no HARQ) |  |
| Rx combining |  | MRC |  |

Table A6: Evaluation assumptions for PUCCH in FR1

| Parameters | Values |  |
| --- | --- | --- |
|  | PF1 | PF3 |
| waveform | CP-OFDM | DFT-S-OFDM |
| performance target | DTX-ACK prob 1% NACK-ACK prob 0.1% ACK-NACK prob 1% | 1% BLER |
| Payload | 2 bits | 11/22 bits |
| Resource allocation | 1 PRB, 14 symbols |  |
| Number of transmissions | 1 |  |
| Rx combining | MRC |  |
| Frequency hopping | Intra-slot FH |  |

Table A7. Simulation assumption for PRACH FR1/FR2

| Parameters | Values |
| --- | --- |
| Physical channel | PRACH |
| Carrier frequency | 700 MHz, 4 GHz – FR1 28 GHz – FR2 |
| Waveform | DFT-s-OFDM |
| PRACH formats | format 0 (FDD), format B4 (TDD, 30kHz SCS) – FR1 format B4 (120kHz SCS)– FR2 |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx for 700MHz and 4Rx for 4GHz for FR1 2Rx for FR2 |
| Propagation channel | FR1: Urban: TDL-C 300 ns Rural: TDL-C 300ns FR2: Indoor scenario: TDL-A 30ns |
| UE velocity | FR1: Urban: 3km/h for indoor Rural: 3km/h for indoor FR2: Indoor: 3km/h |
| Timing offset | 0 as starting point and may take uniform [0, RTT] later. |
| Frequency offset | 0 as starting point and may take 0.05ppm (fixed) at TRP, and 0.1 ppm (fixed) at UE later. |
| Performance metrics | 1% PRACH misdetection probability 0.1% false alarm probability |

Table A8: Evaluation assumptions for Msg2

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 1% iBLER |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling |
| TB scaling | 1/4 |
| PRBs/TBS/MCS | MCS = 0, TBS = 80, PRB = 3 (TB scaling = 1) and 11 (TB scaling = 0.25) |
| PDSCH duration | 12 OS |
| DMRS configuration | Type I, 3 DMRS symbol, no multiplexing with data |
| HARQ configuration | No retransmission |

Table A9: Evaluation assumptions for Msg4

| Parameters | Values |
| --- | --- |
| waveform | CP-OFDM |
| performance target | 10% rBLER |
| Rx combining | MRC |
| Diversity scheme | Precoder cycling |
| PRBs/TBS/MCS | 1040 bits, TBS=1064 MCS is fixed to zero. N_PRB=37 |
| PDSCH duration | 12 OS |
| DMRS configuration | Type I, 2 DMRS symbol, no multiplexing with data |
| HARQ configuration | No retransmission |

Table A10: Evaluation assumptions for Msg3

| Parameters | Values |
| --- | --- |
| waveform | DFT-S-OFDM |
| performance target | 10% rBLER |
| Rx combining | MRC |
| PRBs/TBS/MCS | TBS 56bits, MCS is fixed to zero. N_PRB=2 |
| PDSCH duration | 14 OS |
| DMRS configuration | Type I, 3 DMRS symbol, no multiplexing with data. |
| HARQ configuration | No retransmission |