3GPP TSG RAN WG1 #103-e	R1-2007967

e-Meeting, October 26th – November 13th, 2020

Source:            ZTE, Sanechips

Title:                 Simulation results for NR above 52.6GHz

Agenda Item:   8.2.3

Document for: Discussion and Decision

# Introduction

In RAN1 101 e-meeting and post meeting email discussion, the proposals in Tables 2, 3, 4, 5, 7, 8, 9, 10, 11, 12 of R1-2005185 are agreed, which includes the LLS and SLS simulation assumption and objectives. In RAN1 102-e meeting, the LLS and SLS simulation assumption is updated in R1-2007126 .

In this contribution, we provide some primary simulation results of LLS and SLS based on the agreed simulation assumption in R1-2007126 for above 52.6GHz.

# Link level simulation

## PDSCH

### TDL-A

- Delay spread 5ns

Figure 2.1-1 TDL-A Delay spread 5ns

- Delay spread 10ns

Figure 2.1-2 TDL-A Delay spread 10ns

- Delay spread 20ns

Figure 2.1-3 TDL-A Delay spread 20ns

It can be shown from Figure 2.1-1~2.1-3, for TDL-A channel with different delay spread:

- For QPSK modulation, the performance of SCS 120kHz, 240kHz, 480kHz and 960kHz is similar.

- For 16QAM and 64QAM modulation, larger SCS shows better performance.

- Larger delay spread may cause a performance degradation, but the relative performance of different SCS with delay spread 5ns, 10ns and 20ns is similar.

### CDL-B

- Delay spread 20ns

Figure 2.1-4 CDL-B Delay spread 20ns

- Delay spread 50ns

Figure 2.1-5 CDL-B Delay spread 50ns

It can be shown from Figure 2.1-4 and 2.1-5, CDL-B channel with different delay spread:

- For QPSK modulation, the performance of SCS 120kHz, 240kHz, 480kHz and 960kHz is similar.

- For 16QAM modulation, SCS of 240kHz, 480kHz and 960kHz shows similar performance, performance of 120kHz is a bit worse.

- For 64QAM modulation, larger SCS shows better performance.

- The performance of different SCS shows similar relative performance with delay spread 20ns and 50ns.

Observation 1: Phase noise has limited impact on QPSK and 16QAM modulation, and with PTRS CPE compensation, different SCS (120kHz, 240kHz, 480kHz, 960kHz) shows similar performance.

Observation 2: Phase noise has significant impact on 64QAM modulation, and with PTRS CPE compensation, larger SCS shows better performance.

Observation 3: Various delay spread values don’t affect the relative performance of different SCS.

## PRACH

In this chapter, preamble format A1 with 139 sequence length is used, TDL-A channel and delay spread is 20ns.

Figure 2.2-1 PRACH performance of different SCS

Observation 4: Phase noise and delay spread have limited impact on PRACH performance, the performance of SCS 120kHz, 240kHz, 480kHz and 960kHz is similar.

# System level simulation

## RSRP distribution of different Links

In RAN1 #102e-meeting, it’s agreed that companies are encouraged to submit RSRP distribution for the evaluated scenario in SLS.

The detailed simulation assumptions are listed in Appendix A2.

Figure 3.1-1 RSRP distribution

## Coexistence interference analysis

In this section, the simulation result is performed to study the interference of different LBT schemes. The evaluation scenario is Indoor scenario A as shown in Figure 3.2-1. As we know, in 60 GHz frequency, omni-directional LBT is used in Wi-Fi system such as IEEE 802.11 ad/ay.

Figure 3.2-1 Indoor scenario A

In this part, we provide system-level simulations to evaluate the impact of LBT schemes on the coexistence between NR-U and Wi-Fi systems, where one operator using omni-directional LBT mimics the behavior of Wi-Fi system and the other operator is assumed as NR-U can choose between omni-directional LBT and directional LBT. The detailed simulation assumptions are listed in Appendix A2.

- Case 1 corresponds to omni-directional LBT for Operator 1 (mimic an existed Wi-Fi operator) vs omni-directional LBT for Operator 2 (mimic a new NR-U operator) ;

- Case 2 corresponds to omni-directional LBT for Operator 1(mimic an existed Wi-Fi operator) vs directional LBT for Operator 2 (mimic a new NR-U operator).

In Case 1, gNBs are randomly dropped in a 10m*10m box, UEs are randomly dropped in the 120m*50m room, the minimum distance between BS of different operators is 2 m. In Case 2, the randomization is totally the same with Case 1 in order to show a compatible performance between 2 cases. Simulation assumptions and detailed statistic can be found in Appendix A3 in Table A3-1.

Figure 3.2-2 Coexistence interference analysis

It can be shown in Figure 3.2-2:

- In mean, 5% and 50% UPT, it is shown from Case 1 that Operator 1 has a relatively high throughput with omni-directional LBT compared to the same LBT mechanism used for Operator 2, while the opposite trend is appeared in 95% UPT. The performance difference between the two operators is mainly due to the random distribution of the nodes in this simulation drop which is not friendly to operator 2 in certain cases. However, we can see from Case 2 that such phenomenon is mitigated when directional LBT is used for NR-U instead of omni-directional LBT. Further, we can also observed that the performance of Operator 1 assuming omni-directional LBT was not significantly affected even if directional LBT is used for Operator 2.

Observation 5: Compared to omni-directional LBT, directional LBT is beneficial to increase the probability of channel access and the spatial reuse efficiency for NR-U, and the impact on the performance of the existed Wi-Fi system is negligible.

## CCA threshold and LBT schemes

In this section, the deployment scenario is Indoor scenario A, 2 operators use the same LBT mechanism, directional LBT or Omni-directional LBT. Different CCA threshold such as [-82dBm -72dBm -62dBm] are compared to evaluate the total performance of different LBT schemes. Medium traffic load and high traffic load performance is shown in Figure 3.3-1~3.3-3.

Simulation assumptions and detailed statistic can be found in Appendix.

- Low traffic load

Figure 3.3-1 Low traffic load mean UPT

- Medium traffic load

Figure 3.3-2 Medium traffic load mean UPT

- High traffic load

Figure 3.3-3 High traffic load mean UPT

Observation 6: As the CCA threshold increases, the LBT failure probability gradually decreases and different LBT schemes converge to a similar performance.

Observation 7: With appropriate CCA threshold, directional LBT shows better performance than Omni-directional LBT in NRU-NRU coexistence scheme.

## Single Operator Scenario

In this section, the performance of single operator scenario is evaluated with deployment scenario Indoor C as shown in Figure 3.4-1. Omni-directional LBT and directional LBT are compared in different traffic load with CCA threshold -82dBm.

Figure 3.4-1 Indoor Scenario C

Simulation assumptions and detailed statistic can be found in Appendix.

Figure 3.4-2 Single operator performance

Observation 8: Directional LBT shows better performance than Omni-directional LBT in single operator scenario.

## Different Channel bandwidth

In RAN1 #102e-meeting, another mandatory system simulation bandwidth of 2GHz is agreed, in this chapter, the deployment scenario is Indoor A with 2 operators. Different channel bandwidth are evaluated with Omni-directional LBT and directional LBT are compared in various traffic load.

Simulation assumptions and detailed statistic can be found in Appendix.

Figure 3.5-1 Performance of different bandwidth

Observation 9: Directional LBT shows better performance than Omni-directional LBT in both 400MHz and 2GHz bandwidth.

# Conclusion

Observation 1: Phase noise has limited impact on QPSK and 16QAM modulation, and with PTRS CPE compensation, different SCS (120kHz, 240kHz, 480kHz, 960kHz) shows similar performance.

Observation 2: Phase noise has significant impact on 64QAM modulation, and with PTRS CPE compensation, larger SCS shows better performance.

Observation 3: Various delay spread values don’t affect the relative performance of different SCS.

Observation 4: Phase noise and delay spread have limited impact on PRACH performance, the performance of SCS 120kHz, 240kHz, 480kHz and 960kHz is similar.

Observation 5: Compared to omni-directional LBT, directional LBT is beneficial to increase the probability of channel access and the spatial reuse efficiency for NR-U, and the impact on the performance of the existed Wi-Fi system is negligible.

Observation 6: As the CCA threshold increases, the LBT failure probability gradually decreases and different LBT schemes converge to a similar performance.

Observation 7: With appropriate CCA threshold, directional LBT shows better performance than Omni-directional LBT in NRU-NRU coexistence scheme.

Observation 8: Directional LBT shows better performance than Omni-directional LBT in single operator scenario.

Observation 9: Directional LBT shows better performance than Omni-directional LBT in 400MHz and 2GHz bandwidth.

# Reference

- Chairman's Notes RAN1 92 final, RAN1 #92, Feb. 2018.

- ETSI EN 301 893 V2.1.1 (2017-05)

- TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum, V13.0.0, Jul. 2015.

- TR 36.802, Study on New Radio (NR) Access Technology Physical Layer Aspects, V1.1.0, Jan. 2017.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V14.2.0, Sep. 2017.

# Appendix

## A1. LLS Simulation assumptions

Table A1-1 LLS simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Waveform | CP-OFDM |
| System Bandwidth | 400MHz |
| Subcarrier spacing | 120kHz/240kHz/480kHz/960kHz |
| Transmission bandwidth | 8/4/2/1 PRB |
| CP type | Normal CP |
| Channel Model | TDL-A 5ns, 10ns, 20ns CDL-B 20ns, 50ns |
| PN model | 3GPP TR38.803 example 2 |
| DMRS Configuration | 2 DMRS symbols per slot at (2,11) |
| PTRS Configuration | (K = 2, L = 1) |
| SLIV | (S=0, L=14) |
| Channel Estimation | Realistic |
| PN Estimation | Realistic |
| PN compensation type | CPE |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | For TDL model: 2*2 For CDL model: (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | For TDL model: 2*2 For CDL model: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ |

## A2. SLS Simulation assumptions

Table A2-1 SLS simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 60GHz |
| Channel Bandwidth | 2GHz is used in chapter 3.1~3.5 400MHz is used in chapter 3.5 |
| Subcarrier spacing | 960kHz for channel bandwidth 2GHz; 120KHz for channel bandwidth 400MHz |
| Scenario | Indoor A /Indoor C |
| LBT schemes | Omni-directional LBT Directional LBT |
| CCA threshold | [-82dBm -72dBm -62dBm] |
| Channel Model | The channel model for UE-to-UE links：InH open office: InH – office channel model with LOS probability for indoor - mixed office from TR38.901 The channel model for gNB-to-UE and gNB-gNB links：InH open office: InH – office channel model with LOS probability for indoor - open office from TR38.901 |
| Max. allowed BS Tx power | 40 dBm EIRP |
| Max. allowed UE Tx Power | 25 dBm EIRP |
| BS Antenna gain | 5dBi |
| UE Antenna gain | 5dBi |
| BS Noise Figure | 7dB |
| UE Receiver Noise Figure | 10dB |
| UE receiver | MMSE-IRC |
| BS antenna Array configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna Array configuration | (M, N, P, Mg, Ng) = (2, 2, 2, 1, 2), dH = dV = 0.5 λ |
| Traffic model | FTP Model 3 (27Mbyte file) |

## A3. SLS simulation results

Table A3-1 Coexistence interference analysis

| Cases |  | Case 1：Omni vs Omni |  | Case2:Omni vs Directional |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Operator1 | Operator2 | Operator1 | Operator2 |
| Traffic load Metrics |  | Low load 10%~25% BO |  | Low load 10%~25% BO |  |
| DL UPT (Mbps) | 5%ile | 2727.4854 | 2373.8499 | 2734.7085 | 3030.9807 |
|  | 10%ile | 4643.2490 | 3236.5020 | 4522.3325 | 4083.0208 |
|  | 20%ile | 6492.4668 | 5320.5269 | 6661.6064 | 7527.7227 |
|  | 50%ile | 9411.5508 | 8068.3247 | 8884.0186 | 10866.1943 |
|  | 95%ile | 15086.5273 | 15635.1260 | 14731.7412 | 19536.2930 |
|  | mean | 9506.1719 | 8636.5840 | 9433.5547 | 10886.6279 |
| DL delay (s) | 5%ile | 0.011 | 0.010 | 0.011 | 0.009 |
|  | 50%ile | 0.025 | 0.030 | 0.026 | 0.021 |
|  | 95%ile | 0.143 | 0.172 | 0.144 | 0.097 |
|  | mean | 0.046 | 0.053 | 0.046 | 0.033 |
| Arrival rate (files/s) |  | 1.25 | 1.25 | 1.25 | 1.25 |
| 𝜌DL |  | 100% | 100% | 100% | 100% |
| BO |  | 20.636 | 23.893 | 21.004 | 17.066 |
| Additional report/notes: - LBT procedure and parameters Refer to Section A.2. Subcarrier spacing is 960KHz; - any assumptions/parameters used not as in the agreed baseline This item is not considered： SLS performance evaluations purpose, -71 dBm + 10 log10 (BW/2GHz) is the baseline RSRP threshold for cell selection (UE with RSRP below this threshold are not considered in simulation 3. Details of case: e.g., single or two operators; no-LBT, omni-directional LBT, directional LBT schemes etc. Case1：. two operators,Omni-directional(Operator1) vs Omni-directional(Operator2); Case2：. two operators,Omni-directional(Operator1) vs Directional(Operator2) 4. Other metric(s) and definition if reported 5. Details of COT sharing if used in evaluation: DL Only, no COT sharing |  |  |  |  |  |

Table A3-2 performance of different threshold with CCA threshold -62dBm

| LBT mode |  | omni |  |  | directional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 3546.6826 | 3033.7112 | 975.4548 | 3547.0242 | 3110.3704 | 1207.0940 |
|  | 50%ile | 11305.6396 | 10783.6074 | 7088.4458 | 11371.8018 | 10765.0527 | 8245.8027 |
|  | 95%ile | 18089.7539 | 18282.6270 | 15489.9375 | 18654.7754 | 18886.9160 | 16380.3154 |
|  | mean | 11196.8545 | 10566.7207 | 8016.2710 | 11427.4307 | 10969.6787 | 8994.2236 |
| DL delay (s) | 5%ile | 0.010 | 0.010 | 0.011 | 0.010 | 0.010 | 0.011 |
|  | 50%ile | 0.020 | 0.021 | 0.032 | 0.020 | 0.020 | 0.027 |
|  | 95%ile | 0.072 | 0.109 | 0.589 | 0.070 | 0.099 | 0.429 |
|  | mean | 0.028 | 0.036 | 0.122 | 0.027 | 0.033 | 0.109 |
| Arrival rate (files/s) |  | 1.25 | 2 | 3.5 | 1.25 | 2 | 3.5 |
| 𝜌DL |  | 100% | 100% | 100% | 100% | 100% | 100% |
| BO |  | 14.746% | 25.491% | 56.031% | 14.312% | 24.300% | 50.851 % |
| Additional report/notes: 1. LBT procedure and parameters 2. any assumptions/parameters used not as in the agreed baseline 3. Details of case: e.g., single or two operators; no-LBT, omni-directional LBT, directional LBT schemes etc. 4. Other metric(s) and definition if reported 5. Details of COT sharing if used in evaluation |  |  |  |  |  |  |  |

Table A3-3 performance of different threshold with CCA threshold -72dBm

| LBT mode |  | omni |  |  | directional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 2901.5559 | 1584.9509 | 225.0555 | 3449.4133 | 2440.6877 | 586.0697 |
|  | 50%ile | 9875.0674 | 8321.1289 | 4440.5454 | 10900.7783 | 9365.6563 | 5319.0728 |
|  | 95%ile | 16569.4043 | 14917.2061 | 12050.0732 | 17744.9609 | 17649.1035 | 13868.5537 |
|  | mean | 9927.3428 | 8352.8154 | 4978.7114 | 10831.2520 | 9815.9434 | 6458.9243 |
| DL delay (s) | 5%ile | 0.010 | 0.011 | 0.013 | 0.010 | 0.010 | 0.012 |
|  | 50%ile | 0.023 | 0.030 | 0.070 | 0.021 | 0.024 | 0.044 |
|  | 95%ile | 0.104 | 0.235 | 2.098 | 0.080 | 0.131 | 1.130 |
|  | mean | 0.036 | 0.069 | 0.370 | 0.030 | 0.042 | 0.228 |
| Arrival rate (files/s) |  | 1.25 | 2 | 3.5 | 1.25 | 2 | 3.5 |
| 𝜌DL |  | 100% | 100% | 100% | 100% | 100% | 100% |
| BO |  | 17.989 % | 34.882% | 72.104% | 15.492% | 28.117% | 64.679% |
| Additional report/notes: 1. LBT procedure and parameters 2. any assumptions/parameters used not as in the agreed baseline 3. Details of case: e.g., single or two operators; no-LBT, omni-directional LBT, directional LBT schemes etc. 4. Other metric(s) and definition if reported 5. Details of COT sharing if used in evaluation |  |  |  |  |  |  |  |

Table A3-4 performance of different threshold with CCA threshold -82dBm

| LBT mode |  | omni |  |  | directional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 1468.8210 | 229.8788 | 171.6681 | 2918.3030 | 1458.0205 | 136.3976 |
|  | 50%ile | 6655.7744 | 4163.1836 | 2519.1045 | 9337.8369 | 7474.8154 | 3580.1260 |
|  | 95%ile | 13927.4766 | 10920.2197 | 10165.1152 | 17007.1816 | 13970.7529 | 10623.8594 |
|  | mean | 7034.9761 | 4810.1909 | 3621.6375 | 9284.5352 | 7416.9111 | 4238.0874 |
| DL delay (s) | 5%ile | 0.012 | 0.014 | 0.015 | 0.011 | 0.011 | 0.014 |
|  | 50%ile | 0.038 | 0.072 | 0.110 | 0.026 | 0.037 | 0.091 |
|  | 95%ile | 0.272 | 1.642 | 2.842 | 0.117 | 0.274 | 2.621 |
|  | mean | 0.076 | 0.304 | 0.553 | 0.039 | 0.074 | 0.457 |
| Arrival rate (files/s) |  | 1.25 | 2 | 3.5 | 1.25 | 2 | 3.5 |
| 𝜌DL |  | 100% | 100% | 100% | 100% | 100% | 100% |
| BO |  | 29.025 % | 58.092% | 81.785% | 19.242% | 39.586% | 80.439% |
| Additional report/notes: 1. LBT procedure and parameters 2. any assumptions/parameters used not as in the agreed baseline 3. Details of case: e.g., single or two operators; no-LBT, omni-directional LBT, directional LBT schemes etc. 4. Other metric(s) and definition if reported 5. Details of COT sharing if used in evaluation |  |  |  |  |  |  |  |

Table A3-5 performance of single operator with CCA threshold -82dBm

| LBT mode |  | omni |  |  | directional |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |  |
| DL UPT (Mbps) | 5%ile | 2469.8159 | 1343.2117 | 83.0307 | 2148.6824 | 1935.8322 | 225.7256 |  |
|  | 50%ile | 11222.2646 | 8818.3066 | 4752.3027 | 11596.8965 | 9712.3398 | 6331.9395 |  |
|  | 95%ile | 18383.7070 | 16582.6855 | 14950.5947 | 20836.0254 | 20835.5352 | 17445.4707 |  |
|  | mean | 10292.8906 | 8585.7139 | 5730.4863 | 10906.8916 | 10209.7617 | 7295.3428 |  |
| DL delay (s) | 5%ile | 0.009 | 0.010 | 0.011 | 0.009 | 0.009 | 0.010 |  |
|  | 50%ile | 0.024 | 0.037 | 0.063 | 0.022 | 0.025 | 0.049 |  |
|  | 95%ile | 0.121 | 0.321 | 3.794 | 0.122 | 0.168 | 0.236 |  |
|  | mean | 0.040 | 0.085 | 0.708 | 0.038 | 0.049 | 2.575 |  |
| Arrival rate (files/s) |  | 1.25 | 2 | 3.5 | 1.25 | 2 | 3.5 |
| 𝜌DL |  | 100% | 99.96% | 100% | 99.86% | 100% | 99.78% |
| BO |  | 20.593% | 43.846% | 80.095% | 19.864% | 32.918% | 76.133% |
| Additional report/notes: 1. LBT procedure and parameters 2. any assumptions/parameters used not as in the agreed baseline 3. Details of case: e.g., single or two operators; no-LBT, omni-directional LBT, directional LBT schemes etc. 4. Other metric(s) and definition if reported 5. Details of COT sharing if used in evaluation |  |  |  |  |  |  |  |

Table A3-6 performance of different bandwidth with CCA threshold -82dBm

|  |  | 400M |  |  |  |  |  | 2000M |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LBT mode |  | omni |  |  | Directional |  |  | omni |  |  | Directional |  |  |
| Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
| DL UPT (Mbps) | 5%ile | 690.0958 | 223.3712 | 108.7060 | 1015.2816 | 523.0024 | 143.9919 | 1468.8210 | 229.8788 | 171.6681 | 2918.3030 | 1458.0205 | 136.3976 |
|  | 50%ile | 1800.0000 | 942.9960 | 566.6034 | 2460.7251 | 1720.1998 | 680.7795 | 6655.7744 | 4163.1836 | 2519.1045 | 9337.8369 | 7474.8154 | 3580.1260 |
|  | 95%ile | 3534.6392 | 2624.1802 | 2219.4753 | 4441.9355 | 3317.0732 | 2224.2991 | 13927.4766 | 10920.2197 | 10165.1152 | 17007.1816 | 13970.7529 | 10623.8594 |
|  | mean | 1976.1506 | 1188.0437 | 796.7006 | 2676.9175 | 1838.0183 | 898.8221 | 7034.9761 | 4810.1909 | 3621.6375 | 9284.5352 | 7416.9111 | 4238.0874 |
| DL delay (s) | 5%ile | 0.047 | 0.060 | 0.068 | 0.042 | 0.044 | 0.069 | 0.012 | 0.014 | 0.015 | 0.011 | 0.011 | 0.014 |
|  | 50%ile | 0.112 | 0.247 | 0.484 | 0.088 | 0.141 | 0.378 | 0.038 | 0.072 | 0.110 | 0.026 | 0.037 | 0.091 |
|  | 95%ile | 0.433 | 1.349 | 2.124 | 0.275 | 0.690 | 3.249 | 0.272 | 1.642 | 2.842 | 0.117 | 0.274 | 2.621 |
|  | mean | 0.173 | 0.408 | 0.725 | 0.114 | 0.229 | 0.872 | 0.076 | 0.304 | 0.553 | 0.039 | 0.074 | 0.457 |
| Arrival rate (files/s) |  | 0.3125 | 0.625 | 1.25 | 0.3125 | 0.625 | 1.25 | 1.25 | 2 | 3.5 | 1.25 | 2 | 3.5 |
| 𝜌DL |  | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| BO |  | 20.608% | 52.843% | 83.511% | 15.047% | 41.659% | 82.938% | 29.025 % | 58.092% | 81.785% | 19.242% | 39.586% | 80.439% |
| Additional report/notes: 1. LBT procedure and parameters 2. any assumptions/parameters used not as in the agreed baseline 3. Details of case: e.g., single or two operators; no-LBT, omni-directional LBT, directional LBT schemes etc. 4. Other metric(s) and definition if reported 5. Details of COT sharing if used in evaluation |  |  |  |  |  |  |  |  |  |  |  |  |  |