3GPP TSG RAN WG1 Meeting #103-e  			    			            R1-2007986

e-Meeting, October 26th – November 13th, 2020

# Source:	ETRI

# Title:		Remaining issues on resource allocation mode 2 for NR V2X

# Agenda item:	7.2.4

# Document for:	Discussion/Decision

Introduction

In this contribution, we address the remaining issue on resource allocation mode 2 for NR V2X.

Resource allocation mode 2

Regarding configuration of resource reservation period, the following was agreed in RAN1#101-e meeting [1]:

Agreements: - A UE sets “Resource reservation period” in SCI 1-A to correspond to value of the period provided by higher layers from (pre-)configured set sl-ResourceReservePeriod - RAN1 assumes that at least in cases if higher layer decides not to keep the resource for the transmission in the next period or there is no associated period, then higher layer provides 0 ms periodicity - Send LS to RAN2 to inform this decision

However, the agreement was not captured in higher layer specification i.e., TS 38.321. The reason was that the description in physical layer specification is more suitable since the operation is the same as in LTE, and in LTE such operation has been specified in physical layer specification i.e., TS 36.213. In the last meeting, one company proposed to capture the agreement in physical layer specification i.e., TS 38.213 similar as in LTE [2]. However, it was not discussed due to time limitation. Whether it should be captured in RAN1 or RAN2, we slightly prefer to capture in RAN2 since the agreement is saying that 0ms is provided by higher layer. Therefore, it is proposed to send LS to RAN2 again in order to capture the agreement. However, if the majority view prefers capturing in RAN1, it is also fine to us. In this case, the text proposal for TS 38.213 in [2] can be used. It is copied and pasted in the following for the convenience.

- 16.4 UE procedure for transmitting PSCCH A UE can be provided a number of symbols in a resource pool, by timeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by frequencyResourcePSCCH, for a PSCCH transmission with a SCI format 1-A. A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] sets - If the UE is provided sl-MultiReserveResource-r16, "Resource reservation period" as an index in sl-ResourceReservePeriod-r16 corresponding to - a reservation period provided by higher layers [11, TS 38.321] if the resource in the next period will be used to transmit the next TB - 0 otherwise. - the values of the frequency resource assignment field and the time resource assignment field as described in [6, TS 38.214] to indicate N resources from a set Ry of resources selected by higher layers as described in [11, TS 38.321] with N smallest slot indices yi for 0≤i≤N-1 such that y0<y1<…<yN-1≤y0+31, where:

Proposal 1: Propose to capture the above agreement with one of following options:

- Option 1: Send LS to RAN2

- Option 2: Adopt the above TP

Summary

In this contribution, we made the following proposal for resource allocation mode 2 for NR V2X.

Proposal 1: Propose to capture the above agreement with one of following options:

- Option 1: Send LS to RAN2

- Option 2: Adopt the above TP

References

- Chairman’s notes, RAN1#101-e, May 2020.

- R1-2006770, “Sidelink Resource Allocation Mode 2,” Qualcomm Incorporated