3GPP TSG RAN WG1 #103-e			                         R1-2007993

e-Meeting, October 26th – November 13th, 2020

Agenda item:		8.8.1.1

Source:	China Telecom

Title:	Updated baseline performance for NR coverage enhancements for FR1

Document for:		Discussion

Introduction

In RAN #86 meeting, a new Rel-17 study item on NR coverage enhancements was approved [1] and updated in RAN #88e meeting [2]. The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows.

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

This contribution provides updated baseline performance for NR coverage enhancements for FR1.

Simulation results for PUSCH

- 2.1 The required SNR

The simulation assumptions for PUSCH based on link-level simulation were agreed in RAN1 #101e and RAN1 #102e meeting, which are summarized in Table A-1 and Table A-2 in Appendix for the required SNR calculation. The simulation results of SNR for different scenarios and different services for PUSCH are summarized in Table 2-1 and Table 2-2.

Table 2-1 The required SNR for PUSCH for eMBB

| Scenario | Channel model | Frame structure | Number of receiver chains | Required SNR (dB) |
| --- | --- | --- | --- | --- |
| Urban 4GHz O-to-I | NLOS TDL-C | DDDSUDDSUU | 2 | -1.4 |
|  |  | DDDSUDDSUU | 4 | -4.7 |
|  |  | DDDSU | 2 | 0.1 |
|  |  | DDDSU | 4 | -3.3 |
| Rural 4GHz O-to-I | NLOS TDL-C | DDDSUDDSUU | 2 | -1.5 |
|  |  | DDDSUDDSUU | 4 | -4.8 |
|  |  | DDDSU | 2 | 0.4 |
|  |  | DDDSU | 4 | -3.3 |
| Rural 4GHz O-to-O | NLOS TDL-C | DDDSUDDSUU | 2 | -1.3 |
|  |  | DDDSUDDSUU | 4 | -4.7 |
|  |  | DDDSU | 2 | 0.4 |
|  |  | DDDSU | 4 | -3.7 |
| Rural 2GHz O-to-I | NLOS TDL-C | FDD(UUUUU) | 2 | -2.6 |
|  |  | FDD(UUUUU) | 4 | -6.3 |
| Rural 2GHz O-to-O | NLOS TDL-C | FDD(UUUUU) | 2 | -1.5 |
|  |  | FDD(UUUUU) | 4 | -6.1 |
| Rural 700M with long distance O-to-O | LOS TDL-D | FDD(UUUUU) | 2 | -8.9 |
|  |  | FDD(UUUUU) | 4 | -11.15 |

Table 2-2 The required SNR for PUSCH for VoIP

| Scenario | Channel model | Frame structure | Number of transmission times | Number of receiver chains | Required SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Urban/Rural 4GHz O-to-I | NLOS TDL-C | DDDSUDDSUU | 3 | 2 | -0.25 |
|  |  | DDDSUDDSUU | 3 | 4 | -4.5 |
|  |  | DDDSUDDSUU | 12 | 2 | -5.15 |
|  |  | DDDSU | 2 | 2 | 1.1 |
|  |  | DDDSU | 2 | 4 | -3.5 |
|  |  | DDDSU | 8 | 2 | -4.3 |
| Rural 4GHz O-to-O | NLOS TDL-C | DDDSUDDSUU | 3 | 2 | -3.2 |
|  |  | DDDSUDDSUU | 3 | 4 | -6.3 |
|  |  | DDDSUDDSUU | 12 | 2 | -8.7 |
|  |  | DDDSU | 2 | 2 | -0.8 |
|  |  | DDDSU | 2 | 4 | -4.6 |
|  |  | DDDSU | 8 | 2 | -7.4 |
| Rural 2GHz O-to-I | NLOS TDL-C | FDD(UUUUU) | 8 | 2 | -3.45 |
|  |  | FDD(UUUUU) | 8 | 4 | -6.9 |
|  |  | FDD(UUUUU) | 16 | 2 | -6.1 |
| Rural 2GHz O-to-O | NLOS TDL-C | FDD(UUUUU) | 8 | 2 | -3.4 |
|  |  | FDD(UUUUU) | 8 | 4 | -7.0 |
|  |  | FDD(UUUUU) | 16 | 2 | -6.3 |
| Rural 700M with long distance O-to-O | LOS TDL-D | FDD(UUUUU) | 8 | 2 | -11.55 |
|  |  | FDD(UUUUU) | 8 | 4 | -13.30 |
|  |  | FDD(UUUUU) | 16 | 2 | -13.75 |

- 2.2 Link budget template

Based on the updated link budget template proposed by FL in R1-2007480, we provide the detailed values for different scenarios for PUSCH in the excel sheet attachment.

- 2.3 Target performance

According to the agreements in RAN1 #102e meeting, there are three options for the target performance.

Agreements:

- RAN1 strives for satisfying appropriate targets identified by companies particularly operators

  - The targets may be in the form of one or more of the following:

    - 1. Scenario dependent targets, e.g., ISD/MPL

    - 2. Service dependent targets, e.g., [MCL=147] dB for VoIP;

    - 3. Relative difference between channels, e.g, MIL(/[MCL])

  - Further values and details of such targets will be clarified at RAN1#103-e

  - Note: there is no intention in RAN1 to update the study item objectives due to the identified targets.

In our understanding, compared with relative values, the absolute values can better reflect the requirements from operators. The target MCL/MIL/MPL can be derived from ISD, while ISD can be provided based on operators’ practical deployment. The typical values, e.g., 500m for urban, 1732m for rural, widely used in the simulation can be adopted. Understanding the gap between baseline performance and target performance can provide valuable guidance for practical network deployment.

The target path loss for the target performance can be derived based on the typical ISD for each scenario and path loss formulas in ITU-M.2412 [3].

Table 2-3 Formulas for target performance calculation

| Scenario | ISD(m) | Target range = ISD/sqrt(3) | Target performance(dB) |
| --- | --- | --- | --- |
| Urban 4GHz | 500 | 288.68 | 121.75Note 1 |
| Rural 4GHz | 1732 | 999.97 | 131.57Note 2 |
| Rural 2GHz | 1732 | 999.97 | 125.55Note 2 |
| Rural with long distance | 30000 | 17320.51 | 142.91Note 3 |

Note 1: For urban 4GHz scenario with ISD = 500m, the target performance for NLOS can be obtained as follows:

Target performance dB=161.04-7.1∙log10W+7.5∙log10h-24.37-3.7∙hhBS2∙log10hBS+43.42-3.1∙log10hBS∙log10d3D-3+20∙log10fc-3.2∙log1011.75∙hUT2-4.97-0.6∙hUT-1.5,

where W=20m is the average street width, h=20m is the average building height, hBS=25m is the BS antenna height, hUT=1.5m is the UT antenna height, fc is the carrier frequency, d3D is the target range.

Note 2: For rural scenario with ISD = 1732m, the target performance for NLOS can be obtained as follows:

Target performance dB=161.04-7.1∙log10W+7.5∙log10h-24.37-3.7∙hhBS2∙log10hBS+43.42-3.1∙log10hBS∙log10d3D-3+20∙log10fc-3.2∙log1011.75∙hUT2-4.97,

where W=20m, h=5m, hBS=35m, hUT=1.5m.

Note 3: For rural with long distance scenario with ISD = 30km, the target performance for LOS can be obtained as follows:

Target performance dB=20∙log1040∙π∙dBP∙fc3+min0.03∙h1.72,10∙log10dBP-min0.044∙h1.72,14.77+0.002∙log10h∙dBP+40∙log10d3DdBP,

where W=20m, h=5m, dBP=2∙π∙hBS∙hUT∙fcc and hBS=35m, hUT=1.5m, c=3∙108m/s.

Proposal 1: The target MPL derived from typical ISD, e.g. 500m for urban and 1732m for rural, is defined as the target performance.

- 2.4 Baseline performance for PUSCH

The baseline performance for PUSCH for eMBB are summarized in Table 2-4.

Table 2-4 Baseline performance for PUSCH for eMBB with target MPL derived from ISD

| Scenario | Frame structure | Number of receiver chains | MCL(dB) | MIL(dB) | MPL(dB) | Target MPL dB) | Gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban 4GHz O-to-I | DDDSUDDSUU | 2 | 131.65 | 140.42 | 109.69 | 121.75 | 12.06 |
|  | DDDSUDDSUU | 4 | 131.94 | 140.71 | 109.98 |  | 11.77 |
|  | DDDSU | 2 | 130.15 | 138.92 | 108.19 |  | 13.56 |
|  | DDDSU | 4 | 130.54 | 139.31 | 108.58 |  | 13.17 |
| Rural 4GHz O-to-I | DDDSUDDSUU | 2 | 140.50 | 144.50 | 126.87 | 131.57 | 4.7 |
|  | DDDSUDDSUU | 4 | 140.79 | 144.79 | 127.16 |  | 4.41 |
|  | DDDSU | 2 | 138.60 | 142.60 | 124.97 |  | 6.60 |
|  | DDDSU | 4 | 139.29 | 143.29 | 125.66 |  | 5.91 |
| Rural 4GHz O-to-O | DDDSUDDSUU | 2 | 140.30 | 144.30 | 128.69 |  | 2.88 |
|  | DDDSUDDSUU | 4 | 140.69 | 144.69 | 129.08 |  | 2.49 |
|  | DDDSU | 2 | 138.60 | 142.60 | 126.99 |  | 4.58 |
|  | DDDSU | 4 | 139.69 | 143.69 | 128.08 |  | 3.49 |
| Rural 2GHz O-to-I | FDD(UUUUU) | 2 | 129.56 | 145.60 | 127.97 | 125.55 | -2.42 |
|  | FDD(UUUUU) | 4 | 133.26 | 146.29 | 128.66 |  | -3.11 |
| Rural 2GHz O-to-O | FDD(UUUUU) | 2 | 128.46 | 144.50 | 128.89 |  | -3.34 |
|  | FDD(UUUUU) | 4 | 133.06 | 146.09 | 130.48 |  | -4.93 |
| Rural 700M with long distance O-to-O | FDD(UUUUU) | 2 | 135.86 | 148.89 | 135.10 | 142.91 | 7.81 |
|  | FDD(UUUUU) | 4 | 138.11 | 148.13 | 134.34 |  | 8.57 |

Based on the baseline performance for PUSCH for eMBB, we have the following observations and proposals:

Observation 1: For 4GHz urban scenario, with TDD configuration of DDDSUDDSUU, about 12.06 dB, 11.77 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 2: For 4GHz urban scenario, with TDD configuration of DDDSU, about 13.56 dB, 13.17 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 3: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSUDDSUU, about 4.7 dB, 4.41 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 4: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSU, about 6.60 dB, 5.91 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 5: For 4GHz rural O-to-O scenario, with TDD configuration of DDDSUDDSUU, about 2.88 dB, 2.49 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 6: For 4GHz rural O-to-O scenario, with TDD configuration of DDDSU, about 4.58 dB, 3.49 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 7: For 700MHz rural with long distance O-to-O scenario, about 7.81 dB, 8.57 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Proposal 2: The coverage performance of PUSCH eMBB for 4GHz urban, 4GHz rural and 700MHz rural with long distance should be enhanced.

The baseline performance for PUSCH for VoIP are summarized in Table 2-5.

Table 2-5 Baseline performance for PUSCH for VoIP with target MPL derived from ISD

| Scenario | Frame structure | Number of transmission times | Number of receiver chains | MCL(dB) | MIL(dB) | MPL(dB) | Target MPL (dB) | Gap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Urban 4GHz O-to-I | DDDSUDDSUU | 3 | 2 | 138.75 | 147.52 | 116.79 | 121.75 | 4.96 |
|  | DDDSUDDSUU | 3 | 4 | 139.99 | 148.76 | 118.03 |  | 3.72 |
|  | DDDSUDDSUU | 12 | 2 | 143.65 | 152.42 | 121.69 |  | 0.06 |
|  | DDDSU | 2 | 2 | 137.40 | 146.17 | 115.44 |  | 6.31 |
|  | DDDSU | 2 | 4 | 138.99 | 147.76 | 117.03 |  | 4.72 |
|  | DDDSU | 8 | 2 | 142.80 | 151.57 | 120.84 |  | 0.91 |
| Rural 4GHz O-to-I | DDDSUDDSUU | 3 | 2 | 138.75 | 142.75 | 125.12 | 131.57 | 6.45 |
|  | DDDSUDDSUU | 3 | 4 | 139.99 | 143.99 | 126.36 |  | 5.21 |
|  | DDDSUDDSUU | 12 | 2 | 143.65 | 147.65 | 130.02 |  | 1.55 |
|  | DDDSU | 2 | 2 | 137.40 | 141.40 | 123.77 |  | 7.80 |
|  | DDDSU | 2 | 4 | 138.99 | 142.99 | 125.36 |  | 6.21 |
|  | DDDSU | 8 | 2 | 142.80 | 146.80 | 129.17 |  | 2.40 |
| Rural 4GHz O-to-O | DDDSUDDSUU | 3 | 2 | 141.70 | 145.70 | 130.09 |  | 1.48 |
|  | DDDSUDDSUU | 3 | 4 | 141.79 | 145.79 | 130.18 |  | 1.39 |
|  | DDDSUDDSUU | 12 | 2 | 147.20 | 151.20 | 135.59 |  | -4.02 |
|  | DDDSU | 2 | 2 | 139.30 | 143.30 | 127.69 |  | 3.88 |
|  | DDDSU | 2 | 4 | 140.09 | 144.09 | 128.48 |  | 3.09 |
|  | DDDSU | 8 | 2 | 145.90 | 149.90 | 134.29 |  | -2.72 |
| Rural 2GHz O-to-I | FDD(UUUUU) | 8 | 2 | 129.91 | 145.95 | 128.32 | 125.55 | -2.77 |
|  | FDD(UUUUU) | 8 | 4 | 133.36 | 146.39 | 128.76 |  | -3.21 |
|  | FDD(UUUUU) | 16 | 2 | 132.56 | 148.60 | 130.97 |  | -5.42 |
| Rural 2GHz O-to-O | FDD(UUUUU) | 8 | 2 | 129.86 | 145.90 | 130.29 |  | -4.74 |
|  | FDD(UUUUU) | 8 | 4 | 133.46 | 146.49 | 130.88 |  | -5.33 |
|  | FDD(UUUUU) | 16 | 2 | 132.76 | 148.80 | 133.19 |  | -7.64 |
| Rural 700M with long distance O-to-O | FDD(UUUUU) | 8 | 2 | 138.01 | 151.04 | 137.25 | 142.91 | 5.66 |
|  | FDD(UUUUU) | 8 | 4 | 139.76 | 149.78 | 135.99 |  | 6.92 |
|  | FDD(UUUUU) | 16 | 2 | 140.21 | 153.24 | 139.45 |  | 3.46 |

Based on the baseline performance for PUSCH for VoIP, we have the following observations and proposals:

Observation 8: For 4GHz urban scenario, with TDD configuration of DDDSUDDSUU and 3 retransmissions, about 4.96 dB, 3.72 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 9: For 4GHz urban scenario, with TDD configuration of DDDSU and 3 retransmissions, about 6.31 dB, 4.72 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 10: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSUDDSUU,

- For 3 retransmissions, about 6.45 dB, 5.21 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

- For 12 retransmissions, about 1.55 dB performance gaps are observed for PUSCH with the receiver chains of 2.

Observation 11: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSU,

- For 2 retransmissions, about 7.80 dB, 6.21 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

- For 8 retransmissions, about 2.40 dB performance gaps are observed for PUSCH with the receiver chains of 2.

Observation 12: For 700MHz rural with long distance O-to-O scenario,

- For 8 retransmissions, about 5.66 dB, 6.92 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

- For 16 retransmissions, about 3.46 dB performance gaps are observed for PUSCH with the receiver chains of 2.

Proposal 3: The coverage performance of PUSCH VoIP for 4GHz urban, 4GHz rural O-to-I, rural 700MHz with long distance should be enhanced.

Simulation results for PUCCH

- 3.1 The required SNR

The simulation assumptions for PUCCH based on link-level simulation were agreed in RAN1 #101e and RAN1 #102e meeting, which are summarized in Table A-1 and Table A-3 in Appendix for the required SNR calculation. The simulation results of SNR for different scenarios for PUCCH are summarized in Table 3-1.

Table 3-1 The required SNR for PUCCH

| Format type | Scenario | Channel model | Number of receiver chains | Required SNR (dB) |
| --- | --- | --- | --- | --- |
| Format 1 | Urban 4GHz O-to-I | NLOS TDL-C | 2 | -5.9 |
|  | Rural 4GHz O-to-I | NLOS TDL-C | 2 | -5.9 |
|  | Rural 4GHz O-to-O | NLOS TDL-C | 2 | -5.8 |
|  | Rural 2GHz O-to-I | NLOS TDL-C | 2 | -5.9 |
|  | Rural 2GHz O-to-O | NLOS TDL-C | 2 | -5.8 |
|  | Rural 700M with long distance O-to-O | LOS TDL-D | 2 | -13.8 |
| Format 3 | Urban 4GHz O-to-I | NLOS TDL-C | 4 | -4.7 |
|  | Rural 4GHz O-to-I | NLOS TDL-C | 4 | -4.7 |
|  | Rural 4GHz O-to-O | NLOS TDL-C | 4 | -5.1 |
|  | Rural 2GHz O-to-I | NLOS TDL-C | 4 | -4.8 |
|  | Rural 2GHz O-to-O | NLOS TDL-C | 4 | -5.1 |

- 3.2 Link budget template

Based on the updated link budget template proposed by FL in R1-2007480, we provide the detailed values for different scenarios for PUCCH in the excel sheet attachment.

- 3.3 Baseline performance for PUCCH

The baseline performance for PUCCH are summarized in Table 3-2.

Table 3-2 Baseline performance for PUCCH with target MPL derived from ISD

| Format Type | Scenario | Number of receiver chains | MCL(dB) | MIL(dB) | MPL(dB) | Target MPL dB) | Gap |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Format 1 | Urban 4GHz O-to-I | 2 | 147.35 | 156.12 | 122.31 | 121.75 | -0.55 |
|  | Rural 4GHz O-to-I | 2 | 147.35 | 151.35 | 130.40 | 131.57 | 1.18 |
|  | Rural 4GHz O-to-O | 2 | 147.25 | 151.25 | 131.80 | 131.57 | -0.22 |
|  | Rural 2GHz O-to-I | 2 | 135.31 | 151.35 | 130.40 | 125.55 | -4.84 |
|  | Rural 2GHz O-to-O | 2 | 135.21 | 151.25 | 131.80 | 125.55 | -6.24 |
|  | Rural 700M with long distance O-to-O | 2 | 143.21 | 156.24 | 139.18 | 142.91 | 3.74 |
| Format 3 | Urban 4GHz O-to-I | 4 | 143.14 | 151.91 | 118.10 | 121.75 | 3.66 |
|  | Rural 4GHz O-to-I | 4 | 143.14 | 147.14 | 126.19 | 131.57 | 5.39 |
|  | Rural 4GHz O-to-O | 4 | 143.54 | 147.54 | 128.09 | 131.57 | 3.49 |
|  | Rural 2GHz O-to-I | 4 | 134.21 | 147.24 | 126.29 | 125.55 | -0.73 |
|  | Rural 2GHz O-to-O | 4 | 134.51 | 147.54 | 128.09 | 125.55 | -2.53 |

Based on the baseline performance for PUCCH, we have the following observations and proposals:

Observation 13: For PUCCH format 1, about 1.18 dB, 3.74 dB performance gaps are observed for 4GHz rural O-to-I and rural 700MHz with long distance respectively.

Observation 14: For PUCCH format 3, about 3.66 dB performance gaps are observed for 4GHz urban.

Observation 15: For PUCCH format 3, about 5.39 dB, 3.49 dB performance gaps are observed for 4GHz rural O-to-I and O-to-O respectively.

Proposal 4: The coverage performance of PUCCH format 1 for 4GHz rural O-to-I and rural 700MHz with long distance should be enhanced.

Proposal 5: The coverage performance of PUCCH format 3 for 4GHz urban and 4GHz rural should be enhanced.

Conclusion

In this contribution, we provided baseline performance for NR coverage enhancements for FR1.

Based on the baseline performance for PUSCH for eMBB and VoIP, we have the following observations:

Observation 1: For 4GHz urban scenario, with TDD configuration of DDDSUDDSUU, about 12.06 dB, 11.77 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 2: For 4GHz urban scenario, with TDD configuration of DDDSU, about 13.56 dB, 13.17 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 3: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSUDDSUU, about 4.7 dB, 4.41 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 4: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSU, about 6.60 dB, 5.91 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 5: For 4GHz rural O-to-O scenario, with TDD configuration of DDDSUDDSUU, about 2.88 dB, 2.49 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 6: For 4GHz rural O-to-O scenario, with TDD configuration of DDDSU, about 4.58 dB, 3.49 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 7: For 700MHz rural with long distance O-to-O scenario, about 7.81 dB, 8.57 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 8: For 4GHz urban scenario, with TDD configuration of DDDSUDDSUU and 3 retransmissions, about 4.96 dB, 3.72 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 9: For 4GHz urban scenario, with TDD configuration of DDDSU and 3 retransmissions, about 6.31 dB, 4.72 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

Observation 10: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSUDDSUU,

- For 3 retransmissions, about 6.45 dB, 5.21 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

- For 12 retransmissions, about 1.55 dB performance gaps are observed for PUSCH with the receiver chains of 2.

Observation 11: For 4GHz rural O-to-I scenario, with TDD configuration of DDDSU,

- For 2 retransmissions, about 7.80 dB, 6.21 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

- For 8 retransmissions, about 2.40 dB performance gaps are observed for PUSCH with the receiver chains of 2.

Observation 12: For 700MHz rural with long distance O-to-O scenario,

- For 8 retransmissions, about 5.66 dB, 6.92 dB performance gaps are observed for PUSCH with the receiver chains of 2 and 4 respectively.

- For 16 retransmissions, about 3.46 dB performance gaps are observed for PUSCH with the receiver chains of 2.

Based on the baseline performance for PUCCH, we have the following observations:

Observation 13: For PUCCH format 1, about 1.18 dB, 3.74 dB performance gaps are observed for 4GHz rural O-to-I and rural 700MHz with long distance respectively.

Observation 14: For PUCCH format 3, about 3.66 dB performance gaps are observed for 4GHz urban.

Observation 15: For PUCCH format 3, about 5.39 dB, 3.49 dB performance gaps are observed for 4GHz rural O-to-I and O-to-O respectively.

Based on the above observations, we have the following proposals:

Proposal 1: The target MPL derived from typical ISD, e.g. 500m for urban and 1732m for rural, is defined as the target performance.

Proposal 2: The coverage performance of PUSCH eMBB for 4GHz urban, 4GHz rural and 700MHz rural with long distance should be enhanced.

Proposal 3: The coverage performance of PUSCH VoIP for 4GHz urban, 4GHz rural O-to-I, rural 700MHz with long distance should be enhanced.

Proposal 4: The coverage performance of PUCCH format 1 for 4GHz rural O-to-I and rural 700MHz with long distance should be enhanced.

Proposal 5: The coverage performance of PUCCH format 3 for 4GHz urban and 4GHz rural should be enhanced.

References

- RP-193240, China Telecom, New SID on NR coverage enhancement, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9th – 12th, 2019.

- RP-200861, China Telecom, Revised SID on Study on NR coverage enhancements, 3GPP TSG RAN#88e, June 29th – July 3rd, 2020.

- ITU-M.2412, “Guidelines for evaluation of radio interface technologies for IMT-2020”.

Appendix A

This subclause describes the link-level simulation assumptions for PUSCH and PUCCH for FR1 in Table A-1 ~ A-3.

Table A-1: General parameters for FR1

| Parameter | Value |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| Target data rates for eMBB | Urban: DL 10Mbps, UL 1Mbps Rural: DL 1Mbps, UL 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional) |
| Packet size for VoIP | A packet size of 320 bits with 20ms data arriving interval is adopted. Size (bits). Payload; 256. CRC; 16 (TBS size lower than 3824 bits). MAC; 16 (with 12 bits SN size). RLC; 8 (with 6 bits SN size). PDCP; 16. RTP/UDP/IP; 24 (w RoHC). If applicable, companies report TB size assumed in evaluation. For SIP invite message - Payload of 1500 bytes can be a starting point. - The assumptions (TB size, time period etc.) are reported by companies. - Contributions R1-2003464 and R1-2005259 are taken into account for the evaluation - In addition, 1 second time period can also be considered. |
| Latency requirements for VoIP | Latency requirements assumed in VoIP evaluation for TDD and FDD are reported by companies. |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h (optional 30km/h) for outdoor |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. |

Table A-2: Channel-specific parameters for PUSCH for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| PUSCH duration | 14 OS |
| Repetitions | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Table A-3: Channel-specific parameters for PUCCH for FR1

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% BLER for CSI: 1%, optional for 10%. |
| Number of UE transmit chains | 1 |
| DMRS configuration | FFS: number of DMRS symbols for PUCCH Format 3. |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Repetitions | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |