3GPP TSG RAN WG1 #103-e	                                          R1-2008028

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.8.2.3

Source: 	CMCC

Title: 	Discussion on coverage enhancement for channels other than PUSCH and PUCCH

Document for:	Discussion and Decision

Introduction

At the RAN1#102e meeting, the Coverage enhancement for channels other than PUSCH and PUCCH were discussed. And several agreements had been achieved during the meeting [1].

Agreements:

- Study Msg3 PUSCH enhancement in NR coverage enhancement SI

  - Study at least Msg3 PUSCH repetition

    - FFS the aspects to be enhanced, e.g., signaling indication, repetition pattern, interplay between Msg1 and Msg3, DM-RS enhancements related to repetition etc.

  - FFS multiple-antenna techniques.

In this contribution, we provide our considerations and evaluations on the Msg 3 PUSCH enhancement.

Discussion and Evaluation on Msg 3 enhancements

It was agreed that Msg 3 PUSCH need be studied for the coverage enhancements, which includes at least PUSCH repetition and DMRS enhancements related to repetition. It was concluded in the previous studies such as eMTC and coverage enhancements for LTE that joint channel estimation across the slots could enhance the channel estimation quality and therefore improve the coverage performance. Also from the observation within our companion contribution [2], the joint channel estimation could also improve the coverage performance for the PUSCH with 1Mbps. In this section, the repetition performance and joint channel estimation are evaluated.

The frame structure of 2.6GHz TDD band is considered, which contains 7D1S2U slots and 6D:4S:4U symbols within the special slot. Due to only 2 uplink slot is configured for the system, only 2 repetitions are considered. The simulation assumptions are in the Annex, which is based on the agreements from #101e and #102e.

Case #1 is the baseline performance for the Msg 3 in the Dense Urban O2I scenario. Case #2 considers the repetition transmission in 2 consecutive uplink slots. Case #2a and #2b take single slot channel estimation and cross slot channel estimation respectively. Case #3 and Case #4 considers to reduce the DMRS density in time domain and lower the code rate to enhance the coverage.

Table 1 evaluation results for Msg 3 repetition and cross slot channel estimation

| Scheme | TBS size | Code Rates | Slot/repetition number | DMRS symbols | Channel estimation | SNR (dB) BLER: 10% |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1 (baseline) | 56 | 0.1061 | 1 slot | 3 | Single slot | -5.6 |
| Case #2a | 56 | 0.1061 | 2 slot | 3 | Single slot | -7.85 |
| Case #2b | 56 | 0.1061 | 2 slot | 3+3 | Cross slots | -9.6 |
| Case #3a | 56 | 0.0897 | 2 slot | 1 | Single slot | -7.8 |
| Case #3b | 56 | 0.0897 | 2 slot | 1+1 | Cross slots | -8.95 |
| Case #4a | 56 | 0.0972 | 2 slot | 2 | Single slot | -8.08 |
| Case #4b | 56 | 0.0972 | 2 slot | 2+2 | Cross slots | -9.35 |

From the Table 1, it could be observed that 2 slot repetition could improve the coverage around 2.25dB. And cross slot channel estimation under this condition could improve 1.75dB. Comparing the case #2a, 3a and 4a, a trade-off between channel estimation accuracy and overhead could be observed.

Observation 1:

The 2 slot repetition could improve Msg 3 PUSCH coverage about 2.25dB. And the cross channel estimation could provide additional 1.75dB gain based on 2 slot repetition.

Proposal 1:

Cross slot channel estimation and DMRS enhancements related to repetition should be considered for the coverage enhancement of Msg 3 PUSCH.

Conclusions

In this contribution, we provide our considerations and evaluations on the Msg 3 PUSCH enhancement. The observations and proposals are as below.

Observation 1:

The 2 slot repetition could improve Msg 3 PUSCH coverage about 2.25dB. And the cross channel estimation could provide additional 1.75dB gain based on 2 slot repetition.

Proposal 1:

Cross slot channel estimation and DMRS enhancements related to repetition should be considered for the coverage enhancement of Msg 3 PUSCH.

References

- [1] #102-e RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e e-Meeting, August 17th – 28th, 2020

- [2] R1-2008024, Discussion on the PUSCH coverage enhancement 1, CMCC, RAN1#103e

Annex 1- Simulation assumptions