3GPP TSG RAN WG1 #103			R1-2008150

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.1.2.2

Source: 	Samsung

Title: 	Enhancements on multi-TRP inter-cell operation

Document for:	Discussion and Decision

# Introduction

From WID RP-193133 [1] Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework From RAN1 #102-e agreements R1-2007313 [2] Agreement Study the following aspects of QCL/TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation - Details on configuration of non-serving cell RS; - Allowed source and target RS types for RS transmitted from the non-serving cell TRP; - Allowed QCL types for RS transmitted from the non-serving cell TRP; - Measurement and reporting related to QCL/TCI enhancement except for that in 8.1.1, if any; - Clarification on potential UE behaviour for associating/multiplexing non-serving cell RS with other RS/channels; - Other details not precluded

In this contribution, we provide our views on the highlighted topic in the WID [1] and the agreed items from RAN1 #102-e on inter-cell operation for multi-TRP [2].

# QCL enhancements for inter-cell multi-TRP

Perhaps the most important enhancement over the Rel. 16 QCL/TCI framework for the inter-cell multi-TRP transmission pertains to the scenario when at least one coordinating TRP is not presented with a valid serving cell ID (SCI). Without a valid SCI, the SSBs from the non-serving-cell TRP cannot be used as the QCL source/reference as they cannot be inferred from the TCI-State/QCL-Info in the current TCI framework. Furthermore, how to efficiently measure/monitor the SSBs from the non-serving-cell TRP and make their information available to the UE may also need to be specified. Reusing the Rel. 16 TCI framework to indicate the SSBs from the serving-cell TRP as the QCL source for the TRS from the non-serving-cell TRP could be problematic because the two TRPs are not geographically co-located – not to mention that they could be loosely synchronized in both time and frequency. In Figure 1, examples of the inter-cell multi-TRP transmission under the non-CA framework are provided assuming two coordinating TRPs (serving-cell TRP-1 and non-serving-cell TRP-2).

Figure 1. Illustrative examples of inter-cell multi-TRP transmission

As illustrated on the LHS in Figure 1, the receive timing difference between the two TRPs could be larger than the CP especially in FR2 with short symbol duration and sharp TX beams. In this case, using the RSs from the serving-cell TRP-1 as the QCL source to determine the corresponding time-frequency related parameters such as those defined in QCL-TypeC to receive the downlink transmissions from the non-serving-cell TRP-2 would cause the RX timing offset larger than CP, resulting in significant inter-symbol interference. Note that, however, some level of time-frequency synchronization between the two coordinating TRPs, e.g., the receive timing difference between them is smaller than the CP, can be achieved via tight coordination between the TRPs. In this case, one or more SSBs from TRP-1 could still be used as the QCL source, e.g., QCL-TypeC, for the downlink transmissions from TRP-2. Tight/dynamic coordination across TRPs, however, would be challenging especially for non-ideal backhaul. The problem of reusing the Rel. 16 TCI framework for the inter-cell multi-TRP may be more significant for QCL-TypeD at FR2 as illustrated on the RHS in Figure 1. It is evident from Figure 1 that the SSBs from TRP-1 could not be used as the spatial QCL source for the TRS from TRP-2 for most of the cases because the two TRPs are typically not geographically co-located.

To alleviate the problems stated above, QCL framework needs to be enhanced to allow using SSBs from non-serving-cell TRP as both time/frequency and spatial QCL sources. Since those SSBs should be monitored/measured at UE side in prior to be utilized as QCL references, the details on monitoring/measurement procedure need to be specified.

Proposal 1: Support the use of SSBs from the non-serving-cell TRP as the QCL source/reference for the downlink transmissions from the non-serving-cell TRP depending on the QCL type

- The information of the SSBs from the non-serving-cell TRP need to be available at the UE, and their monitoring/measurement procedure also need to be specified.

- For QCL-typeD, the SSBs from the non-serving-cell TRP should be the only QCL source for the DL transmission, e.g., a TRS, from the non-serving-cell TRP.

- For other QCL types than QCL-typeD, the SSBs from the serving-cell TRP could be used as the QCL source for the DL transmission, e.g., a TRS, from the non-serving-cell TRP.

Figure 2. Illustrative example of (non-serving-cell) SSBs re-indexing

In this contribution, we propose a simple yet effective SSB re-indexing method to better enable the use of SSBs from the non-serving-cell TRP as the QCL source. In the proposed method, the SSBs from the non-serving-cell TRP are re-indexed based on the SSB information of the serving-cell TRP (e.g., the number of available SSBs from the serving-cell TRP). To indicate a SSB from the non-serving-cell TRP, the new SSB index (i.e., after applying the SSB re-indexing) is referred in the TCI-State/QCL-Info, instead of the original SSB index. In Figure 2, one illustrative example of the proposed SSB re-indexing strategy is provided. As can be seen from Figure 2, the serving-cell TRP-1 has SSBs {SSB-1 #0, SSB-1 #1, …, SSB-1 #N-1}, while the non-serving-cell TRP-2 has SSBs {SSB-2 #0, SSB-2 #1, …, SSB-2 #M-1}. After applying the SSB re-indexing, a single SSBs pool is obtained, containing {SSB #0, SSB #1, …, SSB #M+N-1}. In the SSBs pool, {SSB #0, SSB #1, …, SSB #N-1} correspond to the original {SSB-1 #0, SSB-1 #1, …, SSB-1 #N-1} from the serving-cell TRP-1, while {SSB #N, SSB #N+1, …, SSB #M+N-1} correspond to the original {SSB-2 #0, SSB-2 #1, …, SSB-2 #M-1} from the non-serving-cell TRP-2. For instance, if SSB #N is indicated in the TCI state, the UE would know that SSB-2 #0 from the non-serving-cell TRP-2 is actually indicated as the QCL source. It is evident from Figure 2 that by applying the proposed SSB re-indexing method, the UE could easily identify/differentiate the SSBs from the non-serving-cell TRP from those from the serving-cell TRP without relying on any additional information in the TCI state.

Proposal 2: Apply SSB re-indexing to the SSBs from the non-serving-cell TRP. If a SSB from the non-serving-cell TRP is used as the QCL source RS, its new index, i.e., after applying the SSB re-indexing over its original index, is indicated in the TCI state.

# Conclusion

In this contribution, we provide the following proposals regarding Rel. 17 QCL enhancements for inter-cell multi-TRP:

Proposal 1: Support the use of SSBs from the serving-cell TRP as the QCL source/reference for the downlink transmissions from the non-serving-cell TRP depending on the QCL type

- The information of the SSBs from the non-serving-cell TRP may need to be available at the UE, and their monitoring/measurement procedure may also need to be specified.

- For QCL-typeD, the SSBs from the non-serving-cell TRP should be the only QCL source for the DL transmission, e.g., a TRS, from the non-serving-cell TRP.

- For other QCL types than QCL-typeD, the SSBs from the serving-cell TRP could be used as the QCL source for the DL transmission, e.g., a TRS, from the non-serving-cell TRP.

Proposal 2: Apply SSB re-indexing to the SSBs from the non-serving-cell TRP. If a SSB from the non-serving-cell TRP is used as the QCL source RS, its new index, i.e., after applying the SSB re-indexing over its original index, is indicated in the TCI state.

# References

- 3GPP RAN, RP-193133, Samsung, New WID: further enhancements on MIMO for NR

- 3GPP RAN1 #102-e, R1-2007313, Moderator (vivo), Feature lead summary on enhancements on multi-TRP inter-cell operation