3GPP TSG RAN WG1 #103e		R1-2008211

e-Meeting, October 26th – November 13th, 2020

Source:	OPPO

Title:	Text proposals for enhancements on Multi-TRP and panel Transmission

Agenda Item:	7.2.6

Document for:	Discussion and Decision

Introduction

This contribution provides some TPs for multiple TRPs/panels based transmission.

Discussion

Overlapping between HARQ-ACK/PUSCH and CSI/SR

According to current 38.213 [2], if separate HARQ-ACK feedback is configured, PUCCH/PUSCH scheduled by PDCCH associated with a CORESETPoolIndex should not overlap with PUCCH/PUSCH scheduled by PDCCH associated with the other CORESETPoolIndex. For other uplink signal which is not associated with a CORESETPoolIndex, gNB should ensure that uplink signals for different TRPs are TDMed. If two uplink signals overlap in time, UE would multiplex them together based on Rel-15 mechanism. Furthermore, if a CSI is simultaneously overlapped with two PUCCHs within a slot with HARQ-ACK respectively for eMBB and URLLC, the CSI will be multiplexed with the HARQ-ACK with the same priority index firstly, and between the multiplexed information and the other HARQ-ACK, the one with lower priority index will be dropped. Also, if a UE would multiplex UCI in one of multiple PUSCHs, a priority is specified in 38.213 to determine which PUSCH is used to carry the UCI.

However, for M-TRP, if a PUCCH with P/SP CSI is simultaneously overlapped with two PUCCHs within a slot with separate HARQ-ACK respectively for two TRPs (which are associated with different values of CORESETPoolIndex), as shown in Figure 1, the UE behavior on how to multiplex the CSI with the two HARQ-ACKs is not defined, since no Rel-15 behavior can be followed, and not priority index is defined for PUCCH associated with different CORESETPoolIndex. This issue also occurs for other UL signal which is not scheduled by DCI (e.g. PUCCH with SR or type 1 configured grant PUSCH). Similar issue should also be solved for the case that a PUCCH with P/SP CSI/SR is simultaneously overlapped with two PUSCHs associated with different CORESETPoolIndex. A solution is needed otherwise the UE behavior is undefined at all.

Figure 1: Overlapping between two HARQ-ACKs/PUSCHs and CSI

The following three options can be considered for this issue, and option 2 or option 3 is preferred to avoid additional RRC signaling.

- Option 1: Define an association relationship between CORESETPoolindex and UL signal which is not scheduled by DCI. A UL signal will only be multiplexed with HARQ-ACK/PUSCH associated with the same CORESETPoolindex.

- Option 2: UE doesn’t expect two PUCCHs with separate HARQ-ACK or two PUSCHs within a slot associated with different values of CORESETPoolIndex to be simultaneously overlapped with another UL signal.

- Option 3: A UL signal is always multiplexed with the PUCCH/PUSCH associated with CORESETPoolIndex=0 if the UL signal is simultaneously overlapped two PUCCHs/PUSCHs associated with different values of CORESETPoolIndex.

We provide a TP for option 2 here.

Proposed TP1 for 38.213:

### 9.2.5 UE procedure for reporting multiple UCI types A UE that - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ackNackFeedbackMode-r16 = separate does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs to overlap in time with a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs, and does not expect a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the first CORESETs and a PUCCH or a PUSCH transmission triggered by a detection of a DCI format in a PDCCH received in a CORESET from the second CORESETs within the same slot to simultaneously overlap with another uplink signal.

Default close loop index for different CORESETPoolIndex

The UE features list for Rel-16 NR after RAN1#102 e-meeting[1] includes the feature group 16-2a-3 to support of out-of-order operation for PDCCH to PUSCH for multi-DCI based M-TRP transmission. To reduce the UE complexity for close loop power control, a note was added as below to introduce the restriction that same closed loop index for PUSCHs associated with different CORESETPoolIndex is not supported by a UE supporting UL out-of-order. That is, for a UE supporting this FG, different close loop indexes should be configured for PUSCHs associated with different CORESETPoolIndex.

| Features | Index | Feature group | Components | … | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
|  | 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH | … | Note: “Same closed loop index for power control across PUSCHs associated with different CORESETPoolIndex values is not supported by a UE indicating the support of this feature” | Optional with capability signalling |

However, according to current 38.213[2], if SRI-PUSCH-PowerControl is not provided by RRC or SRI is not included in a DCI, only single close loop index  should be assumed by UE for the PUSCH. Then for a UE indicating support of FG 16-2a-3, additional scheduling restriction will be introduced for the following cases:

- Once DCI format 0_0 is scheduled for CORESETPoolIndex=0 (l=0), the DCI format can’t be scheduled by CORESETPoolIndex=1, and only close loop index l=1 can be configured for PUSCH associated with CORESETPoolIndex=1.

- If UE reports capability of single SRS resource in the SRS resource set for codebook (maxNumberSRS-ResourcePerSet equal to 1), SRI will not be included in any UL grant. In this case, only PUSCH associated with one value of CORESETPoolIndex can be scheduled, which means multi-DCI based M-TRP transmission can’t be supported for uplink.

- In FR1, gNB is not likely to configure multiple SRS resources for beam selection in the SRS resource set for codebook. Without SRI in DCI, only PUSCH associated with one value of CORESETPoolIndex can be scheduled by gNB.

In a summary, for a UE supporting out-of-order operation for uplink, PUSCHs scheduled by different CORESETPoolIndex can hardly be supported especially in FR1, since the same default close loop index is defined for different CORESETPoolIndex. There is not such restriction for UEs not supporting this FG. The restriction makes the FG 16-2a-3 meaningless and support of it becomes a block to support PUSCH scheduled with different CORESETPoolIndex. To avoid such unreasonable restriction on scheduling, different default close loop indexes should be defined for different CORESETPoolIndex. That is, if SRI-PUSCH-PowerControl is not provided or SRI is not included in a DCI, close loop index l=0 and l=1 should be respectively applied to PUSCHs associated with CORESETPoolIndex=0 and CORESETPoolIndex=1.It also avoids the same close loop index for PUSCHs targeting different TRPs and allows TRP specific closed loop power control. The mechanism can be directly extended to PUCCH to avoid similar issue considering PUCCH-SpatialRelationInfo is optional.

Proposal: The default close loop index for PUSCH/PUCCH associated with CORESETPoolIndex=1 should be l=1.

Proposed TP2 for 38.213(PUSCH):

### 7.1.1 UE behaviour ### (omitted part) - if the UE is configured with twoPUSCH-PC-AdjustmentStates and if the UE is not configured with twoPUSCH-PC-AdjustmentStates or if the PUSCH transmission is scheduled by a RAR UL grant as described in Clause 8.3 - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, the value of is provided to the UE by powerControlLoopToUse - If the UE is provided SRI-PUSCH-PowerControl, the UE obtains a mapping between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and the value(s) provided by sri-PUSCH-ClosedLoopIndex and determines the value that is mapped to the SRI field value - If the PUSCH transmission is scheduled by a DCI format that does not include an SRI field, or if an SRI-PUSCH-PowerControl is not provided to the UE, l=1 if the PUSCH is scheduled by a PDCCH received in a CORESET which is configured with CORESETPoolIndex equal to 1 otherwise. - If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUSCH-RNTI, the value is provided by the closed loop indicator field in DCI format 2_2

Proposed TP3 for 38.213(PUCCH):

### 7.2.1 UE behaviour ### (omitted part) - is a TPC command value and is included in a DCI format 1_0 or DCI format 1_1 for active UL BWP of carrier of the primary cell that the UE detects for PUCCH transmission occasion or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 36.212], as described in Clause 11.3 - if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and if the UE is not provided twoPUCCH-PC-AdjustmentStates - If the UE is not provided PUCCH-SpatialRelationInfo, l=1 if the PUCCH is scheduled by a PDCCH received in a CORESET which is configured with CORESETPoolIndex equal to 1, l=0 otherwise. - If the UE obtains a TPC command value from a DCI format 1_0 or a DCI format 1_1 and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of through the link to a corresponding p0-PUCCH-Id index

Default pathloss RS for different CORESETPoolIndex

Similar to close loop index, if SRI-PUSCH-PowerControl is not provided to a UE or SRI is not included in a DCI, a default pathloss RS, the pathloss RS with PUSCH-PathlossReferenceRS-Id=0 configured by RRC, will be used for pathloss measurement of PUSCH. If multiple values of CORESETPoolIndex are configured, PUSCHs targeting different TRPs will share the same pathloss RS, which would lead to inaccurate pathloss estimation for PUSCH. Similar issue should also be considered for PUCCH associated with different CORESETPoolIndex when PUCCH-SpatialRelationInfo is not configured. Considering that a PUSCH without indication of SRI-PUSCH-PowerControl or SRI and a PUCCH without PUCCH-SpatialRelationInfo are common cases in FR1, we propose to support TRP specific pathloss RS for PUSCH and PUCCH.

Proposed TP4 for 38.213:

### 7.1.1 UE behaviour ### (omitted part) - If the PUSCH transmission is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP of each carrier and serving cell , as described in Clause 9.2.2, the UE uses the same RS resource index as for a PUCCH transmission in the PUCCH resource with the lowest index - If - the PUSCH transmission is scheduled by a DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or - the PUSCH transmission is scheduled by a DCI format 0_1 that does not include a SRI field, or - SRI-PUSCH-PowerControl is not provided to the UE, the UE determines a RS resource index with a respective PUSCH-PathlossReferenceRS-Id value being equal to one if the PUSCH is scheduled by a PDCCH received in a CORESET which is configured with CORESETPoolIndex equal to 1, and PUSCH-PathlossReferenceRS-Id value being equal to zero otherwise, where the RS resource is either on serving cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking - If - the PUSCH transmission is scheduled by a DCI format 0_0, - the UE is not provided PUCCH resources for the active UL BWP, and - the UE is provided enableDefaultBeamPlForPUSCH0_0 the UE determines a RS resource index providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the scheduling cell for the serving cell - If - the PUSCH transmission is scheduled by a DCI format 0_0, - the UE is not provided a spatial setting for PUCCH resources on the active UL BWP of the primary cell [11, TS 38.321], and - the UE is provided enableDefaultBeamPlForPUSCH0_0 the UE determines a RS resource index providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell - For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking - For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, the UE determines a RS resource index from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission. If the DCI format activating the PUSCH transmission does not include a SRI field, the UE determines a RS resource index with a respective PUSCH-PathlossReferenceRS-Id value being equal to one if the PUSCH is scheduled by a PDCCH received in a CORESET which is configured with CORESETPoolIndex equal to 1, and PUSCH-PathlossReferenceRS-Id value being equal to zero otherwise, where the RS resource is either on serving cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking If the UE is provided enablePLRSupdateForPUSCHSRS, a mapping between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id values can be updated by a MAC CE as described in [11, TS38.321] - For a PUSCH transmission scheduled by a DCI format that does not include a SRI field, or for a PUSCH transmission configured by ConfiguredGrantConfig and activated, as described in Clause 10.2, by a DCI format that does not include a SRI field, a RS resource index is determined from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 ### (omitted part) ### 7.2.1 UE behaviour ### (omitted part) - If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index on the active DL BWP of the serving cell - If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 1 in PUCCH-PathlossReferenceRS if the PUCCH is scheduled by a PDCCH received in a CORESET which is configured with CORESETPoolIndex equal to 1, and the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS otherwise, where the RS resource is either on a same serving cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

Conclusion

In this contribution, we provide some TPs for enhancement on single DCI based M-TRP transmission, multi-DCI based M-TRP transmission and URLLC related enhancement.

References

- R1-2007326，Updated RAN1 UE features list for Rel-16 NR including remaining RAN1 issues, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#102 e-meeting, September 2020.

- 38.213-g30, September 2020.

- 38.214-g30, September 2020.