3GPP TSG RAN WG1 #103-e		R1-2008239

e-Meeting, October 26th – November 13th, 2020

Source:	OPPO

Title:	Power saving mechanisms in NR sidelink

Agenda Item:	8.11.2.1

Document for:	Discussion and Decision

Introduction

In RAN#86 meeting, a new WID on NR Sidelink enhancement was agreed. This study is to enhance NR sidelink for the V2X, public safety and commercial use cases. The objectives of this study include the following part [1]

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following until RAN#90.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The study scope after RAN#90 is to be decided in RAN#90.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

  - Note: RAN2 work will start after [RAN#89].

3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2]

- Define on- and off-durations in sidelink and specify the corresponding UE procedure

- Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other

- Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

In this contribution, we will discuss the mechanisms of power saving for NR SL.

Discussion

  - Random selection and partial sensing

Power saving mechanism was discussed in LTE-V2X for P-UE. To reduce power consumption for P-UE, resource selection based on random selection or partial sensing was agreed. For random selection, there is no sensing or data reception at P-UE, which can reduce power consumption greatly. While random resource selection cannot avoid transmission collision effectively, which degrades system performance. Partial sensing can be taken as a trade-off between power consumption and transmission collision reduction. For partial sensing, UE can sense only part of the resource to determine the available transmission resource within resource selection window. All of the sensing mechanisms are re-used for partial sensing. The only difference is that the sensing range and candidate resource set within the resource selection window are scaled according to a ratio.

For NR-V2X, if power consumption needs to be considered in some cases, such as for commercial use case, and for handhold terminals, resource selection based on random selection and partial sensing can be taken as starting point.

Proposal 1: Random resource selection and partial sensing in LTE-V2X can be taken as starting point for power saving in NR SL.

If partial sensing is applied for NR SL, some enhancement should be studied considering the following aspects:

- Both periodic and aperiodic traffic are supported in NR SL

Partial sensing is more suitable for periodic traffic as UE can estimate when a packet will arrive and the location of corresponding resource selection window, and then the UE can sense X% resource per 100ms (the granularity of transmission periodicity) so that it can guarantee there is sensing result within the resource selection window. While for aperiodic traffic, UE cannot estimate the arriving time of data packet. When the data packet arrives, it is possible that there is no sensing result within the resource selection window. In that case, some enhancement should be studied.

- Short latency requirement of NR SL

In Rel-14 LTE-V2X, the latency requirement is about 100ms, which is reduced to 20ms or 50ms in Rel-15. While in NR SL, the latency is as short as several ms. It is hardly for the UE to obtain sensing results during the resource selection window.

Re-evaluation and pre-emption are supported in NR SL to avoid transmission collision. These mechanisms can be combined with partial sensing or random selection for power saving in NR SL. For example, UE does not do sensing before data packet arrives. In case the data packet arrives, UE can random select resource from resource selection window. Before the slot of the selected resource, UE can determine whether the resource can be used by means of re-evaluation or pre-emption. In that case, UE only need to do partial sensing from data arrival to actual transmission.

Proposal 2: It supports combining partial sensing/random selection and re-evaluation/pre-emption for power saving in NR SL.

  - SL DRX

One of the objectives of SL enhancement is to specify SL DRX mechanism. DRX is a method for power saving in Uu interface. The network configures DRX parameter to UEs. UE wakes up during DRX on-duration for data reception, and go to sleep if there is no data received during DRX on-duration. If SL DRX mechanism is specified for SL, it can also be one of the mechanisms for power saving.

Observation 1: SL DRX mechanism can be one of the mechanisms for power saving in NR SL.

Detailed SL DRX mechanism will be discussed in RAN2. Here we will discuss the relationship between SL DRX and resource selection. The SL DRX is for data reception. To guarantee the transmitter’s data can be received by the receiver, the selected resource should be within the DRX on-duration of the receiver. Otherwise, the RX UE may miss the data.

Figure 1

The TX UE should know RX UE’s DRX parameter so that it can select the resource within the RX UE’s DRX on duration. The DRX parameters should be exchanged between TX UE and RX UE.

Proposal 3: DRX parameters should be exchanged between TX UE and RX UE for power saving.

  - Wake-up signal/Go-to-sleep signal

Wake-up signal (WUS) was introduced in power saving topic in Rel-16. It can be combined with DRX to save power for terminals. Before starting the DRX on duration, gNB can send WUS or go-to-sleep (GTS) signal to UE to indicate whether UE needs to wake up for PDCCH reception. If GTS signal is received, UE will not wake up and keep sleeping, which can save power further.

WUS/GTS mechanism can be applied in sidelink, especially for unicast or groupcast. If there is no SL data to be transmitted from the TX UE, it can send GTS to RX UE to keep RX UE sleeping. If GTS is not received, or WUS is received by RX UE, it should wake up for SL reception.

Proposal 4: WUS or GTS mechanism can be applied to SL for power saving.

Figure 2

In NR Uu, the WUS/GTS is carried in DCI and fix resource is allocated for the DCI transmission so that UE can detect the DCI to determine whether needs to wake-up. While for NR SL, it is not suitable to put the WUS/GTS in SCI since UE needs to select resource for PSCCH/PSSCH transmission and the selected resource is not known by RX UE. RX UE has to do blind detection of the PSCCH/PSSCH which conflict with power saving mechanism.

Observation 2: SL WUS/GTS is not suitable to be carried in PSCCH or PSSCH.

Similar as PSFCH which is sequence based, the WUS/GTS signal can be carried in a sequence. In that case, the multiplexing between WUS/GTS with PSCCH/PSSCH can take the multiplexing between PSFCH and PSCCH/PSSCH as starting point. An illustration is shown in Figure 3.  The period of PSFCH is 4 slots. The associated PSFCH corresponding to PSSCH transmission in slot 0/1/2/3 is in slot 5. The RB set for PSFCH candidate resources is indicated by rbSetPSFCH. The rest PRBs with the same slot can be allocated to WUS/GTS. WUS/GTS sequence is FDMed with PSFCH in the slot.

Proposal 5:

  - WUS/GTS signal is carried in sequence

  - WUS/GTS signal is FDMed with PSFCH in the same slot.

  - The multiplexing between PSFCH with PSCCH/PSSCH can be taken as starting point to study the multiplexing between WUS/GTS and PSCCH/PSSCH.

Figure 3 Illustration of multiplexing between WUS/GTS sequence and PSCCH/PSSCH/PSFCH

  - Resource allocation for power saving

During study phase of R16 NR SL, mode 2d was proposed for resource allocation but not specified during WI phase. In mode 2d, UE1 can allocate resource for UE2, which is similar as mode 1 that gNB allocates resource for UEs. In that case, the UE2 can just follow the allocated resource for transmission and does not need to do sensing for resource selection which is beneficial for power saving.

Observation 3: Resource allocation mechanism Mode 2d is beneficial for power saving.

Proposal 6: Resource allocation mechanism Mode 2d should be considered in NR SL for power saving.

  - SL Power control

Open loop sidelink power control was supported in R16 NR SL for PSCCH and PSSCH. The TX power is determined by SL RSRP measurement. In R17 NR SL, close loop sidelink power control can be further studied for power saving.  Furthermore, the power control for PSFCH can also be studied.

Proposal 7: Close loop sidelink power control can be studied for power saving.

Conclusion

In this contribution, we discuss the mechanisms to reduce power consumption in NR SL. The following observations and proposals are given:

Observation 1: SL DRX mechanism can be one of the mechanisms for power saving in NR SL

Observation 2: SL WUS/GTS is not suitable to be carried in PSCCH or PSSCH.

Observation 3: Resource allocation mechanism Mode 2d is beneficial for power saving.

Proposal 1: Random resource selection and partial sensing in LTE-V2X can be taken as starting point for power sving in NR SL.

Proposal 2: It supports combining partial sensing/random selection and re-evaluation/pre-emption for power saving in NR SL.

Proposal 3: DRX parameters should be exchanged between TX UE and RX UE for power saving.

Proposal 4: WUS or GTS mechanism can be applied to SL for power saving.

Proposal 5:

  - WUS/GTS signal is carried in sequence

  - WUS/GTS signal is FDMed with PSFCH in the same slot.

  - The multiplexing between PSFCH with PSCCH/PSSCH can be taken as starting point to study the multiplexing between WUS/GTS and PSCCH/PSSCH.

Proposal 6: Resource allocation mechanism Mode 2d should be considered in NR SL for power saving.

Proposal 7: Close loop sidelink power control can be studied for power saving.

- Reference

[1] RP-201385 WID revision: NR sidelink enhancement, LG Electronics, RAN#88-e.