3GPP TSG RAN WG1 #103-e	R1- 2008299

Agenda item:		8.5.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Additional scenarios for evaluation of NR positioning

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#86 a new SI was approved on enhancements in Rel-17 to positioning [1]. This contribution discusses our initial views on the additional scenarios for evaluation. Our companion contributions discuss our views on evaluations and potential enhancements in [2], [3]. At RAN1#102-e the following agreements were reached:

Agreement #RAN1 #101-e::

Network efficiency and UE efficiency can be evaluated at least in an analytical manner.

- FFS: the definition of efficiency metric (e.g., the positioning performance (accuracy, latency) vs. PRS/SRS resource utilization etc.)

- Note: It will be up to each company on whether to use other methods (e.g., numerical simulation) for the evaluation.

Agreement #RAN1 #102-e:

PRS/SRS resource utilization is the metric used to evaluate network efficiency

- FFS: what is included in resource utilization, e.g. PRS/SRS/MG configurations, beam sweeping assumptions

In the UE-based definition of the start of the PHY layer latency the following were agreed as alternatives:

Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise,

- Alt. 1: transmission of the PUSCH carrying the MG Request from the UE.

- Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data

- Alt. 3: Start of the Reception of DL PRS

# Discussion

Good progress was made on the scenarios at RAN1#101-e and RAN1#102-e and in this contribution we focus on the small remaining details.

Positioning resource utilization

From RAN1#102e meeting, RAN1 has agreed to use resource utilization for network efficiency metric, however, there is no definition of positioning resource utilization yet. We think the definition can be different from the conventional network efficiency in data communication. Conceptually, the resource utilization is defined as a ratio of radio resources occupied for data/RS transmission over the total amount radio resources. We found some difficulties to apply the same concept to positioning resource utilizations.

One difficulty is found in counting PRS/SRS resources from neighbour cells. Positioning usecases includes the neighbour cell’s transmission resources for UE measurements. While radio resource in a serving cell for data communication can be easily counted, positioning case has a difficulty to identify the given radio resources in positioning usecase. Moreover, radio resources can be overlapped among different cells, there is ambiguity in counting positioning resources in a network.

Observation 1: Positioning resources are configured over multiple cells for most RAT-dependent methods. Positioning resource utilization needs to take into account multiple cells which may have overlapped positioning resources.

In addition, in the previous agreement it was already pointed out that beam sweeping assumptions, MG, and other considerations may complicate the resource utilization for positioning.

Observation 2: The definition for resource utilization needs to consider at least beam sweeping, MG, and other considerations.

With that in mind and taking into account that this is the last meeting of the SI, therefore even with a definition of resource utilization there is no time for detailed analysis, we make the following proposal. Note that if this metric becomes important in Rel-17 that this can be revisited in the WI phase as necessary.

Proposal 1: It is left to individual companies to define the resource utilization for positioning.

UE-based definition of start of the PHY layer latency :

From the last meeting, most of PHY latency definition has been agreed, but the start of UE based method remains with open options. The latency among different methods in the table below should be compared with fairness. If seeing “UE assisted methods” or “UL methods”, the start of the latency definitions is given as the moment when a service request is sent from a serving gNB. Therefore, it is reasonable to adopt a similar start time for the “UE-based method”. For example, assuming mobility situations, the UL methods requires latency that gNBs near a target UE receives NRPP measurement requests, so that it lets measurement gNB nodes get prepared. Similarly, for UE-based method, a target UE needs to be updated with assistance data from gNB under mobile situations, such latency should be fairly considered. Therefore, we think option-2 is a reasonable selection.

Agreement:   Physical Layer Latency Start and End times are defined as follows:

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives. | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

Proposal 2: For UE based methods, set the start of the PHY layer latency as the moment of transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data (Alt. 2 from prior agreement).

# Conclusion

In this contribution we make the following proposals and observations:

Observation 1: Positioning resources are configured over multiple cells for most RAT-dependent methods. Positioning resource utilization needs to take into account multiple cells which may have overlapped positioning resources.

Observation 2: The definition for resource utilization needs to consider at least beam sweeping, MG, and other considerations.

Proposal 1: It is left to individual companies to define the resource utilization for positioning.

Proposal 2: For UE based methods, set the start of the PHY layer latency as the moment of transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data (Alt. 2 from prior agreement).

# References

[1] RP-193237, New SID on NR Positioning Enhancements, Qualcomm.

[2] R1-2008300, Results on evaluation of achievable positioning accuracy and latency, Nokia, Nokia Shanghai Bell

[3] R1-2008301, Views on potential positioning enhancements, Nokia, Nokia Shanghai Bell