3GPP TSG RAN WG1 #103-e	R1- 2008300

Agenda item:		8.5.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 		Results on evaluation of achievable positioning accuracy and latency

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#86 a new SI was approved on enhancements in Rel-17 to positioning [1]. This contribution discusses our results on the evaluations of accuracy and latency. Our companion contributions discuss our views on additional scenarios and potential enhancements in [2], [3].

# Discussion

Good progress was made at RAN1#101-e to agree on simulation assumptions and scenarios. In this contribution those assumptions are followed unless specifically mentioned otherwise. Agreed assumptions from [4] which are used in the results presented here are copied to the Appendix for convenience.

## Accuracy evaluations

As an initial investigation into the accuracy of Rel-16 techniques performance in the IIoT use cases we simulated DL-TDOA using the agreed simulation assumptions from RAN1#101-e and the NR PRS design. We simulated both for InF-SH and InF-DH models which are presented in Figure 1. Additionally for a baseline we simulated the UMi and the Indoor Office (IOO) models from [5].

To produce the InF results we first model the RSTD measurements from all gNBs and then filter the measurements which are used by the positioning algorithm. For IIoT the RSTD measurements are made using Ts/4 level of granularity. For the other scenarios the RSTD measurements are made using Ts level of granularity. The filtering of RSTD measurements is performed using the quality of the links (e.g., PRS-RSRP) to select the 8 best gNBs. So this forms 7 RSTD measurements which are used by the positioning algorithm to estimate the UE location. For these results we use the Taylor Series expansion method to solve using the least squares method as defined in [6]. For the InF results only UEs that fall in the convex hull are considered.

Using the agreed parameter reporting shown below we produce the InF model results shown in this contribution.

| Parameter |  |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL : Comb-6, 6 symbol NR PRS UL : Comb-2, 2 symbol SRS-P |
| Reference signal (type of sequence, number of ports, …) | DL : NR PRS UL : NR SRS-P |
| Number of sites | 18 |
| Number of symbols used per slot per positioning estimate | 6 symbols per slot |
| Number of slots per positioning estimate | 1 slot |
| Power-boosting level | DL: 6 dB UL:fixed power (max) |
| Uplink power control (applied/not applied) | Not applicable |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Thresholding, 0.5, (Oversampling x4) |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Taylor Series, Least Squares |
| Network synchronization assumptions | Perfect Synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | N/A |
| Additional notes, if any | LoS detector and Outlier rejectin algorithm used in UL-TDOA results |

Figure 1. DL TDOA horizontal error for InF Scenarios

Figure 2. DL TDOA horizontal error for UMi and Indoor Office Scenarios

For the results in Figure 2 the IOO and UMi scenarios assumptions are taken from TR 38.855. The results from Figure 1 and 2 are used to derive the summary in Table 1 below.

Table 1. CDF Summary of Initial Results for DL TDOA for Horizontal Error

| Scenario, Fc, BW | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| InF-SH, 3.5 GHz, 100 MHz | 0.26 m | 0.38 m | 0.66 m | 1.65 m |
| InF-DH, 3.5 GHz, 100 MHz | 1.29 m | 2.04 m | 3.34 m | 4.99 m |
| IOO, 3.5 GHz, 100 MHz | 1.17 m | 1.92 m | 3.24 m | 6.50 m |
| UMi, 3.5 GHz, 100 MHz | 5.29 m | 9.59 m | 14.92 m | 23.81 m |

Observation 1: The performance of DL-TDOA is significantly worse in InF-DH compared with InF-SH. Meeting the strictest accuracy requirements for InF-DH may be challenging.

Observation 2: The performance of DL-TDOA is better in the InF-SH scenario compared with IOO.

Observation 3: The performance of DL-TDOA is significantly improved through the use of oversampling.

For UTDOA results, the positioning measurement and localization algorithms are similar as DL-TDOA cases. A UE dropped in convex hull transmit SRS-Pos with the max TX power. In the indoor performance observations, LOS channel indication is an important factor affecting high accuracy performance comparing to other factors. Detection or signalling on LOS/NLOS links can be useful. A method screening LOS profile is used for the simulation results to assist in LoS detection and outlier rejection. PHY can estimate channel impulse response for LOS identification, and a localization algorithm refers to such positioning measurements with priority.

Table 2. CDF Summary of Initial Results for UL TDOA for Horizontal Error [m]

| Scenario, Fc, BW | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- |
| InF-SH, 3.5 GHz, 100 MHz | 0.38 | 0.55 | 1.06 | 1.94 |
| InF-DH, 3.5 GHz, 100 MHz | 1.60 | 2.35 | 3.08 | 4.2 |

Observation 4: UL-TDOA results show similar performance trends to the DL-TDOA results for InF scenarios.

Observation 5: LoS detection and outlier rejection are able to improve the achieveable accuracy in UL-TDOA.

## PHY Latency evaluations

During RAN1#102-e the following agreement on latency evaluations was reached:

Agreement:

- At least the following information is provided for positioning physical layer latency analysis:

  - Source initiating request for positioning measurements/location for a given UE (UE, Network)

  - Destination awaiting for positioning measurements/location for a given UE (UE, Network)

  - Start and end triggers/events for physical layer latency evaluation

    - For Rel.16 solutions, it is based on specification for each solution

  - Initial and final RRC State of positioned UE (RRC IDLE, INACTIVE, CONNECTED) at the start and end time for the physical layer latency evaluation

  - Positioning

    - technique (enumeration): (1) DL-TDOA, (2) DL AoD, (3) UL-TDoA, (4) UL-AoA, (5) Multi-RTT, (6) E-CID

    - type: DL, UL, DL+UL

    - mode: UE-based, UE-assisted

  - Latency component w/ value range and description, including information on any parallel (simultaneous) components

  - Total latency value

- Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest one:

As one example we take the case of Network Initiated location request where the positioning method is DL-TDOA. Table 1 shows the analysis for DL-TDOA. Table 2 shows the analysis for UL-TDOA. Similar PHY latency is expected between DL-TDOA and DL-AoD. The same is assumed about UL-TDOA and UL-AoA.

Table 1:

| Source: NW/Destination: NW Positioning technique: DL-TDOA, type: DL, mode: UE-A Initial and Final RRC States: Initial/Final - CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message |
| Processing of PDSCH | [3-24 symbols] | UE processing – based on N1 values from 38.214 |
| Transmission of DL PRS | [4-10240 slots] | Worst case time between periodic DL PRS based on values from 38.211 |
| Processing of DL PRS at UE | [8-1280 ms] | Assume average value of T=8 (lowest value) is reported by UE as capability and UE is able to process all DL PRS resources (i.e., N is sufficient). |
| Transmit SR | [2 symbols – 1640 slots] | Wait for SR transmission occasion and transmit SR to gNB |
| Processing of SR at gNB | [1 symbol] | SR decoding |
| UL grant preparation and transmission of UL grant by gNB | [3 symbols ] | UL grant generation and waiting for scheduling opportunity (based on CORESET configuration). Typical value. |
| UE processing of PDCCH and UL preparation | [5-36 symbols] | Based on N2 values from 38.214. |
| Scheduling delay - K2 | [0-32 slots] | Scheduling delay from UL grant to PUSCH |
| gNB processing of RSTD Report | [1-2 symbol] |  |
| End trigger |  | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| Total PHY latency | [13.07 – 2956.71] ms | Assuming 15 kHz SCS |

For the analysis performed below we assume that the UE needs to transmit a SR in order to receive an UL grant. It is possible in some scenarios that the UE will already have an UL grant which it can use for the transmission of the RSTD report. In that case the latency may be lower than as shown above. However, this cause can not be counted on in our view as the gNB has no knowledge of the RSTD report upcoming (due to LPP transparency at gNB). In addition, the LMF does not know if UE has an active UL grant or not so for meeting the QoS requirements it should take into account the “worst” case of the UE needing to send the SR.

Observation 6: If 10 ms overall latency is required for positioning then the latency budget for PHY layer alone is already too large and the requirement would not be met.

Observation 7: The DL PRS processing time can be the larget contributor to the PHY layer latency and is independent of the SCS.

Observation 8: The DL PRS periodicity may have a large impact on the PHY layer latency.

Observation 9: Some components of the PHY latency (e.g., SR occasion) depend on gNB configuration of the UE.

Proposal 1: Solutions to enhance the PHY latency for positioning at least for DL methods will be investigated.

Table 2:

| Source: NW/Destination: NW Positioning technique: UL-TDOA, type: UL, mode: UE-A Initial and Final RRC States: Initial/Final - CONNECTED |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  | Reception by the gNB of the NRPPa measurement request message (assumption is that UE has already been configured with periodic SRS-Pos) |
| Wait for transmission of SRS-Pos | [1-81920 slots] | Periodicity of SRS-Pos |
| Transmission of SRS-Pos | [2 symbols- 8 symbols] | Assuming effective comb-1. In FR2 case this will be much longer (e.g., multiple resources per set) |
| Processing of SRS-Pos at gNB/RP-only | [1-5 slots] |  |
| Preparation of UL measurement report | [3 symbols ] | Typical value |
| End trigger |  | The transmission by the gNB of the NRPPa measurement response message |
| Total PHY latency | [2.35 – 81925] ms | Assuming 15 kHz SCS |

Observation 10: PHY latency in UL-TDOA is lower than DL-TDOA but relies on SRS-Pos already being configured (e.g., higher layer latency takes this into account, see RAN2 analysis).

Observation 11: PHY latency in UL-TDOA is also heavily dependent on SRS-Pos periodicity. Therefore, low latency will require high overhead unless considering non-periodic SRS which also had additional delay components.

# Conclusion

In this contribution we make the following proposals and observations:

Observation 1: The performance of DL-TDOA is significantly worse in InF-DH compared with InF-SH. Meeting the strictest accuracy requirements for InF-DH may be challenging.

Observation 2: The performance of DL-TDOA is better in the InF-SH scenario compared with IOO.

Observation 3: The performance of DL-TDOA is significantly improved through the use of oversampling.

Observation 4: UL-TDOA results show similar performance trends to the DL-TDOA results for InF scenarios.

Observation 5: LoS detection and outlier rejection are able to improve the achieveable accuracy in UL-TDOA.

Observation 6: If 10 ms overall latency is required for positioning then the latency budget for PHY layer alone is already too large and the requirement would not be met.

Observation 7: The DL PRS processing time can be the larget contributor to the PHY layer latency and is independent of the SCS.

Observation 8: The DL PRS periodicity may have a large impact on the PHY layer latency.

Observation 9: Some components of the PHY latency (e.g., SR occasion) depend on gNB configuration of the UE.

Proposal 1: Solutions to enhance the PHY latency for positioning at least for DL methods will be investigated.

Observation 10: PHY latency in UL-TDOA is lower than DL-TDOA but relies on SRS-Pos already being configured (e.g., higher layer latency takes this into account, see RAN2 analysis).

Observation 11: PHY latency in UL-TDOA is also heavily dependent on SRS-Pos periodicity. Therefore, low latency will require high overhead unless considering non-periodic SRS which also had additional delay components.

# Appendix – Simulation Assumptions

The layout of the basestations for the InF factor is as follows:

Agreed simulation assumptions for the InF models are here:

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m, (optional) 120x60 m InF-DH: (baseline) 120x60 m, (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): FFS |  |  |
| UE mobility |  | 3km/h (Optional): FFS |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): FFS |  |  |
| Clutter parameters: {density r, height hc,size d_clutter} |  | Low clutter density: {20%, 2m, 10m} High clutter density:FFS |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

# References

[1] RP-193237, New SID on NR Positioning Enhancements, Qualcomm.

[2] R1-2008299, Additional scenarios for evaluation of NR positioning, Nokia, Nokia Shanghai Bell.

[3] R1-2008301, Views on potential positioning enhancements, Nokia, Nokia Shanghai Bell.

[4] RAN1#101-e Chairmen’s Notes.

[5] TR 38.855

[6] “A Simple and Efficient Estimator for Hyperbolic Location”, Y. T. Chan and K. C. Ho.