3GPP TSG RAN WG1 Meeting #103-e 	R1-2008321

E-meeting, October 26th – November 13th, 2020

Agenda Item:	8.5.4

Source:	Huawei, HiSilicon

Title:	Evaluations on additional channel model features

Document for:	Discussion and Decision

Introduction

- In this contribution, we will present introducing additional channel model features for positioning, which is typical for indoor deployments.

Additional channel model features

Ground reflection

Ground reflection (GR) was introduced in section 7.6.8 of TR 38.901 [1], shown in Figure 1.

Figure 1 Illustration of explicit ground reflection

Additional features for the ground reflection are summarized as below

Angle of the reflecting path is based on virtual BS/UE and physical UE/BS

The delay of the reflecting path is based on the propagation length of the reflection.

The reflection coefficient for different polarizations (parallel and perpendicular) is determined by the complex relative permittivity and conductivity for a specific reflecting material.

Ground reflection will cause a reflecting path that is difficult to be separated from the main LOS path due to very close temporal delay and very close angle of arrival at least at gNB side to the main LOS path. The existence of ground reflection will degrade the positioning accuracy.

Wall reflection

In addition, explicit wall reflection can be considered with similar but modified structure compared to the ground reflection, via e.g. turning the perpendicular wall as the “ground” in the rotated LCS (Local coordinate system). Procedures of explicit wall reflection is shown as below.

In case the wall reflection shall be modelled explicitly, the procedure defined in 7.6.8 should be modified assuming the wall is “the ground” in the rotated local coordinate system given by the following coordinate transform matrix R. The exemplary LCS setup is shown in Figure 2 and Figure 3. Orientation of the Wall (perpendicular to the ground); R: ρ=Rρ' where ρ=xyz is the Cartesian representation in GCS, and ρ' is that in LCS; Comments (equivalent LCS transform using bearing α, downtilt β, and slant γ). ϕ; R=0sinϕcosϕ0-cosϕsinϕ100; R-1=RT α=ϕ+πβ=-π2γ=0. ϕ=0 (South wall facing north); R=0010-10100; α=ϕ+πβ=-π2γ=0. ϕ=π2 (East wall facing west); R=010001100; α=ϕ+πβ=-π2γ=0. ϕ=π (North wall facing south); R=00-1010100; α=ϕ+πβ=-π2γ=0. ϕ=3π2 (West wall facing east); R=0-1000-1100; α=ϕ+πβ=-π2γ=0. Figure 2 Illustration of LCS rotation to treat wall as “ground” to reuse ground reflection (ϕ∈-π2,π2) Figure 3 Illustration of LCS rotation to treat wall as “ground” to reuse ground reflection (ϕ∈π2,3π2)

Evaluation

Ground reflection

The cases evaluated are summarized in Table 1.

Table 1 Evaluated parameter combinations

| Parameter | Case 2 [2] (InF-SH, FR1) | Case 401 (InF-SH, FR1) | Case 402 (InF-SH, FR1) | Case 403 (InF-SH, FR1) |
| --- | --- | --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | InF-SH | InF-SH | In-SH | InF-SH |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) | PosSRS (Comb-4, 4 symbol) |
| Reference signal (type of sequence, number of ports, …) | ZC, single port | ZC, single port | ZC, single port | ZC, single port |
| Number of sites | 7 | 7 | 7 | 7 |
| Number of symbols used per slot per positioning estimate | 4 |  |  |  |
| Number of slots per positioning estimate | 1 |  |  |  |
| Power-boosting level | 6dB |  |  |  |
| Uplink power control (applied/not applied) | Not applied |  |  |  |
| interference modelling (ideal muting, or other) | Ideal |  |  |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Super resolution No LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection | Super resolution No LOS/NLOS detection | Super resolution Ideal LOS/NLOS detection |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO | UL-TDOA PSO |
| Network synchronization assumptions | Ideal |  |  |  |
| UE/gNB Tx/Rx Calibration Error | Ideal |  |  |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Tx beam sweeping |  |  |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Tx codebook-based |  |  |  |
| Additional notes, if any | w/o GR | w/o GR | w/ GR | w/ GR |

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 2.

Table 2 Positioning accuracy (in unit of meter)

| Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| 2, LOS/NLOS, w/o GR | 0.0621 | 0.1096 | 0.2410 | 1.0277 | 3.9456 |
| 401, Ideal LOS, w/o GR | 0.0475 | 0.0699 | 0.1079 | 0.1874 | 0.3597 |
| 402, LOS/NLOS, w/ GR | 0.1132 | 0.1678 | 0.2755 | 0.8458 | 3.5387 |
| 403, Ideal LOS, w/ GR | 0.1032 | 0.1379 | 0.1867 | 0.5402 | 5.623 |

Observation 1: The feature of ground reflection will have significant influence on the high accuracy positioning (case 401 0.18m@90% v.s. case 403 0.54m@90%), but less impact on medium accuracy (case 2 1m@90% v.s. case 402 0.84m@90%).

Observation 2: The ground reflection will pose a major thread to achieving <0.5m accuracy for indoor environment.

Wall reflection

Simulation assumption

Considering that too many BSs in the InF environment will cause extreme difficulty in evaluating the reflection, we use the following single-BS square to mimic the reflection around a single BS.

As shown in Figure 4, the physical anchor (PA) is the base station deployed in the centre of a room, whose size is given by ISD x ISD. The four virtual anchors (VA) correspond to the mirror images of the PA for the four respective walls.

Figure 4 Simulation layout for Single BS using wall reflection

The cases evaluated are summarized in Table 3.

Table 3 Evaluated parameter combinations

| Parameter | Case 411 | Case 412 |
| --- | --- | --- |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Joint TOA/AOA estimation for the LOS path | Joint TOA/AOA estimation VA-path association |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | RTT+AOA | Multi-RTT+UL-AoA PSO |
| UE/gNB Tx/Rx Calibration Error | T1 = 7ns ( = 1.4 * (1 + 4)) gNB Tx/Rx group delay error T1(gNB) = 1ns UE Tx/Rx group delay error T1(UE) = 4ns |  |
| Additional notes, if any | Only LOS path | LOS path + NLOS paths with VAs |

Other evaluation assumptions are summarized in Table 4.

Table 4 Evaluation assumptions for wall reflection

| Carrier frequency | 3.5GHz |
| --- | --- |
| Bandwidth | 100MHz |
| Subcarrier spacing | 30kHz |
| gNB Antenna config | (4,4,2,1,1) Single sector ARP Orientation: (0,0,0) |
| UE antenna config | (1,1,2,1,1) Omni-direction ARP Orientation: (alpha,0,0), alpha uniform in [0, pi] |
| ISD | 20m |
| Height | 6m |
| gNB location | Center top of the square room |
| UE distribution | (x,y,z) uniform in ([0,ISD], [0,ISD], [0,6]) |

Algorithm description

Joint TOA/AOA estimation

The delay difference between paths may not correspond to an integer number of samples, causing aliasing, which adds difficulty to identify each path only based on time processing. Joint TOA/AOA estimation will further utilize the spatial separation of paths so that a path is categorized by a pair of (TOA, AOA) in the spatial-temporal domain.

One way to do joint TOA/AOA estimation is 2-demensional (spatial-temporal) search using e.g. MUSIC, but the complexity may be overwhelming even for a gNB. Another alternative is using staged estimation for TOA and AOA iteratively.

In the simulation, we used the following staged TOA and AOA estimation, illustrated by the flow chart in Figure 5 with the following steps.

Step 0: Obtain the CIRs on each respective Rx.

Step 1: Use the CIR to extract the coarse path delay.

Step 2: Estimate the angle(s) for each path.

Step 3: Use the estimated angle as the Rx beamformer and coarse path delay for fine path delay estimation.

Figure 5 Joint TOA/AOA algorithm

VA-path association

The VA-path association is a procedure to determine if a path is generated by a virtual anchor, as in general, some paths are not so regular and thus may not have a corresponding VA. To perform the association attempt, a UE initial location estimate (not as accurate as the final location fix) is needed. Here we use the RTT+AOA based on the LOS path, which may have some bias due to the UE/gNB time calibration error and gNB angle calibration error, but the accuracy is sufficient for VA-path association validation.

The detailed VA-path association can be described as below for the two cases

The number of detected path is larger than or equal to the number of VAs

The number of detected path is smaller than the number of VAs.

For example, if the number of detected path is larger than or equal to the number of VAs, we use the following sequence to c1,…,cNVA to denote that VA i is associated with path ci. For example, the association in the following Figure 6 is represented by c1,…,cNVA=[2,4,1,5].

Figure 6 VA path association

The association optimization is represented by the following equation

c1*,…,cNVA*=argmaxc1,…,cNVA∈Ppc1,…,cNVAX=xLOS

Where

xLOS is the UE location estimate based on LOS path

P is the set containing all possible sequences and cardP=ANpNVA

- To analyze the VA-path association for the case when the number of detected path is smaller than the number of VAs is similar and thus omitted.

Localization

The localization algorithm is similar to the traditional hybrid Multi-RTT+UL-AoA methods, where the multiple TRPs comprise the PA and the VAs with associated paths, whose timing and angle will be fed into the location engine.

Evaluation results

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 5.

Table 5 Positioning accuracy (in unit of meter)

| Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- |
| 411, LOS path | 0.6530 | 0.9686 | 1.2727 | 1.7183 | 2.0250 |
| 412, LOS path + VA paths | 0.5893 | 0.8675 | 1.1595 | 1.5677 | 1.9466 |

Observation 3: Using additional reflecting path can improve the positioning accuracy for single BS positioning.

Summary

Based on the evaluation, we have the following proposal.

- Proposal 1: Ground reflection should be considered in the indoor positioning evaluation.

Proposal 2: Angle information reporting for multi-path should be supported for Rel-17.

Conclusion

In this contribution, we provided our evaluation results for additional channel modeling features, including ground reflection and wall reflection. Based on the evaluation, we have the following observations and proposals.

Observation 1: The feature of ground reflection will have significant influence on the high accuracy positioning (case 401 0.18m@90% v.s. case 403 0.54m@90%), but less impact on medium accuracy (case 2 1m@90% v.s. case 402 0.84m@90%).

Observation 2: The ground reflection will pose a major thread to achieving <0.5m accuracy for indoor environment.

Observation 3: Using additional reflecting path can improve the positioning accuracy for single BS positioning.

- Proposal 1: Ground reflection should be considered in the indoor positioning evaluation.

Proposal 2: Angle information reporting for multi-path should be supported for Rel-17.

- Reference

3TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

R1-2007576, Evaluation results for Rel-16 positioning and Rel-17 enhancement, Huawei, HiSilicon, RAN1#103-e, e-Meeting, 26th October – 11th November, 2020.