3GPP TSG-RAN WG1 Meeting #103-e	R1-2008325

# e-Meeting, October 26–November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the maximum number of CORESETs for Multi-DCI Transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 16-2a was agreed in RAN1 102e as following: 16-2a; Multi-DCI based multi-TRP; - The maximum number of CORESETs configured per BWP per cell in addition to CORESET 0 - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per BWP per cell in addition to CORESET 0 - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot; Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a CP Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex Component 1: Candidate values {2, 3,4,5} Component 2: Candidate values {1,2,3} Component 4: Candidate values {1,2,3,4,7} Note: per SCS, similar with Rel-15. Note: RAN1 will continue discussing how the network will interpret the signaled maximum number of CORESETs in components (1) and (2) (i.e., candidate value 5 for component (1) and candidate value 3 for component (2)) of FG 16-2a, e.g., when CORESET #0 is not configured When CORESET 0 is configured, the gNB can configure a number of UE specific CORESETs per BWP per cell and per TRP, according to reported values in component 1 and component 2 respectively, in addition to CORESET 0. When CORESET 0 is not configured, above reported value of components 1 and 2 are still valid certainly. Based on such an understanding, the description in 38.213 may be updated slightly to match with UE capability description correctly: - In Rel-15, total number of CORESETs per BWP is up to 3 including CORESET0 if configured. - In Rel-16, when CORSET 0 is configured, total number of CORESETs per BWP is up to 6 including CORESET0, e.g. if the UE reports 5 for component 1. When CORSET 0 is not configured, total number of CORESETs per BWP is up to 5. - In Rel-16, for the same TRP, i.e. with the same CORESETPoolIndex, when CORSET 0 is configured, total number of CORESETs per TRP is up to 4 including CORESET0, e.g. if the UE reports 3 for component 2. When CORSET 0 is not configured, total number of CORESETs per TRP is up to 3. Therefore some editorial changes for 38.213 are needed to avoid potential misunderstanding between gNB and UE over UE capability description. |
| Summary of change: |  | Clarify that for the same TRP, i.e. with the same CORESETPoolIndex, when CORSET 0 is configured, total number of CORESETs per TRP is up to 4 including CORESET 0. Clarify that when CORSET 0 is configured, total number of CORESETs per BWP is up to 6 including CORESET 0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interpretation of UE capability can be inconsistent between 38.213 and 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged part omitted>

For each DL BWP configured to a UE in a serving cell, the UE can be provided by higher layer signalling with

-	P≤3 CORESETs if CORESETPoolIndex is not provided

-	P≤3 CORESETs  if a value of CORESETPoolIndex is same for all CORESETs in addition to controlResourceSetZero

-	P≤5 CORESETs if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET in addition to controlResourceSetZero