3GPP TSG RAN WG1 #103-e 	R1-2008332

E-meeting, October 26–November 13, 2020

Agenda Item:	8.11.3

Source:	Huawei, HiSilicon

Title:	Physical layer impacts of sidelink DRX

Document for:	Discussion and Decision

Introduction

As described in the NR sidelink enhancement WID [1], there is an objective to introduce sidelink DRX, along with resource allocation enhancements:

2. Resource allocation enhancement: - Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after [RAN#89]. 3. Sidelink DRX for broadcast, groupcast, and unicast [RAN2] - Define on- and off-durations in sidelink and specify the corresponding UE procedure - Specify mechanism aiming to align sidelink DRX wake-up time among the UEs communicating with each other - Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE

The WID only mentions RAN2 scope. However, in this contribution, we discuss why it is clear that there are impacts of DRX in the physical layer due primarily to its consequences on sensing and related procedures.

Definition of sidelink DRX

In the Uu interface, a DRX cycle is the periodic repetition of the ON duration followed by a possible OFF duration. The MAC entity monitors the PDCCH during ON duration of a DRX cycle [2] [3].

Figure 1 DRX cycle in Uu interface

If basic UE behavior of Uu DRX is reused, sidelink DRX has repeated DRX cycles, and a DRX cycle has ON duration followed by a possible OFF duration. For power saving purposes, the UE should firstly monitor SCI transmissions (1st and 2nd stages) in ON durations of sidelink DRX.

Figure 2 DRX cycle in NR sidelink

Impacts of sidelink DRX in physical layer

Impacts of DRX in sidelink resource allocation

Figure 3 Sidelink resource selection procedure interactions with sidelink DRX

In NR Uu interface, the transmission of certain information during OFF durations is allowed, such as HARQ information [3]. In NR sidelink DRX, whether such allowance is also applicable during OFF duration should be considered carefully, and what kinds of information transmitted needs to be studied further. An evident impact may be on the resource allocation.

In Rel-16 NR sidelink, the mode 2 resource allocation mainly refers to full sensing. In Rel-17 WID, the principle of Rel-14 random resource selection and partial sensing will be introduced to mode 2 resource allocation. For partial sensing and full sensing, the UE needs to monitor SCI transmissions for resource (re)selection. However, we assume the UE is not allowed to monitor SCI transmissions in the OFF duration for sidelink DRX. When the UE is configured with sidelink DRX and uses partial or full sensing for resource (re)selection, the partial sensing slots or full sensing window may be in an OFF duration, and sensing results would be impacted. Accordingly, at least partial sensing and full sensing for sidelink DRX should be studied at physical layer. Whether there is an impact on random selection depends on the extent and nature of changes needed to adapt it from LTE-V2X to NR-V2X.

When UE is configured with DRX states and mode 2 resource allocation, the sensing window and ON duration in DRX may be non-overlapped, partially overlapped, or completely overlapped according to the resource selection trigger time, the length of sensing window, and the length of the ON duration in a DRX cycle. As shown in Figure 3, when the UE is triggered at slot n for resource (re)selection, the sensing window is [n –T0,n–Tproc,0SL) [3]. The UE excludes resources in the resource selection window according to decoded SCIs and SL-RSRP measurements in the sensing window. In this example, the sensing window and ON duration in DRX are partially overlapped. PSCCH1 is in ON duration while PSCCH2 is not. If the UE misses PSCCH2 due to DRX and corresponding resources are not excluded in the resource selection window, the selected transmission resources may collide with the resources reserved by PSCCH2. Thus the transmission of the UE would be impacted regardless of whether the transmission occurs during the ON duration or the OFF duration.

The situation of partial sensing with sidelink DRX is similar to full sensing with sidelink DRX. Based on the baseline of Rel-14 partial sensing, the monitored slots can be a set of discontinuous slots. The partial sensing occasions and ON duration in sidelink DRX may be non-overlapped, partially overlapped, or completely overlapped. For the missed sensing occasion(s) within an OFF duration, the UE cannot recognize the related reserved resources, which may lead to resource collision if only sensing results are used for resource selection. As shown in the given Figure 4 below, a slot which should be monitored is in the OFF duration, thus the UE cannot know whether there is an SCI in the slot or not. This may lead to resource collision.

Figure 4 Partial sensing procedure interactions with sidelink DRX

Observation 1: The full sensing window/partial sensing occasions and OFF duration in sidelink DRX can be non-overlapped, partially overlapped, or completely overlapped, and when they are overlapped, the sensing results derived for resource (re)selection become less accurate.

The sensing results are less accurate compared with a non-DRX UE which has all sensing results in the partial sensing occasions or full sensing window. The resource collision probability would increase if the UE determines resources only according to sensing results in DRX ON duration. RAN1 should consider how to enhance partial sensing and full sensing procedures so they will work well under sidelink DRX, to guarantee the reliability, and reduce the transmission resource collision probability.

When a UE is configured with DRX states in mode 1 resource allocation, the UE transmits sidelink SR to request resources for PSCCH and PSSCH transmission. The scheduled sidelink resources and ON duration of RX UE can be non-overlapped, partially overlapped, or completely overlapped. As shown in Figure 5, the scheduled sidelink resources and ON duration of RX UE are partially overlapped. If the DRX cycle of RX UE is unavailable to the gNB, the transmission in DRX OFF duration would not be accepted by the UE, and the scheduling is wasted. The scheduling of resources by gNB in mode 1 should be further studied in SL DRX mode.

Figure 5 Mode 1 interactions with sidelink DRX

Proposal 1: RAN1 needs to consider how to enhance the sidelink resource allocation schemes considering the impacts of sidelink DRX.

Impacts on physical layer procedure

As we analyzed above, the UE monitors for SCIs in the ON duration of sidelink DRX [3]. Then reception of the UE would be restricted by DRX operation. In LTE Uu interface, UL HARQ information could be not restricted by DRX state and a UE can keep receiving the HARQ information even when in the OFF duration [2]. That is because LTE UL HARQ feedback is synchronous, and the UE receives the information in a dedicated resource without decoding control information. In NR sidelink DRX, the UE would not monitor for SCIs in OFF duration, but whether the UE can receive certain other transmissions during an OFF duration should be considered. For example, the received PSCCH and PSSCH may imply a PSFCH reception in the OFF duration, or a periodic reception due to semi-persistent configuration is scheduled for the UE. Similar as LTE UL HARQ feedback, if the UE receives them, the UE does not also need to monitor control information during the OFF duration, saving power by the same reason as not monitoring PDCCH during a Uu DRX cycle. If the UE is not allowed to receive in an OFF duration, the latency and reliability would be affected. Thus, whether the reception of certain information, e.g. SL HARQ information, is allowed during OFF duration without monitoring for SCIs should be studied.

In LTE and NR Uu interface, the transmission of feedback information including HARQ information and CSI report was specified, in order for further power consumption during Uu DRX OFF duration. Accordingly whether the transmission behavior has any restriction during NR sidelink DRX OFF duration should be studied.

Proposal 2: RAN1 should further study the sidelink reception/transmission behavior during sidelink DRX OFF duration.

Conclusions

In this contribution, we show that there will be impacts in the physical layer due to the introduction of sidelink DRX in Rel-17. Based on above discussions, the following proposals are given, to kick-off RAN1 discussions.

Observation 1: The full sensing window/partial sensing occasions and OFF duration in sidelink DRX can be non-overlapped, partially overlapped, or completely overlapped, and when they are overlapped, the sensing results derived for resource (re)selection become less accurate.

Proposal 1: RAN1 needs to consider how to enhance the sidelink resource allocation schemes considering the impacts of sidelink DRX.

Proposal 2: RAN1 should further study the sidelink reception/transmission behavior during sidelink DRX OFF duration.

- References

- RP-201385 “WID revision: NR sidelink enhancement”, LG Electronics, RAN#88e, June 29 – July 3, 2020.

- 3GPP TS 36.321 “EUTRA; Medium Access Control (MAC) protocol specification”.

- 3GPP TS 38.321 “NR; Medium Access Control (MAC) protocol specification”.

- 3GPP TS 38.214 “Physical layer procedures for data”.