3GPP TSG RAN WG1 Meeting #103-e	R1-2008339

# 26 October – 13 November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | xxxx | rev | - | Current version: | V15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on preamble format indicator presence in NPDCCH order in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh2-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is currently specified that preamble format indicator is only present in DCI format N1 if SIB2-NB or SIB23-NB is configured. If UE signals nprach-Format2 capability, it will always assume the preamble format indicator field is present, because SIB2-NB is always present even when it does not contain any relevant configuration for NPRACH format 2. However, a pre-Rel-15 eNB will not include the field in DCI. As a result, the eNB and UE have different understanding on the DCI contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Presence of the preamble format indicator field is conditioned on the presence of nprach-ParametersListFmt2 (and UE indication of the capability), instead of SIB2-NB or SIB23-NB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE indicating support of NPRACH format 2 will assume presence of the preamble format indicator field in DCI format N1 as SIB2-NB is present, while a pre-Rel-R15 eNB will not include the DCI field. As a result, the UE cannot correctly receive contents of PDCCH order. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 16.3.2	Timing

For the L1 random access procedure, UE's uplink transmission timing after a random access preamble transmission is as follows.

a)	If a NPDCCH with associated RA-RNTI is detected and the corresponding DL-SCH transport block ending in subframe n contains a response to the transmitted preamble sequence, the UE shall, according to the information in the response, transmit an UL-SCH transport block according to Subclause 16.3.3.

b)	If a random access response is received and the corresponding DL-SCH transport block ending in subframe n does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

c)	If no NPDCCH scheduling random access response is received in subframe n, where subframe n is the last subframe of the random access response window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

d)	If an NPDCCH scheduling random access response with associated RA-RNTI is detected and the corresponding DL-SCH transport block reception ending in subframe n cannot be successfully decoded, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe , , where a NPRACH resource is available.

The "PDCCH order" in DCI format N1 indicates to the UE,

-	allocated subcarrier for NPRACH, where  is the subcarrier indication field in the corresponding DCI, is reserved for preamble format 0/1, is reserved for preamble format 2 if nprach-ParametersListFmt2 is configured and the UE indicates the nprach-Format2 capability and Preamble format indicator is set to 1.

-	a repetition number () for NPRACH determined by the repetition number field () in the corresponding DCI according to Table 16.3.2-1 where R1, R2 (if any) and R3 (if any) are given by the higher layer parameter numRepetitionsPerPreambleAttempt for each NPRACH resource, respectively. R1 < R2 <R3.

Table 16.3.2-1: Number of repetitions () for NPRACH following a "PDCCH order"

| 0 | R1 |
| --- | --- |
| 1 | R2 |
| 2 | R3 |
| 3 | Reserved |

The UE shall transmit random access preamble on the NB-IoT carrier indicated by "Carrier indication of NPRACH" field, if the field is present in the "PDCCH order". If the value of "Carrier indication of NPRACH" is non-zero, it indicates a NPRACH carrier derived from SystemInformationBlockType22-NB [11] for which the index in the list is equal to the carrier indication. If the value of "Carrier indication of NPRACH" is zero, the uplink carrier used for NPRACH is derived from SystemInformationBlockType2-NB [11].

If nprach-ParametersListFmt2 is configured and the UE indicates the nprach-Format2 capability, the UE shall transmit the preamble format indicated by "Preamble format indicator" field, otherwise the UE shall transmit preamble format 0/1.

#### < Unchanged parts are omitted >