3GPP TSG RAN WG1 #103-e		R1-2008355

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.3.1.1

Source: 	Sony

Title:	Considerations in HARQ-ACK enhancements for URLLC

Document for:	Discussion / decision

# Introduction

In RAN1#102-e we agreed the following:

Agreements:

Support Rel-17 enhancements to avoid SPS HARQ-ACK dropping for TDD due to PUCCH collision with at least one DL or flexible symbol.

- This topic is to be considered as high priority

- FFS detailed solution(s)

Agreements:

- Simultaneous PUSCH / PUCCH within a cell group (of Sec. 6.13 of R1-2007216) and enhanced (sub-slot) HARQ-ACK multiplexing on PUSCH (of Sec. 4.3 of R1-2007216) can be further discussed as part of AI 8.3.3 in this WI (but not as part of AI 8.3.1.1).

Agreements:

Study further at least the following schemes:

- SPS HARQ skipping for ‘skipped’ SPS PDSCH

- PUCCH repetition enhancements (at least for HARQ-ACK), e.g., sub-slot based, etc.

- Retransmission of cancelled HARQ

- SPS HARQ payload size reduction and / or skipping for ‘non-skipped’SPS PDSCH

- Type 1 HARQ codebook based on sub-slot PUCCH config

- PUCCH carrier switching for HARQ feedback

Companies are encouraged to provide detailed analysis and comparison accordingly

This contribution discusses some considerations on HARQ-ACK enhancements for URLLC.

# Discussions

The following sub-sections discuss aspects of some of the HARQ-ACK enhancements proposed in RAN1#102e [1].

## SPS HARQ-ACK Dropping

Since the K1 value for PDSCH to PUCCH offset is indicated in the activation DCI for a SPS, the K1 value is fixed for all instances of this SPS until it is deactivated and reactivated with a different K1 value.  This may lead to more frequent collision of PUCCH carrying SPS HARQ-ACK with DL or invalid symbols in TDD operation.  One of the proposals is to transmit the dropped SPS HARQ-ACK in the first available PUCCH [1].  However, it is noted that the dropped SPS HARQ-ACKs may accumulate and thereby impact the reliability of the first available PUCCH [2].  An example is shown in Figure 1, where SPS#1 and SPS#2 are activated with K1 values of 1 and 2 slots respectively.  The HARQ-ACKs for the first instances of SPS#1 (Slot n) and SPS#2 (Slot n+1), that are carried by PUCCH P#1 and P#2 respectively are dropped.  These dropped SPS HARQ-ACKs are to be transmitted in the first available PUCCH which is P#3 in this example.  However, P#3 is already scheduled to carry HARQ-ACKs for the second instances of SPS#1 (Slot n+4) and SPS#2 (Slot n+3), i.e. P#3 would need to carry twice the number of SPS HARQ-ACKs since the dropped HARQ-ACKs in P#1 and P#2 are accumulated into P#3.

Figure 1: Accumulation of dropped SPS HARQ-ACK

One way to ensure the reliability of the first available PUCCH so that it is not overloaded with dropped SPS HARQ-ACKs that would impact its reliability is to impose a limit on the number of dropped SPS HARQ-ACK NHARQ that it can be multiplexed into it, where the value NHARQ can be RRC configured.  These NHARQ SPS HARQ-ACKs can come from the latest SPS with dropped HARQ-ACKs prior to the first available PUCCH.  An example is shown in Figure 2, where the HARQ-ACK for SPS#1 in Slot n, SPS#3 in Slot n and SPS#2 in Slot n+1 are dropped due to collision with DL and invalid symbols.  Here, NHARQ = 2 and so the first available PUCCH P#4, which is associated with SPS#3 in Slot n+4, is allowed to carry 2 HARQ-ACKs that are dropped in addition to the HARQ-ACK for SPS#3 in Slot n+4 (labelled as A3-2).  The selection of dropped HARQ-ACK is according to the arrival of the SPS instances where the latest SPS instances are selected and in this example the latest two SPS instances that have dropped HARQ-ACKs are SPS#3 in Slot n and SPS#2 in Slot n+1, where their corresponding HARQ-ACKs are labelled as A3-1 and A2-1 respectively.  Hence P#4 carries HARQ-ACK {A2-1, A3-1, A3-2}.

Figure 2: 1st available PUCCH multiplexes at most NHARQ=2 dropped SPS HARQ-ACKs

Proposal 1: Up to NHARQ of dropped SPS HARQ-ACKs due to collision with DL symbols or invalid symbols in TDD can be retransmitted by multiplexing into the first available PUCCH resource.  Value of NHARQ is FFS.

## SPS HARQ skipping for ‘skipped’ SPS PDSCH

In Rel-16 SPS, an empty SPS, i.e. a ‘skipped’ SPS PDSCH, requires the UE to feedback a NACK. Due to the need to over configure the number of SPS required to support certain traffic such as TSN, where the periodicity is not directly configurable in RRC, it is argued that for such traffic, there will be a lot of ‘skipped’ SPS PDSCH.  Consequently, there will be a lot of NACK being fed back which will waste PUCCH resource.  Hence it was proposed by some companies [1] that NACK is not transmitted for ‘skipped’ SPS PDSCH to reduce PUCCH payload size.

It would be beneficial for the UE to avoid the need to perform PDSCH detection if the UE knows beforehand that a SPS PDSCH is skipped.  One way is to introduce an RRC configuration to indicate which SPS is skipped but this will reduce the flexibility for the gNB to use the SPS.  An alternative is to dynamically indicate a ‘skipped’ SPS using MAC CE in a transmitted SPS PDSCH(s).  For example, an SPS PDSCH can indicate dynamically which SPS’s are skipped.

Proposal 2: If SPS HARQ skipping is supported, consider using MAC CE in a transmitted SPS PDSCH to indicate dynamically which SPSs are skipped.

## Sub-slot PUCCH repetitions

Sub-slot PUCCH repetition is proposed by some companies [1] to improve the reliability of PUCCH carrying HARQ-ACK.  In Rel-16, sub-slot PUCCH is not allowed to cross a sub-slot boundary since it would collide with another sub-slot PUCCH, thereby complicating the intra-UE collision management.  Hence, the same argument applies for sub-slot PUCCH repetitions.

Observation 1: Sub-slot PUCCH repetitions would lead to intra-UE PUCCH collision where PUCCH repetitions in a sub-slot collide with another PUCCH in another sub-slot.

Consider the scenario in Figure 3, where DCI#1 schedules sub-slot based PUCCH P#1 with 4× repetitions, starting at time t7 (sub-slot m+5), to carry HARQ-ACK for PDSCH#1.  P#1 repetitions ends at time t13 in sub-slot m+8.  At time t7 (sub-slot m+5), DCI#2 schedules sub-slot based PUCCH P#2 at time t12 (sub-slot m+8) to carry HARQ-ACK for PDSCH#2.  Here P#2 collides with the 4th (i.e. last) repetition of P#1.  In Rel-16, we introduced L1 priority to manage PUCCH collisions however, in this case we have the following to consider:

- If P#1 and P#2 are High L1 priority, the UCIs are to be multiplexed but here this is not feasible since the 1st P#1 repetition has already started.

- If P#1 is High L1 priority but P#2 is Low L1 priority, then using the Rel-16 rule, P#2 is dropped.  However, in this case, P#1 already had 3 repetitions which are highly likely to have already been received at the gNB whilst P#2 never had a chance to transmit yet.  Dropping P#2 is not efficient as it may result in PDSCH#2 being retransmitted regardless of whether it is decoded or not.

Figure 3: Collision with PUCCH repetitions

Observation 2: The 2 level L1 priority introduced in Rel-16 for UL intra-UE prioritization is not sufficient to handle inter sub-slot PUCCH collisions.

For PUCCH with repetitions, the 1st repetition is important for latency purposes and also has a high probability of being received at the gNB.  Subsequent repetitions are to improve reliability and hence would have lower importance compared to the 1st repetition.  Hence, one way to handle the scenario described in Figure 3 is to assign a lower priority for subsequent repetitions.  For example, each subsequent repetition has lower priority compared to the previous one of the same PUCCH.

Observation 3: The 1st PUCCH repetition has the highest importance compared to subsequent repetitions of the same PUCCH.

Proposal 3: If sub-slot PUCCH repetition is introduced, consider reducing the priority of a repetition according to the number of repetitions that had already been transmitted.

## Retransmission of cancelled HARQ

A HARQ-ACK carried by a PUCCH or PUSCH can be cancelled due to intra-UE or inter-UE prioritization.  A few companies proposed to allow the cancelled HARQ-ACK to be retransmitted [1].  We see benefit for this feature as cancelled HARQ-ACKs may lead to unnecessary retransmissions of PDSCHs.  It was not clear in the previous meeting (RAN1#102e) whether this feature was intended for Low L1 priority HARQ-ACK only or it also includes High L1 priority HARQ-ACK.  Since Low L1 priority HARQ-ACK is more likely to be cancelled, this feature is more suitable for Low L1 priority HARQ-ACKs.  However, the mechanism introduced should be applicable for cancelled High L1 priority HARQ-ACKs.

Proposal 4: Consider retransmission of cancelled Low L1 priority and High L1 priority HARQ-ACKs.

Most companies proposed to use NR-U based HARQ-ACK codebooks, e.g. e-Type 2 and Type 3, for this issue, since they were introduced to handle retransmission of cancelled HARQ-ACKs due to LBT.  This can be used as the starting point for handling of retransmission of cancelled HARQ-ACKs for URLLC in a licensed band.  However, in URLLC, the UE can be configured with two HARQ-ACK codebooks and hence we need to consider whether a Low L1 priority HARQ-ACK in one codebook can be retransmitted in a High L1 priority codebook and vice versa.  We also need to consider whether we can have a mixture of NR-U based codebooks with non-NR-U codebooks, e.g. a HARQ-ACK from a Type2 codebook being retransmitted in a PUCCH associated with a e-Type 2 or Type 3 codebook.

Proposal 5: Consider using e-Type 2 and/or Type 3 HARQ-ACK codebooks as a starting point in designing the mechanism to handle retransmission of cancelled HARQ-ACKs.

Proposal 6: Consider handling of retransmissions of cancelled HARQ-ACK of different L1 priority and/or different codebook type in another HARQ-ACK codebook of different L1 priority and/or different codebook type.

## SPS HARQ payload size reduction and/or skipping for ‘non-skipped’ SPS PDSCH

Some companies proposed that the SPS HARQ-ACK payload size can be reduced, for example:

- Skipping ‘ACK’: [4], [5], [6]

- Skipping ‘NACK’: [4]

- Bundling of multiple SPS feedbacks into one HARQ-ACK [7]

- Allow RRC to turn off HARQ-ACK feedback for SPS [8]

Retransmission of PDSCH is useful for increased reliability with efficient use of resource especially for very low BLER targets.  For example, a very low BLER of 10-6 can be achieved by allowing 2 transmissions of higher BLER each so that the combined BLER meets the target.  Here it is expected that the 1st transmission has a high chance of being received and the gNB only needs to allocate additional resources for a small number a retransmissions, which is a more efficient use of resource compared to using a lot of resource to ensure the very low BLER target.  Turning off HARQ-ACK or skipping ‘ACK’ or ‘NACK’ would have consequences to the operation of HARQ-ACK and thereby affects the efficiency of the resource usage.  That is, trying to save payload for HARQ-ACK may result in worse use of resources elsewhere.  Furthermore, the savings in payload size are unlikely to be high considering especially if a PUCCH needs to be transmitted anyway.  Hence, we do not see strong benefit to support this feature.

Proposal 7: SPS HARQ payload size reduction and/or skipping for ‘non-skipped’ SPS PDSCH are not supported in Rel-17.

# Conclusion

In this contribution, we discuss some features proposed for HARQ-ACK enhancements.  We observe the following:

Observation 1: Sub-slot PUCCH repetitions would lead to intra-UE PUCCH collision where PUCCH repetitions in a sub-slot collide with another PUCCH in another sub-slot.

Observation 2: The 2 level L1 priority introduced in Rel-16 for UL intra-UE prioritization is not sufficient to handle inter sub-slot PUCCH collisions.

Observation 3: The 1st PUCCH repetition has the highest importance compared to subsequent repetitions of the same PUCCH.

We therefore propose the following:

Proposal 1: Up to NHARQ of dropped SPS HARQ-ACKs due to collision with DL symbols or invalid symbols in TDD can be retransmitted by multiplexing into the first available PUCCH resource.  Value of NHARQ is FFS.

Proposal 2: If SPS HARQ skipping is supported, consider using MAC CE in a transmitted SPS PDSCH to indicate dynamically which SPSs are skipped.

Proposal 3: If sub-slot PUCCH repetition is introduced, consider reducing the priority of a repetition according to the number of repetitions that had already been transmitted.

Proposal 4: Consider retransmission of cancelled Low L1 priority and High L1 priority HARQ-ACKs.

Proposal 5: Consider using e-Type 2 and/or Type 3 HARQ-ACK codebooks as a starting point in designing the mechanism to handle retransmission of cancelled HARQ-ACKs.

Proposal 6: Consider handling of retransmissions of cancelled HARQ-ACK of different L1 priority and/or different codebook type in another HARQ-ACK codebook of different L1 priority and/or different codebook type.

Proposal 7: SPS HARQ payload size reduction and/or skipping for ‘non-skipped’ SPS PDSCH are not supported in Rel-17.

# References

[1] R1-2007354, “Feature lead summary #4 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT (AI 8.3.1.1),” Moderator (Nokia), RAN1#102e

[2] R1-2005431, “Discussion on HARQ-ACK enhancements for eURLLC,” ZTE, RAN1#102e

[3] R1-2006799, “HARQ-ACK enhancement for IOT and URLLC,” Qualcomm Incorporated, RAN1#102e

[4] R1-2005243, “UE feedback enhancements for HARQ-ACK,” Huawei, HiSilicon, RAN1#102e

[5] R1-2005967, “UE feedback enhancements for HARQ-ACK,” TCL Communication Ltd., RAN1#102e

[6] R1-2006314, “Discussion on UE feedback enhancement for HARQ-ACK,” LG Electronics, RAN1#102e

[7] R1-2006058, “HARQ-ACK enhancements for Rel-17 URLLC/IIoT,” OPPO, RAN1#102e

[8] R1-2006339, “On the necessity and support of Rel-17 URLLC HARQ-ACK feedback enhancements,” Nokia, Nokia Shanghai Bell