3GPP TSG RAN WG1 #103-e		R1-2008364

e-Meeting, October 26th – November 13th, 2020

Agenda Item: 	8.5.2

Source:	Sony

Title:	Discussion on performance evaluation of Rel-17 positioning

Document for:	Discussion and Decision

# Introduction

# A study item on NR positioning enhancements in Rel-17 was approved [1]. One of the objectives is to study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases:

- Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

- Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

- Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

In RAN1 #101e meeting, the following agreements related to positioning requirements were made [2]:

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

In RAN1 #102e meeting, some of the agreements are as follows [3]:

Agreement:

Capture the following in TR as an observation:

- Performance analysis of baseline I-IoT InF scenarios shows that InF-SH scenario is characterized by high probability of LOS links. In InF-DH the probability of LOS links is reduced substantially while probability of NLOS links is increased accordingly.

Conclusion:

- Evaluations show that high probability of NLOS links and propagation delay offset imposed by NLOS links may cause performance degradation of positioning accuracy, that was especially observed in InF-DH scenario

Agreement:

- At least the following information is provided for positioning physical layer latency analysis:

  - Source initiating request for positioning measurements/location for a given UE (UE, Network)

  - Destination awaiting for positioning measurements/location for a given UE (UE, Network)

  - Start and end triggers/events for physical layer latency evaluation

    - For Rel.16 solutions, it is based on specification for each solution

  - Initial and final RRC State of positioned UE (RRC IDLE, INACTIVE, CONNECTED) at the start and end time for the physical layer latency evaluation

  - Positioning

    - technique (enumeration): (1) DL-TDOA, (2) DL AoD, (3) UL-TDoA, (4) UL-AoA, (5) Multi-RTT, (6) E-CID

    - type: DL, UL, DL+UL

    - mode: UE-based, UE-assisted

  - Latency component w/ value range and description, including information on any parallel (simultaneous) components

  - Total latency value

In this contribution, we provide our simulation results on positioning accuracy based on the agreed simulation scenarios and parameters in the previous meetings and our view on achieving stringent positioning latency for IIoT use-cases. The simulation results and latency analysis are presented according to the agreed format/template in RAN1#102e meeting.

# Performance Evaluation

## Evaluation of Positioning Accuracy

In this contribution, we provide the following simulation results:

- Positioning accuracy using Rel-16 techniques

- Positioning accuracy using potential enhancements in Rel-17

- Horizontal and Vertical accuracy

- UE dropping with convex hull and all areas

- Impact of the gNB height on the vertical positioning accuracy

- Impact of BW size.

The details of general simulation assumption can be found in Appendix A and the details of the simulation results can be found in Appendix B.

### Performance analysis of Rel-16 positioning solutions

In this section, we provide a performance analysis for the distance error in horizontal plane by using the Rel.16 positioning techniques and agreed RAN1 evaluation assumptions. Our research is mainly focused on the DL-TDOA positioning solution. The performance from the Rel.16 technology is considered as the reference in comparison with the performance in Rel.17.

To address the NLOS bias in the TOA measurement, in Rel.16, we use first path average ratio (FPAR) as the indicator indicating if a TOA is measured from LOS or NLOS. In the real application, we consider using a pre-defined threshold to filter out some TOA measurements having low FPAR. The UE would only report the remaining TOA to the Location server (LS).

Table 2.1.1-1 shows the assumption of the Rel.16 positioning. InF-SH, InF-DH and InH-OO are the three main scenarios that we are focused on. The followed table 2.1.2-2 shows the horizontal error for X % of the UEs by using Rel.16 positioning technology. In each case, UEs have two different type of distribution: Convex UE and All UE. The former one means that UEs were uniformly distributed over the convex hull of the horizontal BS deployment and later one means that UE could be anywhere in the horizontal plane.

Table 2.1.1-3 summarize the performance of the Rel.16 technology in term of if the results meet the commercial or the IIoT requirement or not.

Table 2.1.1-1: Rel.16 NR DL positioning - evaluation scenarios and parameters

| Parameter | [Case1, InF- SH, FR2], [Case2, InF-DH, FR2], [Case3, InH-OO, FR2] | [Case4, InF- SH, FR1], [Case5, InF-DH, FR1], [Case6, InH-OO, FR1] |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline (See Table 3) | Baseline (See Table 3) |
| Carrier frequency | 28 GHz | 1.5 GHz |
| Subcarrier spacing | 120 kHz | 30 kHz |
| Reference Signal Transmission Bandwidth | 400 MHz | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6, 6 symbols | Comb-6, 6 symbols |
| Reference signal (type of sequence, number of ports, …) | Pseudo-random sequence, 1 antenna port |  |
| Number of sites | 18 sites in InF, 12 sites in InH |  |
| Number of symbols used per occasion | 6 | 6 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6 dB | 6 dB |
| Uplink power control (applied/not applied) | not applied |  |
| interference modelling (ideal muting, or other) | ideal muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Adaptive threshold-based first peak detection for TOA measurement |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Select the TRP by Fast peak average ratio (FPAR), Maximum likelihood positioning algorithm, AoD measurement |  |
| Network synchronization assumptions | Perfect synchronization |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping at Tx |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | DFT Codebook |  |
| Additional notes, if any |  |  |

Table 2.1.1-2: Rel.16 NR positioning - horizontal location error results from [X]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 1], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.028 | 0.038 | 0.052 | 0.24 |
|  | (Optional) All UEs | 0.036 | 0.059 | 0.12 | 0.39 |
| [Case 2], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.24 | 0.72 | 1.42 | 2.80 |
|  | (Optional) All UEs | 0.30 | 0.68 | 1.31 | 2.83 |
| [Case 3], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.045 | 0.070 | 0.14 | 0.35 |
|  | (Optional) All UEs | 0.068 | 0.13 | 0.28 | 0.50 |
| [Case 4], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.11 | 0.16 | 0.24 | 0.70 |
|  | (Optional) All UEs | 0.14 | 0.21 | 0.37 | 1.06 |
| [Case 5], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.35 | 0.82 | 1.79 | 3.87 |
|  | (Optional) All UEs | 0.31 | 1.14 | 1.93 | 4.29 |
| [Case 6], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.20 | 0.31 | 0.50 | 1.00 |
|  | (Optional) All UEs | 0.21 | 0.36 | 0.64 | 1.17 |

Table 2.1.1-3: Rel.16 NR positioning – horizontal accuracy performance summary [X]

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case 1], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Y | N, 0.047 m | Y |
| [Case 2], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | N, 1.81 m | N, 2.60 m | N, 2.30 m |
| [Case 3], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | Y | N, 0.15 m | Y |
| [Case 4], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Y | N, 0.50 m | N, 0.20 m |
| [Case 5], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | N, 2.87 m | N, 3.67 m | N, 3.37 m |
| [Case 6], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | Y | N, 0.80 m | N, 0.50 m |

The following observations were made based on the aforementioned results:

Observation 1 (Horizontal accuracy of Rel.16 NR positioning):

  - Performance of Rel.16 NR positioning techniques meet the commercial horizontal accuracy requirements in InF-SH and InH-OO scenarios in both DL FR1 and FR2 cases. The performance in InF-DH is relatively worse than the former two scenarios.

  - By using Rel.16 NR positioning techniques, none of these scenarios can achieve the IIoT requirements of 0.2 m horizontal accuracy at 90% of the UE.

### Performance of studied NR positioning enhancements

The system level simulation was performed based on the agreed simulation assumptions and parameters in RAN1 102e meeting [3]. We were evaluating both commercial use cases and IIoT use cases. For commercial use cases, we used Indoor Open Office (IOO) scenario. For IIoT use cases, we used Indoor Factory - Sparse Clutter High base-station placement (InF-SH) and Indoor Factory - Dense Clutter High base-station placement (InF-DH). We evaluate the operation in both FR1 & FR2. All the details on simulation scenarios and its parameters are given in Tables 2.1.2-1.

The CDFs of the horizontal error are shown in the figure 4 to figure 15. Figure 4 to Figure 9 are the simulation from the ‘convex UE’ (UE is uniformly distributed in the convex hull). Figure 10 to Figure 15 are the simulation from the ‘All UE’ (UE is uniformly distributed in the whole area). 4 different positioning techniques are used in each case which are: ‘TDOA’, ‘TDOA/AOD’, ‘TDOA+NLOS’ and ‘TDOA/AOD+NLOS’. ‘TDOA’ means using DL-TDOA only in the location estimate. ‘AOD’ means using DL-AOD to be used together with DL-TDOA. ‘NLOS’ in here is notation for using NLOS mitigation algorithm to further refine the location estimate.

The UE performs both DL-TDOA and AoD measurement. As part of DL-TDOA operation, the UE uses adaptive threshold-based first peak detection for TOA estimate. The power delay profiles are calculated and all the peaks (local maxima) are detected. A predefined threshold which is adaptive to the estimated noise is used to filter out unqualified peaks. Among the rest of the peaks, the first one is selected as the TOA estimate and further used/reported as RSTD to LS. The location server applies the AOD estimation based on the UE measurement report together with some other prior information related to TRP (beam configuration, TRP bearing, etc.) In case of FR2, the UE performs Rx beamforming in by combining the signals received from multiple antenna elements. To obtain the maximum beamforming gain in UE side, we first apply MUSIC algorithm to estimate the AOA and subsequently apply the corresponding combining vector to the signals.

One of the enhancements in Rel.17 DL positioning is adapting the ZoD (Zenith of Departure) detection method to support DL vertical positioning. To measure the ZOD, we consider using Tx beam-sweeping not only in horizontal plane but also in the vertical plane. In the Rel.16, the phase shift applied to two nearby horizontal element layers were zero and therefore, the beams could only point to the horizontal direction. However, in Rel.17, by applying phase shift to each horizontal element layers, we can tilt the beams to a certain vertical angle. In our setup, we use the same beam sweeping pattern as we use before and applied it in 5 different vertical angles. Therefore, in total, 60 beams are used for angle measurement: 12 horizontal beams and 5 vertical beams. To achieve this, we start to use Kronecker product codebook which is basically a Kronecker product of two oversampled DFT codewords.

The Flow chart of the NLOS mitigation technique used in Rel.17 is shown in the figure1:

TOA measurements

Apply Maximum likelihood Positioning algorithm

UE location estimate

Use NSE algorithm in LS side to identify the most likely NLOS TRP. And then remove it.

Select the TOA based on First peak average ratio (FPAR) with a predefined threshold.

Figure 1: Flow Chart of the NLOS mitigation technique.

On the location server operation (LS), we used maximum likelihood estimation (MLE) TDOA+ AOD algorithm for location estimate. Here, it is assumed that the TOA error and the AOD error in the LOS link are Gaussian distributed. A prior information of the standard deviation of the distribution is known. Then, the conditional probability of the UE location given by TOAs and AODs is derived in the location server. The MLE will search for the best UE estimate that can maximize the probability. We also introduce a method to enhance the positioning estimation accuracy by utilizing a NLOS detection estimator (NLOS). Based on N TRPs, there is a probability that a RSTD measurement is from NLOS link. In total, there are 2^N possible combinations which is denoted as ‘State’. The NLOS detection algorithm will calculate all the UE location candidates and select one as the best estimate.

Table 2.1.2-1: NR positioning enhancements - evaluation scenarios and parameters

| Parameter | [Case7, InF- SH, FR2], [Case8, InF-DH, FR2], [Case9, InH-OO, FR2] | [Case10, InF- SH, FR1], [Case11, InF-DH, FR1], [Case12, InH-OO, FR1] |
| --- | --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline (See Table 3) | Baseline (See Table 3) |
| Carrier frequency | 28 GHz | 1.5 GHz |
| Subcarrier spacing | 120 kHz | 30 kHz |
| Reference Signal Transmission Bandwidth | 400 MHz | 100 MHz |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6, 6 symbols | Comb-6, 6 symbols |
| Reference signal (type of sequence, number of ports, …) | Pseudo-random sequence, 1 antenna port |  |
| Number of sites | 18 sites in InF, 12 sites in InH |  |
| Number of symbols used per occasion | 6 | 6 |
| number of occasions used per positioning estimate | 1 | 1 |
| Power-boosting level | 6 dB | 6 dB |
| Uplink power control (applied/not applied) | not applied |  |
| interference modelling (ideal muting, or other) | ideal muting |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Adaptive threshold-based first peak detection for TOA measurement |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | Select the TRP by FPAR, Maximum likelihood positioning algorithm, NLOS Detection algorithms, AoD measurement |  |
| Network synchronization assumptions | Perfect synchronization |  |
| UE/gNB Tx/Rx Calibration Error | 0 |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | Beam sweeping at Tx, MUSIC based signal combining at Rx |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) | Kronecker Product Codebook (Kronecker product of two oversampled DFT codewords) |  |
| Evaluated Enhancement for Rel.17 | NLOS Detection algorithms | NLOS Detection algorithms |
| Additional notes, if any |  |  |

The simulation results in terms of horizontal location error are summarized in Table 2.1.2-2 and Table 2.1.2-3.

Table 2.1.2-2: NR positioning enhancements - horizontal location error results from [X]

|  |  | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.025 | 0.0344 | 0.046 | 0.074 |
|  | (Optional) All UEs | 0.033 | 0.048 | 0.075 | 0.23 |
| [Case 8], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.044 | 0.075 | 0.53 | 1.96 |
|  | (Optional) All UEs | 0.063 | 0.31 | 1.31 | 2.12 |
| [Case 9], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.041 | 0.062 | 0.11 | 0.31 |
|  | (Optional) All UEs | 0.054 | 0.095 | 0.18 | 0.39 |
| [Case 10], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.10 | 0.14 | 0.20 | 0.38 |
|  | (Optional) All UEs | 0.13 | 0.19 | 0.30 | 0.67 |
| [Case 11], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.20 | 0.46 | 1.49 | 2.13 |
|  | (Optional) All UEs | 0.29 | 0.75 | 1.60 | 3.62 |
| [Case 12], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | Convex UEs | 0.20 | 0.29 | 0.42 | 0.69 |
|  | (Optional) All UEs | 0.20 | 0.33 | 0.53 | 0.98 |

Table 2.1.2-3: Rel.17 NR positioning – horizontal accuracy performance summary [X]

| Simulation case (Horizontal Error) | Commercial horizontal accuracy requirements [1]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.2]m @[90]%are met - Yes/No. If no, provide performance gaps | IIoT horizontal accuracy requirements of [0.5]m @[90]%are met -Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Y | Y | Y |
| [Case 8], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | N, 0.96 m | N, 1.76 m | N, 1.46 m |
| [Case 9], [InH-OO], [FR2], [400 MHz], [Comb-6], [6dB PB] | Y | N, 0.11 m | Y |
| [Case 10], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Y | N, 0.18 m | Y |
| [Case 11], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | N, 1.13 m | N, 1.93 m | N, 1.63 m |
| [Case 12], [InH-OO], [FR1], [100 MHz], [Comb-6], [6dB PB] | Y | N, 0.49 m | N, 0.19 m |

Observation 2 (Horizontal accuracy of Rel.17 NR positioning):

  - The IIoT horizontal requirement (< 0.2 m accuracy @ 90% CDF) can be achieved in InF-SH FR2 scenario.

  - In 3 scenarios, InF-SH FR2, InF-SH FR1 and InH-OO FR2, the IIoT, the achievable accuracy is < 0.5 m at 90% CDF.

  - The enhancement of NLOS detection algorithm from Rel.16 to Rel.17 improves the horizontal positioning accuracy, especially in InF-DH scenario. The improvement can be up to 1.7 m in InF-DH FR1 case.

### Performance of the vertical positioning:

In this section, we show the result of the vertical positioning measured from InF-SH and InF-DH scenarios in both FR1 and FR2 cases in Table 2.1.3-1 to 2.1.3-2. The CDFs of the vertical error in various scenarios and gNB deployments are shown in the figure 16 to figure 26. In each of the scenario, we present both CDF of the vertical location error (The left subplot) and the CDF of the horizontal location error (The right subplot).

In RAN1#101e meeting [2], the gNB height is recommended using a fixed height which is 8 meter or using various heights which is in the range of {4, 8} m. Therefore, these two deployments would be the main deployments and we simulate both of these deployments in each scenario. Additionally, we propose our optional gNB deployment which allows the gNB having 4 different various height in a range {2, 4, 6, 8} m. In the table below, the ‘gNBH’ stands for the gNB height. ‘gNBH ∈ {4,8} m’ means that the gNBs have two different height. Half of them is 4-meter-high and the other half are 8 meters. ‘UEH’ in here stands for the UE antenna height which is assumed to be uniformly distributed within the range of [0, 8] m.

Table 2.1.3-1: NR positioning enhancements - vertical location error results from [X]

| Case | Deployment | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | gNBH ∈ {2, 4, 6, 8} m, UEH ∈ [0, 8] m (Proposal) | 0.45 | 0.58 | 0.72 | 0.93 |
|  | gNBH ∈ {4,8} m, UEH ∈ [0, 8] m | 0.84 | 1.01 | 1.18 | 1.34 |
|  | gNBH = 8 m, UEH ∈ {0, 8} m | 0.75 | 1.12 | 1.51 | 1.88 |
| [Case 8], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | gNBH ∈ {4,8} m, UEH ∈ [0, 8] m | 0.85 | 1.22 | 1.62 | 2.11 |
|  | gNBH = 8 m, UEH ∈ {0, 8} m | 1.56 | 2.02 | 2.53 | 3.04 |
| [Case 10], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | gNBH ∈ {4,8}m, UEH ∈ [0, 8] m | 0.96 | 1.25 | 1.50 | 1.73 |
|  | gNBH = 8 m, UEH ∈ {0, 8} m | 0.98 | 1.38 | 1.78 | 2.16 |
| [Case 11], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | gNBH ∈ {4,8}m, UEH ∈ [0, 8] m | 0.85 | 1.25 | 1.77 | 2.57 |
|  | gNBH = 8 m, UEH ∈ {0, 8} m | 1.31 | 1.74 | 2.18 | 2.69 |

Table 2.1.3-2: Rel.16 NR positioning – vertical accuracy performance summary [X]

| Simulation case (Vertical Error) | Commercial vertical accuracy requirements [3]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT vertical accuracy requirements of [0.2]m @[90]% are met - Yes/No. If no, provide performance gaps | IIoT vertical accuracy requirements of [1]m at @[90]% are met - Yes/No. If no, provide performance gaps |
| --- | --- | --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Y | N, 1.14 m | N, 0.34 m (Y if using proposal deployment) |
| [Case 8], [InF-DH], [FR2], [400 MHz], [Comb-6], [6dB PB] | Y | N, 1.91 m | N, 1.11 m |
| [Case 10], [InF-SH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Y | N, 0.73 m | N, 1.16 m |
| [Case 11], [InF-DH], [FR1], [100 MHz], [Comb-6], [6dB PB] | Y | N, 2.37 m | N, 1.57 m |

Observation 3 (Vertical accuracy of Rel.17 NR positioning):

  - The commercial requirements (< 3 m accuracy) can be fulfilled in all scenarios under evaluation.

  - The performance gap between two IIoT requirement and the simulation results are still very large. None of the cases with default gNB height can achieve the IIoT requirement.

  - In one of gNB height deployment configuration (Case 7) and in InF-SH FR2 scenario, the achievable performance is less than  1 m accuracy.

### Investigation on the impact of the gNB height on the vertical positioning accuracy:

From the previous simulation, we found that the proposal gNB deployment ‘gNBH ∈ {2, 4, 6, 8} m’ is better than ‘gNBH ∈ {4,8} m’ in terms of vertical positioning accuracy, and which arouse our interest in investigating on the impact of the gNB height. In this section, we show our simulation results using the following gNB deployment in InF-SH FR2 scenario. Note that in each of these deployments, the amount of the gNBs having the same height would be the same. Figure 2 is the comparison among Deployment 1 to 5 in terms of CDF of the vertical error. Table 2.1.4-1 shows the vertical error at X% in the CDF from these 5 deployments.

Deployment 1: gNB height = 8 m,

Deployment 2: gNB height = {4, 8} m,

Deployment 3: gNB height = {2, 4, 6, 8} m,

Deployment 4: gNB height = {1, 2, 3, 4, 5, 6, 7, 8} m,

Deployment 5: gNB height = {1: 0.5: 8} m. (= {1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5, 6, 6.5, 7, 7.5, 8} m)

In all the test deployment, the UE antenna height is within the range of [0, 8].

Figure 2: The vertical positioning error using various gNB deployments in InF-SH FR2 scenario.

Table 2.1.4-1: NR positioning enhancements - vertical location error results from [X]

|  | Deployment Number | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [InF-SH], [FR2], [400 MHz], [Comb-6], [6dB PB] | 1 | 0.75 | 1.12 | 1.51 | 1.88 |
|  | 2 | 0.84 | 1.01 | 1.18 | 1.34 |
|  | 3 | 0.45 | 0.58 | 0.72 | 0.93 |
|  | 4 | 0.35 | 0.48 | 0.65 | 0.86 |
|  | 5 | 0.26 | 0.39 | 0.57 | 0.79 |

Observation 4: (the impact of gNB height):

  - The vertical positioning become more accurate as the gNB height are getting diverse. In other word, using gNBs with different height can improve the vertical positioning accuracy.

  - In the deployment 3 to 5 and in InF-SH FR2 scenario,  the achievable performance is less than  1 m accuracy.

### Investigation on the impact of the bandwidth

The simulation results for InF-SH in FR2 with various bandwidth configuration is shown in Figure 3. Here, we consider the positioning technique: DL-TDoA+AoD with NLOS detection. It can be observed that the PRS bandwidth configuration is heavily affecting the positioning accuracy performance. The performance accuracy target cannot be made especially with the usage of PRS in smaller bandwidth configuration. This basically reflect the real deployment as the operator is expected to utilize a portion of its cell bandwidth for PRS allocation.

Figure 3: Simulation results in Indoor Factory Sparse High (InF-SH) FR2, using Comb4 PRS signal, applying different bandwidth.

Table 2.1.6-1: NR positioning enhancements - horizontal location error results from [X]

|  | Bandwidth | 50% | 67% | 80% | 90% |
| --- | --- | --- | --- | --- | --- |
| [Case 7], [InF-SH], [FR2], [Comb-4], [6dB PB] | 400 MHz | 0.0250 | 0.0345 | 0.0467 | 0.0863 |
|  | 100 MHz | 0.1448 | 0.1953 | 0.2556 | 0.3656 |
|  | 50 MHz | 0.2019 | 0.2772 | 0.3851 | 0.7773 |
|  | 20 MHz | 0.3945 | 0.6006 | 0.9000 | 1.4567 |

Observation 5: (the impact of PRS bandwidth):

  - The usage of PRS configuration in a smaller bandwidth degrades the positioning accuracy performance. In the other word, by changing the bandwidth, we can control the positioning accuracy.

# Evaluation of Physical Layer Latency

The physical layer latency for DL-TDOA in legacy NR Rel-16 is studied in this section.  The signaling is grouped into 4 main time entities.

- LMF send Location request to UE.

- UE request/receive and wait for a configured measurement gap.

- UE measures the PRS

- UE request to send and send a measurement report

A signaling diagram describing the signaling together with time variables for physical layer latency evaluation is shown in Figure X below.

Figure 4: Signalling diagram for a DL TDOA from Location Request to Measurement Report

Table 1 show the minimum achievable time values as of release 16.  The lowest achievable latency sets the boundary of what Rel-16 NR is capable of.

Table 1: Physical layer latency breakdown

| Time name | Min Time (ms) | Min Time (% of total) | Description |
| --- | --- | --- | --- |
| t_lr | 10 | 18 % | Assumed similar to an RRC message according to 38.331 section 12. |
| t_mg_conf | 11 | 20 % | The measurement gap request and configuration are on RRC level. The RRC reconfiguration is 10 ms according to 38.331 section 12. The request is assumed to take more than 1 ms to transmit. |
| t_prs | 22.5 | 40 % | PRS periodicity ∈ [4, 10240] ms MGRP ∈ [20, 160] ms with mglFR1 ∈ [3, 6]ms and mglFR2 ∈ [1.5, 6] ms Repetition factor for all configuration is in the range of 1 to 32 For a minimum value of t_prs no repetitions, shortest PRS period and smallest MGRP is assumed. The time value consist of a worst case waiting of one full MGRP after the MG is configured (20 ms). Then shortest mgl is 1.5 ms for FR2, enough PRS resources are assumed to fit in here. PRS measurements are then assumed to occur partly in parallel while received in L1 baseband. The processing depends on number of PRS to measure. Capability for processing signaling applies in TS 37.355 but here we have chosen a tail L1 processing of 1 ms. |
| t_report | 12.5 ms | 22 % | UL Scheduling Request transmission assumed to 1 ms, gNB delay for UL Grant 3 ms + 1 ms transmission of grant, UE decode of grant 3 ms and 1 ms transmission of UL measurement report and 3 ms decoding of measurement report at network side |
| Total | 56 ms |  |  |

Observation 6: Rel 16 physical layer latency for DL PRS based positioning is estimated to be more than 56 ms. Approximately, 60 % of the physical layer latency is for signaling and 40 % is for measuring and handling of measurement gap.

In rel-17, the target of physical layer latency is expected to be 10 ms which is far below from the lowest achievable latency 56 ms. Hence, a significant modification to the existing procedure is required. This may include shortening the existing signaling / measurement and removing some of the signaling.

Observation 7: A significant modification to the existing procedure is required in order to achieve 10ms physical layer latency. This may include shortening the existing signaling / measurement and removing some of the signaling.

# Conclusion

In this contribution, we have provided our simulation results in term of positioning accuracy in both vertical and horizontal for various scenarios. Additionally, latency analysis for downlink-based positioning were also presented. Our observations are as follows:

Observation 1 (Horizontal accuracy of Rel.16 NR positioning):

  - Performance of Rel.16 NR positioning techniques meet the commercial horizontal accuracy requirements in InF-SH and InH-OO scenarios in both DL FR1 and FR2 cases. The performance in InF-DH is relatively worse than the former two scenarios.

  - By using Rel.16 NR positioning techniques, none of these scenarios can achieve the IIoT requirements of 0.2 m horizontal accuracy at 90% of the UE.

Observation 2 (Horizontal accuracy of Rel.17 NR positioning):

  - The IIoT horizontal requirement 1 (< 0.2 m accuracy @ 90% CDF) can be achieved in InF-SH FR2 scenario.

  - In 3 scenarios, InF-SH FR2, InF-SH FR1 and InH-OO FR2, the IIoT, the achievable accuracy is < 0.5 m at 90% CDF.

  - The enhancement of NLOS detection algorithm from Rel.16 to Rel.17 improves the horizontal positioning accuracy, especially in InF-DH scenario. The improvement can be up to 1.7 m in InF-DH FR1 case.

Observation 3 (Vertical accuracy of Rel.17 NR positioning):

  - The commercial requirements (< 3 m accuracy) can be fulfilled in all the scenarios.

  - The performance gap between two IIoT requirement and the simulation results are still very large. None of the cases with default gNB height can achieve the IIoT requirement.

  - In one of gNB height deployment configuration (Case 7) and in InF-SH FR2 scenario, the achievable performance is less than  1 m accuracy.

Observation 4: (the impact of gNB height):

  - The vertical positioning become more accurate as the gNB height are getting diverse. In other word, using gNBs with different height can improve the vertical positioning accuracy.

  - In the deployment 3 to 5 and in InF-SH FR2 scenario,  the achievable performance is less than  1 m accuracy.

Observation 5: (the impact of PRS bandwidth):

  - The usage of PRS configuration in a smaller bandwidth degrades the positioning accuracy performance. In the other word, by changing the bandwidth, we can control the positioning accuracy.

Observation 6: Rel 16 physical layer latency for DL PRS based positioning is estimated to be more than 56 ms. Approximately, 60 % of the physical layer latency is for signaling and 40 % is for measuring and handling of measurement gap.

Observation 7: A significant modification to the existing procedure is required in order to achieve 10ms physical layer latency. This may include shortening the existing signaling / measurement and removing some of the signaling.

# References

- RP-193237 SID: NR Positioning Enhancements

- TR 38.855 Study on NR positioning support

- TS 22.261 “Service requirements for the 5G system”

- TR 22.804 “Study on Communication for Automation in Vertical Domains”

- https://sites.google.com/a/co911rc.org/co911rc/resources/wireless-9-1-1

- Appendix A

Table 2: Common simulation parameters

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Description of applied algorithms for estimation of signal location parameters (See Table 1). |  |
| Network synchronization | Perfect Synchronization |  |
|  | Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |

Table 3: Assumptions of InF scenario

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  |  |
| Layout | Hall size | InF-SH: (baseline) 300x150 m InF-DH: (baseline) 120x60 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: {40%, 2m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

# Appendix B

1: Evaluation of accuracy for the Convex UE

Figure 5: Simulation results in Indoor Factory Sparse High (InF-SH) FR2, using Comb-6 RE pattern with 400 MHz BW.

Figure 6: Simulation results in Indoor Factory Sparse High (InF-SH) FR1, using Comb-6 RE pattern with 100 MHz BW.

Figure 7: Simulation results in Indoor Factory Dense High (InF-DH) FR2, using Comb-6 RE pattern with 400 MHz BW.

Figure 8: Simulation results in Indoor Factory Dense High (InF-DH) FR1, using Comb-6 RE pattern with 100 MHz BW.

Figure 9: Simulation results in Indoor Open Office (InH-OO) FR2, using Comb-6 RE pattern with 400 MHz BW.

Figure 10: Simulation results in Indoor Open Office (InH-OO) FR1, using Comb-6 RE pattern with 100 MHz BW.

2. Figures: Evaluation of accuracy for All UEs (i.e. not only convex hull UEs)

Figure 11: Simulation results in Indoor Factory Sparse High (InF-SH) FR2, using Comb-6 RE pattern with 400 MHz BW, All UE.

Figure 12: Simulation results in Indoor Factory Sparse High (InF-SH) FR1, using Comb-6 RE pattern with 100 MHz BW, All UE.

Figure 13: Simulation results in Indoor Factory Dense High (InF-DH) FR2, using Comb-6 RE pattern with 400 MHz BW, All UE.

Figure 14: Simulation results in Indoor Factory Dense High (InF-DH) FR1, using Comb-6 RE pattern with 100 MHz BW, All UE.

Figure 15: Simulation results in Indoor Open Office (InH-OO) FR2, using Comb-6 RE pattern with 400 MHz BW, All UE

Figure 16: Simulation results in Indoor Open Office (InH-OO) FR1, using Comb-6 RE pattern with 100 MHz BW, All UE

  - Evaluation of the vertical positioning:

Figure 17: Vertical and horizontal positioning accuracy in Indoor Factory Sparse High (InF-SH) FR2, using Comb-6 RE pattern with 400 MHz BW, UE height is within the range [0, 8] m, gNB height is 8 m.

Figure 18:Vertical and horizontal positioning accuracy in Indoor Factory Sparse High (InF-SH) FR2, using Comb-6 RE pattern with 400 MHz BW, UE height is within the range [0, 8] m, gNB height range {4,8} m.

Figure 19: Vertical and horizontal positioning accuracy in Indoor Factory Dense High (InF-DH) FR2, using Comb-6 RE pattern with 400 MHz BW, UE height is within the range [0, 8] m, gNB height is 8 m.

Figure 20: Vertical and horizontal positioning accuracy in Indoor Factory Dense High (InF-DH) FR2, using Comb-6 RE pattern with 400 MHz BW, UE height is within the range [0, 8] m, gNB height range {4,8} m.

Figure 21:Vertical and horizontal positioning accuracy in Indoor Factory Sparse High (InF-SH) FR1, using Comb-6 RE pattern with 100 MHz BW, UE height is within the range [0, 8] m, gNB height is 8 m.

Figure 22: Vertical and horizontal positioning accuracy in Indoor Factory Sparse High (InF-SH) FR1, using Comb-6 RE pattern with 100 MHz BW, UE height is within the range [0, 8] m, gNB height range {4,8} m.

Figure 23: Vertical and horizontal positioning accuracy in Indoor Factory Dense High (InF-DH) FR1, using Comb-6 RE pattern with 100 MHz BW, UE height is within the range [0, 8] m, gNB height is 8 m.

Figure 24: Vertical and horizontal positioning accuracy in Indoor Factory Dense High (InF-DH) FR1, using Comb-6 RE pattern with 100 MHz BW, UE height is within the range [0, 8] m, gNB height range {4,8} m.

Figure 25: Vertical and horizontal positioning accuracy in Indoor Factory Sparse High (InF-SH) FR2, using Comb-6 RE pattern with 400 MHz BW, UE height is within the range [0, 8] m, gNB height range {2, 4, 6, 8} m.

Figure 26: Vertical and horizontal positioning accuracy in Indoor Factory Sparse High (InF-SH) FR2, using Comb-6 RE pattern with 400 MHz BW, UE height is within the range [0, 8] m, gNB height range {1, 2, 3, 4, 5, 6, 7, 8} m.

Figure 27: Vertical and horizontal positioning accuracy in Indoor Factory Sparse High (InF-SH) FR2, using Comb-6 RE pattern with 400 MHz BW, UE height is within the range [0, 8] m, gNB height range {1: 0.5: 8} m.