3GPP TSG RAN WG1 #103-e		R1-2008369

e-Meeting, October 26th – November 13th, 2020

Source: 	Sony

Title:	On TRS/CSI-RS occasion(s) for idle/inactive UEs

Agenda Item:	8.7.1.2

Document for:	Discussion & Decision

# Introduction

In the 3GPP TSG RAN#86 meeting, a new WID on power saving enhancement was approved [1]. One of the objectives is shown below:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100-2 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In the RAN1#102e meeting [2], the following agreements related to potential TRS/CSI-RS occasion(s) were made:

Agreements:

Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as:

-           AGC, time/frequency tracking

-           FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication

The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling

-           FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.)

-           FFS for other signalling candidates (e.g., pre-configuration, etc.)

-           FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.)

Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly).

- Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

In this contribution, we discuss aspects related to power saving enhancement for idle/inactive-mode, particularly on the possibility of using TRS/CSI-RS occasion(s) when the UE is in idle/inactive mode.

# Discussions

## Functionalities of potential TRS/CSI-RS for idle/inactive UEs

During idle/inactive mode, a UE is configured with DRX operation, in which during the ON duration, the gNB transmits a paging indication in case there is downlink data for that UE or system information updates. During the DRX opportunity (OFF period), the UE basically can sleep (i.e. expect no data) to save energy. However, the UE needs to be ready / synchronized during the ON period. Hence, it is expected that the UE performs gain control, synchronization and measurement prior to the ON duration. When a legacy NR UE is in idle/inactive mode, as opposed to LTE, no cell specific reference signals are available and the UE, as shown in Figure 1, can only rely on synchronization signal block (SSB) reception for the automatic gain control (AGC), synchronization, beam selection and measurement operations. This can lead to an increase in UE power consumption. Depending on the scenarios, a UE in bad coverage (e.g. cell edge) may required to read multiple SSBs (e.g. 3 SSBs) to ensure the UE is ready for paging reception. In good coverage, a UE may require only one SSB prior to the paging reception.

Figure 1: Idle mode with DRX operation and SSBs reception in legacy NR.

The legacy operation in NR can be described below:

- In IDLE mode, the UE monitors the initial bandwidth part (BWP) to receive SSB for synchronization purposes and cell detection/measurement and also monitors for potential paging following DRX configuration.

- During the initial access phase, the UE performs a random-access procedure, where the UE gets attached to the network, receives RRC messages and enters the connected mode state. When in connected mode, the UE can move to another BWP that may have a wider bandwidth than the initial BWP (here we call it an active BWP).

- One of the RRC messages contains the reference signal configuration (CSI-RS, TRS) that the UE needs for measurement and further time/frequency synchronization.

- TRS is configured to have an association with the SSB. This association is described by the quasi-colocation (QCL) information. The UE can assume the same QCL configuration for the TRS reception and the associated SSB. Multiple TRS (CSI-RS) resources can be mapped to an SSB.

- Once the UE has finished data communication, the UE goes back to idle mode and only monitors the initial BWP, and uses SSB to measure and monitor for potential paging and cell re-selection.

In idle mode in legacy NR operation, the UE is not configured to receive CSI-RS/TRS. The UE monitors the initial BWP and can only rely on SSB reception for the automatic gain control (AGC), synchronization, beam selection and measurement operations. Furthermore, the SSB bandwidth is limited to the initial BWP bandwidth (e.g. 5 MHz for 15 kHz subcarrier spacing). while TRS can have much wider bandwidth. Hence, TRS can provide finer measurement estimation results than SSB.

During IDLE mode, the UE monitors one PO per paging DRX cycle for potential paging and SI update. In multi beam operation (or in FR2) the UE needs to wake up much earlier than the paging occasion in order to establish a suitable beam to be used during the paging occasion. SSB availability can be sparse since the lowest periodicity can be 20 msec. UEs at the cell edge or when operating in FR2 may need to utilize multiple SSBs prior to the on-duration of the paging DRX cycle, when it receives control and data channels from gNB. The time needed for the UE to wake-up is highly determined by the location of SSB and PO. This means that if the SSB and POs are not aligned, the UE needs to wake-up multiple times or cannot turn to deep sleep between SSB and PO.

The availability of TRS for idle/inactive UEs is expected to assist UE functionality particularly to simplify the aforementioned legacy operation with the main objective being to reduce UE power consumption. We consider the TRS used by idle/inactive UEs has the following main characteristics:

- These TRS are not specifically for idle/inactive UEs. These are TRS with a primary purpose for the connected mode UEs and these TRS can also be used by the idle/inactive UEs. TRS is expected to provide fine frequency/time synchronization. This is typically accomplished by allocating a relatively larger amount of TRS resource than for SSB (e.g. wider bandwidth than SSB).

- The legacy TRS periodicity is configurable with a period of 10, 20, 40, or 80 ms. Hence, depending on the SSB configuration, there could be one or more TRS occasion during the period of an SSB burst. There could also be one TRS occasion during multiple SSB bursts.

We support previous agreements that the TRS can be used by the UE for AGC function and time/frequency synchronization functionality. Furthermore, the UE can also use the TRS for RRM measurement for the serving cell. The UE knows both the SSB and TRS occasions. Hence, it is up to the UE implementation how to utilize SSB and TRS for the aforementioned functionalities. The usage of TRS for RRM measurement for neighbor cells should not be considered as the UE would be required to obtain the TRS configuration of neighbor cells. The problem is the TRS configuration of neighbor cells can be changed. It can be changed frequently as it is basically the TRS available in connected mode of that neighbour cells. The changes of neighbour cell TRS configuration that can be used for idle/inactive UEs can significantly increase signalling. The usage of TRS for other functionality, such as for paging indication should also not be considered. This would require a specific TRS for idle/inactive UEs and it should be discussed in the paging enhancements agenda item.

Proposal 1: TRS/CSI-RS occasions for idle/inactive UEs can be used for AGC, time/frequency tracking, and RRM for the serving cell. It is up to the UE implementation whether to utilize SSB and/or TRS/CSI-RS for such functionalities.

## Configuration of Potential TRS/CSI-RS for idle/inactive UEs

The legacy TRS configuration for connected mode UEs is configured via RRC messaging. The TRS configuration contains mandatory fields, such as:

- TCI states configuration where the QCL relation between TRS and SSB is provided.

- CSI-RS configuration where the resource allocation configuration of individual CSI-RS is provided.

- CSI-RS resource-set configuration where the assignment/allocation of CSI-RS as TRS is provided.

There are also optional parameters in the legacy TRS configuration. The details of parameters and the required bits for the above mandatory fields (and excluding optional parameters) are given in Appendix A.

We observed that the estimated minimum configuration size of TRS configuration for each spatial filter is around 27 bytes with the following break-down:

- TCI states: 			4 bytes

- CSI-RS config: 		20 bytes

- CSI-RS Resource set config: 	3 bytes

Depending on the SSB configuration, a gNB with 64 SSBs requires a TRS configuration size of 1728 bytes. Furthermore, a gNB may support multiple TRS configurations as there can be many connected UEs. Each or some of the connected UEs may have its own TRS configuration. During idle/inactive mode, a UE can be configured with one of those multiple TRS configurations. We consider that the configuration of potential TRS/CSI-RS for idle/inactive mode is provided by dedicated RRC signaling. This is in a similar fashion to conveying the TRS for connected mode UEs in legacy NR. In legacy NR, when the UE switches to idle mode, the UE is expected to flush the connected mode configuration. Particularly in this case, we expect the UE to keep the TRS configuration so that the UE can use it while the UE is in idle/inactive mode. There is also another case where the UE in idle/inactive mode uses the new TRS configuration. The new TRS configuration is not the same as the TRS configuration when the UE in connected mode.

Observation 1: TRS/CSI-RS configuration size with mandatory parameters is not small and it can have a size of up to 1728 bytes.

We do not support providing the TRS/CSI-RS configuration in system information (SIB). When the UE switches from connected to idle/inactive mode, it may be the case that the previously configured TRS in connected mode is reused in idle/inactive mode. In this case, it becomes redundant if the TRS/CSI-RS is provided in SIB as the UE has already obtained it in an RRC message. Furthermore, we have a concern on TRS/CSI-RS configuration size which means that it should not be provided via SIB.

Proposal 2: The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signaling in dedicated RRC Messaging (i.e. the same mechanism as for the legacy TRS/CSI-RS occasion(s) for connected mode UE(s)).

In RAN1#102e, the following observation was made in the Chairman’s notes [2]:

Observation:

It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.)

A remaining aspect related to this agreement is on the UE assumption of whether the UE can assume the TRS is always present or not. Considering the transmission of TRS/CSI-RS to idle/inactive UEs is up to gNB implementation, there could be a “mismatch” case where the idle/inactive UEs assume the existence of a TRS/CSI-RS occasion while the gNB has decided to stop the TRS/CSI-RS transmission. In order for the UE to properly utilize TRS, the UE needs to detect whether there is a TRS occasion or not. This requires the UE to perform blind detection. From the UE point of view, a blind detection operation is preferably avoided or minimized. This can be avoided/minimized if the activation or deactivation of TRS transmission is provided in system information (i.e. SIB 1). Here, we consider it only requires a few bits and it should be feasible to be added as part of SIB. However, we also need to understand some potential limitations from the network side, for example how often is the possible change of TRS configuration. A frequent change may require frequent SIB transmission as well as SIB reading by the UE.

Proposal 3: Activation and De-activation of TRS/CSI-RS in SIB is further investigated to avoid/minimize UE blind detection operation.

# Conclusion

In this contribution, we discuss power saving enhancement for UEs in IDLE or INACTIVE mode. Our observation and proposals are listed below:

Proposal 1: TRS/CSI-RS occasions for idle/inactive UEs can be used for AGC, time/frequency tracking, and RRM for the serving cell. It is up to the UE implementation whether to utilize SSB and/or TRS/CSI-RS for such functionalities.

Observation 1: TRS/CSI-RS configuration size with mandatory parameters is not small and it can have a size of up to 1728 bytes.

Proposal 2: The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signaling in dedicated RRC Messaging (i.e. the same mechanism as for the legacy TRS/CSI-RS occasion(s) for connected mode UE(s)).

Proposal 3: Activation and De-activation of TRS/CSI-RS in SIB is further investigated to avoid/minimize UE blind detection operation.

# References

- RP-193239, New WID: UE Power Saving Enhancements.

- RAN1-102e Chairman’s notes

- TR 38.840, “Study on UE power saving”, RAN1.

# Appendix A

TCI states Configuration:

--------------------------

tci-StatesToAddModList

{

...

{

tci-StateId [0-127],				(7 bit)

qcl-Type1

{

bwp-Id [0-3],				(2 bit)

referenceSignal csi-rs : [0-255],		(8 bit)

qcl-Type [typeA-D] 			(2 bit)

},

qcl-Type2

{

bwp-Id [0-3],				(2 bit)

referenceSignal csi-rs : [0-255],		(8 bit)

qcl-Type [typeA-D]			(2 bit)

}

For each beam a TCI state (type1 + type2) ~ 32 bits/beam ~4 bytes/beam:

CSI-RS Configuration:

--------------------------

csi-MeasConfig setup :

{

nzp-CSI-RS-ResourceToAddModList

{

{

nzp-CSI-RS-ResourceId [0-255],				(8 bit)

resourceMapping

{

frequencyDomainAllocation rowX : [0-15],	(4 bit)

nrofPorts [p1-p7],					(3 bit)

firstOFDMSymbolInTimeDomain [0-15],			(4 bit)

cdm-Type [noCDM, …, cdm8-FD2-TD4],			(2 bit)

density three : [NULL,…],				(2 bit)

freqBand

{

startingRB [0-511],					(9 bit)

nrofRBs [0-511]					(9 bit)

}

},

powerControlOffset [0-31],				(5 bit)

powerControlOffsetSS [db0,..],				(2 bit)

scramblingID [0-1023],					(10 bit)

periodicityAndOffset slotsX : [0-16383],		(14 bit)

qcl-InfoPeriodicCSI-RS [0-127]				(7 bit)

},

{

nzp-CSI-RS-ResourceId 1,

resourceMapping

{

frequencyDomainAllocation rowX : '[0-15],

nrofPorts [p1-p7],

firstOFDMSymbolInTimeDomain [0-15],

cdm-Type [noCDM, …, cdm8-FD2-TD4],

density three : [NULL,…],

freqBand

{

startingRB [0-511],

nrofRBs [0-511]

}

},

powerControlOffset [0-31],

powerControlOffsetSS [db0,..],

scramblingID [0-1023],,

periodicityAndOffset slotsX : [0-16383],

qcl-InfoPeriodicCSI-RS [0-127]

},

Note: Each beam has 2 CSI-RS associated: ~ 2 x ( 80 bit) ~ 20 bytes/beam

CSI-RS Resource set Configuration:

--------------------------

nzp-CSI-RS-ResourceSetToAddModList

{

{

nzp-CSI-ResourceSetId [0-63], 		(6 bit)

nzp-CSI-RS-Resources

{

[0-255],				(8 bit)

[0-255]				(8 bit)

},

repetition [off-on],			(1 bit)

trs-Info [true-false]			(1 bit)

},

Note: Each beam has 1 resource set associating the 2 CSI-RS for the beam: 24 bit/beam ~3bytes/beam