3GPP TSG RAN WG1 #103-e		                                    		  R1-2008488

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.5.1

Source:	InterDigital Inc.

Title:	Discussion on evaluation assumptions for latency

Document for:	Discussion and Decision

# Introduction

In RAN1#102e, the following agreement was made related the alternatives for the starting occasion for latency evaluation for UE based positioning.

Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives.

Different from UE-assisted positioning, the UE based positioning focus on specific use cases. Thus, different start time may be defined for UE-based positioning. In this contribution, we present our view on each alternative for the start time in the latency analysis for UE based positioning.

# Start time for latency evaluation for UE-based positioning

In this section, each alternative is examined. As a reference, a procedure for DL-TDOA/AoD positioning method introduced in RAN2 email discussion [2] is used.

Alt.1 : “transmission of the PUSCH carrying the MG Request from the UE” as the start time for latency analysis

In this alternative, it is assumed that the UE autonomously requests configuration for measurement gap to receive PRS. The gNB will perform configuration of the measurement gap, and PRS is sent subsequently. It is also assumed that the UE has already received PRS configurations from the network and the UE may perform positioning at preferred timing. Another possibility could be the case where the UE may have been pre-configured with the PRS configurations and upon receiving the request for location information from higher layer, the UE could send the MG request in PUSCH. Such use case may exist if the UE is stationary and surrounding environment does not change frequently. The UE may use the MG request to trigger the positioning process.

Alt.2 : “Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data” as the start time for latency analysis

In this alternative, transmission of LPP Request Assistance Data becomes the trigger for positioning. It is assumed that the UE has already received LPP Request Location Information. This alternative may be the most time consuming alternative in the physical layer among all alternatives. On the other hand, this alternative is the most general as it does not assume any optimizations in terms of the availability of PRS configurations or the use of pre-configured PRS configurations at UE. It also provides flexibility for the network to configure PRS per UE’s request whereas in other alternative, it is assumed that the PRS configuration is nearly static. For practical deployment, the latency analysis based on Alt. 2 may be suitable.

Figure 1 Procedure for DL-TDOA/DL-AoD [2]

Alt.3 : “Start of the Reception of DL PRS” as the start time for latency analysis

This alternative provides the least amount of latency. It assumes that PRS and measurement gaps are already configured. It also possible that Alt. 3 could be applicable only for certain scenarios (e.g. idle/inactive mode operation), where the configuration of measurement gap may not be necessary. Similar to Alt. 1, the analysis considering Alt. 3 may be useful for the UE which is located in the static environment. It may be assumed that the UE is configured statically and the network assumes the start of the positioning as soon as it send PRS.

Based on the above analysis, we make the following proposal.

Proposal 1 : For UE-based positioning, support Alt. 2, “Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data”, as the start time for the physical layer latency analysis

# Conclusion.

The following proposal is made in this contribution.

Proposal 1 : For UE-based positioning, support Alt. 2, “Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data”, as the start time for the physical layer latency analysis

Reference

[1] RAN1 Chairman’s notes, RAN1#102e, Aug., 2020.

- [2] “FL Summary for additional scenarios for evaluation of NR positioning enhancements,” Intel, Report of [Post111-e][625][POS] End-to-end latency analysis (Intel), Oct. 2020.