3GPP TSG RAN WG1 Meeting #103-e  		R1-2008510

e-Meeting, October 26th – November 13th, 2020

Agenda Item: 8.6.1

Source: MediaTek Inc.

Title: On complexity reduction features for NR RedCap UEs

Document for: Discussion and Decision

# Introduction

The Rel-17 study item on Reduced Capability NR devices was approved during the RAN plenary meeting #86 [1]. The objectives have been updated in RAN plenary meeting #88e [2]. One of the objectives of the SI is to study solutions for complexity reduction.

# Use cases

Below, we reiterate the application scenarios targeted by NR RedCap as specified in the SID [1]. For each of them, we address the most relevant aspects, i.e., the primary challenges or potential product category differentiators:

- IWSN: 99.99% availability, 100ms E2E latency (5-10 ms if safety critical), DR<2Mbps, stationary

  - Relevant aspect: Power consumption (impacting on the maintenance cost of battery operated devices)

  - UE complexity/cost and form factor do not need to be fully optimized.

- Video surveillance: 99-99.9% availability, E2E latency <500ms, 2-4Mbps (HD) or 7.5-25Mbps (UHD)

  - Relevant aspect: Reduced cost

  - Power consumption and form factor are not critical

- Wearables: average DL 5-50Mbps / UL 2-5Mbps; peak device bit rate [6] up to DL 150Mbps / UL 50Mbps [2]

  - Relevant aspect: Small form factor (essential for wearables), reduced cost and power consumption

  - Arguably, wearables represent the most stringent targets due to the sever form factor limitations and the additional sensitivity to cost and power consumption. However, if these difficulties can be overcome successfully wearables have the potential to offer a large market, which may accelerate the evolution of other market segments across the board.

Economies of scale are essential in the cost optimization of UE’s. Therefore, the applications targeted by RedCap should be supported by as few device types as possible. Ideally, a single type of device should address all the applications.

Observation 1: RedCap SI/WI should aim at bringing the following relevant enhancements by application scenarios:

- Reduced power consumption for IWSN

- Reduced cost for surveillance cameras

- Reduced form factor, cost and power consumption for wearables

Observation 2: Economies of scale can drive the cost reduction for RedCap UE modems. Device types should be defined so as not to fragment the UE modem market. Evolution of a single market segment (e.g. wearables) may play an essential role in enabling other markets across all application scenarios through economies of scale for RedCap UE modems.

# UE Complexity Reduction in FR1

## UE bandwidth reduction

Agreements RAN1#102: - For RedCap UEs in FR1, - The baseline UE bandwidth capability is 20 MHz, which can be assumed during the initial access procedure. - Discuss further by email whether there is an issue or a necessity in achieving up to 150Mbps assuming a 20MHz and rank 1 transmission. Agreements: - For the baseline UE bandwidth capability of RedCap UEs, the same maximum UE bandwidth in a band applies to both RF and baseband. - This maximum UE bandwidth applies to both data and control channels. - This maximum UE bandwidth is assumed for both DL and UL. - Complexity analyses with other mixes of bandwidths are not precluded. The plenary meeting RAN#89e has also clarified [2] that any restrictions on bandwidth below 20 MHz will not be considered. “Note1: The work defined above should not overlap with LPWA use cases. The lowest data rate and bandwidth capability considered should be no less than an LTE Category 1bis modem.”

For FR1, 20 MHz maximum UE bandwidth seems sufficiently large based on the following considerations:

- For wearables, to achieve the targeted peak device data rate [6] of 150 Mbps in downlink, 20 MHz UE bandwidth is sufficient when assuming two layers and 64-QAM or 256-QAM modulation rate.

- UE configuration with 20 MHz maximum bandwidth and 1 Rx layer is sufficient to achieve the application bit rate of 5-50 Mbps targeted in wearables in DL even in TDD operation, and can support a peak device data rate of 109 Mbps assuming 256QAM, which is higher than the peak DL data rate of Cat 3 LTE (102 Mbps).

- The initial BWP is 20MHz wide or less in most existing deployments. Maximum UE bandwidth of 20 MHz is sufficient to align with this.

- The lower the bandwidth, the more constrained the scheduling. However, 20 MHz does not seem to be too restrictive, considered that the operation bandwidth of NR may vary in the range of 5-50 MHz for SCS of 15 kHz, and 10-100 MHz when SCS is 30 or 60 kHz ([5], TS38.101, Table 5.3.5-1 Channel bandwidths for each NR band).

Observation 3: UE configuration with 20 MHz maximum bandwidth and 1 Rx layer is sufficient to achieve the targeted application bit rate of 5-50 Mbps in DL even in TDD operation, and can support a peak device data rate of 109 Mbps assuming 256QAM, which is higher than the peak DL data rate of Cat 3 LTE (102 Mbps).

Proposal 1: For RedCap UEs in FR1, there is no issue if the UEs do not achieve 150Mbps.

## Reduced number of UE RX chains

Agreements: - For FR1, study two antenna configurations for RedCap UEs, namely 1Rx/1Tx and 2Rx/1Tx.

Above 2496 MHz (i.e. for wavelengths below 12 cm, such as for channels n7, n38, n41, n77, n78, n79) the standard mandates 4 UE Rx antennae. Below 2496 MHz the requirement is only 2 UE Rx antennae, since at longer wavelength designing uncorrelated antennae is more constrained for the same form factor whereas the better signal penetration relaxes the receiver sensitivity requirements. For a proportional relaxation of the requirements, it seems a reasonable compromise to consider the following reduction of the number of antennae in FR1 for wearables:

- Above 2496 MHz in FR1: from 4 Rx to 2 Rx antennae

  - 1 or 2 spatial Rx layers?

- Below 2496 MHz: from 2 Rx to 1 Rx antenna

Reduction in the number of Rx antennae is the other primary driver for complexity reduction besides reduced bandwidth. DBB and RF complexity are reduced alike. In FR1 TDD use case, the main complexity reduction (40%-45%) is from 4Rx down to 2Rx, along with the bandwidth reduction. Further reducing the number of Rx antennae to 1Rx may yield 45%-50%, thus hardly justifies the increased loss in performance.

The main concern in reducing the number of antennae is the loss in receive diversity, hence the reduced coverage and potential problem with PDCCH blockage. These adverse effects will probably require enhancements to the current standard (e.g. PDCCH repetitions or extended CORESETs) Reduction to a single Rx chain incur important losses in spectral efficiency, as well, which can only be remedied by baring RedCap UE’s from joining the cell.

Observation 4: The main concern in reducing the number of antennae is the loss in receive diversity, hence the reduced spectral efficiency, coverage and potential problem with PDCCH blockage. These adverse effects will probably require enhancements to the current standard.

The loss in performance may be particularly relevant in the case of the downlink control channel because Release 16 provides limited means to enhance the signal to noise ratio. While the data channels can use repetition and the uplink control channel can use repetitions and long PUCCH formats, the PDCCH is very restricted in time for obvious reasons.  Table 1 shows the simulated decoding statistics for different aggregation levels comparing three UE Rx antennae configurations. At a target reliability 99%, using aggregation level AL16 the difference in performance can be read as 2.28 dB between 4 and 2 antennae and 5.61 dB between 4 and 1 antennae (Figure 1) for 4GHz. The loss between 2 and 1 Rx antenna is 2.56 dB at 700 MHz (Figure 2).

Table 1: Performance loss from reducing the number of antennae.

| PDCCH BLER=1% | 700 MHz |  | 4GHz |  |
| --- | --- | --- | --- | --- |
|  | Req. SNR [dB] | Loss [dB] | Req. SNR [dB] | Loss [dB] |
| 4Rx | - | - | – 11.67 | - |
| 2Rx | – 8.97 | - | – 9.39 | 2.28 |
| 1Rx | – 6.38 | 2.56 | – 6.06 | 5.61 |

Figure 1: PDCCH BLER for AL16 with 4, 2 and 1 Rx antennae at a carrier frequency of 4 GHz.

Figure 2: PDCCH BLER for AL16 with 2 and 1 Rx antennae at a carrier frequency of 700 MHz.

Table 2 presents the performance loss in FR1 TDD for the RedCap configuration cases of 2 Rx chains with 2 Rx MIMO layer and 1 Rx chain, respectively. We observe the sever impact onto the cell-edge coverage, the average spectral efficiency (38% loss), and the dramatic drop of 5%-ile UPT (by 95%) when the number of receive antennae is further reduced from 2Rx to 1Rx. The ~5% additional cost reduction mentioned before must be weighed against such a dramatic performance loss.

Table 2: Number of Rx antennae: Performance vs. complexity reduction trade off

| UE Type | UE BW (MHz) | # of RX Chains | # of Rx layers | DL QAM | Ave. SE (bps/Hz) | UPT 50%-ile (Mbps) | UPT 5%-ile (Mbps) | PDCCH AL16 SINR loss | Peak DL data rate (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RedCap UE FR1 | 20 | 2 | 2 | 256 | 3.086 | 63.05 | 15.55 | - 2.3 dB | 218 |
|  |  | 1 | 1 | 256 | 1.91 | 21.45 | 0.72 | - 5.6 dB | 109 |
| Loss |  | - | - | - | 38% | 66% | 95% | - | 50% |

Proposal 2: In FR1 the number of mandatory Rx antennae is reduced from 2 to 1 in channels below 2496 MHz.

Proposal 3: In FR1 the number of mandatory Rx antennae is reduced from 4 to 2 in channels above 2496 MHz.

Table 3 presents the performance loss in FR1 TDD for 2 Rx chains with 2 Rx MIMO layer and 1 Rx MIMO layer. The drop in median UPT is significant (41%), and the peak device data rate is halved (109 Mbps). For the cell-edge there is minor difference, the 5%-ile UPT is satisfactory w.r.t. the service requirements.  Meanwhile, the reduction to 1 MIMO layer offers negligible cost benefit over the 2Rx RedCap configuration. Once the bandwidth has been reduced and the number of Rx chains is halved, the baseband throughput is reduced by a factor 10, and the complexity gets dominated by components which are unaffected (FFT, CE, DL control, UL, code footprint, etc.)

Table 3: Number of Rx MIMO layers: Performance vs. complexity reduction trade off

| UE Type | UE BW (MHz) | # of RX Chains | # of Rx layers | DL QAM | Ave. SE (bps/Hz) | UPT 50%-ile (Mbps) | UPT 5%-ile (Mbps) | PDCCH AL16 SINR loss | Peak DL data rate (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RedCap UE FR1 | 20 | 2 | 2 | 256 | 3.086 | 63.05 | 15.55 | -2.3 dB | 218 |
|  |  | 2 | 1 | 256 | 2.67 | 36.78 | 15.24 | -2.3 dB | 109 |
| Loss |  | - | - | - | 13% | 41% | 2% | - | 50% |

The above results are illustrated also in Figure 3, along with further scenarios varying the maximum downlink modulation order.

Figure 3: User perceived throughput (UPT) for low-moderate network load (4UEs, FTP3 traffic) simulated with different RedCap UE configurations at a time

Observation 5: The reduction in the number of MIMO layers below the number of receive antennae may only achieve marginal cost saving at the expense of significant drop in peak device data rate and median UPT.  Proposal 4: The number of Rx MIMO layers mandated for RedCap UEs in FR1 is the same as the applicable number of Rx chains mandated.

## Half-duplex FDD

Agreements: - Study HD-FDD operation Type A and Type B (as defined in LTE) in RAN1, where study of Type A is prioritized.

By limiting the UE capability to half-duplex operation in the paired spectrum bands we can dispense with the duplexer, which decreases the link budget. The insertion loss thus spared on the antenna shared between transmit and receive typically amounts to 1-2 dB. This may represent a considerable gain for the coverage recovery, especially as the numbed of Rx antennae is reduced. Replacing the duplexer with a switch in each RF band also saves on cost and area of the RF frontend. The saving may amount to 5% of the reference modem depending on the number of bands.

Observation 6: Half-duplex FDD allows sparing the 1-2 dB insertion loss of the duplexer in the UE for the antenna shared with Tx. Dropping the duplexer per each band also saves on cost and area of the RF frontend.

The operation could be similar to half-duplex FDD LTE: the UE turns into transmit mode when it has an uplink transmission and turns back to reception immediately afterwards. This behavior is different from TDD operation configured in Section 11.1 of TS38.213. The UE needs to report the lack of full-duplex capability to the gNB. With this expectation, the scheduler makes sure that uplink and downlink transmissions of the same RedCap UE do not overlap. Furthermore, CG and SR must not conflict with DCI monitoring or beam tracking. Each half-duplex UE switches between DL and UL at independent points in time, thus scheduling effectiveness may be preserved.

If only the duplexer is dropped then the same UE modem IC can be reused for full duplex and half-duplex equipment design, thus avoiding UE modem market fragmentation. This option matches Type-A operation, where switching between uplink and downlink does not require a significant guard gap in time.

Observation 7: Scheduling effectiveness is not jeopardized by supporting Type-A half-duplex UE’s in paired spectrum, since each UE could switch between DL and UL at independent points in time, according to their respective scheduled or configured uplink transmissions.

Observation 8: With Type-A HD FDD, only the duplexer is dropped, and the same (full-duplex) UE modem can be reused in full-duplex and half-duplex FDD UE designs, thus avoiding UE modem market fragmentation.

To further reduce UE complexity and power peaks in the consumption, a guard gap can also be specified before and after the UE turns into uplink, which is called Type-B operation in LTE. In itself this feature does not reduce the complexity sufficiently to consider a tailored design and does not risk market fragmentation. Type-B HD FDD should not be studied unless necessary for a tailored design that reduces complexity via UE processing timeline and capability restrictions.

Proposal 5: Type-B HD FDD should not be studied further.

A side benefit of half-duplex operation is that it reduces the maximum power peaks in the consumption. This may offer some additional benefits for the design and for the battery life (Li+ batteries deliver less charge at currents above their nominal rating).

## UE processing timeline

Agreements: - For UE complexity reduction through relaxed UE processing time, study a more relaxed UE processing time in terms of N1/N2 compared to capability #1.

The standard specifies several requirements on the UE processing timelines: N1 (5.3, TS38.214), N2 (6.4, TS38.214), CSI (5.4, TS38.214), bandwidth part switching delay (8.6.5, TS38.133), etc. Of these, only N1 (Table 5) and N2- with Capability 1 - matter for the discussion for their direct impact on complexity and latency.

In the IWSN scenario safety critical links require 5-10 ms packet delay budget, and reliability of 99.999%. For such a high reliability and relatively frequent packets one shot transmissions should be avoided unless scheduling encounters some corner cases. That is, at least one retransmission (assuming adaptive HARQ) should fit in comfortably within the 5 ms delay budget. With e.g. half-duplex FDD this can be comfortably achieved in view of the short communication range (no need for repetitions, etc.).

The worst case scenario is assumed by the 2.6 GHz TDD deployment (despite that SCS = 30 kHz), which has 7:1:2 (DL:SP:UL) frame configuration in agreed use case. The goal is to ensure that most of the time the scheduler can retransmit a PDSCH initially transmitted at the end of the DL, in the next DL. In other words the UL needs to be at least as long as N2 + PUCCH alignment delay + PUCCH duration. With N2 = 12 symbols, 0 PUCCH alignment delay, and PUCCH Format 1 or 3 with a length of at least 4 symbols this totals at least 16 symbols. Thus there is some room left for PUCCH alignment delay and/or longer PUCCH format.

Table 4 Safety critical monitoring service requirements in IWSN (from Table 5.3.8.1-1, TR22.804)

| Scenario | E2E Laten-cy | Priority | Data Update Time | Communication service availability | Connections per gateway | Network scalability | Node density | Communication range per node |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition monitoring for safety | 5 ms to 10 ms | Highest | Up to 100 packets/sec | > 99,9999% to 99,999999% | 10 to 100 | > 100 nodes to 1000 nodes | 0,05 m-2 to 1 m-2 | < 30 m |

Table 5: PDSCH processing timeline N1 (Table 5.3-1 and 5.3-2 TS38.214 [10]) and PUSCH preparation time for PUSCH timing N2 (Table 6.4-1 and 6.4-2 TS38.214 [10])

| SCS | N1 with Capability 1 |  | N2 with Capability 1 |
| --- | --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 configured |  |  |
|  | No | Yes |  |
| 15 kHz | 13 | 8 | 10 |
| 30 kHz | 13 | 10 | 12 |
| 60 kHz | 20 | 17 | 23 |
| 120 kHz | 24 | 20 | 36 |

Furthermore, it is difficult to predict how the scope of applicability would be impaired by the relaxation / reduction: take for instance, potential low-latency edge services in the future. As it has been seen for the safety critical monitoring scenario, some IWSN services do not tolerate any relaxation at all, and eventually the brand of RedCap NR could be tarnished in the wider IWSN realm by not supporting these tight latency requirements.

Lastly we need to weigh the constraints imposed on the scheduler and the standardization effort required by a third UE processing capability class, especially given that Capability#1 for N1/N2 is the baseline operation for NR.

As an example for the impact on the standard, consider Section 5.1 in TS 38.214, where the highlighted numbers are based on capability 1:

“In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, and N=24 for =3.”

In view of the above, N1 and N2 UE processing timelines should not be relaxed for RedCap UEs below the baseline NR processing capability. The benefits expected from such a relaxation would not be in proportion to the standardization effort, the impact on NR L1 procedures, the impact on scheduling and the potential limitation on scope of applicability. NR’s low latency allows relatively fast exchanges with server on the edge, which could be required in novel application features and services.

The estimated cost reduction by doubling N1, N2 is in the order of ~1%.

Observation 9: The estimated cost reduction by doubling N1, N2 is in the order of ~1%. Therefore, UE processing timelines relaxation should not be supported in RedCap NR. The benefits would not be in proportion to the standardization effort, the impact on scheduling and the potential limitation on scope of applicability.

## UE processing capability

Following bandwidth and Rx chain reduction, further reduction of the maximum modulation order from 256QAM to 64QAM in the downlink may result in ~2% reduction in the downlink. Meanwhile, the peak data rate is reduced by 25%. As a trade-off between complexity and performance, it is proposed that 256QAM is limited to 64QAM only when the transmission is scheduled onto two layers.

Proposal 6: As a trade-off between complexity and performance, maximum modulation order in the downlink in FR1 is limited to 64QAM only when the transmission is scheduled onto two layers.

The savings are similarly small (~2%) in the uplink when the modulation order is reduced form 64QAM to 16QAM. Meanwhile the peak data rate is reduced by 33%. It seems reasonable that performance is favored over cost reduction because surveillance cameras generate uplink heavy traffic.

The 16 HARQ processes mandated for NR should be maintained for relaxed RTT in the case of RedCap NR as well. The soft buffer complexity estimation should assume that external memory is used, so that it does not scale with the number of HARQ processes.

Observation 10: The soft buffer complexity does not scale with the number of HARQ processes as the implementation can off-load storage to external memory.

# UE Complexity Reduction in FR2

## UE bandwidth reduction

Table 6 compares the performance of the bandwidth options of 100 MHz and 50 MHz. The peak device data rate drops from 808 Mbps to 404 Mbps from 100MHz to 50MHz. Otherwise the DL performance degradation is not relevant.

Table 6: Performance difference between bandwidth option

| FR2 RedCap | Avg. SE bps/Hz | DL UPT 50%-ile Mbps | DL UPT 5%-ile Mbps | Peak DL data rate Mbps |
| --- | --- | --- | --- | --- |
| 200 MHz -> 100 MHz | 1.047 | 114.13 | 61.3 | 808 |
| 200 MHz -> 50 MHz | 0.991 | 44.38 | 21.09 | 404 |

However, the bandwidth option of 50 MHz imposes sever restriction on FDM and causes performance degradation on transmissions that will not fit in entirely in the bandwidth (e.g. PBCH at SCS=240 kHz or CORESET configurations spanning 67 MHz). Potential coverage recovery techniques for initial access and downlink control scheduling could benefit from FDM, too, which 50 MHz UE bandwidth would not allow. This is a serious drawback in a system where TDM is costly due to analog beamforming.

Observation 11: The 50MHz option may degrade performance of initial access and imposes sever restrictions on FDM, which could be a handicap for coverage recovery solutions, too.

The difference in complexity is not too significant since the throughput is already diminished by a factor 2 by the 100 MHz configuration, impacting on the baseband whereas the RF path has a large share in the cost and hardly scales with the bandwidth.

## Reduction in number of UE Rx chains

The reduction in receive chains has a high impact on the complexity of the RF path and DBB alike.  However, the impact on the performance is also considerable, both for UEs near the BS and those at the edge of the cell (Table 7).

Table 7: Performance difference between bandwidth option

| FR2 RedCap | Avg. SE bps/Hz | DL UPT 50%-ile Mbps | DL UPT 5%-ile Mbps | Peak DL data rate Mbps |
| --- | --- | --- | --- | --- |
| 100 MHz; 2 Rx, 2 MIMO layers | 3.4 | 114.13 | 61.3 | 808 |
| 100 MHz; 1 Rx | 1.8 | 33.55 | 11.28 | 404 |

To assess the best trade-off between performance and complexity, it should be studied whether application scenarios with potentially down-link heavy traffic scenarios (such as wearables) would be targeted by RedCap devices, on one hand. On the other hand, coverage and reliability for DL control channel must be ensured for uplink-heavy traffic scenarios, as well. Therefore, coverage recovery techniques for PDCCH may also influence the design choice. The number of Rx chains in FR2 may be reduced if uplink heavy traffic is targeted and the DL control channel coverage and reliability can be ensured.

Observation 12: The number Rx chains in FR2 may be reduced if uplink heavy traffic is targeted and the DL control channel coverage and reliability can be ensured.

## Reduction in maximum modulation order

Following bandwidth and Rx chain reduction, further reduction of the maximum modulation order from 64QAM to 16QAM in the downlink may result in ~3% reduction in the downlink. Meanwhile, the peak data rate is reduced by 33% and the spectral efficiency drops by a similar amount for median UEs..

Table 8: Performance impact of maximum modulation order in FR2

| UE Type | UE BW (MHz) | # of RX Chains | # of Rx layers | DL QAM | Ave. SE (bps/Hz) | Cell edge SE (bps/Hz) | Peak DL data rate (Mbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 indoors | 100 | 2 | 2 | 64 | 3.433 | 0.149 | 808 |
|  | 100 | 2 | 2 | 16 | 2.357 | 0.130 | 540 |
|  |  |  |  |  | -31% | -12% |  |
|  | 50 | 2 | 2 | 64 | 3.135 | 0.132 | 392 |
|  | 50 | 2 | 2 | 16 | 2.193 | 0.130 | 260 |
|  |  |  |  |  | -30% | -2% |  |

The savings are small (1-2%) in the uplink when the modulation order is reduced form 64QAM to 16QAM. Meanwhile, the peak data rate is reduced by 33%. It seems reasonable that performance is favored over cost reduction because surveillance cameras generate uplink heavy traffic.

Proposal 7: Maintain 64-QAM as mandatory in both downlink and uplink for RedCap UEs in FR2.

# Conclusions

In this contribution we made the following observations and proposals.

On RedCap use cases and targets:

Observation 1: RedCap SI/WI should aim at bringing the following relevant enhancements by application scenarios:

- Reduced power consumption for IWSN

- Reduced cost for surveillance cameras

- Reduced form factor, cost and power consumption for wearables

Observation 2: Economies of scale can drive the cost reduction for RedCap UE modems. Device types should be defined so as not to fragment the UE modem market. Evolution of a single market segment (e.g. wearables) may play an essential role in enabling other markets across all application scenarios through economies of scale for RedCap UE modems.

On the relaxation of the maximum RedCap UE bandwidth in FR1:

Observation 3: UE configuration with 20 MHz maximum bandwidth and 1 Rx layer is sufficient to achieve the targeted application bit rate of 5-50 Mbps in DL even in TDD operation, and can support a peak device data rate of 108 Mbps assuming 256QAM, which is higher than the peak DL data rate of Cat 3 LTE (102 Mbps).

Proposal 1: For RedCap UEs in FR1, there is no issue if the UEs do not achieve 150Mbps.

On the number of mandated RedCap UE Rx antennae and MIMO layers in FR1:

Observation 4: The main concern in reducing the number of antennae is the loss in receive diversity, hence the reduced coverage and potential problem with PDCCH blockage. These adverse effects will probably require enhancements to the current standard.

Proposal 2: In FR1 the number of mandatory Rx antennae is reduced from 2 to 1 in channels below 2496 MHz.

Proposal 3: In FR1 the number of mandatory Rx antennae is reduced from 4 to 2 in channels above 2496 MHz.

Observation 5: The reduction in the number of MIMO layers below the number of receive antennae may only achieve marginal cost saving at the expense of significant drop in peak device data rate and median UPT.

Proposal 4: The number of Rx MIMO layers mandated for RedCap UEs in FR1 is the same as the applicable number of Rx chains mandated.

On half-duplex FDD operation for RedCap UE in FR1 FDD channels:

Observation 6: Half-duplex FDD allows sparing the 1-2 dB insertion loss of the duplexer in the UE for the antenna shared with Tx. Dropping the duplexer per each band also saves on cost and area of the RF frontend.

Observation 7: Scheduling effectiveness is not jeopardized by supporting Type-A half-duplex UE’s in paired spectrum, since each UE could switch between DL and UL at independent points in time, according to their respective scheduled or configured uplink transmissions.

Observation8: With Type-A HD FDD, only the duplexer is dropped, and the same (full-duplex) UE modem can be reused in full-duplex and half-duplex FDD UE designs, thus avoiding UE modem market fragmentation.

Proposal 5: Type-B HD FDD should not be studied further.

On RedCap UE timelines:

Observation 9: The estimated cost reduction by doubling N1, N2 is in the order of ~1%. Therefore, UE processing timelines relaxation should not be supported in RedCap NR. The benefits would not be in proportion to the standardization effort, the impact on scheduling and the potential limitation on scope of applicability.

On RedCap UE capability:

Proposal 6: As a trade-off between complexity and performance, maximum modulation order in the downlink in FR1 is limited to 64QAM only when the transmission is scheduled onto two layers.

Observation 10: The soft buffer complexity does not scale with the number of HARQ processes as the implementation can off-load storage to external memory.

On the relaxation of the maximum RedCap UE bandwidth in FR2:

Observation 11: The 50MHz option may degrade performance of initial access and imposes sever restrictions on FDM, which could be a handicap for coverage recovery solutions, too.

On the number of mandated RedCap UE Rx chains in FR2:

Observation 12: The number Rx chains in FR2 may be reduced if uplink heavy traffic is targeted and the DL control channel coverage and reliability can be ensured.

On the mandated maximum modulation order in FR2:

Proposal 7: Maintain 64-QAM as mandatory in both downlink and uplink for RedCap UEs in FR2.

# References

- RP-193238 “New SID on support of reduced capability NR devices”, Ericsson, RAN#86, December 2019

- RP-201386 “Revised SID on Study on support of reduced capability NR devices”, Ericsson, RAN#88e, July 2020

- R1-2004962, skeleton of TR38.875 “Study on support of reduced capability NR devices”, RAN1#101-e, May, 2020

- 3GPP TR 36.888, “Study on provision of low-cost Machine-Type Communications (MTC) User Equipments (UEs) based on LTE (Release 12)”, 2013

- 3GPP TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)”, V16.3.0 (2020-03)

- 3GPP TS 38.306, “NR, User Equipment (UE) radio access capabilities” V15.8

- 3GPP TS 38.211 “NR; Physical channels and modulation”

- 3GPP TS 38.212 “NR; Multiplexing and channel coding”

- 3GPP TS 38.213 “NR; Physical layer procedures for control”

- 3GPP TS 38.214 “NR; Physical layer procedures for data”

- 3GPP TR 38.802 “Study on new radio access technology Physical layer aspects”

- 3GPP TR 38.804 “Study on new radio access technology Radio interface protocol aspects”

- 3 GPP TR 22.804 “Study on Communication for Automation in Vertical domains (CAV)” V16.2.0, 2018

- 3GPP TS 38.104 “NR; Base Station (BS) radio transmission and reception”

# Appendix

Link-level simulation settings for PDCCH performance evaluation are listed in Table 9 and Table 10.

Table 9: Link-level simulation settings for PDCCH performance evaluation at 4 GHz

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 700 MHz |
| Channel model | Urban TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h |
| BS antenna configuration | 4 TxRU’s 16 antenna elements (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| UE antenna configuration | 4, 2 and 1 Rx antenna ports |
| System bandwidth | 20 MHz |
| Sub-carrier spacing | 15 kHz |
| Channel estimation | Practical |
| PDCCH | 40 bits payload, AL16 |
| CORESET | 2 symbols, 48 PRBs |

Table 10: Link-level simulation settings for PDCCH performance evaluation at 4 GHz

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4 GHz |
| Channel model | Urban TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h |
| BS antenna configuration | 64 TxRU’s 192 antenna elements (M,N,P,Mg,Ng) = (12,8,2,1,1) |
| UE antenna configuration | 4, 2 and 1 Rx antenna ports |
| System bandwidth | 20 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |
| PDCCH | 40 bits payload, AL16 |
| CORESET | 2 symbols, 48 PRBs |

Agreements: - Use the TR 36.888 methodology for UE cost/complexity evaluation as a starting point and determine what major updates are needed. - Cost/complexity breakdowns can be separate for FR1 and FR2 if found beneficial. Agreements: The reference NR device for evaluation of cost/complexity reduction supports the following: - All mandatory Rel-15 features (with or without capability signalling) - Single RAT - Operation in a single band at a time - Maximum bandwidth: - For FR1: 100 MHz for DL and UL - For FR2: 200 MHz for DL and UL - Antennas: - For FR1 FDD: 2Rx/1Tx - For FR1 TDD: 4Rx/1Tx - For FR2: 2Rx/1Tx - Power class: PC3 - Processing time: Capability 1 - Modulation: - For FR1: support 256QAM for DL and 64QAM for UL - For FR2: support 64QAM for DL and 64QAM for UL - Access: Direct DL/UL access between UE and gNB Note: The study will consider impacts on the cost/complexity reduction from support of multiple RF bands within FR1 or FR2.

Table 11: Configuration used for system level performance evaluation at 2.6GHz

| Parameters | FR1 values |  |
| --- | --- | --- |
| Layout | Single layer Macro layer: Hex. |  |
| Inter-BS distance | 500 m |  |
| Scenario and Frequency | Dense Urban: 2.6 GHz (TDD) | 4 GHz (TDD) Secondary choice |
| SCS | 30 kHz |  |
| TDD Configuration | DDDDDDDSUU (S: 6D:4G:4U) | DDDSUDDSUU (S: 10D:2G:2U) |
| Channel Model | 3D UMa / TDL-C |  |
| UE distribution and number per cell | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h {0%, 100%} RedCap UE’s; 10 UEs in total |  |
| UE BW, layers, modulation-order | eMBB UE: 100 MHz, Up to 4 layers, DL 256QAM and UL 64QAM. RedCap UE: 20MHz, Up to 2 layers for 2Rx, 1 layer for 1RX, DL 64QAM and UL 16QAM |  |
| UE antenna configurations | eMBB UE: 4Rx/1Tx ; (M,N,P,Mg,Ng, Mp,Np) = (1,2,2,1,1,1,2) 4R for DL RedCap UE: {2Rx/1Tx , 1Rx/1Tx} ; (M,N,P,Mg,Ng, Mp,Np) = (1,1,2,1,1,1,1) 2R for DL 1TX: (1,1,1,1,1,1,1) for UL |  |
| UE antennae correlation and height | correlation: low; height: 1.5m |  |
| BS antenna configurations, height | (M,N,P,Mg,Ng) = (12,8,2,1,1), 64TXRUs; height: 25 m 4T/4R |  |
| Scheduling | SU-PF |  |
| BS antenna gain – 3dB connector loss [dBi] | 17.77 dB | 17.77 dB |
| UE antenna gain – 1dB connector loss [dBi] | eMBB: -1 dB ; RedCap: -4 dB | eMBB: -1 dB ; RedCap: -4 dB |
| BS Tx power density [dBm/MHz] , (sys. BW) | 33dBm/MHz (100MHz) | 24dBm/MHz (100MHz) |
| UE Tx Power [dBm] | 23 | 23 |
| BS receiver noise figure [dB] | 5 | 5 |
| UE receiver noise figure [dB] | 7 | 7 |
| Receiver, CE | MMSE-IRC; Realistic CE and feedback |  |
| Data Traffic Model | FTP Model 3 with 0.5Mbyte payload and mean inter-arrival time of 200 ms; Full buffer traffic is optional |  |

Table 12: Configuration used for system level performance evaluation at 28GHz

| Parameters | FR2 values |
| --- | --- |
| Layout | Single layer ; Indoor floor: (12BSs per 120m x 50m) , 12 TRP |
| Inter-BS distance | 20 m |
| Scenario and Frequency | Indoor: 28 GHz (TDD) |
| SCS | 120 kHz |
| TDD Configuration | DDDSU (S: 10D:2G:2U) |
| Channel Model | 5GCM office / TDL-A, CDL-A (optional) |
| UE distribution and number per cell | 100% Indoor: 3km/h {0%, 100%} RedCap UEs; 10 UE’s in total |
| UE BW, layers, modulation-order | eMBB UE: 100 MHz, Up to 2 layers, DL 64QAM and UL 64 QAM RedCap: UE { 50 or 100 } MHz, Up to 2 layers, DL 16QAM and UL 16QAM |
| UE antenna configurations | eMBB and RedCap UE: 2Rx/1Tx ; (M,N,P,Mg,Ng, Mp,Np) = (2,2,2,1,1,1,1) for 2RX and (2,2,1,1,1,1,1) for 1RX 1TX: (2,2,1,1,1,1,1) for UL |
| UE antennae correlation and height | correlation: low; height:1.5m |
| BS antenna configurations, height | (M,N,P,Mg,Ng) = (8,8,2,1,1), 2TXRUs; height: 3 m |
| Scheduling | SU-PF |
| BS antenna gain – 3dB connector loss [dBi] | 23.06 |
| UE antenna gain – 1dB connector loss [dBi] | Both eMBB and RedCap UEs: 10.02dB |
| BS Tx power density [dBm/MHz] , (sys. BW) | 3dBm/MHz (PDSCH of 60PRB --> ~23dBm) |
| UE Tx Power [dBm] | 23 |
| BS receiver noise figure [dB] | 7 |
| UE receiver noise figure [dB] | 13 |
| Receiver, CE | MMSE-IRC; Realistic CE and feedback |
| Data Traffic Model | FTP Model 3 with 0.5Mbyte payload and mean inter-arrival time of 200 ms; Full buffer traffic is optional |