3GPP TSG RAN WG1 #103e			R1-2008558

e-Meeting, October 26th – November 13th, 2020

Source:	NTT DOCOMO, INC.

Title:	Baseline coverage performance for FR2

Agenda Item:	8.8.1.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#86 meeting, the new SI on NR coverage enhancement was approved [1]. The objectives of the SI are captured as follows.

The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows. - The target scenarios and services include - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1 - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2. - TDD and FDD for FR1. - VoIP and eMBB service for FR1. - eMBB service as first priority and VoIP as second priority for FR2. - LPWA services and scenarios are not included. - Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation - UL channels (including PUSCH and PUCCH) are prioritized for FR1. - Both DL and UL channels for FR2. - Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services - The target channels include at least PUSCH/PUCCH - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions) - Study the additional enhanced solutions for FR2 if any - Evaluate the performance of the potential solutions based on link level simulation.

At the RAN1#102-e meeting, the general parameters for the MIL and MCL evaluation were defined [2], and the link budget template was also defined in the post-meeting e-mail discussion. In this contribution, we discuss on the link budget evaluation for baseline coverage performance for both DL and UL based on MIL derived with the defined link budget template for FR2.

- Discussion on link budget evaluation for baseline coverage performance for FR2

2.1 Target performance

At the RAN1#102-e meeting, link budget template and target performance were discussed and following agreements were made [2].

- Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters - For LLS based methodology, coverage bottleneck(s) identification is performed using at least MIL or MCL (assuming the set of simuation assumptions) - Even when SLS is used to obtain some components of MIL or MCL, it is categorized as LLS based methodology. - MCL values can also be used to identify the coverage bottleneck(s) when applicable - “applicable” above means the following situation: - [comparing channels with similar antenna (and antenna array) gain, and/or - the simulation results with MIL from companies are diverse, and the comparison with MIL is not easy]

At the meeting, MIL and MCL were selected as candidate metrics for the coverage bottleneck(s) identification. In case of including antenna and beamforming gains in the link level simulation, they can be considered in the link budget. Therefore, MIL is sufficient to define the target performance for enhancement.

Proposal 1: MIL is used for the target metric of link budget for NR coverage enhancement.

Defining the target performance for the link budget metric is also one of the important discussion points and following two options were discussed at the last RAN1 meeting.

Opt.1: Absolute value (e.g. derived based on ISD)

Opt.2: Relative value (e.g. value of 2nd best channel is defined as target performance)

Defining the target performance based on ISD (Opt.1) seems the straightforward approach to define the target performance for the link budget metric. On the other hand, it may not reflect the real environment / deployments that have large difference over scenarios and variability due to e.g. channel conditions, and hence it may be very difficult to define a single appropriate value to consider the variety of conditions. In addition, it is concerned that no issue for the coverage performance may be found if a lower target performance is defined by considering a certain environment/condition.

We will find the performance difference between physical channels, and the improvement of coverage performance will be realized by minimizing the performance difference i.e., enhancing the performance of bottleneck channel(s). Therefore, deriving the target performance based on relative value between physical channels is preferable for the coverage enhancement.

Proposal 2: Relative value is used for the target performance definition for NR coverage enhancement.

2.2 PUCCH short format (format 0 and 2)

From a NW operation point of view, since large number of BS antenna beams would be used in FR2 to compensate coverage performance, PUCCH with long duration may not be appropriate for NW operation in FR2 due to large overhead with analogue beam constraint. Figure 1 shows PUCCH resource mapping for SR with considering BS antenna beam sweeping. As shown in Fig.1, in case of 14 symbols for PUCCH long format, PUCCH for SR needs to be configured for e.g., 24 UL slots (in case that the number of BS antenna beams is 24), while 4 slots are sufficient for the same number of beams with PUCCH short format with 2 symbols duration. Therefore, PUCCH short format should be considered for the baseline coverage performance for FR2 with considering practical NW operation of using large number of BS antenna beams.

Figure 1: PUCCH resource mapping with considering BS antenna beam sweeping.

Proposal 3: PUCCH short format should be considered for baseline coverage performance for FR2 with considering practical NW operation of using large number of BS antenna beams.

- MIL evaluation for FR2

In this section, our MIL evaluation results are introduced. The defined simulation assumptions are used for the link level simulation as listed in Table 1.

Table 1: Channel-specific parameters for initial MIL evaluation

| Channel Assumptions |  | PDCCH | PDSCH | PUCCH | PUSCH |
| --- | --- | --- | --- | --- | --- |
| QoS Target | VoIP | 1% BLER | 2% rBLER | NACK to ACK probability: 0.1% ACK missed detection probability: 1% | 2% rBLER |
|  | eMBB |  | 10% iBLER | 1% BLER | 10% iBLER |
| Max number of HARQ retransmissions | VoIP | - | 8 | - | 8 |
|  | eMBB |  | - |  | - |
| Number of RBs |  | 48 | 1 for VoIP 40 for eMBB | 1 | 4 for VoIP 30 for eMBB |
| MCS number |  | - | 10 | - | 11 |
| Aggregation level |  | 16 | - | - | - |
| Frequency hopping |  | - | - | 32 | 24 for VoIP |
| Number of repetition |  | - | - | 4 | - |
| PUCCH format |  | - | - | Format 0/1 for VoIP Format 2/3 for eMBB | - |
| Symbol duration |  | 2 | 12 | 14 for format 1/3 2 for format 0/2 | 14 |
| Payload size |  | 40 bits | - | 2 bits for VoIP 11 bits for eMBB (short format) 22 bits for eMBB (long format) | - |

Figure 2 shows the link level simulation results of BLER for PDSCH/PUSCH/PDCCH channels for Indoor and Urban scenarios with VoIP and eMBB. As shown in Fig.2, different scenarios (Indoor and Urban scenarios) do not provide remarkable difference for BLER performance. Based on the simulated BLER results, corresponding MILs are derived using the calculation template.

Figure 2: Link level simulation results for PDSCH/PUSCH/PDCCH channels

Figure 3 shows the link level simulation results for PUCCH for Indoor and Urban scenarios with VoIP and eMBB. The performance metric for VoIP is NACK to ACK probability and ACK missed detection probability, whereas the metric in eMBB is BLER. As shown in Fig.3, the performance of PUCCH channel gets worse as the OFDM symbol duration is shorter. The required SINR is derived so as to achieve both of the target NACK to ACK probability (0.1%) and the target ACK missed detection probability (1%). Based on the simulated results, corresponding MILs are derived using the link budget calculation template as in the zip archive.

Figure 3: Link level simulation results for PUCCH channels

Figure 4 shows the MIL evaluation results for FR2 for Indoor and Urban scenarios with VoIP and eMBB where PUCCH long format is considered. According to the proposal 2, the MIL performance target is set at the 2nd best value among different channels for each scenario, so that an improvement of worst and 2nd worst channels need to be considered. As shown in Fig.4, the improvement of PUSCH (approximately 21 dB) and PDSCH (approximately 8 dB) is the most essential point for the coverage enhancement for Urban eMBB for FR2. It should also be noted that the improvement of PDSCH (approximately 17 dB) and PDCCH (approximately 19 dB) is also essential for Indoor VoIP, where 23 dBm/100MHz is defined as BS Tx power density for Indoor scenario.

Figure 4: MIL evaluation results for FR2 (PUCCH long format).

Figure 5 shows the MIL evaluation results for FR2 for Indoor and Urban scenarios with VoIP and eMBB where PUCCH short format is considered. The MIL performance target is set at the value of 2nd best channel for each scenario, so that an improvement of worst and 2nd worst channels need to be considered. As shown in Fig.5, the improvement of PUCCH short format (approximately 13 dB) is also essential for the coverage enhancement for Urban VoIP for FR2.

Figure 5: MIL evaluation results for FR2 (PUCCH short format).

Proposal 4: Improvement of both PDSCH and PDCCH should be considered for VoIP scenario, and improvement of both PDSCH and PUSCH should be considered for eMBB scenario for FR2 coverage enhancement.

Proposal 5: Improvement of PUCCH short formats should be also considered for FR2 coverage enhancement.

- Conclusion

In this contribution, we discussed on the simulation assumptions and results for baseline coverage performance for both DL and UL for FR2. Based on the discussion, we made following proposals.

Proposal 1: MIL is used for the target metric of link budget for NR coverage enhancement.

Proposal 2: Relative value is used for the target performance definition for NR coverage enhancement.

Proposal 3: PUCCH short format should be considered for baseline coverage performance for FR2 with considering practical NW operation of using large number of BS antenna beams.

Proposal 4: Improvement of both PDSCH and PDCCH should be considered for VoIP scenario, and improvement of both PDSCH and PUSCH should be considered for eMBB scenario for FR2 coverage enhancement.

Proposal 5: Improvement of PUCCH short formats should be also considered for FR2 coverage enhancement.

References

- 3GPP, RP-193240, China Telecom, “New SID on NR coverage enhancement,” Dec. 2019.

- 3GPP, RAN1#102e, RAN1 Chairman’s Notes, Sep. 2020.