3GPP TSG RAN WG1 #103-e	R1-2008568

e-Meeting, October 26th – November 13th, 2020

Agenda item:		8.3.2

Source:		Nokia, Nokia Shanghai Bell

Title:	UL enhancements for IIoT/URLLC in unlicensed controlled environment

Document for:		Discussion and Decision

# 1	Introduction

The revised work item description for “Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR” was agreed at RAN#88-e [1].

Specifically, the objectives regarding the support of unlicensed operation on FR1, considering controlled environments, were defined as follows:

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]: - Specify support for UE-initiated COT for FBE with minimum specification effort - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In RAN1 meeting #102-e the following agreements related to these objectives were reached [2]:

Agreements: - For semi-static channel access mode, - If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity. - For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported. Agreements: - For semi-static channel access mode, - When gNB operates as an initiating device - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT - When a UE operates as an initiating device - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB - When the gNB shares a COT initiated by a UE during an FFP associated with the UE - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE - FFS whether/how to support additional restrictions to the idle period Agreements: - For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode - FFS the case when the UE is IDLE/INACTIVE mode Agreements: - A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst. Update on 8/26 Agreements: - At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum. Conclusion: Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision. Agreements: - Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles. Agreements: - UE-to- gNB COT sharing in semi-static channel access mode is supported. - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP. - FFS details - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP. - FFS whether/how UE to gNB COT sharing when the gap is >16us Update from 8/28 GTW Agreements: For semi-static channel access mode, o Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT. o FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT. Agreements: - For semi-static channel access mode, - FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling. - FFS on to be provided by SIB-1 - FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

In this contribution we discuss FFSs from the RAN1 meeting #102-e agreements, and further details about the UL enhancements for URLLC in unlicensed controlled environments which are part of the WI.

# 2	UL enhancements for URLLC in unlicensed band

## 2.1 NR/NR-U CG enhancements harmonization

The assumption for the WI is that the operation in the unlicensed spectrum occurs in a controlled environment, i.e. where interference from other systems is not expected. In that sense, as observed in [3] the possibility of LBT failure would depend on the deployment. If in a controlled facility it can be ensured that LBT failures are negligible, then NR IIoT based CG (defined primarily for licensed spectrum use) can potentially be employed without NR-U specific enhancements. While if the effect of potential LBT failures are considered, due to intra-system interference or sporadic interference from other RAT, it is beneficial for IIoT/URLLC to use the NR-U CG mechanisms, which were designed to overcome the impact of frequent LBT failures.

HARQ and COT sharing impact

For the case where potential LBT failures are not considered and NR-U CG mechanisms are not utilized, the HARQ operation can follow NR IIoT Rel-16 design for CG. That includes HARQ process selection based on CG PUSCH timing, implicit HARQ-ACK feedback, etc. In that case, the UE wouldn’t need to indicate HARQ related information such as HARP Process ID, RV and NDI through CG-UCI. However, the CG-UCI is also used in NR-U for indicating COT sharing information, so the impact of exempting the UE to transmit CG-UCI when using NR IIoT Rel-16 based CG in unlicensed band is that gNB cannot make use of UE acquired COT.

Proposal 1: Two operation modes can be considered independently; NR IIoT Rel-16 based CG with NR based HARQ procedure and without CG-UCI, and NR-U based CG including CG-UCI and possibility of UE COT sharing.

Time domain consecutive allocations vs. Intra-slot repetition

There are clear differences between the types of time domain resource allocations supported in NR-U CG and Rel-16 IoT:

- NR-U introduced support for an allocation of a burst of consecutive slots (cg-nrofSlots) and multiple CG PUSCH resources within a slot (cg-nrofPUSCH-InSlot) configured by RRC,

- Rel-16 for IIoT introduced a support for PUSCH repetition type-B for intra-slot repetition including actual repetitions across slot boundary.

At least, the use of PUSCH repetition type-B can be considered for unlicensed band operation, if no NR-U specific enhancements are used for the case of no potential LBT failure. The combined design of these features was left out of the scope of Rel-17 WI according the plenary discussions.

Proposal 2: PUSCH repetitions type-B should be supported for unlicensed band operation when using NR IIoT Rel-16 based CG, without NR-U specific enhancements. FFS: required spec changes, if any.

Proposal 3: The use of PUSCH repetition type-B together with NR-U based multi-slot allocations should not be considered.

CG retransmission timer

As agreed in RAN1#102-e meeting, at least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated for CG. In our view, it is up to gNB to decide whether cg-RetransmissionTimer should be configured, for example, depending on whether the effect of potential LBT failures are assumed to be negligible or not. In case cg-RetransmissionTimer is not configured, at least autonomous retransmission and explicit ACK by DFI should clearly not be used for NR-U CG.

However, it is not clear whether other NR-U CG functionalities can be used such as HARQ process selection by UE, CG-UCI (including COT sharing), NR-U CG specific time domain resource allocation, etc. In our view, in deployments where LBT failures are not considered so that cg-RetransmissionTimer is not configured, then other NR-U enhancements to overcome LBT failures are also not needed, and NR IIoT based CG should be adopted. It is up to RAN2 to decide which parameter should be used for determining if NR-U CG or NR IIOT CG procedure is to be followed.

Proposal 4: When cg-RetransmissionTimer is not configured, NR-U specific CG features such as CG-UCI, COT sharing indication, UE-selected HARQ process ID,  and consecutive allocations are not supported.

CG-UCI and PUCCH multiplexing and prioritization

Rel-16 NR-U CG features include CG-UCI multiplexing with PUCCH on the CG-PUSCH, when a PUCCH carrying HARQ-ACK overlaps with the CG-PUSCH within a PUCCH group if the higher layer parameter cg-UCI-Multiplexing is configured. Otherwise, if that parameter is not configured and the overlap occurs, the CG-PUSCH is skipped, i.e. the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information on a PUCCH transmission or on another PUSCH transmission according TS 38.213.

Rel-16 IIoT CG features include UCI enhancements which allow two HARQ-ACK codebooks of different priorities to be constructed. Separate PUCCH configurations for each priority are provided to the UE, and the priority is indicated in the scheduling DCI. If two physical channels of different priority, e.g. PUCCH carrying HARQ-ACK and CG-PUSCH, overlap in time, the UE should transmit the one of higher priority. On the other hand, if the channels have the same priority, the UE should multiplex the information, e.g. HARQ-ACK is multiplexed on PUSCH.

For Rel-17 IIoT, the PHY layer multiplexing of channels of different priorities  is being discussed under Agenda Item 8.3.3 . In RAN1#102-e meeting, different multiplexing scenarios were agreed including the multiplexing of HARQ-ACK and PUSCH with/without UCI. In addition, the following FFS item was also listed, which may relate to NR-U CG:

Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only). - Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only) - Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI. - Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI. For the above multiplexing scenarios, - Support separate configurations of at least beta-offset values (FFS for alpha) for multiplexing with different priority combinations. - FFS for other separate configurations. - FFS: value range of beta-offset (e.g. <1). - FFS the conditions, if needed, for multiplexing, e.g. - FFS: Whether to support multiplexing in case a PUCCH/PUSCH overlaps with more than one PUCCH/PUSCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling)? - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding). - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority). - How to handle multiplexing of UCI of different priorities and CG-UCI in a CG-PUSCH

We see that prioritization of UCI introduced in Rel-16 for licensed spectrum should also be considered in the case of unlicensed spectrum operation, including the case when CG-UCI is present and NR-U specific CG enhancements are used for deployments where sporadic LBT issues are non-negligible for IIOT. The exact UE behavior for UCI multiplexing/prioritization in the presence of CG-UCI is FFS.

Proposal 5: PHY multiplexing/prioritization introduced in Rel-16 is supported also with NR-U CG. Interaction of CG-UCI and HARQ-ACK codebooks of different priorities is FFS.

Although the related FFS point was listed under the Agenda Item 8.3.3 (intra-UE multiplexing/prioritization), it is not very clear whether this topic should also be discussed under the agenda item 8.3.2 (URLLC unlicensed).

Proposal 6: RAN1 should decide which Agenda Item (8.3.2 or 8.3.3) to handle the issue of UCI multiplexing and prioritization on unlicensed spectrum.

## 2.2 On the support for UE-initiated COT for FBE

Introducing support for UE-initiated COT with semi-static channel occupancy requires the configuration of a UE FFP.  The UE FFP configuration should at least include an FFP periodicity (equal to the sum of the semi-static channel occupancy time and the idle period) and an FFP start. It has already been agreed in RAN1 meeting #102-e that the start of the FFP for UE-initiated COT can be different from the start of the FFP for gNB-initiated COT.

The FFP periodicity for UE-initiated COT determines how often UL resources may be available. Therefore, it impacts the latency of transmission when using e.g. UL CG with UE-initiated COT and semi-static channel occupancy. On the other hand, to reduce the number of LBT checks at the gNB when operating in a controlled environment, a gNB may prefer to operate with relatively long FFP periodicity.

Therefore, we propose that the FFP periodicity for UE-initiated COT can be configured to be different from the FFP periodicity for gNB-initiated COT.

Proposal 7: FFP periodicity for a UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Based on the agreements in RAN1 meeting #102-e:

- An initiating device is not allowed to transmit during the idle period of any FFP associated with the initiating device in which the initiating device initiates a COT

- A responding device is not allowed to transmit during the idle period of that FFP in which the responding device shares the COT initiated by the initiating device

It is FFS whether and how to support additional restrictions. For example, TS 37.213 currently specifies that the gNB and UEs shall not transmit any transmissions during the idle period of the gNB FFP. Based on what was agreed for NR-U in Rel-16, these restrictions assume the gNB is always operating as initiating device, and the UEs always as responding devices. However, with the introduction of support for UE-initiated COT with semi-static channel occupancy, these restrictions should be reconsidered.

For instance, assuming that the semi-static channel occupancy time of the FFP of one device can collide with the idle period of the FFP of another device, it should be discussed, whether an initiating device should be allowed to transmit during the idle period of another device. As with semi-static channel occupancy the target is not to exceed an overall channel occupancy higher than 90% to 95% (depending on the FFP periodicity and duration of the idle period), it seems enough to prevent initiating devices from transmitting during their idle periods. However, in order to allow gNBs to initiate a COT, and possibly share it with other UEs, at least as a default, UEs should avoid transmitting in correspondence of the serving gNB idle period, even when operating as initiating devices.

Proposal 8: As initiating device, the gNB can transmit during any UE FFP idle periods.

Proposal 9: As initiating device, a UE can transmit during other UEs FFP idle periods.

Proposal 10: By default, as the initiating device, a UE cannot transmit during its serving gNB’s idle period.

On the other hand, avoiding UE-initiated transmissions in correspondence of the serving gNB idle period may introduce unnecessary delays and lower the overall spectral efficiency if, for example, the gNB has no intention to acquire the COT in the subsequent FFP. Therefore, it is suggested to introduce support for gNB-controlled UE-initiated UL transmissions during gNB idle periods. Details are FFS.

Proposal 11: Introduce support for gNB-controlled UE-initiated UL transmissions during gNB idle periods, when the gNB has no intention to acquire the COT in the subsequent FFP.

In RAN1 meeting #102-e it was agreed to introduce support for UE-to-gNB COT sharing. During a UE-initiated COT, the gNB may still have the possibility to initiate its own COT. If the gNB can initiate its own COT and transmits e.g. a grant towards the UE, the UE may not be able to reliably determine whether it should operate as responding device based on the shared gNB COT, or as initiating device based on the UE-initiated COT. In LBE, when scheduled to perform an UL transmission, the UE can always determine based on the LBT type indicated in the DCI, whether the corresponding PUSCH transmission is within a shared gNB COT (Type 2 LBT) or outside a shared gNB COT (Type 1 LBT). This provides additional robustness on top of determination of presence of gNB shared COT based on detection of other DL signal(s). For example, the UE may not perform additional checks if the DCI scheduling PUSCH transmission indicates a Type 2A/B/C LBT. If this property is to be maintained for FBE operation, RAN1 should discuss how the UE can determine, based on information in the DCI, whether a scheduled PUSCH transmission is within a shared gNB COT, or should be transmitted based on UE-acquired COT (i.e. UE FFP).

Proposal 12: A UE should be able to determine, exclusively from information in the scheduling DCI, whether a scheduled PUSCH transmission should be transmitted according to shared gNB COT or UE-initiated COT.

In RAN1 meeting #102-e it was agreed to support UE-initiated COT for semi-static channel access mode by any UL transmission in RRC_CONNECTED mode, while the case when the UE is in IDLE/INACTIVE mode was left FFS. The latter would essentially correspond to introducing support for UE-initiated PRACH transmissions using semi-static channel access mode.

Not allowing UE-initiated PRACH transmissions when UE is in IDLE/INACTIVE mode presents two main disadvantages. First, the gNB may be required to acquire the COT with high frequency in order to allow the UEs to transmit on PRACH with acceptable latency. This may introduce unnecessary interference, as well as potentially increase the power consumption at the gNB. Second, as UEs would need to detect a DL transmission by the gNB at the beginning of the gNB FFP before they can transmit on PRACH, this may also introduce additional delays to the RACH procedure. Therefore, we suggest to also support UE-initiated COT for semi-static channel access mode in IDLE/INACTIVE mode. Details are FFS.

Proposal 13: Support UE-initiated COT for semi-static channel access mode in IDLE/INACTIVE mode.

If proposal x7 is agreed, FFP parameters for UE-initiated COT would need to be provided to the UE also by SIB-1, and not only by dedicated RRC signalling.

Proposal 14: Assuming support of UE-initiated COT for semi-static channel access mode in IDLE/INACTIVE mode is agreed, FFP parameters for UE-initiated COT also need to be provided to the UE by SIB-1.

Whether the UE FFP start and periodicity are explicitly configured or (at least partially) implicitly determined based on other higher layer parameters, is very much dependent on whether FFP parameters for UE-initiated COT are only provided by RRC signalling, or also by SIB-1, which on the other hand also depends on whether or not UE-initiated COT for semi-static channel access mode in IDLE/INACTIVE mode is supported. Therefore, we propose to still keep this issue FFS, at least until an agreement is reached on IDLE/INACTIVE mode and SIB-1 signalling.

It can be FFS whether the duration of the UE FFP is explicitly configured using SIB1 or RRC, or can be implicitly determined based on other configurations such as RACH configuration, UL CG configuration, etc.

Proposal 15: FFS whether the UE FFP start and periodicity are explicitly configured, or implicitly determined based on other higher layer configurations such as RACH configuration, UL CG configuration, etc.

In order to avoid potential conflicts between a serving gNB and its connected UEs, if the serving gNB acquires a COT during a gNB FFP, all its connected UEs should by default operate as responding devices according to gNB-shared COT during the corresponding gNB FFP.

Proposal 16: a UE may initiate a COT, by transmitting at the beginning of the configured UE FFP, if it has not detected an overlapping gNB-initiated COT or if gNB has previously allowed UE to do so. Otherwise, the UE is still allowed to transmit, but following the principles of gNB-shared COT.

# 3	Conclusions

In this contribution, we discussed uplink enhancements for IIOT/URLLC in unlicensed band including and CG harmonization and support for UE initiated COT. Based on the discussions we have the following proposals:

NR/NR-U CG enhancements harmonization:

Proposal 1: Two operation modes can be considered independently; NR IIoT Rel-16 based CG with NR based HARQ procedure and without CG-UCI, and NR-U based CG including CG-UCI and possibility of UE COT sharing.

Proposal 2: PUSCH repetitions type-B should be supported for unlicensed band operation when using NR IIoT Rel-16 based CG, without NR-U specific enhancements. FFS: required spec changes, if any.

Proposal 3: The use of PUSCH repetition type-B together with NR-U based multi-slot allocations should not be considered.

Proposal 4: When cg-RetransmissionTimer is not configured, NR-U specific CG features such as CG-UCI, COT sharing indication, UE-selected HARQ process ID,  and consecutive allocations are not supported.

Proposal 5: PHY multiplexing/prioritization introduced in Rel-16 is supported also with NR-U CG. Interaction of CG-UCI and HARQ-ACK codebooks of different priorities is FFS.

Proposal 6: RAN1 should decide which Agenda Item (8.3.2 or 8.3.3) to handle the issue of UCI multiplexing and prioritization on unlicensed spectrum.

On the support for UE-initiated COT for FBE

Proposal 7: FFP periodicity for a UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT

Proposal 8: As initiating device, the gNB can transmit during any UE FFP idle periods.

Proposal 9: As initiating device, a UE can transmit during other UEs FFP idle periods.

Proposal 10: By default, as the initiating device, a UE cannot transmit during its serving gNB’s idle period.

Proposal 11: Introduce support for gNB-controlled UE-initiated UL transmissions during gNB idle periods, when the gNB has no intention to acquire the COT in the subsequent FFP.

Proposal 12: A UE should be able to determine, exclusively from information in the scheduling DCI, whether a scheduled PUSCH transmission should be transmitted according to shared gNB COT or UE-initiated COT.

Proposal 13: Support UE-initiated COT for semi-static channel access mode in IDLE/INACTIVE mode.

Proposal 14: Assuming support of UE-initiated COT for semi-static channel access mode in IDLE/INACTIVE mode is agreed, FFP parameters for UE-initiated COT also need to be provided to the UE by SIB-1.

Proposal 15: FFS whether the UE FFP start and periodicity are explicitly configured, or implicitly determined based on other higher layer configurations such as RACH configuration, UL CG configuration, etc.

Proposal 16: a UE may initiate a COT, by transmitting at the beginning of the configured UE FFP, if it has not detected an overlapping gNB-initiated COT or if gNB has previously allowed UE to do so. Otherwise, the UE is still allowed to transmit, but following the principles of gNB-shared COT.

# References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, TSG RAN#88-e, July 2020.

- R1-2007391, Summary#5 on enhancements for unlicensed band URLLC/IIoT for R17, Moderator (Ericsson), 3GPP TSG RAN WG1 Meeting #102-e, August 2020.

- R1-2008843, On UL intra-UE prioritization and multiplexing enhancements, Nokia, Nokia Shanghai Bell, 3GPP TSG RAN WG1 Meeting #103-e, October 2020.