3GPP TSG RAN WG1 #103-e		                   R1-2008572

e-Meeting, October 26th – November 13th, 2020

Agenda item:	7.2.6

Source:	LG Electronics

Title:	Text proposals on full Tx power UL transmission

Document for:	   Discussion and Decision

- Introduction

In this contribution, we provide text proposals on full Tx power UL transmission for TS 38.306.

- Text proposals on full Tx power UL transmission

In the RAN1#99 meeting, following agreements for supporting ULFPTx with Mode 2 operation are captured chairman’s note as follows:

Agreement

For 4 ports, number of bits to indicate TPMI(s) which can deliver UL full power:

  - Non Coherent 2 bits

  - Partial coherent 4 bits

    - Additional entries on top of existing entries may be added to table 1 and table 2

  - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions

Table 1.

| 4Tx, nonCoherent | 4Tx, partial coherent (4bit) |
| --- | --- |
| G0 | G0 |
| G1 | G1 |
| G2 | G2 |
| G3 | G3 |
|  | G4 |
|  | G5 |
|  | G6 |

Definition of G0~G6 can be found in the table below.

Table 2.

Since above agreed TPMI groups only cover limited UE PA architecture, it was intensively discussed how to efficiently resolve this issue during the last several e-meetings. As a result, it was agreed that 4 port UE can report up to two TPMI groups based on the UE capability as capture in TS 38.306. However, in the current version of TS 38.306, it does not explicitly describe/define TPMI groups (G0-G6). Therefore, in order to eliminate ambiguity, we propose following TP below.

Proposal: Send an LS to RAN2 for clarifying the definition of G0-G6 in TS38.306 with attaching the following TP for TS 38.306.

4.2.7.7	FeatureSetUplink parameters

-------------------------------------------------------- Omitted -----------------------------------------------------

| ul-FullPwrMode2-TPMIGroup-r16 Indicates the UE supported TPMI group(s) which delivers full power. The capability signalling comprises the following values: - twoPorts-r16 indicates a 2-bit bitmap - fourPortsNonCoherent-r16 indicates the TPMI groups {G0-3} - fourPortsPartialCoherent-r16 indicates the TPMI groups (G0-6) where TPMI groups (G0-6) are defined as TPMI groups. G0; 121000,. G1; 121000, 120010, 1210000100,. G2; 121000, 120100, 120010,1210000100, 1210010000,1200100100, 12100010001000. G3; 1210000100, 1210010000,1200100100, 12100010001000. G4; 121010, 1210-10,1210j0,1210-j0,1210000100. G5; 121010, 1210-10,1210j0,1210-j01210000100, 1210010000,1200100100, 12100010001000. G6; 121010, 1210-10,1210j0,1210-j0, 120101, 12010-1,12010j,12010-j 1210010000, 1210000100, 1210000001,1200100100,1200100001, 1200001001,12100010001000. UE indicates support of this feature shall also indicate support of ul-FullPwrMode2-MaxSRS-ResInSet. NOTE 1: When a full coherent UE operates in mode 2, it reports TPMIs the same as a partial-coherent UE. NOTE 2: For 4 port partial-coherent or full-coherent UE, UE can report: 2-port {2-bit bitmap} and one of 4-port non-coherent {G0~G3} and one of 4-port partial-coherent {G0~G6} For 4 port non-coherent UE, UE can report: 2-port {2-bit bitmap} and one of 4-port non-coherent {G0~G3} For 2 port UE, UE can report: 2-port {2-bit bitmap} NOTE 3: A UE that supports this feature must report at least one of the values. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

-------------------------------------------------------- Omitted -----------------------------------------------------