3GPP TSG RAN WG1 #103-e		R1-2008664

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	TP on physical DL channel design in unlicensed spectrum

Agenda Item:	7.2.2

Document for:	Discussion and Decision

- Introduction

This contribution discusses TPs related with CSI-RS measurement aspect.

- Discussion

In RAN1 #103-e [1], the following agreements are made on CSI-RS measurement:

Agreement:

If at least one of SFI and CO duration fields in DCI 2_0 is configured, the UE may average measurements of two or more instances of a periodic or semi-persistent NZP CSI-RS for channel measurement or for interference measurement that occur in the indicated remaining CO duration.  The UE shall not average measurements of two or more instances of a periodic or semi-persistent NZP CSI-RS for channel measurement or for interference measurement that do not occur in the indicated remaining CO duration.

Agreement:

If neither SFI nor CO duration fields in DCI 2_0 are configured but CSI-RS-ValidationWith-DCI-r16 is configured, the UE may average measurements of two or more instances of a periodic or semi-persistent NZP CSI-RS for channel measurement or for interference measurement that occur in a time duration for which all OFDM symbols are occupied by a set of PDSCH and/or CSI-RS(s) that are scheduled/triggered to the UE, including the scheduling/triggering PDCCH(s)]. The UE shall not average measurements of two or more instances of a periodic or semi-persistent NZP CSI-RS for channel measurement or for interference measurement that occur in a time duration for which not all OFDM symbols are occupied by a set of PDSCH and/or CSI-RS(s) scheduled/triggered to the UE, including the scheduling/triggering PDCCH(s).

However, they are not reflected in current spec and thus the following TP is proposed:

Proposal 1: RAN1 adopts TP1 for section 5.2.1.4.2 of TS 38.214.

----------------------------------------TP1: Start TP for Section 5.2.1.4.2 of TS 38.214 -------------------------------------

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging instances of any periodic/semi-persistent nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement located in different DL transmissions bursts (defined in [16, TS 37.213])., where

-  the nzp-CSI-RS Resource is not in the same channel occupancy durations indicated by DCI format 2_0 if the UE is provided at least one of SlotFormatCombinationsPerCell or CO-DurationList-r16; or

- the set of symbols for PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats overlapped with nzp-CSI-RSResource is not continuous if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatCombinationsPerCell.

-	the interference measurements for computing CSI value

-  based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213]; and

-  without averaging instances of any periodic/semi-persistent CSI-IM-Resource in the corresponding CSI-IM-ResourceSet for interference measurement located in different DL transmissions, where

-	the CSI-IM-Resource is not in the same channel occupancy durations indicated by DCI format 2_0 if the UE is provided at least one of SlotFormatCombinationsPerCell or CO-DurationList-r16; or

- the set of symbols for PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats overlapped with CSI-IM-Resource is not continuous if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatCombinationsPerCell.

----------------------------------------TP1: End TP for Section 5.2.1.4.2 of TS 38.214 --------------------------------------

- Conclusion

In this contribution, we focus on the remaining issues of physical DL channel design, and have the following proposal:

Proposal 1: RAN1 adopts TP1 for section 5.2.1.4.2 of TS 38.214.

----------------------------------------TP1: Start TP for Section 5.2.1.4.2 of TS 38.214 -------------------------------------

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging instances of any periodic/semi-persistent nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement located in different DL transmissions bursts (defined in [16, TS 37.213])., where

-  the nzp-CSI-RS Resource is not in the same channel occupancy durations indicated by DCI format 2_0 if the UE is provided at least one of SlotFormatCombinationsPerCell or CO-DurationList-r16; or

- the set of symbols for PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats overlapped with nzp-CSI-RSResource is not continuous if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatCombinationsPerCell.

-	the interference measurements for computing CSI value

-  based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213]; and

-  without averaging instances of any periodic/semi-persistent CSI-IM-Resource in the corresponding CSI-IM-ResourceSet for interference measurement located in different DL transmissions, where

-	the CSI-IM-Resource is not in the same channel occupancy durations indicated by DCI format 2_0 if the UE is provided at least one of SlotFormatCombinationsPerCell or CO-DurationList-r16; or

- the set of symbols for PDSCH(s) and/or aperiodic CSI-RS(s) indicated by DCI formats overlapped with CSI-IM-Resource is not continuous if the UE is provided CSI-RS-ValidationWith-DCI-r16, is not provided CO-DurationPerCell-r16, and is not provided SlotFormatCombinationsPerCell.

---------------------------------------- TP1: End TP for Section 5.2.1.4.2 of TS 38.214 -------------------------------------

- References

- RAN1 chairman notes, RAN1 #102-e, e-Meeting, August 17th – 28th, 2020.