3GPP TSG RAN WG1 #103-e		R1-2008670

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	PDCCH enhancements for URLLC

Agenda Item:	7.2.5

Document for:	Discussion and Decision

- Introduction

In RAN1#102 e-meeting, clarification on span is discussed. The following agreement is achieved [1]:

Agreement

A UE expects the same combination (X, Y) to apply in every slot, including cross-slot boundary, on the active DL BWP of a cell.

In this contribution, we share our view on remaining issues of PDCCH enhancements for URLLC.

- Clarification on aligned span and unaligned span case

In Rel-15, the same span pattern repeats in every slot, as defined in UE feature 3-5b. In Rel-16, span pattern is not defined. In a slot, a span starts at a first symbol where a PDCCH monitoring occasion starts and ends at a last symbol where a PDCCH monitoring occasion ends. Since the PDCCH monitoring occasions may be different in different slots, if the spans in each slot are determined based on the actual PDCCH monitoring occasions in each slot, the span patterns may be different in different slots. This may lead to the aligned spans in some slots and unaligned spans in other slots.

In such case, UE needs to buffer the PDCCH monitoring occasions information and determine the spans of all slots. After that, UE determines the number of BD and CEs per span depending on whether the PDCCH monitoring occasions belong to aligned span case or un-aligned span case. If UE performs online processing, i.e., it does not buffer all information in advance, when the sets of spans in the first 10 slots are aligned while the ones in the 11th slot are unaligned, UE would assume that this is aligned case and to obtain a wrong number of BD and CEs by aligned span case assumption, which is not desired from gNB.

Considering complexity of UE implementation, it is expected that there would be aligned spans case for Rel-16 PDCCH monitoring capability in all slots. For example, when UE determines aligned spans case in the first one or more slots, UE expects aligned spans case for all the other subsequent slots. On the other hand, in case of unaligned spans, it is not necessary to restrict the spans in all slots are unaligned or the same, otherwise it would put limit on the network flexibility of configuration. Therefore, when UE determines unaligned spans case in the first one or more slots, there is no restriction for all the other subsequent slots.

Proposal 1:

- When UE determines the aligned spans case in the first one or more slots, UE expects aligned spans case for all the other subsequent slots.

- When UE determines unaligned spans case in the first one or more slots, there is no restriction for all the other subsequent slots.

- The bit size misalignment between counter DAI and UL DAI

In the last meeting, the pseudo code issue for Type 2 codebook was discussed and there was no agreement on it. In Rel-16 for DCI format 1-2, the counter DAI field with the configurable size is supported and 1 bit can be configured, while the UL DAI bits in UL scheduling DCI are still 2 bits. This leads to the size misalignment between counter DAI and UL DAI.  The pseudo code, highlighted by yellow colour in the following part, bases on the same field size. When the different field sizes are configured between counter DAI and UL DAI, the error occurs. A method is proposed to align the 2 bits UL DAI to 1 bit to correct pseudo code as above mentioned. This method only addresses the case of 1 missed DCI and can not protect 3 missed DCI case. In our opinion, it is hardly to handle 3 missed DCI case by 1 bit counter DAI, because there is no enough states of codepoint to differentiate 3 missed DCI case. From simplicity, it is preferred that gNB implementation handles this issue by omitting the MSB bit of UL DAI without pseudo code correction .

Proposal 2: It is up to gNB implementation to handle the bit size misalignment between counter DAI and UL DAI, e.g. by omitting the MSB bit of UL DAI.

38.213 - 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel …… Set to the number of PDCCH monitoring occasion(s) while Set – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while if PDCCH monitoring occasion is before an active DL BWP change on serving cell or an active UL BWP change on the PCell and an active DL BWP change is not triggered in PDCCH monitoring occasion ; else if there is a PDSCH on serving cell associated with PDCCH in PDCCH monitoring occasion , or there is a PDCCH indicating SPS PDSCH release on serving cell if end if if else end if if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1 elseif harq-ACK-SpatialBundlingPUCCH is provided to the UE and is a monitoring occasion for PDCCH with a DCI format that supports PDSCH reception with two transport blocks and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks in at least one configured DL BWP of a serving cell, oTD⋅j+VC-DAI,c,mDL-1ACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 else oTD⋅j+VC-DAI,c,mDL-1ACK = HARQ-ACK information bit of this cell Vs=Vs∪TD⋅j+VC-DAI,c,mDL-1 end if end if end if end while end while if end if …….

- Conclusion

In the contribution, we have some investigations on remaining issues for URLLC PDCCH enhancement, and propose that,

Proposal 1:

- When UE determines the aligned spans case in the first one or more slots, UE expects aligned spans case for all the other subsequent slots.

- When UE determines unaligned spans case in the first one or more slots, there is no restriction for all the other subsequent slots.

Proposal 2: It is up to gNB implementation to handle the bit size misalignment between counter DAI and UL DAI, e.g. by omitting the MSB bit of UL DAI.

------------------------------------------------------------------------------------------------------------------------------------

- References

[1] Chairman notes RAN1 #102 e-meeting.