3GPP TSG RAN WG1 #103-e			R1-2008672

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Enhancement for Scheduling/HARQ

Agenda Item:	7.2.5

Document for:	Discussion and Decision

- Introduction

In the previous e-meeting, some remaining issues about Scheduling and HARQ are discussed. The following agreements are achieved [1]:

RAN1#101 e-meeting

Agreement

After the UE determines the overlapping PUCCH or PUSCH for multiplexing/prioritization, the UE cancels the PUCCH or PUSCH that has overlapping with semi-static configured DL symbols or SSB symbols, and then the multiplexing/prioritization is performed among the non-cancelled overlapping transmissions

RAN1#102 e-meeting

Agreement

Confirm the following working assumption and remove the brackets as follows:

- A UE behavior of handling intra-UE prioritization/multiplexing for overlapping UL transmissions on semi-static flexible symbols is not affected by UL cancellation due to dynamic SFI or [DL grant]

- Note: The UE performs prioritization/multiplexing first and once done applies dynamic SFI

The processing order between semi-static TDD configuration and intra-UE prioritization/cancellation is discussed and no agreement is achieved. In this contribution, we share our view on this issue.

- CR related to processing order between semi-static TDD configuration and intra-UE prioritization/cancellation

Based on the previous agreement, UE will have 3-step processing for to handle the processing order between intra-UE multiplexing/prioritization, and cancellation due to semi-static TDD configuration.

- Step 1: UE follows Rel-15 behaviours for any intermediate procedure to determine the overlapping PUCCH or PUSCH for multiplexing/prioritization

- Step 2: UE cancels the ones that collides with semi-static DL symbols,

- Step 3: UE performs multiplexing/prioritization among the non-cancelled overlapping channels.

After step 2, UE should remove all the configured PUCCH/PUSCH resources overlapping with semi-static DL symbols/SSBs, which is better to ensure final PUCCH/PUSCH transmission. Otherwise, final PUCCH/PUSCH may be invalid due to collision with semi-static DL symbols or SSB symbols, which leads to the dropping of UL channel(s). On the other hand, gNB scheduling should guarantee an available final PUCCH/PUSCH resource.

Proposal 1: To consider the following text proposal

TS 38.213 9 UE procedure for reporting control information … If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority indexes, the UE cancels the PUCCH or the PUSCH transmission if the UE determines a PUCCH or a PUSCH transmissions overlapping with DL symbols indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. Then the UE resolves the overlapping for PUCCH and/or PUSCH transmissions of a same priority index as defined in Clause 9.2.5. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE cancels the PUCCH or the PUSCH transmission if the UE determines a PUCCH or a PUSCH transmissions overlapping with DL symbols indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, then the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of a same priority index. Then, if the UE determines to transmit - a first PUCCH of larger priority index scheduled by a first DCI format in a first PDCCH reception, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE cancels the transmission of the PUSCH or the second PUCCH before the first symbol overlapping with the first PUCCH transmission. The UE expects that the transmission of the first PUCCH does not start before Tproc,2+d1 after a last symbol of the first PDCCH reception - a PUSCH of larger priority index scheduled by a first DCI format in a first PDCCH reception, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE cancels the transmission of the PUCCH before the first symbol overlapping with the PUSCH transmission. The UE expects that the transmission of the PUSCH does not start before Tproc,2+d1 after a last symbol of the first PDCCH reception where Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability …

- Conclusion

In the contribution, we have investigation on the processing order between semi-static TDD configuration and intra-UE prioritization/cancellation, and propose that,

Proposal 1: To consider the following text proposal

TS 38.213 9 UE procedure for reporting control information … If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of the same priority indexes, the UE cancels the PUCCH or the PUSCH transmission if the UE determines a PUCCH or a PUSCH transmissions overlapping with DL symbols indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. Then the UE resolves the overlapping for PUCCH and/or PUSCH transmissions of a same priority index as defined in Clause 9.2.5. When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE cancels the PUCCH or the PUSCH transmission if the UE determines a PUCCH or a PUSCH transmissions overlapping with DL symbols indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, then the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of a same priority index. Then, if the UE determines to transmit - a first PUCCH of larger priority index scheduled by a first DCI format in a first PDCCH reception, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE cancels the transmission of the PUSCH or the second PUCCH before the first symbol overlapping with the first PUCCH transmission. The UE expects that the transmission of the first PUCCH does not start before Tproc,2+d1 after a last symbol of the first PDCCH reception - a PUSCH of larger priority index scheduled by a first DCI format in a first PDCCH reception, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE cancels the transmission of the PUCCH before the first symbol overlapping with the PUSCH transmission. The UE expects that the transmission of the PUSCH does not start before Tproc,2+d1 after a last symbol of the first PDCCH reception where Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability …

- References

[1] Chairman notes RAN1 #101 e-meeting

[1] Chairman notes RAN1 #102 e-meeting