3GPP TSG RAN WG1 #103-e                                                R1-2008674

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Remaining issues and corrections on multi beam related issues

Agenda Item:	7.2.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss several remaining issues on multi-beam and text proposals in 3GPP Rel-16.

- UE behavior for L1-SINR measurement

RAN1 #99 Conclusion For one L1-SINR report, the RRC parameters timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements can be configured. Agreement - For L1-SINR report, the UE applies the QCL-TypeD configured to the CMR on the associated NZP IMR when measuring interference - Note: UE is not expected to measure one NZP CSI-RS transmission instance with multiple different QCL-TypeD. - There is no spec impact for the note above RAN1 #102-e Agreement The text proposal on measurement restriction for L1-RSRP for TS38.214 subclause 5.2.1.4.3 is endorsed in R1-2007132 (TS38.214, Rel-15, CR#0110, Cat. F) and R1-2007133 (TS38.214, Rel-16, CR#0111, Cat. A).

TS 38.214 5.2.2.1 Channel quality indicator (CQI) If a UE is not configured with higher layer parameter timeRestrictionForChannelMeasurements, the UE shall derive the channel measurements for computing CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. If a UE is configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. If a UE is not configured with higher layer parameter timeRestrictionForInterferenceMeasurements, the UE shall derive the interference measurements for computing CSI value reported in uplink slot n based on only the CSI-IM and/or NZP CSI-RS for interference measurement no later than the CSI reference resource associated with the CSI resource setting. If a UE is configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting.

TS 38.214 5.2.1.4.3 L1-RSRP Reporting If a UE is not configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. If a UE is configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

In RAN1 #102-e meeting, the UE behavior for L1-RSRP report with measurement restriction is captured in Rel-15, which is based on the same principle as CSI measurement. For L1-RSRP measurement in section 5.2.1.4.3 of TS38.214 described as above, the UE behavior for SSB or NZP CSI-RS for channel measurement is determined based on the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig.

For CQI measurement in section 5.2.2.1 of TS38.214, in addition to the channel measurement, the UE behavior for CSI-IM and/or NZP CSI-RS for interference measurement is determined based on the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig.

According to conclusion in RAN1 #99 meeting, the RRC parameters timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements can be configured for L1-SINR report. The timeRestrictionForChannelMeasurements parameter in CSI-ReportConfig can be configured to enable time domain restriction for channel measurements and timeRestrictionForInterferenceMeasurements can be configured to enable time domain restriction for interference measurements.

However, the UE behavior with measurement restriction for L1-SINR is not defined. Similar to CQI measurement, for interference measurement of L1-SINR report, the UE behavior can be defined according to the configuration of the higher layer parameters timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements, except for the case when one resource setting is configured for L1-SINR measurement.

When one resource setting is configured, the resource setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel and interference measurement for L1-SINR computation. UE may assume that same 1 port NZP CSI-RS resource(s) with density 3 REs/RB is used for both channel and interference measurements. It is not clear for the measurement behavior for this resource setting according to the measurement restriction. One possibility is the measurement behavior for this resource setting always follows the higher layer parameter timeRestrictionForChannelMeasurements regardless of the resource setting used for channel measurement or for interference measurement. Another possibility is the measurement behavior for this resource setting follows the higher layer parameter timeRestrictionForChannelMeasurements when this resource setting is used for channel measurement, and follows the higher layer parameter timeRestrictionForInterferenceMeasurements when this resource setting is used for interference measurement.

- Clarify whether the following measurement restriction could be configured simultaneously when one resource setting is associated with L1-SINR:

- timeRestrictionForChannelMeasurements

- timeRestrictionForInterferenceMeasurements

- Support the proposed TP.

TS 38.214 5.2.1.4.4 L1-SINR Reporting For L1-SINR computation, for channel measurement the UE may be configured with NZP CSI-RS resources and/or SS/PBCH Block resources, for interference measurement the UE may be configured with NZP CSI-RS or CSI-IM resources. - for channel measurement, the UE may be configured with CSI-RS resource setting with up to 16 resource sets, with a total of up to 64 CSI-RS resources or up to 64 SS/PBCH Block resources. For L1-SINR reporting, if the higher layer parameter nrofReportedRSForSINR in CSI-ReportConfig is configured to be one, the reported L1-SINR value is defined by a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and if the higher layer parameter nrofReportedRSForSINR is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-SINR based reporting, where the largest measured value of L1-SINR is quantized to a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and the differential L1-SINR is quantized to a 4-bit value. The differential L1-SINR is computed with 1 dB step size with a reference to the largest measured L1-SINR value which is part of the same L1-SINR reporting instance. When NZP CSI-RS is configured for channel measurement and/or interference measurement, the reported L1-SINR values should not be compensated by the power offset(s) given by higher layer parameter powerControOffsetSS or powerControlOffset. When one or two resource settings are configured for L1-SINR measurement - If a UE is not configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. - If a UE is configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. - If a UE is not configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing L1-SINR reported in uplink slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting. - If a UE is configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing the L1-SINR reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting.

- TCI state update for CORESET

TS 38.321 6.1.3.15 TCI State Indication for UE-specific PDCCH MAC CE The TCI State Indication for UE-specific PDCCH MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields: - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1-r16 or simultaneousTCI-UpdateList2-r16 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1-r16 or simultaneousTCI-UpdateList2-r16, respectively; - CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits; - TCI State ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to 0, this field indicates a TCI-StateId for a TCI state of the first 64 TCI-states configured by tci-States-ToAddModList and tci-States-ToReleaseList in the PDSCH-Config in the active BWP. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits.

TS 38.213 10.1 UE procedure for determining physical downlink control channel assignment For each CORESET, the UE is provided the following by ControlResourceSet: - a CORESET index p, by controlResourceSetId, where - 0<p<12 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided; - 0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET; - a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID; - a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity; - a number of consecutive symbols provided by duration; - a set of resource blocks provided by frequencyDomainResources; - CCE-to-REG mapping parameters provided by cce-REG-MappingType; - an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a respective CORESET; - if the UE is provided by simultaneousTCI-UpdateList-r16 or simultaneousTCI-UpdateListSecond-r16 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by TCI-States with same activated tci-StateID value to CORESETs with index p in all configured DL BWPs of all configured cells in a list determined from a serving cell index provided by a MAC CE command - an indication for a presence or absence of a transmission configuration indication (TCI) field for a DCI format, other than DCI format 1_0, that schedules PDSCH receptions or indicates SPS PDSCH release and is transmitted by a PDCCH in CORESET p, by tci-PresentInDCI or tci-PresentInDCI-ForDCIFormat1_2-r16.

In section 6.1.3.5 of TS38.321, the TCI state indication for UE-specific PDCCH MAC CE is provided. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1-r16 or simultaneousTCI-UpdateList2-r16, this MAC CE applies to all the Serving Cells in the set simultaneousTCI-UpdateList1-r16 or simultaneousTCI-UpdateList2-r16, respectively. For TCI state ID, the field of CORESET ID can be set to 0, which means the simultaneous TCI state update can also be applied to CORESET#0. However, in section 10.1 of TS 38.213, the simultaneous TCI state update for CORESET is described for the CORESET with index p>0 which is not aligned with TS38.321.

- Clarify whether the simultaneous TCI state update can be applied to CORESET#0 due to the following restriction in the same paragraph:

a CORESET index p, by controlResourceSetId, where - 0<p<12 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided; - 0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;

- Support the following TP.

TS 38.213 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > For each CORESET, the UE is provided the following by ControlResourceSet: - a CORESET index p, by controlResourceSetId, where - 0<p<12 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided; - 0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET; - a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID; - a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity; - a number of consecutive symbols provided by duration; - a set of resource blocks provided by frequencyDomainResources; - CCE-to-REG mapping parameters provided by cce-REG-MappingType; - an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a respective CORESET with index p≥0; - if the UE is provided by simultaneousTCI-UpdateList-r16 or simultaneousTCI-UpdateListSecond-r16 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by TCI-States with same activated tci-StateID value to CORESETs with index p p≥0 in all configured DL BWPs of all configured cells in a list determined from a serving cell index provided by a MAC CE command - an indication for a presence or absence of a transmission configuration indication (TCI) field for a DCI format, other than DCI format 1_0, that schedules PDSCH receptions or indicates SPS PDSCH release and is transmitted by a PDCCH in CORESET p, by tci-PresentInDCI or tci-PresentInDCI-ForDCIFormat1_2-r16. < Unchanged parts are omitted >

- Conclusion

In this contribution, we have the following proposals for multi-beam transmission schemes.

- Clarify whether the following measurement restriction could be configured simultaneously when one resource setting is associated with L1-SINR:

- timeRestrictionForChannelMeasurements

- timeRestrictionForInterferenceMeasurements

- Support the proposed TP.

TS 38.214 5.2.1.4.4 L1-SINR Reporting For L1-SINR computation, for channel measurement the UE may be configured with NZP CSI-RS resources and/or SS/PBCH Block resources, for interference measurement the UE may be configured with NZP CSI-RS or CSI-IM resources. - for channel measurement, the UE may be configured with CSI-RS resource setting with up to 16 resource sets, with a total of up to 64 CSI-RS resources or up to 64 SS/PBCH Block resources. For L1-SINR reporting, if the higher layer parameter nrofReportedRSForSINR in CSI-ReportConfig is configured to be one, the reported L1-SINR value is defined by a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and if the higher layer parameter nrofReportedRSForSINR is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-SINR based reporting, where the largest measured value of L1-SINR is quantized to a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and the differential L1-SINR is quantized to a 4-bit value. The differential L1-SINR is computed with 1 dB step size with a reference to the largest measured L1-SINR value which is part of the same L1-SINR reporting instance. When NZP CSI-RS is configured for channel measurement and/or interference measurement, the reported L1-SINR values should not be compensated by the power offset(s) given by higher layer parameter powerControOffsetSS or powerControlOffset. When one or two resource settings are configured for L1-SINR measurement - If a UE is not configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. - If a UE is configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-SINR reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. - If a UE is not configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing L1-SINR reported in uplink slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting. - If a UE is configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing the L1-SINR reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting.

- Clarify whether the simultaneous TCI state update can be applied to CORESET#0 due to the following restriction in the same paragraph:

a CORESET index p, by controlResourceSetId, where - 0<p<12 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided; - 0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;

- Support the following TP.

TS 38.213 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > For each CORESET, the UE is provided the following by ControlResourceSet: - a CORESET index p, by controlResourceSetId, where - 0<p<12 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided; - 0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET; - a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID; - a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity; - a number of consecutive symbols provided by duration; - a set of resource blocks provided by frequencyDomainResources; - CCE-to-REG mapping parameters provided by cce-REG-MappingType; - an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a respective CORESET with index p≥0; - if the UE is provided by simultaneousTCI-UpdateList-r16 or simultaneousTCI-UpdateListSecond-r16 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by TCI-States with same activated tci-StateID value to CORESETs with index p p≥0 in all configured DL BWPs of all configured cells in a list determined from a serving cell index provided by a MAC CE command - an indication for a presence or absence of a transmission configuration indication (TCI) field for a DCI format, other than DCI format 1_0, that schedules PDSCH receptions or indicates SPS PDSCH release and is transmitted by a PDCCH in CORESET p, by tci-PresentInDCI or tci-PresentInDCI-ForDCIFormat1_2-r16. < Unchanged parts are omitted >

References

- Chairman’s notes for 3GPP TSG RAN WG1 #99 Meeting.

- Chairman’s notes for 3GPP TSG RAN WG1 #102-e Meeting.

- 3GPP TS 38.214 V16.3.0 (2020-09)

- 3GPP TS 38.213 V16.3.0 (2020-09)

- 3GPP TS 38.321 V16.2.1 (2020-09)