3GPP TSG RAN WG1 #103-e		                                    R1-2008684

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.6.1

Source:	InterDigital, Inc.

Title:	UE complexity reduction features for reduced capability NR devices

Document for:	Discussion

# Introduction

In RAN#86, a new SI on reduced capability (RedCap) NR devices was approved [1]. The objective of the SI is to:

- Identify and study potential UE complexity reduction features

- Study UE power saving and battery lifetime enhancement for reduced capability UEs

- Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited

In RAN1 #102-e, the following agreements were taken [2]:

Agreements:

- For cost/complexity reduction analysis, the RF-to-baseband cost ratio for an FR1 UE is assumed to be 40:60.

- For cost/complexity reduction analysis, the RF-to-baseband cost ratio for an FR2 UE is assumed to be approximately 50:50.

Conclusion:

- The study of reduced number of UE (physical) antenna elements and panels in FR2 is not prioritized in the RedCap study item.

Agreements:

- For RedCap UEs in FR1,

  - The baseline UE bandwidth capability is 20 MHz, which can be assumed during the initial access procedure.

  - Discuss further by email whether there is an issue or a necessity in achieving up to 150Mbps assuming a 20MHz and rank 1 transmission.

Agreements:

- For the purpose of evaluation, the UE processing time in terms of N1/N2 can be assumed to be doubled compared to those of capability #1, i.e.,

  - N1 = 16, 20, 34, and 40 symbols for 15, 30, 60, and 120 kHz SCS (assuming only front-loaded DMRS)

  - N2 = 20, 24, 46, and 72 symbols for 15, 30, 60, and 120 kHz SCS

Agreement:

- Study of relaxed UE processing time related to CSI computation is not prioritized in the RedCap study item.

Agreements:

- For FR1 DL, study relaxation of maximum mandatory modulation to 64QAM instead of 256QAM.

- For FR1 UL, study relaxation of maximum mandatory modulation to 16QAM instead of 64QAM.

- For FR2 DL, study relaxation of maximum mandatory modulation to 16QAM instead of 64QAM.

- For FR2 UL, study relaxation of maximum mandatory modulation to 16QAM instead of 64QAM.

- Restriction to 1 or 2 MIMO layers in DL can be studied.

- No TBS restriction is considered in this SI beyond the implicit TBS restrictions resulting from reduced UE bandwidth or reduced number of MIMO layers.

Agreements:

- Assume the detailed cost breakdown for FR1 FDD/TDD and FR2 in the table below:

| Functional block | FR1 FDD (2Rx) | FR1 TDD (4Rx) | FR2 |
| --- | --- | --- | --- |
| RF |  |  |  |
| Antenna array for FR2 |  |  | ~33% |
| Power amplifier | ~25% | ~25% | ~18% |
| Filters | ~10% | ~15% | ~8% |
| RF transceiver (including LNAs, mixer, and local oscillator) | ~45% | ~55% | ~41% |
| Duplexer / Switch | ~20% | ~5% | ~0% |
| Baseband |  |  |  |
| ADC / DAC | ~10% | ~9% | ~4% |
| FFT/IFFT | ~4% | ~4% | ~4% |
| Post-FFT data buffering | ~10% | ~10% | ~11% |
| Receiver processing block | ~24% | ~29% | ~24% |
| LDPC decoding | ~10% | ~9% | ~9% |
| HARQ buffer | ~14% | ~12% | ~11% |
| DL control processing & decoder | ~5% | ~4% | ~5% |
| Synchronization / cell search block | ~9% | ~9% | ~7% |
| UL processing block | ~5% | ~5% | ~7% |
| MIMO specific processing blocks | ~9% | ~9% | ~18% |

Agreements:

- In potential cost evaluations for a UE, it is assumed that the multi-band support affects the RF cost but not the baseband cost significantly.

- In the TR, at least include a qualitative statement; relevant numerical results can also be considered.

Agreements:

- For the baseline UE bandwidth capability of RedCap UEs, the same maximum UE bandwidth in a band applies to both RF and baseband.

  - This maximum UE bandwidth applies to both data and control channels.

  - This maximum UE bandwidth is assumed for both DL and UL.

  - Complexity analyses with other mixes of bandwidths are not precluded.

In this contribution, we further discuss complexity reduction features for RedCap UEs.

# Complexity reduction

## Reduced bandwidth

In NR, a UE needs to support 100 MHz in FR1 and 200 MHz in FR2. These large bandwidths are not necessary to achieve throughput requirements of RedCap UEs. So, UE cost can be reduced by utilizing narrower bandwidths. In RAN1#101e [2], it was agreed to study 20 MHz BW in FR1 and 50 MHz or 100 MHz in FR2.

Table 1 provides relative complexity reduction analysis for reducing the FR1 BW to 20 MHz from 100 MHz. According to the analysis, about 40% cost reduction can be achieved by utilizing a narrower bandwidth.

Table 1 Relative complexity reduction analysis for BW reduction

| RF | Functional block | 20 MHz (FR1) |
| --- | --- | --- |
|  | Power amplifier | 20% |
|  | Filters | 0% |
|  | RF transceiver | 20% |
|  | Duplexer / Switch | 0% |
|  | Other | 0% |
|  | Total RF cost | 84% |
| Baseband | ADC / DAC | 80% |
|  | FFT/IFFT | 85% |
|  | Post-FFT data buffering | 80% |
|  | Receiver processing block | 80% |
|  | LDPC decoding | 55% |
|  | HARQ buffer | 55% |
|  | DL control processing & decoder | 0% |
|  | Synchronization / cell search block | 0% |
|  | UL processing block | 55% |
|  | MIMO specific processing blocks | 0% |
|  | Other | 0% |
|  | Total Baseband cost | 44% |
| Total | Overall relative complexity reduction | 40% |

The impact of reduced BW on DL and UL channels would not be large; some negligible loss may be observed due to reduced frequency diversity. Since most of the existing channels and configurations can be reused, specification impact is expected to be small.

The major impact of reduced BW may be on the broadcast channels. In RAN1 #101-e, it was agreed to study maximum bandwidth of 50 MHz and 100 MHz for FR2, at least for initial access. In NR, when 240 kHz subcarrier spacing is used, the PBCH spans a BW of 57.6 MHz, exceeding the 50 MHz BW. One straightforward option for the RedCap UE is to ignore the PBCH subcarriers beyond the 50MHz transmission bandwidth, resulting in potential coverage loss. The impact of this approach has been evaluated with simulations provided in Figure 1. In the simulations, 1, 2, and 4 receive antennas are used. The receive BW is set to 57.6 MHz or 50 MHz. The detailed simulation assumptions are provided in the Appendix. We can see from this figure that the loss in SNR as a result of not receiving all PBCH subcarriers is less than 1 dB which can be mitigated by collecting more energy via accumulating several copies of the PBCH.

In addition to the PBCH, CORESET0 may also span a BW beyond the 50 MHz: in certain configurations (e.g., 96 RBs, 1 OFDM symbol, 60 kHz subcarrier spacing), the BW of CORESET0 may be as large as 69.12 MHz. In addition, since message 2/3/4 are carried within CORESET0, initial access may also be impacted. One potential solution is to define a separate CORESET0 for RedCap UEs. This could simply be achieved by mapping the controlResourceSetZero in pdcch-ConfigSIB1, included in MIB, to a new set of RBs for RedCap UEs.

Figure 1 PBCH performance with reduced BW for 1, 2, 4 receive antennas

Observation 1: 50 MHz maximum UE bandwidth does not result in significant SNR loss for PBCH in FR2.

## Reduced number of antennas

Reducing the number of RF chains is expected to significantly reduce device complexity and cost. Removing an RF transceiver, the associated filter and ADC would reduce RF complexity. Removal of the baseband processing performed per RF chain such as FFT, channel estimation, etc. would result in less complexity in the baseband. An estimation of the cost reduction due to reducing the number of antennas is provided in Table 2 for FR1 when the number of Rx antennas is reduced from 4 to 2 and 1. We can see from the analysis that significant cost savings can be achieved: about 50% for reducing the number of Rx antennas from 4 to 1; and about 40% for reducing the number of Rx antennas from 4 to 2.

Table 2 Relative complexity reduction analysis for reducing the number of Rx antennas

| RF | Functional block | 1 Rx antenna | 2 Rx antennas |
| --- | --- | --- | --- |
|  | Power amplifier | 0% | 0% |
|  | Filters | 75% | 50% |
|  | RF transceiver | 75% | 50% |
|  | Duplexer / Switch | 0% | 0% |
|  | Other | 0% | 0% |
|  | Total RF cost | 47.5 | 65 |
| Baseband | ADC / DAC | 60% | 40% |
|  | FFT/IFFT | 75% | 50% |
|  | Post-FFT data buffering | 75% | 50% |
|  | Receiver processing block | 75% | 50% |
|  | LDPC decoding | 0% | 0% |
|  | HARQ buffer | 0% | 0% |
|  | DL control processing & decoder | 0% | 0% |
|  | Synchronization / cell search block | 75% | 50% |
|  | UL processing block | 0% | 0% |
|  | MIMO specific processing blocks | 75% | 50% |
|  | Other | 0% | 0% |
|  | Total Baseband cost | 49% | 66 |
| Total | Overall relative complexity reduction | 51.6% | 38% |

Reducing the number of antennas shall have a detrimental impact on the coverage and spectral efficiency. The acquisition time for the synchronization signals may increase and the reception of the broadcast channel may be impacted. The reduction in the received signal power and loss of diversity would reduce the coverage of the control channel and the random-access channel during the Message 2/4 reception. In addition, the BLER of the data channel would increase.

One effective method to limit or mitigate the coverage and BLER loss of the control and data channels is to use repetitions and/or relaxed acquisition time. NR already supports some type of slot aggregation for PDSCH and PUSCH, but such a technique is not available for control channels. Repetitions would increase the acquisition time for both data and control channels, resulting in more energy being used; therefore, using repetitions would reduce the power saving gains.

## Half-duplex FDD

Another method that can be considered for complexity reduction is half-duplex FDD in which using a switch instead of a duplexer reduces device complexity. Furthermore, employing a single oscillator can be considered to attain more complexity reduction.  One issue that would arise with a single oscillator is that a larger switching time from DL to UL, or vice-versa, may be required, resulting in some necessary changes in the L1 design; however these changes are expected to be minimal.

The cost reduction of half duplex FDD would be realized due to the cost reduction of the duplexer/switch; the total cost reduction is estimated to be about 5%.

From the above analysis, it becomes clear that bandwidth reduction and reducing the number of receive antennas result in significant cost savings. Therefore, we propose:

Proposal 1: Bandwidth reduction and reducing the number of receive antennas are adopted for RedCap UEs.

# Conclusion

This contribution has discussed several complexity reduction features for reduced capability devices. Based on the discussion, we propose the following:

Proposal 1: Bandwidth reduction and reducing the number of receive antennas are adopted for RedCap UEs.

In addition, from simulations, we have observed the following:

Observation 1: 50 MHz maximum UE bandwidth does not result in significant SNR loss for PBCH in FR2.

# References

- RP-193238, “New SID on support of reduced capability NR devices”, Ericsson, RAN#86.

- Chairman’s Notes, 3GPP WG1 #102-e.

- Chairman’s Notes, 3GPP WG1 #101-e.

- Appendix

Table 1 Simulation assumptions

| Frequency region | FR2 |
| --- | --- |
| Carrier frequency | 28 GHz |
| Subcarrier spacing | 240 kHz |
| Bandwidth | 50 MHz |
| Antenna configuration | 4 at the Tx; and 1,2, 4 at the Rx |
| Precoding | Fixed |
| Channel estimation | Ideal |
| Channel model | TDL-A |
| UE speed | 3 km/h |
| TDD slot format | DDDSU |