3GPP TSG-RAN WG1 Meeting #103e	R1-2008761

e-Meeting, October 26 – November 13, 2020

| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.214 | CR |  | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 38.214 for NR positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-10-12 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The reasons for the changes proposed are as follows: - there is misalignment between TS 37.355 and TS 38.214 when it comes to the fields present in positioniong frequency layer, DL PRS resource set, and DL PRS resource. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - The fields dl-PRS-CombSizeN-r16, dl-PRS-ResourceBandwidth-r16, and dl-PRS-StartPRB-r16 are moved to positioning frequency layer to align with TS 37.355. - The field dl-PRS-ResourceList-r16 is moved to DL PRS resource set to align with TS 37.355. - Reference clause numbers related to TS 38.211 are corrected - Reference clause numbers related to TS 37.355 are corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be misalignment between TS 37.355 and TS 38.214. The Reference clause numbers referred to in 38.214 would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.6.5	PRS reception procedure

---- Unchanged texts omitted ----

The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters nr-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-ResourceSet-r16 and nr-DL-PRS-Resource-r16 defined by Clause 6.4.3 [17, TS 37.355].

A positioning frequency layer consists of one or more DL PRS resource sets and it is defined by Clause 6.4.3 [17, TS 37.355]:

-	dl-PRS-SubcarrierSpacing-r16 defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing-r16. The supported values of dl-PRS-SubcarrierSpacing-r16 are given in Table 4.2-1 of [4, TS38.211].

-	dL-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL PRS positioning frequency layer have the same value of DL-PRS-CyclicPrefix. The supported values of DL-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211].

-	dl-PRS-PointA-r16 defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS resource set have common Point A and all DL PRS resource sets belonging to the same DL PRS positioning frequency layer have a common Point A.

-	dl-PRS-CombSizeN-r16 defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-CombSizeN-r16.

-	dl-PRS-ResourceBandwidth-r16 defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resource sets within a positioning frequency layer have the same value of dl-PRS-ResourceBandwidth-r16.

-	dl-PRS-StartPRB-r16 defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA-r16. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-StartPRB-r16.

The UE expects that it will be configured with dl-PRS-ID-r16 each of which is defined such that it is associated with multiple DL PRS resource sets from the same cell. The UE expects that one of these dl-PRS-ID-r16 along with a nr-DL-PRS-ResourceSetId-r16 and a nr-DL-PRS-ResourceId-r16 can be used to uniquely identify a DL PRS resource.

A DL PRS resource set consists of one or more DL PRS resources and it is defined by Clause 6.4.3 [TS 37.355]:

-	nr-DL-PRS-ResourceSetId-r16 defines the identity of the DL PRS resource set configuration.

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing-r16=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity.

-	dl-PRS-ResourceRepetitionFactor-r16 defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32,. All the DL PRS resources within one resource set have the same resource repetition factor.

-	dl-PRS-ResourceTimeGap-r16 defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceSetId-r16 within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap-r16 if dl-PRS-ResourceRepetitionFactor-r16 is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet-r16 is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap-r16.

-	dl-PRS-MutingOption1-r16 and dl-PRS-MutingOption2-r16 define the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. If mutingOption1 is configured, each bit in the bitmap of mutingOption1 corresponds to a configurable number provided by higher layer parameter dl-PRS-MutingBitRepetitionFactor-r16 of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. The length of the bitmap can be {2, 4, 6, 8, 16, 32} bits. If mutingOption2 is configured each bit in the bitmap of mutingOption2 corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet-r16 and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor-r16. Both mutingOption1 and mutingOption2 may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211].

-	nr-DL-PRS-SFN0-Offset-r16 defines the time offset of the SFN0 slot 0 for the transmitting cell with respect to SFN0 slot 0 of reference cell.

-	dl-PRS-ResourceList-r16 determines the DL PRS resources that are contained within one DL PRS resource set.

-	dl-PRS-CombSizeN-r16 defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.1 of [TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-CombSizeN-r16.

-	dl-PRS-ResourceBandwidth-r16 defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a positioning frequency layer have the same value of dl-PRS-ResourceBandwidth-r16.

-	dl-PRS-StartPRB-r16 defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA-r16. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-StartPRB-r16.

A DL PRS resource is defined by:

-	dl-PRS-ResourceList-r16 determines the DL PRS resources that are contained within one DL PRS resource set.

-	nr-DL-PRS-ResourceId-r16 determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set.

-	dl-PRS-SequenceId-r16 is used to initialize cinit value used in pseudo random generator [4, TS38.211, 7.4.1.7.2] for generation of DL PRS sequence for a given DL PRS resource.

-	dl-PRS-CombSizeN-and-ReOffset-r16 defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS38.211].

-	dl-PRS-ResourceSlotOffset-r16 determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset

-	dl-PRS-ResourceSymbolOffset-r16 determines the starting symbol of a slot configured with the DL PRS resource.

-	dl-PRS-NumSymbols-r16 defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211].

-	dl-PRS-QCL-Info-r16 defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be 'QCL-Type-D' with a DL PRS or SS/PBCH Block from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-Type-C' with a SS/PBCH Block from a serving or non-serving cell. If the DL PRS is configured as both 'QCL-Type-C' and 'QCL-Type-D' with a SS/PBCH Block then the SSB index indicated should be the same.

---- Unchanged texts omitted ----