3GPP TSG RAN WG1 Meeting #103-e                                          	R1-2008816 e-Meeting, October 26th – November 13th, 2020

Agenda item       : 8.10.2

Source                 : CEWiT, Tejas Networks, Reliance Jio, IITM, Saankhya Labs, IITH

Title	         : Discussion on simultaneous operation of IAB-node’s child and parent links

Document for     : Discussion

# 1. Introduction

The agreements in RAN 1 meeting #102-e on IAB enhancements were as follows [1]:

Agreement

The Rel-16 semi-static and dynamic resource allocation mechanisms are the starting point for supporting Rel-17 multiplexing cases.

- FFS: Applicability for different IAB-DU resource types

- FFS: Cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT

Agreement

- Based on the WID, the following multiplexing cases are in scope for potential support in Rel-17:

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

- Further study  for Case A and Case B at least the following scenarios:

  - Single or multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- Further study for Case C and Case D at least for the following scenarios:

  - Multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- FFS: Required level of specification impact to support the different cases. Any additional specification support in Rel-17 should be conditioned on feasibility from an interference and reliability perspective on a per-link and network basis

For companies to further consider:

Whether the following characteristics of the IAB node implementation will impact the operation of different resource multiplexing cases, including resource partitioning (i.e. identify whether there is a need for potential specification impact/enhancements compared to Rel-16 if the characteristic is or is not supported by an IAB node):

- Baseband (mis)timing alignment between IAB-MT and IAB-DU

- Separate vs. shared antenna panels/RF front-end for IAB-MT/IAB-DU

- Separate vs. shared baseband for IAB-MT/IAB-DU

- Transmitter/receiver implementation

- Self-interference cancellation

- Power control mechanisms

For companies to further consider:

Different resource partitioning scenarios for access and backhaul links, including their respective implication on interference, for different resource multiplexing cases. Examples include:

- Whether a given case is only applicable for certain resource types: e.g. DL only, UL only, DL + UL

- Whether a given case is only applicable for backhaul links or both access and backhaul links

- Note: This should have no impact on legacy UE behavior

Agreement

- Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx)

- Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx)

  - RAN1 should strive to minimize specification impact due to this feature

- FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)

Based on the above agreements and topics to further consider, this contribution describes the potential enhancements related to simultaneous tx and/or rx modes of operation in IAB networks where the different simultaneous tx and/or rx modes of operation are

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

# 2. Enhancements in handling interference

# a. Cross-link interference (CLI):

# When the IAB nodes in a network operate in simultaneous Tx and/or and Rx mode, it will lead to interference among the IAB nodes termed as cross-link interference (CLI). Rel. 16 CLI management mainly focuses on UE-to-UE CLI measurement on SRS and its reporting. However, in case of IAB network, some enhancements to the existing Rel. 16 CLI management mechanism to improve the CLI measurement accuracy is needed. Further, resource partitioning is also an effective method to handle CLI specially to access UEs.

1. CLI measurement accuracy: In Rel. 16, CLI RSRP is measured on SRS transmitted by UE. The receiving UE advances its reception boundaries based on implementation to receive this SRS. However, due to factors like network synchronisation error, unknown propagation delays between the UEs, distance of UE from the gNB etc., the SRS received might be misaligned beyond the CP duration. This will be very prominent in case of IAB networks, where the child IAB node may get connected to a far-off parent IAB node because of the line of sight (LoS) path typically associated with a backhaul link. Also, the different timing alignment requirements across IAB nodes will add to the error in alignment. Further, the CP duration will be very less for FR2 due to the usage of large subcarrier spacing to handle large bandwidth. Thus, the received SRS from such a node may lose samples from the start point. This will degrade the measurement accuracy of SRS RSRP.

Observation 1: Using Rel. 16 CLI management scheme, which is not designed specific to IAB network, the CLI measurement accuracy of SRS RSRP will be degraded due to factors like  network synchronisation error, unknown propagation delays between the IAB nodes, very less CP duration in FR2, different timing alignment across nodes, large distance between child and parent node etc.

Proposal 1: Mechanism specific to IAB network to improve the CLI measurement accuracy as compared to Rel. 16 CLI management is needed.

2. Resource partitioning: Simultaneous operation of IAB node along with UE result in severe interference and performance degradation for UE. For e.g., when a child-MT and a UE Tx to the IAB node at the same time, then at the IAB-DU, the Rx from child-MT will be dominating due to higher Tx power and better channel conditions (mostly LoS). Therefore, UL-Rx from child-MT can overpower UL-Rx from UE, causing performance deterioration for UE. Further, simultaneous operation of IAB node requires support for new timing cases, where there are changes in the timing advance (TA) values and additional cases like negative TA, which the UE cannot understand. Therefore, IAB node cannot perform all simultaneous operation along with UE. Resource partitioning is an approach to overcome the impact of simultaneous operation of IAB node (i.e., CLI, SI, negative TA etc.) on access UE.

Observation 2: Resource partitioning methods can help to overcome the impact of simultaneous operation of IAB node on access UEs.

# b. Self-Interference (SI):

# In Case C and Case D, backhaul and child links of an IAB node are active simultaneously with reception in one link and transmission in the other link. In that case, transmission in one link can interfere with the other link causing self-interference (SI) leading to performance degradation. Moreover, apart from the CLI present in the network, there will be SI within the IAB node. For e.g., in case of an IAB node in DU-Rx-MT-Tx mode, the reception at IAB-DU is affected by the transmission occurring at IAB-MT. The interference issue is much severe in DU-Tx-MT-Rx mode compared to DU-Rx-MT-Tx mode, as Tx power of IAB-DU is higher compared to IAB-MT. Moreover, MT Rx from backhaul in some cases has much less power than a DU-Tx towards UE, especially due to LoS backhaul and when the panel used has high gain. Having different panels at the DU and MT, will ensure more isolation as compared to single panel depending upon the panel structure, position, distance etc. resulting in passive cancellation of SI. On the other hand, SI in single panel systems will be more severe and active SI cancellation circuits are needed, where the exact amount of SI cancellation depends upon the hardware circuit. Thus, in both the cases, the amount of cancellation of SI is implementation specific. However, it is possible in both the cases that some residual SI remains uncancelled depending upon the scenario, power levels at MT and DU etc. This residual SI can be handled by some other techniques like power control to ensure better performance.

# Observation 3: The amount of SI cancellation is implementation specific. Having multi-panel does not fully ensure that there will be no residual SI.

Observation 4: Techniques to handle the residual amount of SI will be independent of whether the system is single panel or multi-panel. The technique should be equally applicable to both single and multi-panel to ensure better performance.

Proposal 2: SI handling methods should be supported for both single panel or multi-panel systems to ensure better performance.

Further, proper measurement of the SI must be carried in IAB node to work effectively in Case C and Case D. For e.g., in Case C, IAB-MT must measure the interference caused by the Tx at IAB-DU. To obtain accurate measurements, IAB-MT has to stop Tx/Rx in the BH link while measuring SI. This can be thought of as equivalent of a measurement gap used by UE to perform handover measurements in other inter-frequency cells. The parent node should not schedule IAB-MT in the SI measuring occasions. Therefore, proper coordination among parent-DU and IAB-MT is required in this context, to avoid conflict and loss of data. Either parent should configure measurement occasions to IAB-MT or IAB-MT should report the measuring occasions to the parent-DU in advance.

Observation 5: IAB node MT might need time-frequency resources for SI measurement, which are free from backhaul reception and transmission. This requires cooperation with the parent.

Proposal 3: SI measurement occasions are required at an IAB node operating in Case C and Case D. The following options can be considered in configuring SI measurement occasions

Alt 1: Parent node configures measurement occasions to IAB-MT at regular intervals.

Alt 2: IAB node requests for measurement occasions to parent node and parent-DU configures it.

Alt 3: IAB node configures measurement occasions and report it to parent node in advance.

Sometimes SI/CLI may become very severe, and an IAB node in simultaneous Tx and/or Rx mode of operation cannot achieve desired performance. In that case, the IAB node will switch to/fallback to the TDM mode. To switch, the IAB node signals fallback request to the parent and donor to indicate that it wants to fallback to TDM mode.  The default configuration of the fallback TDM mode is configured by the parent node either semi-statically or dynamically. Once, the IAB node receives confirmation from the parent regarding the fallback, the IAB node uses the default configuration and start operating in TDM mode.

Observation 6:  Severe SI/CLI will not always allow an IAB node to work in simultaneous Tx and/or Rx mode of operation efficiently.

Proposal 4:  In case of severe interference, IAB node signals fallback request to parent and donor node, and switches to TDM mode with default configuration after receiving confirmation from the parent node. The default configuration of the fallback TDM mode is configured by the parent node either semi-statically or dynamically.

# 3. Enhancements related to power control

In the previous section, the interference issues that can arise in an IAB network were discussed. Power control is one of the methods to handle such interference. For e.g., in case of an IAB node in DU-Rx-MT-Tx (Case D) mode, the reception at IAB-DU is affected by the transmission occurring at IAB-MT. The interference might cause severe degradation in performance when the IAB-DU receives from UEs which transmit at limited power and the IAB-MT transmit at higher power as instructed by the parent IAB node. In the above example, if the IAB node can inform the parent about the interference experienced at the IAB node, then the interference can be mitigated. Thus, there should be a mechanism to inform the parent about the interference level at the IAB node so that the parent node can adjust the Tx power for child-MT. The parent decides on the adjustment of Tx power of child-MT depending upon the scenario. It is important that the parent should be informed about the interference caused at the IAB node. Thus, this can be the starting point for power control mechanisms.

Proposal 5: For Case D, there should be a feedback mechanism regarding the interference at an IAB node from MT to the parent to aid power control.

# 4.  Handling of timing misalignment

Timing alignment procedures for simultaneous MT-Rx-DU-Tx (Case C) and MT-Tx-DU-Rx (Case D) will be useful in reducing the circuit complexity and processing specially in terms of handling SI. However, ensuring timing alignment for these 2 cases might bring up new issues. For eg., in the previous meeting RAN1 #102-e, certain proposals recommended usage of Case 7 timing for aligning reception at IAB-DU with the transmission at IAB-MT in multiplexing Case D. In that case, the TA signaled by IAB-DU to child-MT should ensure that UL-Rx at IAB-DU aligns with UL-Tx at IAB-MT. Therefore, TA signaled by IAB-DU is dependent on the TA used in the backhaul link and will keep on increasing down the hop. Hence, this will not be an efficient method for timing alignment for Case D. Similarly, aligning the reception at MT with transmission from DU in Case C is not possible because DL transmission from all DUs are supposed to be aligned.

Considering the issues explained above, it might not be possible to handle Case C and Case D timings using standards. However, this should not restrict us to implement Case C and Case D modes in IAB. In our understanding, timing alignment for Case C and Case D can be handled in an implementation specific way. However, if any standard specific timing alignment technique is studied which can potentially reduce circuit complexity for Case C and Case D is welcome.

# 5. Conclusion

Observation 1: Using Rel. 16 CLI management scheme, which is not designed specific to IAB network, the CLI measurement accuracy of SRS RSRP will be degraded due to factors like  network synchronisation error, unknown propagation delays between the IAB nodes, very less CP duration in FR2, different TA across nodes, large distance between child and parent node etc.

Proposal 1: Mechanism specific to IAB network to improve the CLI measurement accuracy as compared to Rel. 16 CLI management is needed.

Observation 2: Resource partitioning methods can help to overcome the impact of  simultaneous operation of IAB node on access UEs.

Observation 3: The amount of SI cancellation is implementation specific. Having multi-panel does not fully ensure that there will be no residual SI.

Observation 4: Techniques to handle the residual amount of SI will be independent of whether the system is single panel or multi-panel. The technique should be equally applicable to both single and multi-panel to ensure better performance.

Proposal 2: SI handling methods should be supported for both single panel or multi-panel systems to ensure better performance.

Observation 5: IAB node MT might need time-frequency resources for SI measurement, which are free from backhaul reception and transmission. This requires cooperation with the parent.

Proposal 3: SI measurement occasions are required at an IAB node operating in Case C and Case D. The following options can be considered in configuring SI measurement occasions

Alt 1: Parent node configures measurement occasions to IAB-MT at regular intervals

Alt 2: IAB node requests for measurement occasions to parent node and parent-DU configures it

Alt 3: IAB node configures measurement occasions and report it to parent node in advance

Observation 6:  Severe SI/CLI will not always allow an IAB node to work in simultaneous Tx and/or Rx mode of operation efficiently.

Proposal 4:  In case of severe interference, IAB node signals fall back request to parent and donor node, and switches to TDM mode with default configuration after receiving confirmation from the parent node. The default configuration of the fall back TDM mode is configured by the parent node either semi-statically or dynamically.

Proposal 5: For Case D, there should be a feedback mechanism regarding the interference at an IAB node from MT to the parent to aid power control.

# 6. References

# [1] RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e, August 2020.