3GPP TSG RAN WG1 #103-e		                                    		 R1-2008819

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.5.4

Source:	InterDigital Inc.

Title:	Discussion on potential enhancements for supporting XR

Document for:	Discussion and Decision

# Introduction

The Rel-17 SID on Study on XR Evaluations has the following objectives [1]:

The following applications are to be considered as starting points for this study: - VR1: “Viewport dependent streaming” - VR2: “Split Rendering: Viewport rendering with Time Warp in device” - AR1: “XR Distributed Computing” - AR2: “XR Conversational” - CG: Cloud Gaming Note: Use cases in quotes are from TR26.928. The following traffic parameters for the different applications are to be considered as starting point for the study: Traffic characteristics: - UL and DL File Size distribution (e.g., Pareto with given parameters) - UL and DL File arrival time distribution (e.g., Periodic every 1/60 seconds) Traffic requirements: - Round-trip-time or UL and DL one-way Packet delay budget (PDB) - UL and DL Packet error rate (PER) The objective of this study item are as follows: - Confirm XR and Cloud Gaming applications of interest - Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc. - Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios - Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs Note 1: eURLLC SI/WI work relevant to XR should be taken into consideration. Note 2: Traffic model for the performance evaluation shall be based on the standardization in SA WG4

In this contribution, by considering of the assumptions for XR traffic and XR devices identified in SA4 TR 26.928 [2], we discuss the potential enhancements related to coverage, capacity, power consumption, support for sidelink and positioning that can be considered for supporting the XR applications/use cases.

# Discussion

## Enhancements for Coverage and Service Continuity

From the description in SID [1] and the SA4 TR [2], it is apparent that several traffic requirements for XR are shared with those for eMBB (e.g., high data rates) and URLLC (e.g., high reliability and low latency). As such, the assumptions related to coverage and service continuity applied for eMBB and URLLC can also be used as baseline for identifying the gaps for further enhancements.

For VR use cases, the data rate requirement estimated per-user in good coverage deployment scenarios range between 0.1 to 1 Gbps. In this regard, small cells/indoor hotspot deployment and FR2 can be considered for supporting high data rates for VR. However, given the susceptibility to blockages for FR2 links and the consequent loss in coverage, other enhancements to improve the robustness of the links should be considered for XR, especially solutions that can leverage on the sensors available in the XR device (e.g. for identifying presence of blockages).

In the case of AR, it is expected that high data rates are consistently and continuously supported even during mobility with minimal service interruption. This can be an issue for AR in outdoor/urban scenarios where shadowing, deep fades and blockages to LOS links can be prevalent. Considering the use of split rendering architecture, it can be more challenging to maintain high data rates and low latency due to degradation of coverage at cell edge during mobility when part of the rendering functionality is offloaded to an edge function/server. In these scenarios, preemptive/predictive mechanisms for indicating to the higher layers/application based on the detection of change in coverage or mobility at lower layers in UE and RAN can be considered to minimize any service interruption events.

Proposal 1: 	The enhancements considering the capabilities of XR device and proactive/predictive mechanisms for preventing loss in coverage and supporting service continuity should be studied

## Enhancements for Capacity

Achieving high capacity is a key challenge when it comes to wide deployment and adoption of XR and Cloud Gaming use cases. Given the similarities between XR and URLLC traffic requirements, the evaluation methodology applied (TR 38.824) for determining the capacity of URLLC, defined as the maximum offered cell load under which Y% (e.g. 95%) of URLLC UEs in a cell with target link reliability R under latency bound L, can be used as a starting point for capacity evaluations for XR.

In addition, the enhancement considered for improving capacity of eMBB and URLLC UEs, including the use of massive MIMO/M-TRP and/or FR2, can also be considered for XR. However, depending on XR device form factor (e.g. the use of AR glasses) and the possible antenna configurations, the existing capacity enhancements may still impose a limitation on the number of XR UEs supported per cell. Other enhancements that can be further studied include allowing interaction between RAN and higher layers to support RAN awareness of UE viewport and pose information for more efficient spatial multiplexing, beam tracking and beam switching.

Proposal 2: 	The enhancements for supporting interaction between RAN and higher layers for improving capacity for XR should be studied

## Power Consumption Enhancements for XR

For XR and Cloud Gaming use cases, it is extremely challenging to support broadband URLLC (eMBB+URLLC) traffic requirements combined with low power consumption in the XR device. Given the quasi-periodic nature of XR traffic (e.g. high inter-packet arrival rate) in DL and UL for transmitting video/media and pose information, the opportunities for operating in low power mode (e.g. idle/inactive modes) or prolonged sleep DRX cycles are minimal.

For wearable XR devices such as lightweight HMDs and AR glasses, the battery capacity can be limited due to form factor restrictions. Additionally, depending on the XR use case and architecture, the available battery capacity is used either mainly for internal processing/computation related to rendering and display or for communication. For example, for VR using standalone device the internal processing for rendering and pose correction upon receiving the video/media in DL is expected to consume most of available battery capacity. In split rendering architecture used for AR and certain VR applications, the power consumption for internal processing is traded off for supporting higher communication due to transmission of video in UL and reception of pre-rendered video/media in DL. While the use of tethering between the XR device and a UE (e.g. smartphone) can alleviate the power consumption issue by offloading certain processing to the UE, the expected power savings may not be adequate for prolonging the device usage due to supporting higher communications on the SL for carrying the XR traffic.

In this regard, the solutions from Rel-16, such as BWP-based power reduction from reduced DCI monitoring and dormant SCells can be considered as baseline. However, the existing solutions may be incompatible for supporting high number of XR UEs per cell requiring high throughput and satisfaction rate (e.g. ≥ 95%), hence justifying the need for enhancements. The other aspects that can be considered in the study are enhancements to C-DRX configuration to increase sleep duration and the support for more efficient scheduling solutions which allow timely resource allocation (e.g. for minimizing DCI monitoring occasions) based on the awareness of quasi-periodic XR traffic characteristics.

Proposal 3: 	Power saving enhancements (e.g. XR traffic aware scheduling, optimized C-DRX sleep duration) considering the different XR traffic/architecture models (e.g. split, non-split) should be studied

## Sidelink Enhancements for XR

AR applications (e.g. AR1 and AR2) typically require the use XR devices with small form factor (e.g. XR glasses) for transmitting pose/video and displaying the received media. However, given the limitation in the battery capacity and processing capability in the XR device, a wireless tethering mechanism to a paired UE (e.g. smartphone) may be used to relay the traffic to and from the XR device with high data rate and low latency. In addition, the tethered UE could also be used to perform local processing (e.g. post rendering of video/media) prior to transmitting to XR device.

In this regard, the Rel-16 NR sidelink (SL) along with Rel-17 NR SL relays may be considered as a baseline for supporting sidelink-based tethering for XR. For example, NR SL can be used as the wireless interface between XR device and UE for supporting high data rate and low latency transmissions. Similarly, NR SL relays can be used for ensuring end-to-end QoS between the XR device and RAN via the relay UE. Further enhancements that may be considered for NR SL and NR SL relaying, given the stringent traffic requirements for XR, include the use of relay UE assisted scheduling for the XR device (e.g. Mode 2 b/d) for fast SL resource allocation, SL assisted low power operation for XR device (e.g. aligned C-DRX), and SL assisted positioning for improving the positioning accuracy of XR device viewport.

Proposal 4: 	The enhancements for NR SL and SL relaying, considering the use of wireless tethering for supporting XR traffic requirements, should be studied

## Positioning Enhancements for XR

The different XR applications and use cases considered in the SID [1] and the SA4 TR [2] require UE positioning information (e.g. user viewport and pose) for determining the video or media to be streamed and rendered in the XR device. The type of the UE positioning information, including the UE location (i.e. geographic coordinates), viewport orientation, and other pose related information (e.g. 6DoF), may vary depending on the XR application and the capability of the XR device. For example, for VR1 applications (i.e. viewport dependent streaming) the user pose along with the viewport information is sent in the UL and in response, the video/media which is optimized to the user pose/viewport is received interactively in DL. Similarly, for AR1 application (i.e. distributed computing), the tracking and sensor information related to the user’s orientation and pose in indoor/outdoor scenarios is sent in UL for in-network computation and the pre-rendered video/media is received in the DL.

In the existing XR applications, the UE positioning information is typically determined using RAT-independent positioning methods including GNSS and internal sensors. Specifically, the XR applications may rely on either inside-out tracking (e.g. using device-oriented beacons) or outside-in tracking (e.g. using beacons from external transmitters) for determining user/viewport position. The determined positioning information is used in the device (e.g. for pose correction) or sent in UL as high layer/application traffic flow, transparent to access stratum and RAN. While the use of RAT-independent positioning methods may be adequate for certain simple XR applications, the degree of accuracy and latency achievable for positioning (e.g. < 3m and >1s) may not be sufficient to provide high QoE to the user, especially for advanced XR applications such as XR gaming and industrial AR where the precise placement of 3D objects is vital.

For the considered advanced XR applications (e.g. VR2, AR2), higher precision tracking (e.g. <10 cm) of the user’s position and mobility, including the 6DoF rotational and translational movements may be necessary. Additionally, the positioning information should be determined and delivered with low latency for supporting the low motion-to-photon latency (e.g. <20ms) requirements. Considering the case for small form-factor XR devices (e.g. AR glasses), which may have limitation in the sensors and processing capability for determining positioning with high precision and low latency, alternative positioning techniques may be considered for further study.

In this regard, the Rel-16 RAT dependent positioning methods along with the Rel-17 enhancements, based on DL PRS measurement or UL SRS for positioning transmission, may be considered as a baseline to support the high accuracy and low latency positioning requirements expected for XR. Further enhancements that may be considered for RAT-dependent positioning methods for XR may include computation of other 6DoF parameters with high accuracy, independent of the direction or location of UE. Additionally, the coordination and multiplexing of positioning RS transmission along with the XR traffic in UL and DL can be considered for further enhancements.

Proposal 5: 	The enhancements for achieving high accuracy and low latency positioning for XR use cases should be studied

# Conclusion

The following are the conclusions made in this contribution:

Proposal 1: 	The enhancements considering the capabilities of XR device and proactive/predictive mechanisms for preventing loss in coverage and supporting service continuity should be studied

Proposal 2: 	The enhancements for supporting interaction between RAN and higher layers for improving capacity for XR should be studied

Proposal 3: 	Power saving enhancements (e.g. XR traffic aware scheduling, optimized C-DRX sleep duration) considering the different XR traffic/architecture models (e.g. split, non-split) should be studied

Proposal 4: 	The enhancements for NR SL and SL relaying, considering the use of wireless tethering for supporting XR traffic requirements, should be studied

Proposal 5: 	The enhancements for achieving high accuracy and low latency positioning for XR use cases should be studied

# References

- RP-201145, Rel-17 SID on Study on XR Evaluations, Qualcomm, June 2020

- 3GPP TR 26.928, Extended Reality (XR) in 5G, v.16.0.0, Mar. 2020