3GPP TSG RAN WG1 #103-e                                      R1-2008855

e-Meeting, October 26th – November 13th, 2020

Source:	ZTE

Title:	Discussion on enhancements for IoT-NTN

Agenda Item:	8.15.2

Document for:   Discussion

# Introduction

In RAN#86 meeting, the SI for IoT-NTN [1] has been endorsed with two sets of objectives including study for link budget, timing, etc. In this contribution, related technical issues including synchronization, timing, power consumption etc. are identified and potential solutions are discussed.

# General considerations for IoT-NTN

Comparing to the terrestrial IoT network, the NTN based solution is targeted to provide the IoT service with larger coverage and flexibility (e.g., isolated region) for various commercial cases. For well supporting such usage, detailed studies with consideration on the intrinsic characteristics of NTN system should be conducted for both NB-IoT and eMTC.

In this way, before the triggering of discussion on each topic, a general investigation on the KPIs for the IoT-NTN should be conducted firstly. For example, w.r.t the TN based on network, according to the ITU requirements [2], massive devices with density equating to 1,000,000 device/km2, should be supported, which may be not achievable for the NTN case with much larger coverage per cell. Meanwhile, to provide the service for UE with deep fading (e.g., MCL = 164 dB), different UE Cat/coverage level/transmission configuration are supported in TN case. For the NTN, with enlarged Tx-Rx separation, the link budget for NTN may not fulfill the needs for all services. The corresponding enhancements will also lead to potential impacts on the power consumption, similar as the needs for pre-compensation.

Proposal 1: Investigation on the target KPIs (e.g., UE density, power consumption, etc.) for the target scenarios should be prioritized.

# Technical issues to support IoT over NTN

## Coverage enhancement

As aforementioned, the existing specification is targeted for the coverage with MCL up to 164 dB. But according to the preliminary results shown in Figure 1 (according to the same assumption listed in [3]), it can be found that for the basic scenarios (e.g., evaluation angle = 10 degree), the MCL of around 5% (e.g., for GEO) UEs is larger than 164 dB even without consideration of additional vegetation loss for rural scenario as highlighted in [3]. After taking more realistic assumption, e.g., to adding the vegetation loss for NLoS UE by assuming d = 10m, the coupling loss is degraded.

Figure 1 Illustration on the CL for DL

In this case, potential enhancement on the existing channel should be carefully checked, e.g., NPSS/NSSS or NPDSCH/NPUSCH channel. As one simple solution, enlarged repetition number may be needed for eMTC and NB-IoT.

Proposal 2: Potential enhancement to enhance the coverage for IoT should be studied.

## Synchronization

In NR-NTN, evaluation on the synchronization performance for both DL and UL has been conducted. For the IoT-NTN, since more sparse RS are assigned in the system, e.g., non-SRS for UL in NB-IoT, non-TRS for NB-IoT and eMTC, the performance for initial synchronization and tracking will be impacted due to the movement of satellite, e.g., in LEO case. And more detailed studies are needed for both DL and UL.

## DL Synchronization

As aforementioned, it has been proved that the robust performance can be provided for DL synchronization (at least during the initial access stage) via SSB design with assuming the pre-compensated DL Doppler w.r.t the beam center by gNB via implementation. The similar assumption can also be considered in the evaluation for IoT.

Meanwhile, in [4], detailed methodology and evaluation are provided to evaluate the performance of NPSS for NB-IoT design with different assumptions on the coupling loss, e.g.,164, 154 and 144 dB, which corresponds to required SINR of -12.6, -2.6 and 7.4 dB is given in Table 1.

In TN based IoT, such evaluation mainly focus on the impact of inaccurate UE oscillator with assumption of slow fading channel (e.g., Doppler frequency = 1Hz [5]). In this way, except for the frequency sweep to match the sync raster, coherent combination over time domain (for multiple NPSS periods) is also considered to deal with power SINR for NPSS detection.

Table 1 Example of MCL table for NB-IoT NPSS [5]

| Parameter | NPSS |  |  |
| --- | --- | --- | --- |
| (1) Tx Power in Occupied Bandwidth (dBm) | 35 | 35 | 35 |
| (2) Thermal Noise Density (dBm) | -174 | -174 | -174 |
| (3) Occupied Bandwidth (kHz) | 180 | 180 | 180 |
| (4) Receiver Noise Figure (dB) | 5 | 5 | 5 |
| (5) Interference Margin (dB) | 0 | 0 | 0 |
| (6) Effective Noise Power (dBm) = (2)+10log10((3))+(4)+(5) | -116.4 | -116.4 | -116.4 |
| (7) Required SINR (dB) | -12.6 | -2.6 | 7.4 |
| (8) Receiver Sensitivity (dB) = (6)+(7) | -129 | -119 | -109 |
| (9) Rx processing gain (dB) | 0 | 0 | 0 |
| (10) Maximal Coupling Loss (dB) = (1)-(8)+(9) | 164 | 154 | 144 |

However, for IoT over NTN, additional impacts on the satellite movement should also be considered as one key factor. For example, after the implementation beam specific pre-compensation on the Doppler shift, the impact is mainly determined by the distribution of residual Doppler for each UE. And it can be modelled as the uniform distribution, where the range is determined by the residual Doppler shift for beam edge UE. Then, the performance for the synchronization signal detection along with potential ambiguity on cell search via frequency sweep on the sync raster should be evaluated.

In addition, for the normally PDCCH/PDSCH reception in NB-IoT, the limited RS, e.g., NRS, will be considered for channel tracking (e.g., timing and frequency) to ensure the decoding performance for data.  Situation may be better for eMTC since additional RS as CSI-RS/CRS can be considered. But for all cases, the corresponding performance with consideration on the potential Doppler shifts/Timing drift should also be studied. Otherwise, even with larger repetition, the combination gain cannot be achieved due to the variation of Doppler and unaligned timing.

Proposal 3: DL synchronization mechanism should be evaluated with consideration on the characteristic of NTN system.

## UL synchronization

In IoT-NTN, since UEs are assumed with GNSS capability. Therefore, pre-compensation methods proposed for UL synchronization in NR-NTN, e.g., geometric based and timestamp based solutions, can also be considered in IoT-NTN.

In general, according to these solutions, the UL pre-compensation is performed with GNSS signal and/or assistance information. In the IoT-NTN SI [1], it is highlighted that simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. Therefore GNSS receiving and corresponding pre-compensation may be done before UL data transmission. However, the duration for each transmission, e.g., PRACH or PUSCH with long duration (e.g. 256 ms, 1 s) may happen for IoT devices, especially with large repetition number. In this case, with consideration on the satellite movement (at least), updates on the pre-compensation value of time delay and frequency offset during the transmission time interval is expected. Otherwise, the accumulated error, e.g., TA, will decrease the system performance as shown in Figure 2. For example, the TA error will exceed the CP length for corresponding channel with long transmission duration.

- Pre-compensation per 256 ms

- Pre-compensation per 2560 ms

Figure 2 Illustration on the timing error for NB-IoT with different granularity for pre-compensation

Observation 1: Performance degradation will occur for the continuous transmission with larger repetition.

Then, receiving GNSS signal or assistance information during a continuous repetition transmission should be considered to enable the updates of TA and Doppler estimation in connected mode. Moreover, segmented pre-compensation should also be considered. Under these conditions, the long repetition transmission can be divided to multiple segments, then the TA/Doppler for each segment can be well estimated and pre-compensated.

Considering the limitation of receiving time of GNSS, and also the legacy gap of 40 ms for NB-IoT, which is inserted after a period of continuous PRACH and/or PUSCH transmission, GNSS reception during the gap can be considered. Further, dedicated gap may be configured in order to ensure the HD-FDD UE receiving GNSS signal or assistance information. e.g., legacy gap, can be extended forward or afterward with a length of propagation delay.

Proposal 5: Segmented pre-compensation for long continuous repetition transmission should be considered.

Proposal 6: Receiving GNSS signal or assistance information during a continuous repetition transmission should be considered.

## Beam management

In NTN deployment, the larger coverage of a NTN cell can be implemented by either a single or multiple satellite beams. Similar as identified in NR, obvious benefits can be achieved to support the latter one (multiple beams per cell), especially for LEO cases, for example:

- As aforementioned, for ensuring the DL synchronization performance, the pre-compensation on the Doppler shift w.r.t the beam center is assumed. In this way, the UE only needs to deal with the residual part. If the whole coverage of one satellite is done by single cell with one beam, the remaining Doppler shift will be larger and corresponding performance will be degraded.

- From another aspect, if multiple cells are deployed under the coverage of one satellite with small beam, due to movement of satellite, frequently handover will be considered for the UE within the either connected or idle mode with larger overhead and power consumption. More specifically, in legacy NB-IoT, connected mobility (e.g., handover and measurement report) is not supported, since both NB-IoT UEs and eNBs are assumed to be generally stationary on Earth. This assumption is invalid in IoT over NTN. Thus, corresponding enhancement with additional impacts will be definitely needed for IoT over NTN scenarios.

Then, support the deployment with multiple beams per cell should be considered for IoT-NTN with corresponding investigation for beam management.

Furthermore, following the NR discussion, the beam may also be mapped to certain frequency resource, e.g., anchor/non-anchor carrier for NB-IoT and narrowband for eMTC. Meanwhile, for the IoT, additional index can also be considered for the beam distinguish, which is similar as the NR SSB index.

In addition, in case of transmission with larger repetition, the beam switching may occur during the transmission time interval, which is different as NR. Then, for IoT-NTN, corresponding enhancements should also be studied to ensure the performance.

Proposal 7: Deployment with multiple beams per cell should be studied with potential enhancement to enable the beam management.

## Timing relationship

In general, the timing relationship enhancement is mainly to deal with the impacts of larger RTT, which is common for NTN regardless of RAT. Then, the concluded cases in NR-NTN can be followed for study in IoT case. For example, in NB-IoT, since the PUCCH is not supported in NB-IoT, the timing relationship for HARQ-ACK can be handled by introduction of K_offset on NPUSCH transmission.

W.r.t the solution, as discussed in [6], both implicit and explicit indication of value of K_offset can be considered. And beam-specific Koffset via SIB can also be applied once the concept of beam is supported via either explicit or implicit way. Meanwhile, in order to saving signaling overhead, different units should be also considered for each scenarios.

Proposal 8: Investigation on the time relationship based on the progress of NR-NTN can be considered.

## Channel capacity enhancement

For NTN, larger coverage per cell is expected with more potential UEs. In this way, the existing capacity for channel, especially the common channel, will be limited [7] and corresponding enhancements should be considered. For example, in existing design for NPRACH as shown in Figure 3, the corresponding hopping pattern is used to enable the TA estimation. However, with assumption on the pre-compensation, such needs may not be urgent. Then, resource reuse to enhance the capacity of NPRACH can be considered.

Figure 3 An example of PRACH preamble and PRACH occasions in NB-IoT

Proposal 9: Investigation to enhance the channel capacity can be considered.

## Power consumption

As one key KPI for IoT device, the long battery life is critical to enable the IoT service. However, according to the preliminary study in [7], additional power consumption may occurs in case of IoT-NTN comparing to the legacy TN system. Then, potential solution to improve the power consumption should be evaluated from either specification-based or implementation based way.

Proposal 10: Investigation on the power consumption along with potential enhancement can be can be considered.

# Conclusions

In this contribution, related technical issues and solution to support NB-IoT and eMTC over satellite is conducted with following observation and proposals:

Observation 1: Performance degradation will occur for the continuous transmission with larger repetition.

Proposal 1: Investigation on the target KPIs (e.g., UE density, power consumption, etc.) for the target scenarios should be prioritized.

Proposal 2: Potential enhancement to enhance the coverage for IoT should be studied.

Proposal 3: DL synchronization mechanism should be evaluated with consideration on the characteristic of NTN system.

Proposal 5: Segmented pre-compensation for long continuous repetition transmission should be considered.

Proposal 6: Receiving GNSS signal or assistance information during a continuous repetition transmission should be considered.

Proposal 7: Deployment with multiple beams per cell should be studied with potential enhancement to enable the beam management.

Proposal 8: Investigation on the time relationship based on the progress of NR-NTN can be considered.

Proposal 9: Investigation to enhance the channel capacity can be considered.

Proposal 10: Investigation on the power consumption along with potential enhancement can be can be considered.

# References

- RP-193235, Study item on NB-IoT/eMTC support for NTN, MediaTek Inc.

- RP-191524 3GPP ITU-R Contribution - Final IMT-2020 Submission Overview

- R1-2008854 Preliminary views on the scenarios and assumption for IoT-NTN

- TR 45.820 v1.3.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”

- R1-161918, NB-PSS and NB-SSS Design, Qualcomm Incorporated.

- R1-2008850 Discussion on timing relationship for NTN, ZTE.

- R1-2008856 Discussion on power consumption and NPRACH capacity for NTN