3GPP TSG RAN WG1 #103-e			R1-2008863

e-Meeting, October 26th – November 13th, 2020

Agenda item:		8.10.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancements for resource multiplexing among IAB backhaul and access links

Document for:		Discussion and Decision

# 1	Introduction

IAB Rel.17 WID [1] has following objectives led by RAN1:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In RAN1 #102-e meeting [2-6], several agreements related to resource multiplexing were made and they are captured in the Annex I. In this contribution, we discuss detailed enhancements to the resource multiplexing between child and parent links of an IAB node. The multiplexing cases for simultaneous DU and MT operation are named as

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

Section 2 focuses on simultaneous half-duplex operation of the child and parent links (Case A and B). The full duplex support (Cases C and D) we see as an implementation issue. The problem there is the interference from DU Tx to MT Rx or from MT Tx to DU Rx, and we see nothing to specify for resource allocation that would help solving this problem.

Section 3 discusses resource allocation enhancements related to dual connectivity scenarios.

# 2	Multiplexing operations of the child and parent links

## 2.1	Case A/B: Semi-static and dynamic resource allocation mechanisms

With TDM operation, IAB node MT and DU are not active simultaneously: MT DL Rx, MT UL Tx, DU DL Tx, and DU UL Rx take place in different symbols. With SDM/FDM half-duplex operation (multiplexing Case A and B) MT and DU may be transmitting or receiving simultaneously. FDM operation means that MT and DU are receiving or transmitting at different frequency resources while with SDM the DU Rx/Tx is spatially isolated from MT Rx/Tx.

A RAN1 #102-e agreement is:

Agreement

The Rel-16 semi-static and dynamic resource allocation mechanisms are the starting point for supporting Rel-17 multiplexing cases.

- FFS: Applicability for different IAB-DU resource types

- FFS: Cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT

The first of the above points refers to the Hard (H), Soft (S), and Not Available (NA) character of DU symbols.

With TDM multiplexing, H symbols are allocated exclusively for DU to schedule for child MTs or access UEs i.e. MT scheduling does not affect the DU resource use. With FDM/SDM multiplexing, MT scheduling does not necessarily exclude DU resource use but may anyway seriously limit that. An example is SDM in FR2 and the situation that IAB node must serve a child node or UEs with the same antenna panel that is used for communication with the parent node. In this situation, the simplest way of providing guaranteed DU service opportunities would be configuring some symbols with H character.

NA symbols configured at a given IAB node are not available for DU and can be reserved for MT. On the other hand, when an IAB DU has been configured with NA symbol, a child node and/or parent node, without beyond-TDM capability, has the symbol available for DU scheduling. Thus, a NA character would be useful at least when SDM/FDM capable node is operating with nodes without such capability.

S symbols are conditionally available for DU scheduling. They may be made available by parent signaling them available using DCI 2_5 or the availability may be determined implicitly by the IAB node when (a) the IAB-node MT does not transmit or receive in the symbol or (b) the IAB-node MT is to transmit or receive in the symbol, and the transmission or reception in the symbol is not changed due to use of the symbol by IAB-node DU. The implicit availability of S symbols would be important for utilization of the FDM/SDM capability of a node.

To summarize we propose:

Proposal 2.1: The S/H/NA resource types are applied with FDM/SDM multiplexing.

According to Rel-16 specification, the symbols with Cell-specific/semi-static signals and channels at the IAB-DU (TX of SS/PBCH block, PDCCH for Type0-PDCCH CSS sets, periodic CSI-RS, or RX of PRACH or a SR) are considered as hard. Furthermore, the parent node may know the child DU configurations of these signals and channels. There seems no point to completely forbid MT being active when DU is handling the Cell-specific signals and channels. Instead, DU should be given the priority and MT would be allowed to operate according to Case A or B configuration if that can be done without compromising DU RX or TX.

Proposal 2.2: For all multiplexing modes, cell-specific/semi-static signals and channels of the IAB-DU shall be considered as hard resources (like in Rel-16). The parent may be required to know these cell-specific signals and channels only when IAB node has a certain restriction on supporting the multiplexing mode due to cell-specific/semi-static signal and channels.

As discussed above, even with FDM/SDM multiplexing the DU scheduling could be based on S/H/NA resource characters. However, indication of frequency and spatial availability must be introduced on top of S/H definitions for efficient dividing of the resources for FDM and SDM between MT and DU:

- For FDM, the frequency availability signaling allows IAB node to decide which frequency resources are available or excluded for its DU.

- For SDM, the availability signaling would mean restrictions in the beam and/or panel selection of the DU indicated by an angular restriction.

The frequency availability may be semi-static or dynamic. The semi-static availability applies with hard UL or DL symbol types and is controlled by CU that signals frequency resources that are available for DU scheduling. The following ways of dividing resources may be used:

- Different CCs than those active for IAB MT may be used by the DU: RRC shall not configure any CCs for the child nodes (served by IAB DU) which are overlapping with the CCs configured to the IAB MT.

- If a CC is configured for both the MT of the IAB node and child nodes (served by IAB DU), it is possible to configure non-overlapping BWPs in parent and child links.

- If a CC is configured with overlapping BWPs for the MT of the IAB node and child nodes, CU configuration may restrict the hard resource availability to only certain PRBs.

The first two in the list above are supported by present specification and, for flexibility, we propose adding support for the third one:

Proposal 2.3: For FDM operation of the IAB node, the availability of PRBs in hard symbols shall be additionally indicated via semi-static CU signaling.

Dynamic frequency and spatial availability applies with soft symbols and is at least partly under the control of the parent node. The presently specified implicit soft symbol availability determination allows FDM or SDM in a limited fashion. For instance, when MT is configured for DL, DU may be allowed to schedule spatially separate child UL that does not interfere with MT Rx. For supporting FDM or SDM in other situations, the availability indicator for DU soft symbols should be modified:

For FDM in DU soft symbols, parent frequency availability signaling allows IAB node to decide which frequency resources are available or excluded for its DU soft symbols. When scheduling DL resources on the IAB DU, the resource availability can be determined implicitly at the IAB node provided K2 < K0, but in general explicit signaling will be necessary to coordinate multiplexing. The available RBs would be those which are not scheduled for the MT and not needed as a guard between MT and DU RBs. A new message is required to signal the RBs that will be scheduled for MT DL or, alternatively, that will be free for DU UL scheduling.

Proposal 2.4: For FDM operation, a dynamic indication of frequency availability of soft resources shall be controlled by the parent via introducing frequency availability indication.

For SDM in DU soft symbols, the new availability signaling would mean parent restricting certain angular directions that the IAB DU should not be using with its transmission beams. This could also be associated with a panel recommendation coming from the parent node. However, we shall further investigate the possibility of reusing NR MIMO framework to enable such indication. This type of SDM availability signaling enhancement may be more useful for SDM with MT in DL, where the IAB node may decide by itself how to schedule DU UL without excessive interference between MT and DU Rx signals. For this, the parent should signal in advance that it intends to schedule MT DL in a slot. For the normal MT UL, the scheduling may come early enough to be taken into account for DU DL scheduling but in general, as with FDM, explicit signaling will be necessary to coordinate multiplexing.

Proposal 2.5: For SDM operation, a dynamic indication of spatial restrictions (or availability) of soft resources shall be further studied to enable efficient SDM operation at the IAB node.

For both FDM and SDM, when child DU has F-S resources, availability signaling should tell which link direction is available. In general, link direction of an F-S resource of the child DU is decided by the child node. However, allowing full freedom of deciding the directions that child DU can transmit or scheduled UL transmissions would impact parent link(s), including the BH link performance. For example, certain angular restrictions (or availability) on F-S could further instruct the directions (DL/UL) available for the IAB node to allow SDM operation.

Proposal 2.6: For FDM/SDM operation, allowed direction of the transmission for the IAB DU in F-S resources may be further controlled by the parent node by using a dynamic indication.

## 2.2	Case A and B: Additional considerations

Another set of questions to clarify after RAN1#102-e [2] is shown below in Italic with our comments included after each question.

For companies to further consider:

Whether the following characteristics of the IAB node implementation will impact the operation of different resource multiplexing cases, including resource partitioning (i.e. identify whether there is a need for potential specification impact/enhancements compared to Rel-16 if the characteristic is or is not supported by an IAB node):

- Baseband (mis)timing alignment between IAB-MT and IAB-DU

At least considering Case A and B together with the respective timing modes, it is not clear what impacts companies foresee under this. Also, discussion on baseband timing alignments are fully implementation-related and should not be the main focus of RAN1 discussions.

- Separate vs. shared antenna panels/RF front-end for IAB-MT/IAB-DU

With shared MT and DU antenna panels (in FR2), MT using a panel prevents the IAB node from serving UEs and child nodes using the same panel. In beam management, it is well understood that simultaneous reception or transmission of beams using the same panel is not possible. For example, if the parent schedules DL reception for the MT of an IAB node whose DU schedules UL transmission (i.e. there is SDM multiplexing Case B) and the reception beams of the MT and DU are at the same panel, the IAB node may not receive either of the transmissions. This means that a parent should be made aware of its child node’s restrictions on supporting the reception by certain beams in DL (and UL). One method to allow this knowledge is frequent beam measurements and reporting of the parent link such that IAB node can decide which beams/panels to use for parent link transmissions and scheduling of the child nodes. However, it may not be a feasible solution due to unnecessary overhead/complexity. Without extra beam measurements, if the IAB node can report the changes on activated beams for the parent link, that information could be used by the parent to avoid any overlap and still support the IAB MT with any other activated beam. In another method, the child IAB node might indicate that MT panel use does not prevent its DU serving any UEs or child nodes. Or if a parent could be received through more than one panel, child could indicate which of the parent beams is optimal for it not just taking into account the signal quality but also its DUs needs of panel use.

- Separate vs. shared baseband for IAB-MT/IAB-DU

Maybe the question could be taken into account by processing capability classes i.e. IAB nodes with limited processing power due to shared baseband or because of other reasons could indicate a limited SDM support.

- Transmitter/receiver implementation

No specification impacts.

- Self-interference cancellation

No specification impacts.

- Power control mechanisms

For simultaneous transmission, the existing methods of power sharing should be used. Setting of IAB DU TX and IAB MT power could be handled like in DAPS or other situations of simultaneous UL. Proper power balancing would be useful for simultaneous reception by DU and MT (FDM or FR1 only). This is in limited extend possible through controlling the child MT TX power.

From the above set of questions, at least optimizing the operation with shared antenna panels is worth studying:

Proposal 2.7: For SDM operation, RAN1 shall consider the case of sharing of panels and further investigate the required beam reporting enhancements to enable simultaneous transmission/reception of parent and child links.

Proposal 2.8: For sharing of antenna panels between MT and DU, consider child node indicating for the parent node the changes of beams or panels used for reception.

Yet another question after RAN1#102-e is [2]:

For companies to further consider:

Different resource partitioning scenarios for access and backhaul links, including their respective implication on interference, for different resource multiplexing cases. Examples include:

- Whether a given case is only applicable for certain resource types: e.g. DL only, UL only, DL + UL

- Whether a given case is only applicable for backhaul links or both access and backhaul links

- Note: This should have no impact on legacy UE behavior

We assume that UL and DL in the question refer to the UL and DL parts of TDD pattern. Assuming further that UEs would always follow the TDD pattern, the multiplexing Cases A and B would be as shown in Figure 1. Considering that IAB-MT is only allowed to transmit in UL or Flexible slots as it follows most of the behaviors defined for access UEs, it is not fully clear how this would impact on specifications when defining IAB only behavior for UL transmission in DL resources or DL reception in UL resources. For example, SSB measurements, RACH transmissions, DCI monitoring and impacts on RAN4 requirements should be carefully checked for possible impacts. Therefore, it is reasonable to follow the same principles as access UEs even in Case A and B multiplexing modes.

Proposal 2.9: For Case A and B, no additional relaxation is supported for using DL resources in uplink transmission (Case A) or using UL resource in DL reception (Case B) by the IAB-MT.

Figure 1: Multiplexing Cases A and B when BH transmissions are allowed in UL part of TDD pattern.

# 3	Resource configurations for IAB DC operation

RAN1#102 discussed the DC enhancements with following conclusion and agreements:

Conclusion

At least the inter-carrier DC scenario can be considered in Rel-17. Further discussion in RAN3/RAN Plenary may be necessary for the intra-carrier DC scenario:

Agreement

Reuse by IAB-MT of existing Inter-frequency DC is considered as a starting point to support concurrent BH links to two parents.

- FFS: Reuse of multi-TRP transmission resource allocation features (if intra-freq DC scenario is supported for IAB)

- FFS: Additional specification effort to support IAB

For companies to further consider:

The following categories of enhancements have been proposed to support DC scenarios (not an exhaustive list):

- Inter-parent DU resource coordination mechanisms and signaling

- Resource allocation/scheduling conflict resolution rules at the parent or child node

- Per-link IAB-DU resource configurations at the parent node

For RAN#96 there was also an email discussion [RP-202083] on the support for intra-frequency DC but there was no agreement leaving further discussion/decision to RAN1.

As identified earlier [R1-2005535], the resource configuration and allocations with intra-frequency DC would require enhancements in following areas:

- Rules of using resources with two MT configurations and scheduling from MCG and SCG; this is an issue with inter-donor DC with two CUs

- SDM/FDM support including the multiplexing between the parent links

- Determination how to schedule the child link connections being part of the child node DC connections, either MCGs or SCGs

- Coordination of DU configurations of the two parent nodes to avoid conflicting allocations causing issues with the DC reception at the IAB-MT; with single donor DC this can be handled within the CU

- Timing issues, e.g. guard periods required on the parent links and TA control on the child link(s)

Out of these the first one and the “multiplexing between parent links” of the second bullet would be applicable for possible UE intra-frequency DC, the rest being IAB specific.

Observation 3.1: Due to missing support for intra-frequency DC for UE, the baseline is not complete for the IAB specific enhancements required for intra-frequency DC.

Up to now intra-frequency operation on two simultaneous connections has been specified for DAPS which is a mobility enhancement minimizing the service break rather than providing option for simultaneous UP connections. As known, intra-frequency DC is not supported for UEs in current specifications.

DAPS has following specific features differing from the DC operation:

- Being an enhancement for the HO procedure aiming to minimize the UP break by having duplicated functions at the PDCP layer

- The control is at one CU at a time; no scenario corresponding to inter-donor case in IAB and therefore no conflicting resource configurations

- On the source link, there is only control signalling for UL feedback (e.g. HARQ) for possible remaining DL data transmission buffered at the source gNB

- For any overlapping UL TX, the target link is prioritized i.e. no simultaneous TX on the source and target links

- UP UL TX is switched from source to target, i.e. no parallel UL data connections

Despite supporting intra-frequency operation of DAPS, considering the specific behaviour and target use case, it cannot be used as the starting point for the intra-frequency operation of IAB. Furthermore, DAPS support for IAB has not been agreed in RAN2 and it would not be applicable as such for backhauling as the PDCP layer is not terminated at the IAB-node.

Observation 3.2: DAPS operation as such would not provide a feasible starting point of intra-frequency operation.

Based on the discussion above, the baseline operation for intra-frequency DC for UE requires additional features to be complemented on top of which IAB specific enhancements would then be defined. As the first step, further details on the specification impacts could be done to assess the consequences of the intra-frequency support. Based on such analysis, RAN1 can make the decision whether the enhancement would be feasible in the Rel.17 timeframe.

Proposal 3.1: RAN1 to make an assessment about the required specification impacts for the decision of whether intra-frequency DC would be feasible within the Rel.17 timeframe.

In the context of multi-TRP transmission, it is reasonable to assume that IAB-MT shall also be capable of supporting multi-TRP modes as they are generally appliable already in Rel-16 for UEs. When the IAB MT supports multi-TRP operation, the IAB MT may be scheduled by two different TRPs and Rel-16 defined multi-TRP framework considering both single DCI based multi-TRP and multi-DCI based multi-TRP operations. Multi-TRP operation can be viewed as intra-frequency DC scenario where scheduling of each TRP is done independently towards the IAB-MT.

Proposal 3.2: IAB-MT shall support both single DCI based and multi-DCI based multi-TRP transmission schemes.

Overall, there is nothing special that RAN1 needs to rework when enabling IAB MT to support the multi-TRP operation. On resource multiplexing, the IAB MT sees both TRPs using the same resource configuration, i.e., the same MT configuration is applied at the IAB MT. For multi-DCI based multi-TRP operation, the IAB MT may receive PDCCH from two different TRPs, where each TRP uses a group of CORESETs for PDCCH transmission (distinguished via coresetPoolIndex in CORESET configuration).

In multi-DCI based multi-TRP operation, the only concern we foresee is the indication of soft resource availability via different TRPs, where the DCI 2-5 monitoring may be associated with both TRPs. Each TRP can use resources independently per each link (without coordinating with other TRP) as all possibilities of reception (fully/partially/non overlapping) are allowed. In such situations, each TRP may send the soft resource availability based on usage of resources of the backhaul link towards the IAB MT. It may be reasonable to assume that the IAB DU in a TDM mode shall only use a soft resource only when availability indications are received from both TRPs.

Proposal 3.3: For multi-DCI based multi-TRP reception supported IAB node, the explicit indication of a soft resource is determined based on indications received from both TRPs, and a soft resource is available only when IAB-MT receives DCI 2-5 indications allowing the use of the soft resource.

# 4	Conclusions

Our proposals on the FDM/SDM half-duplex operation of the child and parent links are:

Proposal 2.1: The S/H/NA resource types are applied with FDM/SDM multiplexing.

Proposal 2.2: For all multiplexing modes, cell-specific/semi-static signals and channels of the IAB-DU shall be considered as hard resources (like in Rel-16). The parent may be required to know these cell-specific signals and channels only when IAB node has a certain restriction on supporting the multiplexing mode due to cell-specific/semi-static signal and channels.

Proposal 2.3: For FDM operation of the IAB node, the availability of PRBs in hard symbols shall be additionally indicated via semi-static CU signaling.

Proposal 2.4: For FDM operation, a dynamic indication of frequency availability of soft resources shall be controlled by the parent via introducing frequency availability indication.

Proposal 2.5: For SDM operation, a dynamic indication of spatial restrictions (or availability) of soft resources shall be further studied to enable efficient SDM operation at the IAB node.

Proposal 2.6: For FDM/SDM operation, allowed direction of the transmission for the IAB DU in F-S resources may be further controlled by the parent node by using a dynamic indication.

Proposal 2.7: For SDM operation, RAN1 shall consider the case of sharing of panels and further investigate the required beam reporting enhancements to enable simultaneous transmission/reception of parent and child links.

Proposal 2.8: For sharing of antenna panels between MT and DU, consider child node indicating for the parent node the changes of beams or panels used for reception.

Proposal 2.9: For Case A and B, no additional relaxation is supported for using DL resources in uplink transmission (Case A) or using UL resource in DL reception (Case B) by the IAB-MT.

Our observations and proposals on resource configurations for IAB DC operation are:

Observation 3.1: Due to missing support for intra-frequency DC for UE, the baseline is not complete for the IAB specific enhancements required for intra-frequency DC.

Observation 3.2: DAPS operation as such would not provide a feasible starting point of intra-frequency operation.

Proposal 3.1: RAN1 to make an assessment about the required specification impacts for the decision of whether intra-frequency DC would be feasible within the Rel.17 timeframe.

Proposal 3.2: IAB-MT shall support both single DCI based and multi-DCI based multi-TRP transmission schemes.

Proposal 3.3: For multi-DCI based multi-TRP reception supported IAB node, the explicit indication of a soft resource is determined based on indications received from both TRPs, and a soft resource is available only when IAB-MT receives DCI 2-5 indications allowing the use of the soft resource.

# References

- RP-193251, New WID on Enhancements to Integrated Access and Backhaul.

- Chairman’s notes RAN1#102-e

- R1-2007241, Summary #1 of [102-e-NR-eIAB-01],	Moderator (AT&T)

- R1-2007322, Summary #2 of [102-e-NR-eIAB-01], Moderator (AT&T)

- R1-2007287, Summary of [102-e-NR-eIAB-02], Moderator (Qualcomm)

- R1-2007352,	Summary #3 of [102-e-NR-eIAB-02],	Moderator (Qualcomm)

# Annex I

Conclusion

At least the inter-carrier DC scenario can be considered in Rel-17. Further discussion in RAN3/RAN Plenary may be necessary for the intra-carrier DC scenario.

Agreement

Reuse by IAB-MT of existing Inter-frequency DC is considered as a starting point to support concurrent BH links to two parents.

- FFS: Reuse of multi-TRP transmission resource allocation features (if intra-freq DC scenario is supported for IAB)

- FFS: Additional specification effort to support IAB

For companies to further consider:

The following categories of enhancements have been proposed to support DC scenarios (not an exhaustive list):

- Inter-parent DU resource coordination mechanisms and signaling

- Resource allocation/scheduling conflict resolution rules at the parent or child node

- Per-link IAB-DU resource configurations at the parent node

Agreement

At least existing Rel-16 bands supporting IAB can be considered when evaluating the feasibility/impact of supporting different multiplexing cases.

Simultaneous operations

R1-2007322	Summary #2 of [102-e-NR-eIAB-01]	Moderator (AT&T)

Agreement

The Rel-16 semi-static and dynamic resource allocation mechanisms are the starting point for supporting Rel-17 multiplexing cases.

- FFS: Applicability for different IAB-DU resource types

- FFS: Cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT

Agreement

- Based on the WID, the following multiplexing cases are in scope for potential support in Rel-17:

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

- Further study for for Case A and Case B at least the following scenarios:

  - Single or multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- Further study for Case C and Case D at least for the following scenarios:

  - Multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- FFS: Required level of specification impact to support the different cases. Any additional specification support in Rel-17 should be conditioned on feasibility from an interference and reliability perspective on a per-link and network basis

For companies to further consider:

Whether the following characteristics of the IAB node implementation will impact the operation of different resource multiplexing cases, including resource partitioning (i.e. identify whether there is a need for potential specification impact/enhancements compared to Rel-16 if the characteristic is or is not supported by an IAB node):

- Baseband (mis)timing alignment between IAB-MT and IAB-DU

- Separate vs. shared antenna panels/RF front-end for IAB-MT/IAB-DU

- Separate vs. shared baseband for IAB-MT/IAB-DU

- Transmitter/receiver implementation

- Self-interference cancellation

- Power control mechanisms

For companies to further consider:

Different resource partitioning scenarios for access and backhaul links, including their respective implication on interference, for different resource multiplexing cases. Examples include:

- Whether a given case is only applicable for certain resource types: e.g. DL only, UL only, DL + UL

- Whether a given case is only applicable for backhaul links or both access and backhaul links

- Note: This should have no impact on legacy UE behavior

Agreement

- Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx)

- Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx)

  - RAN1 should strive to minimize specification impact due to this feature

- FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)