3GPP TSG RAN WG1 #103-e	                                                                 R1-2008873

# e-Meeting, August 17th – 28th, 2020

Agenda item:	8.2.3

Source:	Samsung

Title:	Evaluation results for extending NR to up to 71GHz

Document for:	Discussion and Decision

# Introduction

In RAN #86, a new study item for Rel-17 was approved to extend NR to up to 71 GHz [1]. This contribution provides link level evaluations with different numerologies and system level evaluation with various channel access mechanisms.

This contribution is a revision of R1-2008158.

# Link level evaluation results

## LLS for PDSCH to study candidate numerologies

In this section, link level evaluation results for TDL-A channel with different delay spread are provided. For link-level evaluations, PTRS configuration (K=4 and L=1) and CPE compensation are considered. Further details evaluation assumptions are summarized in Appendix A.

In Figure 1~3, BLER performances according to MCS 7, MCS 16, and MCS 22 are provided with different BWs. For 400MHz evaluation, 256/128/64/32 RBs are used for 120/240/480/960kHz SCS. For 2000MHz evaluation, 360/160 RBs are used for 480/960kHz SCS. Based on the results, it is observed that link performance of smaller sub-carrier spacing is more degraded especially for higher MCS because of the phase noise impact. It is also observed that higher sub-carrier spacing can mitigate phase noise impact.

Figure 1: BLER for TDL-A with 5ns

Figure 2: BLER for TDL-A with 10ns

Figure 3: BLER for TDL-A with 20ns

This set of evaluation results is captured in Table 1.

Table 1: SINR in dB achieving PDSCH BLER of 10% /1%

| Tdoc / Source | MCS | Channel | 120KHz /400MHz | 240KHz /400MHz | 480KHz /400MHz | 960KHz /400MHz | 960KHz /2GHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2008873 / Samsung | 7 | TDL-A, 5ns | 4.53/6.89 | 4.66/7.24 | 5.75/8.32 | 6.24/8.47 | 2.65/4.08 |
|  |  | TDL-A, 10ns | 2.79/5.42 | 2.95/5.76 | 3.99/6.06 | 4.40/6.97 | 2.51/4.27 |
|  |  | TDL-A, 20ns | 2.83/4.91 | 3.27/5.49 | 4.01/5.95 | 4.47/6.67 | 3.00/3.88 |
|  | 16 | TDL-A, 5ns | 11.77/14.46 | 11.09/13.69 | 11.70/13.55 | 12.18/13.90 | 9.85/12.34 |
|  |  | TDL-A, 10ns | 11.41/13.08 | 10.18/12.74 | 10.38/12.27 | 10.67/12.55 | 10.13/11.61 |
|  |  | TDL-A, 20ns | 10.99/12.40 | 10.10/12.19 | 10.21/12.39 | 11.16/13.19 | 10.24/10.96 |
|  | 22 | TDL-A, 5ns | 23.44/NaN | 19.39/NaN | 17.81/20.81 | 17.15/21.12 | 15.73/19.37 |
|  |  | TDL-A, 10ns | 23.72/NaN | 18.27/NaN | 16.74/19.62 | 16.11/17.74 | 15.87/18.84 |
|  |  | TDL-A, 20ns | 24.28/NaN | 18.83/NaN | 16.62/19.39 | 16.41/18.28 | 16.65/19.02 |
|  | Additional report/notes: - Normal CP - CPE compensation only - PTRS configuration: symbol 3-13, K=4, L=1 - DMRS configuration: symbol 2 - NaN in the table refers to a large SINR value exceeding the range of interest (i.e., >25 dB) |  |  |  |  |  |  |

Proposal 1: Capture Table 1 of the link-level evaluation results for PDSCH in TR 38.808.

## LLS for PDSCH to study PT-RS pattern

A set of simulations for studying the PT-RS pattern is shown in Figure 4 and Figure 5, for the SCS of 120 kHz and 240 kHz, respectively. In this study, we focus on MCS 22, as this is the case with most phase noise impacts, and other evaluation assumptions are as in the Appendix A. For the PT-RS patterns, we choose NPTRS=12 (1 RB) and the size of each chunk to be 50 RB, which roughly gives 2% overhead in frequency domain and roughly the same overhead as Rel-15 PT-RS patterns with K = 4 and L = 1. For the ICI compensation, we only compensate up to the first none-CPE ICI components for practical considerations.

It can be observed that phase noise has great impacts in these scenarios and there are clear gaps between the CPE only based compensation and ICI compensation. We can conclude that CPE compensation in these scenarios is not sufficient and additional ICI compensation could achieve reasonable performance. Hence, supporting new PT-RS pattern to enable ICI compensation is essential for enhancing BLER performance in the higher frequency range.

Figure 4 Study of PT-RS pattern for 120 kHz SCS.

Figure 5 Study of PT-RS pattern for 240 kHz SCS.

This set of evaluation results is captured in Table 2 and Table 3.

Table 2: SINR in dB achieving PDSCH BLER of 10% /1%

| Tdoc / Source | MCS | Channel | 120KHz /400MHz New pattern No comp | 120KHz /400MHz New pattern CPE comp | 120KHz /400MHz New pattern ICI comp | 120KHz /400MHz R15 pattern CPE comp |
| --- | --- | --- | --- | --- | --- | --- |
| R1-2008873 / Samsung | 22 | TDL-A, 5ns | NaN/NaN | 22.71/NaN | 18.79/22.05 | 23.45/NaN |
|  |  | TDL-A, 10ns | NaN/NaN | 22.68/NaN | 17.65/21.49 | 23.81/NaN |
|  | Additional report/notes: - Normal CP - PTRS configuration: Rel-15 pattern: K=4, L=1; new pattern: 1 RB in 50 RB chunk - DMRS configuration: symbol 2 - NaN in the table refers to a large SINR value exceeding the range of interest (i.e., >25 dB) |  |  |  |  |  |

Table 3: SINR in dB achieving PDSCH BLER of 10% /1%

| Tdoc / Source | MCS | Channel | 240KHz /400MHz New pattern No comp | 240KHz /400MHz New pattern CPE comp | 240KHz /400MHz New pattern ICI comp | 240KHz /400MHz R15 pattern CPE comp |
| --- | --- | --- | --- | --- | --- | --- |
| R1-2008873 / Samsung | 22 | TDL-A, 5ns | NaN/NaN | 19.25/NaN | 17.11/19.98 | 19.41/NaN |
|  |  | TDL-A, 10ns | NaN/NaN | 18.76/NaN | 16.20/19.16 | 18.29/NaN |
|  | Additional report/notes: - Normal CP - PTRS configuration: Rel-15 pattern: K=4, L=1; new pattern: 1 RB in 50 RB chunk - DMRS configuration: symbol 2 - NaN in the table refers to a large SINR value exceeding the range of interest (i.e., >25 dB) |  |  |  |  |  |

Proposal 2: Capture Table 2 and Table 3 of the link-level evaluation results for studying PT-RS pattern in TR 38.808.

# System level evaluation results

In this section, a set of evaluation results for different channel access mechanisms is shown in Figure 4 and Figure 5, for DL and UL UPT respectively, wherein the system-level simulation is for Scenario indoor-A, and the LBT based channel access procedure is using EN 302 567 as baseline. Detailed evaluation assumptions are reported in the Appendix B.

It can be observed that directional LBT performs better than omni-directional LBT, in terms of the DL and UL throughput (both mean and 5% tile), and directional LBT has performance gain over no LBT for high load case. It can also be observed that directional LBT has higher gain in term of the 5% tile performance, which implies a more accurate channel interference estimation for the 5% tile UEs.

Figure 4. DL UPT results for different channel access mechanisms.

Figure 5. UL UPT results for different channel access mechanisms.

This set of evaluation results is captured in Table 4.

Table 4: System level evaluation results for scenario Indoor-A

| Tdoc / Source | Cases |  | Case 1: no LBT |  |  | Case 2: omni-directional LBT |  |  | Case 3: directional LBT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2008873 / Samsung | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 3693.6 | 2131.4 | 1387.1 | 3420.3 | 1903.6 | 1288.3 | 3606.2 | 2184.3 | 1639.7 |
|  |  | 50%ile | 7720.5 | 6641.6 | 4006.2 | 7462.8 | 6370.2 | 3822.1 | 7646.8 | 6801.9 | 4579.4 |
|  |  | 95%ile | 14264.1 | 14208.3 | 14031.9 | 13595.5 | 13472.0 | 13039.9 | 14196.5 | 14355.6 | 14156.4 |
|  |  | mean | 8002.7 | 7161.2 | 5248.9 | 7643.7 | 6897.7 | 4573.9 | 7894.3 | 7197.6 | 5431.5 |
|  | DL delay (s) | 5%ile | 0.015 | 0.017 | 0.019 | 0.015 | 0.017 | 0.020 | 0.015 | 0.017 | 0.018 |
|  |  | 50%ile | 0.030 | 0.038 | 0.050 | 0.033 | 0.042 | 0.053 | 0.030 | 0.037 | 0.046 |
|  |  | 95%ile | 0.063 | 0.078 | 0.095 | 0.064 | 0.080 | 0.097 | 0.064 | 0.076 | 0.090 |
|  |  | mean | 0.033 | 0.040 | 0.051 | 0.036 | 0.044 | 0.055 | 0.034 | 0.038 | 0.047 |
|  | UL UPT (Mbps) | 5%ile | 1189.4 | 465.8 | 277.2 | 1125.1 | 401.5 | 246.0 | 1135.4 | 485.4 | 295.3 |
|  |  | 50%ile | 2850.8 | 1748.4 | 927.7 | 2413.5 | 1687.9 | 882.6 | 2813.2 | 1791.0 | 977.9 |
|  |  | 95%ile | 5698.3 | 5466.0 | 5125.8 | 5537.3 | 5348.2 | 5011.1 | 5613.9 | 5495.2 | 5213.8 |
|  |  | mean | 2436.7 | 1734.0 | 1029.5 | 2297.6 | 1627.7 | 922.6 | 2404.5 | 1783.4 | 1174.2 |
|  | UL delay (s) | 5%ile | 0.032 | 0.041 | 0.054 | 0.032 | 0.042 | 0.055 | 0.032 | 0.041 | 0.051 |
|  |  | 50%ile | 0.068 | 0.102 | 0.143 | 0.070 | 0.107 | 0.146 | 0.069 | 0.100 | 0.141 |
|  |  | 95%ile | 0.150 | 0.205 | 0.296 | 0.152 | 0.209 | 0.301 | 0.150 | 0.202 | 0.291 |
|  |  | mean | 0.089 | 0.117 | 0.152 | 0.091 | 0.120 | 0.158 | 0.090 | 0.114 | 0.146 |
|  | Arrival rate (files/s) |  | 0.4 | 0.8 | 1.2 | 0.4 | 0.8 | 1.2 | 0.4 | 0.8 | 1.2 |
|  | 𝜌DL |  | 0.99 | 0.99 | 0.98 | 0.99 | 0.98 | 0.98 | 0.99 | 0.99 | 0.98 |
|  | 𝜌UL |  | 0.98 | 0.97 | 0.97 | 0.98 | 0.96 | 0.96 | 0.99 | 0.98 | 0.98 |
|  | BO |  | 0.15 | 0.44 | 0.74 | 0.17 | 0.48 | 0.80 | 0.15 | 0.43 | 0.71 |
|  | Additional report/notes: 1. LBT procedure and parameters are following ETSI 302 567 v2.1.20 2. Details of cases: - Case 1: Indoor-A with two operators, no-LBT - Case 2: Indoor-A with two operators, omni-directional LBT - Case 3: Indoor-A with two operators, directional LBT 3. No COT sharing 4. Other parameters to report: - Carrier frequency: 60 GHz - Carrier bandwidth: 2 GHz - Numerology: 960 kHz SCS with NCP |  |  |  |  |  |  |  |  |  |  |

Proposal 3: Capture Table 4 of the system-level evaluation results for channel access mechanisms in TR 38.808.

# Conclusion

In this contribution, we collect the link-level and system-level evaluation results for extending NR to 71 GHz. Proposals of this contribution are summarized as follows.

Proposal 1: Capture Table 1 of the link-level evaluation results for PDSCH in TR 38.808.

Proposal 2: Capture Table 2 and Table 3 of the link-level evaluation results for studying PT-RS pattern in TR 38.808.

Proposal 3: Capture Table 4 of the system-level evaluation results for channel access mechanisms in TR 38.808.

# Reference

[1] RP-193259, New SID: Study on supporting NR from 52.6GHz to 71 GHz, Intel Corporation, 2019.

# Appendix A

Table 5 Evaluation assumptions in LLS

| Parameters | Values |
| --- | --- |
| Carrier frequency | 60 GHz |
| Waveform | CP-OFDM |
| Antenna configuration | 2x2 (for TDL model) |
| MIMO Layer | 1 |
| Subcarrier spacing | 120/240/480/960 kHz |
| CP length | NCP |
| Phase noise model | TR38.803 Example 2 |
| MCS | NR Rel-15 MCS Table |
| MIMO reception algorithm | MMSE |
| Channel model | TDL-A |
| RB allocation | 2 GHz: 320/160 400 MHz: 256/128/64/32 |
| FFT size | 2 GHz: 4096/2048 400 MHz: 4096/2048/1024/512 |
| PDSCH symbol indexes | 2-13 |
| DMRS symbol index | 2 |
| DMRS bundling size | Full allocated BW |
| PTRS symbol indexes | 3-13 |
| PTRS density | One tone per 4 RBs for Rel-15 pattern, one RB in 50 RB chunk for new pattern |
| Channel code | LDPC |
| Channel estimation | MMSE |
| Transmission mode | QPSK:526/1024; MCS 7 16QAM: 658/1024; MCS 16 64QAM:666/1024; MCS 22 |

# Appendix B

Table 4 Evaluation assumptions in SLS

| Parameters | Values |
| --- | --- |
| Carrier frequency | 60 GHz |
| Subcarrier spacing | 960 KHz |
| Bandwidth | 2 GHz |
| Number of RBs | 160 |
| Development scenario | Indoor-A |
| UE distribution | 5 per BS |
| Channel model | InH open office |
| Mobility | 1 km/hr |
| BS antenna configuration | (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH) |
| BS antenna element gain | 5 dBi |
| UE antenna configuration | (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) |
| UE antenna element gain | 5 dBi |
| BS power limitation | 40 dBm EIRP |
| UE power limitation | 25 dBm EIRP with 21 dBm max TxP |
| BS NF | 7 dB |
| UE NF | 10 dB |
| PDCCH overhead | 2 symbol per slot |
| DMRS overhead | 1 symbol per slot |
| Traffic model | FTP model 3 (27 Mbyte file) |
| DL/UL traffic radio | 50% DL, 50% UL |
| LBT based channel access | EN 302 567 |