3GPP TSG RAN WG1 #103-e		                  		                     R1-2008900

e-Meeting, October 26th – November 13th, 2020

Source: 	Fraunhofer IIS, Fraunhofer HHI

Title:	Enhancements on SRS for coverage and capacity

Agenda item:       8.1.3

Document for:	Decision

# Introduction

In RAN1#102-e [1], the following agreements were made for the enhancements on SRS coverage and capacity.

Agreement For SRS coverage/capacity enhancements, evaluate and, if needed, specify one or more from three categories based on the following definition. - Class 1 (Time bundling): Utilize relationship among two or more occasions of one or more SRS resources in one or more slots to enable joint processing within time domain. - Study aspects include the issue of phase discontinuity, interruption of SRS transmission by other UL signals, etc.. - Class 2 (Increase repetition): Change the legacy SRS pattern in one resource and one occasion from time domain by increasing SRS symbols for repetition. - Study aspects include to use TD-OCC to compensate the negative impact on SRS capacity, inter-cell interference randomization, whether these SRS symbols are in one slot or consecutive slots, etc.. - Class 3 (Partial frequency sounding): Support more flexibility on SRS frequency resources to allow SRS transmission on partial frequency resources within the legacy SRS frequency resources. - Study aspects include the partial frequency resources are with RB level or subcarrier level (e.g., larger comb, partial bandwidth), PAPR issue, etc..

In this contribution, we present our views on partial frequency sounding and on repetition.

# Coverage and capacity enhancements

## Partial frequency sounding

SRS sounding is performed by the UE on a specific set of frequency domain resources defined by the bandwidth configuration. Each SRS resource is configured with a set of higher layer bandwidth configuration parameters cSRS, bSRS, bhop, nshift, nRRC. These parameters are indicated to a UE the RRC information element ‘SRS-Config’. The total bandwidth assigned for the SRS resource is determined by the parameter CSRS, while the actual sounding bandwidth is determined by the parameters bhop and bSRS. Regardless of the propagation channel, once the UE is configured with the bandwidth parameters, the SRS transmission takes place on the entire configured sounding bandwidth. For cell-edge UEs, due to the limited transmit power, the channel estimation quality obtained at the base station from the SRS is often not sufficient. Moreover, the channel is typically frequency selective and varying in the frequency domain. Therefore, not all of the configured frequency domain resources may be useful for the SRS transmission. Therefore, the SRS resource may be configured to be transmitted only on partial frequency resources of the configured bandwidth, the partial frequency resources are either a set of contiguous subbands or non-contiguous subbands. The advantage of such partial sounding is two-fold, one is the power boosting on the partial frequency resources and the other one is the increase in the multiplexing capacity of the UEs as the un-used frequency resources can be configured to be used by other UEs.

Partial frequency sounding may also be useful in case of strong frequency correlation among subbands of the configured bandwidth. In this case, adjacent subbands need not to be sounded, as the channel remains almost identical for a number of subbands. Therefore, only a few non-contiguous subbands may be sounded and the channel on the remaining subbands can be calculated by interpolation.

The partial frequency sounding may be performed at a subband level. Per current specification, the size of a subband is 4 PRBs. Accordingly, the number of subbands N is given by N=D/4, where D is the number of PRBs in the configured actual sounding bandwidth. In order to perform SRS sounding on specific subbands, the UE may be indicated by a N-length bitmap, where a ‘1’ in the n-th position sounds the n-th subband in the configured bandwidth.

As only few subbands are sounded with power boosting, partial frequency sounding of subbands may enhance the coverage.

Observation: SRS sounding on a subset of subbands within the configured SRS bandwidth enhances the coverage due to power boosting.

Partial frequency sounding can be realized with the current SRS framework for some bandwidths by properly choosing the bandwidth parameters nRRC, however with limitations in selecting the subbands. In order to achieve complete flexibility in sounding the subbands for all parameter combinations, an explicit indication of the desired subbands to the UE may be needed.

Observation: For flexibility in sounding the partial frequency resources within the configured bandwidth, an explicit indication of the subbands to the UE may be needed.

Proposal: Study partial frequency sounding at subband level.

Apart from partial sounding at subband level, partial sounding can also be performed on a PRB level within a subband. According to the current SRS framework, the granularity of a subband is 4 PRBs. For coverage and capacity enhancements, a straightforward extension is to support large comb factors other than the values of 2 and 4.  Increasing the comb factor allows multiplexing of multiple UEs and results in a power boosting. On the other hand, a pattern-based sounding of partial PRBs within a subband may be considered to enhance the coverage and capacity. For the pattern-based sounding, a subset of g<4 PRBs can be sounded, instead of sounding all 4 PRBs within a subband. For g=2, 2 different patterns {1,0,0,1}, {0,1,1,0} may be considered. Pattern {1,0,0,1} considers the SRS sounding on PRB 1 and 4, whereas {0,1,1,0} considers sounding on PRB 2 and PRB 3. Assuming a comb factor of 4, pattern-based sounding results in multiplexing of 8 users in total, whereas only 4 users can be multiplexed when all PRBs are sounded. In addition, as only half of the resources are sounded by each UE within a subband as opposed to sounding all PRBs, power boosting by a factor of 2 is obtained, which results in a coverage enhancement. Moreover, pattern-based sounding with increased comb factor may be also considered to further enhance the multiplexing capacity.

Observation: Pattern-based partial sounding of PRBs within a subband enhances both coverage and capacity.

Observation: For the same comb factor, in addition to enhanced coverage, pattern-based sounding with g=2  results in twice the multiplexing capacity to that of sounding all PRBs within a subband.

Proposal: Further study pattern based partial PRB sounding.

## Repetition

SRS coverage can be enhanced by repeating the SRS transmission on adjacent symbols. Especially, for cell-edge UEs, due to limited transmit power, frequency hopping combined with repetition greatly enhances the received SNR thus increasing the coverage. In the current specification, repetition values of 2 and 4 are supported. Although, supporting repetition values greater than 4 increases the coverage, it has the disadvantage that the multiplexing capacity of the UEs may be reduced as repetition requires multiple symbols. Therefore, repetition combined with partial frequency sounding may help to expand UE coverage as well as capacity primarily for cell-edge UEs.

Observation: Increasing the value of repetition factor decreases the multiplexing capacity of the UE.

Proposal: Repetition combined with partial frequency sounding may expand both coverage and capacity especially for cell-edge UEs.

# Conclusions

Based on the above discussion, we have the following observations and proposals.

Observation: SRS sounding on a subset of subbands within the configured SRS bandwidth enhances the coverage due to power boosting.

Observation: For flexibility in sounding the partial frequency resources within the configured bandwidth, an explicit indication of the subbands to the UE may be needed.

Proposal: Study partial frequency sounding at subband level.

Observation: Pattern-based partial sounding of PRBs within a subband enhances both coverage and capacity.

Observation: For the same comb factor, in addition to enhanced coverage, pattern-based sounding with g=2  results in twice the multiplexing capacity to that of sounding all PRBs within a subband.

Proposal: Further study pattern based partial PRB sounding.

Observation: Increasing the value of repetition factor decreases the multiplexing capacity of the UE.

Proposal: Repetition combined with partial frequency sounding may expand both coverage and capacity especially for cell-edge UEs.

# References

- Chairman Notes, RAN1#102-e, E-meeting, 17th – 28th August 2020.