3GPP TSG RAN WG1 #103-e	R1-2008940

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.12.1

Source: 		Moderator (CMCC)

Title: 	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS

Document for:	Discussion/decision

Introduction

The WI NR_MBS was approved in RAN plenary #86 meeting [1], and the WID was revised in RAN plenary #88 e-meeting [2]. One of the objective is to specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service, and this objective also includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

The following email thread for group scheduling is announced by chairman in RAN1#103 e-meeting:

[103-e-NR-MBS-01] Email discussion/approval for mechanisms to support group scheduling for RRC_CONNECTED UEs– Fei (CMCC)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

In this contribution, we summarize the related issues and proposals based on the contributions submitted in RAN1#103 e-meeting under the agenda item 8.12.1 [3]-[24]. The following sections are structured as follows.

From section 2 to 8, we categorized the key issues raised by contributions into 7 kinds and each section covers one kind of issues. In each section, we first provide the background and a short summary for the issue in sub-section X.1, then one or several initial proposals related to this issue are recommended by moderator in sub-section X.2, and then in sub-section X.3 one or more tables are provided to collect company views for the initial proposals in the 1st round email discussion, and then in sub-section X.4 the proposals will be updated based on companies’ input. As email discussion goes on, we may add more sub-sections for companies to provide views for the next round email discussions and for moderator to provide further updated proposals.

In section 9, some issues and proposals not belonging to the above 7 kinds are categorized as “other issues”, and they can be low priority and can be discussed based on more progress of previous issues.

In section 10, some proposals will be selected for discussion in the 1st GTW session, and more sections will be added based on discussion for next GTW sessions.

Issue #1: Frequency resource configuration for multicast

Background and summary

Considering that frequency resource configuration for broadcast in RRC_CONNECTED state may be correlated to the frequency resource configuration for broadcast in RRC_IDLE/INACTIVE which needs more discussions and progresses in RAN2, here we first focus on the frequency resource configuration for multicast, and based on the agreements for multicast, later we can further discuss or extend it to the frequency resource configuration for broadcast.

In RAN1#102-e, it was agreed to define/configure common frequency resource for group-common PDSCH for RRC_CONNECTED UEs, but there are some FFS points as below.

- FFS: whether to reuse the BWP framework or not

- FFS: the relation between the common frequency resource and UE dedicated BWP, e.g., the common frequency resource is a MBS specific BWP, or the common frequency resource is confined within UE’s dedicated BWP, etc.

- FFS: whether more than one common frequency resource can be configured per UE

Firstly, regarding the relation between the common frequency resource and UE dedicated unicast BWP, based on the contributions submitted in RAN1#103-e, there are basically five options.

- Option 1: The common frequency resource for group-common PDSCH is defined as an MBS specific BWP, which may or may not be overlapped with the dedicated unicast BWP(s)

  - The numerology (SCS and CP) of the MBS specific BWP may or may not be the same as that of the dedicated unicast BWP(s)

  - The requirements of BWP switch delay when switching between MBS specific BWP and dedicated unicast BWP follow the Rel-15/16 requirements.

- Option 2A: The common frequency resource for group common PDSCH is defined as an MBS specific BWP, which is associated with a dedicated unicast BWP. The MBS specific BWP is confined within its associated dedicated unicast BWP and using the same numerology (SCS and CP).

  - FFS whether there is a BWP switch delay or not when switching between the MBS specific BWP and its associated dedicated unicast BWP, i.e., an LS to RAN4 is needed if this option is selected.

- Option 2B: The common frequency resource for group common PDSCH is defined as a new parameter ‘MBS common frequency resource’ which is confined within a dedicated unicast BWP, but it is not defined as a BWP.

  - The MBS common frequency resource is configured per dedicated unicast BWP

  - FFS: How to indicate the starting PRB and the length of MBS common frequency resource

- Option 3: Support both option 1 and option 2A or 2B.

- Option 4: It’s up to network implementation to ensure all UEs in the same MBS group use the same BWP for group-common PDSCH reception based on existing framework for BWP management, e.g., by RRC (re)configuration or BWP switching.

For option 1, the advantage is that it can maximally reuse the RRC signaling of BWP configuration and it can also flexibly support different numerologies for MBS services and unicast services, while the disadvantage is that it cannot support the simultaneous reception of MBS service and unicast service if UE do not support two active BWPs since UE has to switch back and forth between the MBS specific BWP and dedicated unicast BWP to receive MBS service and unicast service.

For option 2A, its motivation and advantage is to maximally reuse the RRC signaling of BWP configuration for MBS, and at the same time to avoid the BWP switch delay when switching between the MBS specific BWP and its associated dedicated unicast BWP. However, even for the case that MBS specific BWP is contained in the associated dedicated unicast BWP, BWP switch delay is not zero when switching between the MBS specific BWP and its associated dedicated unicast BWP according to the current RAN4 spec. In order to achieve zero BWP switch delay, the newly introduced MBS specific BWP is actually a special BWP which is different from the existing Rel-15/16 BWP,  new requirements of BWP switch delay (gap=0) may need to be defined by RAN4, but RAN4 is not involved in this objective according to the WID.

For option 2B, the advantage is that UE can simultaneously receive the MBS service on the MBS common frequency resource and unicast service on the associated dedicated unicast BWP without BWP switch delay, but how to indicate the starting PRB and the length of MBS common frequency resource and also how to configure the parameters related to group-common PDCCH / group-common PDSCH need to be discussed further.

In summary, both option 2A and 2B aim to support the simultaneous reception of MBS service and unicast service when UE operates in a dedicated unicast BWP which contains a common frequency resource for group-common PDSCH, regardless of the common frequency resource is a special MBS specific BWP (i.e., Option 2A) or newly defined parameter ‘MBS common frequency resource’ (i.e., Option 2B).

For option 3, it seems some companies think both option 1 and option 2A/2B can be supported for NR MBS.

For option 4, the advantage is no spec impact for common frequency resource configuration, while the disadvantage is that it may be restrictive since RRC (re)configuration will be needed if network want to group the UEs configured with partial overlapped but not exactly the same BWPs into one MBS group. Regardless of option 1/2/3 is agreed or not, it seems still possible to allow network to (re)configure all UEs in the same MBS group with a same BWP for unicast and MBS using option 4. It may be implicitly supported by other companies who proposed other more advanced options (e.g., option 1/2/3), so we can check with companies if option 4 can be first agreed and on top of that down selection can be performed from option 1/2/3.

The supporting companies for different options are summarized in the table below.

| Options for common frequency resource | Companies |
| --- | --- |
| Option 1 | Apple, Convida |
| Option 2A | ZTE, Qualcomm, Intel, Chengdu TD Tech |
| Option 2B | FUTUREWEI, Huawei, vivo, CATT, Samsung, Lenovo, MTK, Intel, CMCC |
| Option 3 (both Option 1 and Option 2A or 2B) | LG, OPPO, Nokia |
| Option 4 | Ericsson |

Secondly, regarding whether more than one common frequency resource can be configured per UE, 4 companies [Huawei, Nokia, Qualcomm, CMCC] support more than one common frequency resource can be configured per UE, one company [CMCC] proposes to further study whether more than one common frequency resource can be configured per dedicated unicast BWP if above option 2A/2B is supported, and one company [Qualcomm] supports to configure more than one common frequency resource per dedicated unicast BWP.

| Proposals for common frequency resource | Companies |
| --- | --- |
| More than one common frequency resource for group-common PDSCH can be configured per UE | Huawei, Nokia, Qualcomm, CMCC |

Initial Proposals based on contributions

    - Proposal 1-1

Based on the majority view, the following moderator recommendation is made.

[Moderator’s recommendation]

Proposal 1-1: For RRC-CONNECTED UEs, for common frequency resource definition / configuration for group-common PDSCH, support Option 4 as the basis and support one of Option 2A and Option 2B in addition.

- Option 2A: The common frequency resource for group common PDSCH is defined as an MBS specific BWP, which is associated with a dedicated unicast BWP. The MBS specific BWP is confined within its associated dedicated unicast BWP and using the same numerology (SCS and CP).

  - FFS whether there is a BWP switch delay or not when switching between the MBS specific BWP and its associated dedicated unicast BWP, i.e., an LS to RAN4 is needed if this option is selected.

- Option 2B: The common frequency resource for group common PDSCH is defined as a new parameter ‘MBS common frequency resource’ which is confined within a dedicated unicast BWP, but it is not defined as a BWP.

  - The MBS common frequency resource is configured per dedicated unicast BWP

  - FFS: How to indicate the starting PRB and the length of MBS common frequency resource

- Option 4: It’s up to network implementation to ensure all UEs in the same MBS group use the same BWP for group-common PDSCH reception based on existing framework for BWP management, e.g., by RRC (re)configuration or BWP switching.

    - Proposal 1-2

Based on the majority view, the following Moderator recommendation is made.

[Moderator’s recommendation]

Proposal 1-2: For RRC-CONNECTED UEs, if option 2A or option 2B for common frequency resource is agreed, support more than one common frequency resource for group-common PDSCH can be configured per UE.

Company Views (1st round of email discussion)

    - Company Views on Proposal 1-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | We support the intention of proposal 1-1 and believe that Option 1, is simply a special case for Option 2 (2a or 2b), where the unicast parts are reduced to zero. I.e. Option 3 in your original discussion, is an option you get by default with Option2. Between options 2b and 2a, we have a slight preference for option 2b in order to avoid any type/delay associated with explicit BWP switching. But we think: - If Option 2a is about reusing RRC signaling, then maybe if you support the more advanced/optimized Option 2b, you will already have Option 2a as a default backup alternative. - Option 2a needs more study/feedback from RAN4 (and RAN1) regarding the pros and cons, e.g. can it save power for the UE when there is no unicast? What is the switching delay associated with overlapping unicast and Mcast BWPs? With respect to option 4, we agree that we should reuse as much of the existing BWP framework as possible, but we already see scope for enhancements (e.g. how to define the frequency resource to support option 2B) Moving forwards, we suggest the following simpler proposal: Proposal 1-1a: For RRC-CONNECTED UEs, a common frequency resource definition / configuration for group-common PDSCH is supported, that enables the support of simultaneous unicast and multicast in the same slot, where: - The unicast support can be configured to be zero. FFS: If separate but overlapping BWPs are used to support the MCast and Unicast parts. Rationale: Get some agreement on the common frequency resource to make progress. Support more discussion and investigation into the pros and cons of options 2a and 2b, e.g RAN4 feedback about switching delays. As we suggested earlier, we may eventually support both. Avoid vague “based on existing framework for BWP management” statement that currently is the last bullet. “based on” in our view covers enhanced/modified frameworks (i.e. more option 2b) as well as largely untouched framework (more option 2a) |
| MTK | Support Option 2B. In last meeting, we had agreed that at least supporting FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. From this perspective, Optional 2B without BWP switching delay is better than other candidate options. |
| CATT | We are generally fine with FL’s proposal 1-1, and we support option 2B. There are some unclear parts that need clarification/modifications as follows. Option 2A: The intention of Option 2A is the following case 1-1 in the figure. Option 2B: - Option 4 is pre-defined as the baseline which describes that “all UEs in the same MBS group use the same BWP”, but Option 2B states that the common frequency resource is not defined as a BWP. We suggest to align the definition on the common frequency resource/MBS BWP. - “MBS common frequency resource is configured confined within per dedicated unicast BWP” may have different design cases as follows. From one UE’s perspective, a UE can be configured with multiple unicast BWPs. - A clarification is needed that option 2B includes case 1 and/or case 2? - Case 1: In general, it should not be mandatory to configure the MBS common frequency resource on each UE unicast BWP. It is also difficult to guarantee that a common frequency resource can always be found among those configured unicast BWPs, because it is too restrict for the network to configure that there is always an overlap among those unicast BWPs. BWP4 in case 1 is not configured with MBS frequency resource. - Case 2: The wording in Option 2B is more like case 2 below. It is flexible for configuration on unicast and MBS BWPs. However, it is not flexible when scheduling MBS and BWP switch is needed. Option 2B can be modified by adding one sub-bullet which can address case 1 above. - Option 2B: The common frequency resource for group common PDSCH is defined as a new parameter ‘MBS common frequency resource’ which is confined within a dedicated unicast BWP, but it is not defined as a BWP. - The MBS common frequency resource is configured per dedicated unicast BWP - A UE can be configured with multiple dedicated unicast BWPs which include the same MBS common frequency resource. FFS: how to configure. - FFS: How to indicate the starting PRB and the length of MBS common frequency resource |
| Ericsson | We have a preference to support only Option 4 in the sense of reusing the existing BWP framework, i.e. without any additional specification impact related to BWPs. With this, unicast and MBS could use the same BWP without BWP switching or spec impact. This maximizes the commonality with NR unicast. Regarding 2A we wish to point out that the required BWP switching, even if supported by the UE (subject to RAN4 agreement) also requires RRC reconfiguration with current specifications (since DCI-based switching is optional) and such, potentially frequent, RRC reconfigurations (i.e. UE-specific) would be impractical. Regarding 2B, for group-common PDCCH we wish to point out that this requires a common DCI size, but DCI FDRA is currently BW/BWP-dependent, so with different UE BWP sizes group-common PDCCH would not work without further spec impact. |
| Qualcomm | For Opt2A The motivation to configure a special multicast BWP within its associated dedicated BWP is to enable simultaneous reception of unicast and multicast. It is similar as we support simultaneous reception in CORESET0 and dedicated BWP if including CORESET0. The active BWP is not switched from dedicated BWP to the special multicast BWP, i.e., there is no RF retuning here. We try to use the functionality of NR BWP configuration, which includes the configuration of group common PDCCH (CORESET/SS) and group common PDSCH within the multicast BWP. For Opt2B The wording of Opt2B seems only mention the frequency resource allocated to mutlicast reception per dedicated BWP. Also, it does not say anthing on how to define the other configurations for GC-PDCCH/GC-PDSCH. For GC-PDSCH, the configuration could be different than of unicast PDSCH in the same dedicated BWP, e.g., TDRA, MIMO, MCS, DMRS/PTRS, rate matching, etc.. Comparing Opt2A and Opt2B, we slightly prefer Opt2A since it is clear that the common configuration parameters are for multicast reception of a group of UEs and it does not limit the flexible configuration of unicast PDSCH. If companies have concern on the name of BWP, the MBS specific BWP can use another name, e.g., ‘MBS common configuration’, with similar configuration funcationality as BWP but no BWP switching is needed. For Opt4 It means the same BWP is used for unicast and multicast reception, i.e., the UE is configured with a dedicated BWP (e.g., full BW) and the same BWP is used for MBS. Then, it is just a special case of Opt2A or Opt2B where the common frequency resource for multicast is same as that of dedicated BWP for unicast. But we still need to configure GC-PDCCH/GC-PDSCH parameters for MBS reception. Proposal 1-1: For RRC-CONNECTED UEs, for common frequency resource definition / configuration for group-common PDSCH, support one of Option 2A and Option 2B. - Option 2A: The common frequency resource for group common PDSCH is defined as a MBS specific BWP, which is associated with a dedicated unicast BWP. The Multicast specific BWP is confined within its associated dedicated unicast BWP and using the same numerology (SCS and CP). - No BWP switching between the multicast reception in the MBS BWP and unicast reception in its associated dedicated BWP. - Option 2B: The common frequency resource for group common PDSCH is defined as a new parameter ‘MBS common frequency resource’ which is confined within a dedicated unicast BWP, but it is not defined as a BWP. - The MBS common frequency resource is configured per dedicated unicast BWP - FFS: How to indicate the starting PRB and the length of MBS common frequency resource |
| ZTE | Our understanding is that Option 2A should be sufficient. Different configurations may be needed for unicast and multicast, e.g., frequency resource, TDRA table, MIMO layer, etc. Option 2B is only applied to address the frequency resource configuration issue. If Option 2B is adopted, then we may have to discuss all the configurations that may be different between unicast and multicast one by one. However, since most of the configurations are on top of BWP since Rel-15, it is straightforward to reuse the BWP framework for MBS, which is more efficient. From our perspective, Option 2A is a superset of Option 2B and Option 4. Network can configure the MBS BWP the same as or different from the unicast BWP by implementation. Further, if Option 2A is adopted, the unified definition on common frequency resources for MBS transmission can be applicable to UEs in both RRC_CONNECTED state and RRC_IDLE/INACTIVE state. |
| Samsung | For both options, the frequency resource is associated with a dedicated unicast BWP and there is no switching delay (same numerology). We agree with that framework. Option 2A re-uses the BWP framework for MBS while option 2B introduce an MBS frequency resource within a dedicated BWP. We are open to discuss trade-offs further but for now the more important aspect is to agree on the common framework of receiving MBS PDSCH within a dedicated BWP without switching delay. |
| Convida | We support option 1 but can accept option 3, i.e., ‘support both option 1 and option 2A’ as compromise. We think the option 2A is a sub-case for option 1. The main issue for option 2A and option 2B is that they may not work for broadcast use case. Although for multicast use case, the gNB may manage to group the UEs smartly and can find such common frequency resource confined within the UEs’ dedicated BWPs. However, for broadcast use case, the PTM PDCCH is supposed to be received by all the UEs within the cell, where each UE may be configured and activated with different BWPs. Since the gNB may most likely distribute the UEs’ unicast BWPs to utilize the full spectrum as much as possible, it is not feasible to find an overlapping portion for all the UEs. Therefore, we think either option 2A or option 2B can not work for all MBS use cases standalone, and option 1 should be supported. |
| Lenovo, Motorola Mobility | Option 2B is supported. We still have concern on BWP switching effort when Option 2A is used. |
| OPPO | We support Option 4 but open to further discuss Option 2A/2B. Option 4 is what has been supported in current specification, it is straightforward that this option can be reused for MBS as well. For Option 2A, as the center frequency and bandwidth of MBS specific BWP can be different from the associated unicast BWP, seems BWP switching is still needed. We agree with the benefits of Option 2B, but it would also introduce some specification impacts, such as on FDRA determination and RRC, and it is only applicable when dedicated BWPs of UEs with the MBS group are overlapping and configured with same numerology. |
| LG | We prefer Option 1. We can also live with Option 2A. The MBS specific BWP can be introduced based on the existing concept of BWP. For UE in RRC_CONNECTED, the MBS specific BWP can be one of DL BWPs configured by either UE dedicated signaling or broadcast signaling such as SIB/MCCH. To avoid switching time, gNB could ensure that the MBMS specific BWP is overlapped with one or more of UE dedicated unicast BWPs using the same numerology (SCS and CP), as proposed by Option 2A. The concept of MBS BWP would be also useful for a UE in RRC_IDLE/RRC_INACTIVE receiving a MBS transmission via MBS common frequency resource which may or may not be confined within initial DL BWP. The MBS BWP can be wider than the initial DL BWP with a different numerology for MBS services, but it may be confined within unicast BWPs of connected UEs receiving that MBS transmission. Nevertheless, we wonder if gNB could ensure that the MBMS specific BWP is overlapped with ‘all’ UE dedicated unicast BWPs using the same numerology (SCS and CP). If not, a UE in RRC_CONNECTED may perform R15/16 BWP switching to another UE dedicated unicast which is not overlapped with the MBS specific BWP. So, the UE may not receive MBS transmissions. |
| vivo | Option 2B is supported. Option 4 is a specific case of option 2A or option 2B, if option 2A or 2B is supported, it can be up to gNB implementation to configure as option 4. No need to support this option explicitly. For option 2B, based on current BWP framework, it conflicts with our previous agreements to support FDM unicast PDSCH and group-common PDSCH reception since UE can only works on one active DL BWP at a given time. In addition, the BWP switching issue has to be solved before we discuss whether/how to support 2B, that means we have to send LS to RAN4 to ask the feasibility if there is no BWP switching between unicast dedicated BWP and MBS specific BWP. |
| Huawei/HiSilicon | It is better not to complicate the issue further. If option 4 is up to implementation, then there is no need or should not be the basis because it is up to implementation. NW can either implement it in this way or not. If option 2A is originated from reusing the BWP configuration framework, the benefit is doubtable if it causes specification efforts in other ways. For example, defining the association between unciast BWP and MBS BWP, when unicast BWP is active and when MBS BWP is active, and whether there is BWP swithcing delay and whether the switching delay is accpetable, etc... Also, how the resouce is configured from signaling perspective should not be the concern from RAN1 perspective at this stage. Instead, RAN1 should focus on the functionality of MBS and trust RAN2 can design the signaling in whatever RAN1 agreeds/prefers. We concerned the restriction in option 4 and the switching delay in option 1, so we support the direction of option 2B, opition 2B can be further simplied as follows: - Option 2B: The common frequency resource for group common PDSCH is defined as a new parameter ‘MBS common frequency resource’ which is confined within a dedicated unicast BWP, but it is not defined as a BWP. - The MBS common frequency resource is configured per dedicated unicast BWP - FFS: How to indicate the starting PRB and the length of MBS common frequency resource The reason for this modification is that whether it is a parameter as suggested or it is a “special BWP” within dedicated unicast BWP does not matter much and can be up to RAN2 for signaling design. |
| CMCC | We support option 2B For option 2A and 2B, the common motivation is not introduce BWP switching delay between unicast and multicast reception, but the option 2A will also introduce RAN4’s work. Therefore, wo prefer option 2B which has no RAN4 impact and can realize the simultaneous operation between unicast and multicast. Regarding option 4, we think the option 4 is an exceptional case of option 2B, which the common frequency resource equals with the UE specific BWP. |
| Chengdu TD Tech, TD Tech | - We support option 2A From our point of view, no BWP switch is needed between an MBS specific BWP and a dedicated/activated BWP which contains the MBS specific BWP. In our views on Option 2A, it can be specified as below. If UE has at least one unicast service, it works on the dedicated BWP for both the unicast services and the MBS with receiving the group-common PDSCH within the frequency resource defined by the MBS specific BWP. If UE has no unicast service and enters the RRC_CONNECTED state just for receiving a multicast service in an MBS specific BWP, it can work on the MBS specific BWP with this BWP as its dedicated/activated BWP or work on a dedicated/activated BWP for the subsequent possible unicast service with the current MBS specific BWP contained in the dedicated/activated BWP. If UE works on the MBS specific BWP, the subsequent unicast service can be provided at this MBS specific BWP or provided with a new dedicated/activated BWP which contains the MBS specific BWP through the RRC reconfiguration or BWP switch. After the RRC reconfiguration or BWP switch, UE can receive both the unicast service and the MBS on the new dedicated/activated BWP. In our view, no RAN4 study on the MBS specific BWP is needed with the work assumption that the MBS specific BWP is a special BWP with the following relationship with its associated dedicated/activated BWP: - For an MBS specific BWP, its associated dedicated/activated BWP contains it. - If a UE is configured with a dedicated/activated BWP with an MBS specific BWP contained, UE works on the dedicated/activated BWP with receiving the MBS within the frequency resource defined by the MBS specific BWP. - As for Option 4, we have the following points of view: We think RRC (re)configuration or BWP switching needs to be supported for the RRC_CONNECTED UE in a multi-cast group to receive the group-common PDSCH on the same BWP. But in some scenarios, more than one BWPs are needed with each BWP having the group-common PDSCH to send the same MBS. For example, no BWP exists for providing both the MBS and the unicast services of each UE in the multi-cast group. We suggest that option 4 is specified as below. Option 4: In order to make all UEs in the same MBS group use the same BWP for group-common PDSCH reception, the existing framework for BWP management, e.g., by RRC (re)configuration or BWP switching needs to be supported. |
| Moderator | Firstly, we need to first focus on the frequency resource configuration for multicast, since everything related to broadcast in RRC_CONNECTED state may be correlated to that of broadcast in RRC_IDLE/INACTIVE state which needs more progresses in RAN2 and AI 8.12.3. Secondly, based on companies’ views, the first observation is that majority prefer to have a common frequency resource definition/configuration to at least enable simultaneous reception of unicast service and multicast service in the same slot, i.e., without BWP switch delay, regardless of common frequency resource is a ‘special MBS-specific BWP’ or ‘MBS common frequency resource’. Thirdly, to avoid companies’ concern on the BWP switch delay issue and potential RAN4 impact of Option 2A, it seems the proponents of Option 2A are also fine to not use the name of BWP and replace ‘special MBS specific BWP’ with another name ‘MBS common configuration’. However, regardless what the name is, as long as a separate BWP ID is used for it to reuse the RRC signaling structure of BWP, RAN4 confirmation is needed regarding whether the BWP switch delay can be zero or not. Fourthly, for Option 2B, one of the concern is that it only mentions the frequency resource allocated to multicast per dedicated unicast BWP but not for the configuration of group-common PDCCH/PDSCH for multicast which may be different from that of unicast PDSCH in the same dedicated BWP. ZTE’s comment is that if Option 2B is adopted, then we may have to discuss the configurations such as TDRA table, MIMO layer, etc., one by one. I think these parameters may need to be discussed for both option 2A and 2B. Option 2A tries to reuse BWP’s RRC signaling structure to configure the parameters for multicast, but if we look at those parameters in BWP in TS38.331, the main parameters are PDCCH-Config and PDSCH-Config, the details of these parameters can also be reused directly for option 4 or reused with some reinterpretation of the frequency region they refer to for Option 2B. In addition, the concrete RRC signaling can be up to RAN2 design. Although the original intention of proposal 1-1 is to focus on the discussion of common frequency resource of group-common PDCCH / PDSCH, we can also broad the discussion to include other configurations of group-common PDCCH / PDSCH. The other concern from Ericsson is that further spec effort is needed regarding the DCI size since DCI FDRA is currently BWP dependent, but companies already see the enhancement based on the agreement of last meeting to define/configure the common frequency resource for PDSCH-common PDSCH, and some companies also provide related proposal for this issue. Based on these observations and taking into account all the comments from companies, we can try the following updated Proposal 1-1: Updated Proposal 1-1: For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH can be configured per dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot - If the common frequency resource is not configured and the multicast reception is enabled, the dedicated unicast BWP is used as the common frequency resource for group-common PDCCH / PDSCH - FFS whether UE can be configured with no unicast reception in the common frequency resource - FFS on details of the configurations of group-common PDCCH / PDSCH - FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities - Down select from the two options for the definition of common frequency resource for group-common PDCCH/ PDSCH - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is confined within its associated dedicated unicast BWP and using the same numerology (SCS and CP) - Send LS to RAN4 to confirm whether the BWP switch delay can be zero or not when switching between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP if this option is adopted - Option 2B: The common frequency resource is defined as a ‘MBS common frequency resource’ with a number of contiguous PRBs, which is confined within the dedicated unicast BWP. - FFS: How to indicate the starting PRB and the length of PRBs of the MBS common frequency resource Most comments from companies have been taken into account in the updated proposal. Below are some responses to other detailed comments. @CATT, to your comments and questions on Option 2B, the current updated proposal 1-1 does not mandated that a dedicated unicast BWP has to be configured with a common frequency resource for group-common PDCCH/PDSCH based on the current wording ‘can be configured’ in the main bullet, i.e., a dedicated unicast BWP can be configured without common frequency resource and only for unicast reception. For your case 1 and case 2, they are completely up to network configuration. It needs further study whether more than one common frequency resource can be configured per dedicated unicast BWP / per UE. Option 2A and Option 2B aim to have the same effect. |

    - Company Views on Proposal 1-2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
|  | We support the intention of the proposal. We believe that - subject to the limits of the UE capabilities, the network can configure multiple group-common PDSCHs in the same BWP or across different BWPs. But, as we had indicated in our contribution, perhaps we need to discuss and understand the motivation behind having this proposal, as compared to leaving it to gNB implementation. We support a clarification/discussion regarding the PDCCH to schedule these PDSCHs. Is the MCast PDCCH always confined to the PDSCH BWP? Don’t forget there is dci supported BWP switching. Moving forwards only when a variant of proposal 1-1/1-1a is agreed, do we suggest a proposal like: Proposal 1-2a: For RRC-CONNECTED UEs, support for more than one common frequency resource for group-common PDSCH can be configured per UE, subject to the UE capabilities. |
| MTK | Support proposal 1-2. For supporting different MBS services, UE should support more than one common frequency resource as UE has multiple dedicated BWPs in unicast. MBS common frequency resources can be configured as subset of each dedicated BWP. |
| CATT | A clarification is needed for proposal 1-2: - When supporting more than one MBS common frequency resources per UE, does it mean that supporting more than one MBS common frequency resources on a unicast BWP of the UE (case 1-2a), or supporting one MBS common frequency resource on each unicast BWP but different resources on different BWPs of the UE (case 1-2b), or the most general one as in case 1-2c below? Based on our understanding, the intention is to introduce at most one MBS common frequency resource on each UE/CC. To make the proposal more clear, we would like to suggest to modify the proposal 1-2 as: Proposal 1-2: For RRC-CONNECTED UEs, if option 2A or option 2B for common frequency resource is agreed, support at most one common frequency resource for group-common PDSCH can be configured per carrier. |
| Ericsson | We disagree with Proposal 1-2 now, since we would like to first have the proposal clarified. If it is proposed to have the same MBS service spread over two or more common frequency resources, for group-common PDSCH within the allocated BWP, it seems this case could already be resolved with the FDRA bitmap configuration in PDCCH. If it is proposed to support multiple MBS services per UE, then we should first agree to that. |
| Qualcomm | We support the proposal 1-2. |
| ZTE | Wait for the outcome of Proposal 1-1 first. We prefer to discuss Proposal 1-2 after RAN1 finalizes Proposal 1-1 because different options in Proposal 1-1 may lead to different understandings of Proposal 1-2. |
| Samsung | We understand this to be a parallelization of the multiple unicast BWPs of Rel-15 - we agree. As a side note, we do not see any need to have multiple common frequency resources per BWP. |
| Convida | We suggest to discuss this issue after proposal 1-1 is clarified. |
| Lenovo, Motorola Mobility | We can wait for the conclusion of Proposal 1-1 then discuss Proposal 1-2. |
| OPPO | This proposal is based on an assumption that one of option 2A or option 2B is supported, we prefer to discuss this proposal after proposal 1-1 is confirmed. |
| LG | We support more than one MBS specific BWPs for group-common PDSCH for a single UE. The UE may receive multiple MBS service on multiple MBS BWPs depending on UE capability and UE’s interest in those services. However, if multiple MBS BWPs can be configured, Option 1 needs to be also considered. |
| vivo | We support the proposal 1-2 in principle. It would be appreciated if the question from CATT, i.e. how to interpreted this proposal (case 1-2a, case 1-2b, case 1-2c?) is clarified. |
| Huawei/HiSilicon | We support UE can be configured with more than one common frequency resource for group-common PDSCH. To be clearer, this is per UE be configurable to up to four unicast BWP perspective. We prefer to have a single common resource for group-common PDSCH per unicast BWP per UE for simplicity. If it is the common ground of proposals 1-2 proponents, I suggest modifying the proposal as follows: Proposal 1-2: For RRC-CONNECTED UEs, if option 2A or option 2B for common frequency resource is agreed, support more than one common frequency resource for group-common PDSCH can be configured per UE with a single common frequency resource configured per unicast dedicated BWP. |
| CMCC | Support proposal 1-2, we can wait it after the discussion of proposal 1-1. |
| Chengdu TD Tech, TD Tech | We think proposal 1-2 NEEDS the further clarification according to the comments from the above companies. |
| Moderator | Based on companies’ comments, Proposal 1-2 has been merged in Proposal 1-1 as an FFS. - FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to the UE capabilities Regarding CATT’s questions, my understanding is that, either case 1-2b or case 1-2c is up to network implementation. Network can configure the UE similar to case 1-2c so that UE can receive all the MBS services regardless it switches to BWP1, BWP2 or BWP3. Network can also configure the UE similar to case 1-2b so that UE can receive different MBS services in different BWPs. For Case 1-2a, the necessity needs to be discussed. However, this is just my understanding, it seems companies still need time to consider it further, so I put it in FFS. |

Updated Proposals (1st round of email discussion)

    - [High] Updated Proposal 1-1

[High] Updated Proposal 1-1: For multicast of RRC-CONNECTED UEs, a common frequency resource for group-common PDCCH / PDSCH can be configured per dedicated unicast BWP to support simultaneous reception of unicast and multicast in the same slot

- If the common frequency resource is not configured and the multicast reception is enabled, the dedicated unicast BWP is used as the common frequency resource for group-common PDCCH / PDSCH

- FFS whether UE can be configured with no unicast reception in the common frequency resource

- FFS on details of the configurations of group-common PDCCH / PDSCH

- FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

- Down select from the two options for the definition of common frequency resource for group-common PDCCH/ PDSCH

  - Option 2A: The common frequency resource is defined as an MBS specific BWP, which is confined within its associated dedicated unicast BWP and using the same numerology (SCS and CP)

    - Send LS to RAN4 to confirm whether the BWP switch delay can be zero or not when switching between the multicast reception in the MBS specific BWP and unicast reception in its associated dedicated BWP if this option is adopted

  - Option 2B: The common frequency resource is defined as a ‘MBS common frequency resource’ with a number of contiguous PRBs, which is confined within the dedicated unicast BWP.

    - FFS: How to indicate the starting PRB and the length of PRBs of the MBS common frequency resource

Company Views (2nd round of email discussion)

    - Company Views on Updated Proposal 1-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

Updated Proposals (2nd round of email discussion)

To be added…

Issue #2: Group scheduling mechanism

Background and summary

In RAN1#102-e, it was agreed to at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI. There was a FFS whether to support UE-specific PDCCH to schedule a PDSCH for MBS.

Firstly, regarding using UE-specific PDCCH to schedule a PDSCH for MBS, based on companies’ contributions, there could be different possibilities, e.g., using UE-specific PDCCH to schedule UE-specific PDSCH (same as legacy unicast transmission) for initial transmission or for retransmission, using UE-specific PDCCH to schedule group-common PDSCH for initial transmission or for retransmission, etc. In order to facilitate of RAN1 discussion, one company proposes to consider the following clarification as RAN1 common understanding only for discussion purpose.

- PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme.

- PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme.

From Moderator’s perspective, it may be also beneficial to consider the following clarification for discussion purpose to achieve a common understanding from RAN1 point of view, since many companies use similar terminologies, such as PTP transmission, unicast transmission, etc.

- PTP transmission: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

Secondly, for initial transmission of MBS service, besides PTM transmission scheme 1 which was agreed in RAN1#102-e, 7 companies [vivo, CATT, Oppo, Nokia, APT, Convida, CMCC] propose to also support PTM transmission scheme 2, but 7 companies [Huawei, ZTE, ETRI, Qualcomm, Ericsson, Lenovo, MTK] propose to not support it.

4 companies [LG, Apple, MTK, Qualcomm] propose to support PTP transmission for initial transmission of MBS service, but considering that dynamic switch between PTP and PTM has been in the scope of WID, and it was also extensively discussed in RAN2#111-e post email discussion (refer to email thread in RAN2 [Post111-e][904][MBS] L2 Architecture (Huawei)), it can be assumed that PTP transmission can be supported for initial transmission for MBS. It seems not necessary to have an additional agreement in RAN1 on this. If more companies suggest to have such an agreement in RAN1, we can add a separate proposal for it later.

One company [CATT] proposes to support sub-group-common PDCCH based group scheduling for initial transmission of MBS service. In this scheme, the whole MBS group can be divided into several sub-groups with small number of UEs, and a Sub-G-RNTI can be used to scramble a group-common PDCCH for a small group scheduling, but all UEs in the MBS group are scheduled with a group-common PDSCH which is scrambled with G-RNTI.

| Proposals for initial transmission for MBS | Companies |
| --- | --- |
| Support PTM transmission scheme 2 for initial transmission for MBS | Support: vivo, CATT, Oppo, Nokia, APT, Convida, CMCC Object: Huawei, ZTE, ETRI, Qualcomm, Ericsson, Lenovo, MTK |
| Support PTP transmission for initial transmission for MBS | LG, Apple, MTK, Qualcomm |
| Support sub-group-common PDCCH based group scheduling for initial transmission for MBS. | CATT |

Thirdly, 8 companies [Huawei, vivo, Apple, Lenovo, MTK, Qualcomm, CMCC, Nokia] propose that, if initial transmission is based on PTM transmission scheme 1, retransmission could use PTP transmission or PTM transmission scheme 1. 2 companies [MTK, CMCC] also proposes to support PTM transmission scheme 2 for retransmission.

| Proposals for retransmission for MBS | Companies |
| --- | --- |
| If initial transmission is based on PTM transmission scheme 1, retransmission can use PTP transmission or PTM transmission scheme 1 | Huawei, vivo, Apple, Lenovo, MTK, Qualcomm, CMCC, Nokia |
| If initial transmission is based on PTM transmission scheme 1, retransmission can use PTM transmission scheme 2 | CMCC |

Initial Proposals based on contributions

    - Proposal 2-1

[Moderator’s recommendation]

Proposal 2-1: For discussion purpose, consider the following clarification as RAN1 common understanding.

- PTP transmission: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

- PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme.

- PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme.

    - Proposal 2-2

[Moderator’s recommendation]

Proposal 2-2a: Support PTM transmission scheme 2 for initial transmission of MBS service in RRC_CONNECTED state.

Proposal 2-2b: Further study whether to support sub-group-common PDCCH based group scheduling for initial transmission of MBS service in RRC_CONNECTED state.

- FFS: details of sub-group-common PDCCH based group scheduling

    - Proposal 2-3

[Moderator’s recommendation]

Proposal 2-3: For RRC_CONNECTED UEs, if initial transmission of MBS service is based on PTM transmission scheme 1, at least support retransmission can use PTP transmission or PTM transmission scheme 1.

- FFS: whether to support PTM transmission scheme 2 for retransmission.

- FFS: How to indicate the association between PTM scheme 1 and PTP transmitting the same TB.

- FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different subgroups of UEs in the same MBS group?

Company Views (1st round of email discussion)

    - Company views on Proposal 2-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | We support the agreement of these definitions for subsequent discussions. However, would an additional note be helpful: Note: The feedback mechanisms (PTM and/or PTP) for these potential transmission schemes is still FFS |
| MTK | We agree with this clarification. From RAN1’s perspective, we think the difference b/w PTP and PTM (including PTM scheme 1 and scheme 2) is that how to scramble PDSCH, e.g., C-RNTI and G-RNTI corresponds to PTP and PTM, respectively. For PTM transmission scheme, the difference b/w PTM scheme 1 and scheme2 is how to scramble PDCCH, e.g., C-RNTI and G-RNTI corresponds to PTM scheme 1 and PTM scheme 2, respectively. From our understanding, both PTM and PTP can transmit MBS packet. |
| CATT | We are also generally okay with the proposal 2-1. As we proposed and explained the benefit of multi-group-common scheduling scheme in our contribution R1-2007835, we would like to suggest to update the second bullet of proposal 2-1 as: PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCHs with CRC scrambled by group-common RNTIs to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called (Multi-) group-common PDCCH(s) based group scheduling scheme. |
| Ericsson | We agree with Proposal 2-1 |
| Qualcomm | It can be used as a clarification for discussion. |
| ZTE | We are fine to define the terminology for MBS. For PTM transmission scheme 1, we believe it can also be applied for RRC IDLE/RRC_INACTIVE UEs. We prefer to reflect this in this proposal. |
| Samsung | We think the description of the PTM schemes 1 and 2 needs to be further discussed. A UE will be configured the RNTI by UE-dedicated RRC - then it can only be UE-specific. But that does not mean PTM scheme 2. The main question is whether the UE monitors PDCCH according to CSS or USS (that does not mean C-RNTI). It needs to be discussed whether for allocation of the UE’s PDCCH monitoring capability and search space set dropping, PDCCH monitoring for MBS PDSCH is always prioritized over PDCCH monitoring for unicast PDSCH. For PTP, we would like to discuss what additional aspects are needed relative to Rel-16 and relative to PTM schemes 1 and 2. If none/marginal, it can be considered with a lower priority as it is not a main use case. |
| Convida | We are OK with the clarification |
| Lenovo, Motorola Mobility | We are Ok with the definition. |
| OPPO | We are basically fine to reach a common understnading on the 3 schemes. However, for PTP and PTM scheme 2, although the PDCCH is UE-specific, we do not think the PDCCH CRC has to be scrambled with UE-specific RNTI. More specifically, as the PDCCH is supposed to be transmitted in USS, the CRC can be scrambled by the group-common RNTI as the scheduled PDSCH, such that no field in the DCI is needed to indicate which RNTI is used for the scrambling of scheduled PDSCH. Hence we prefer to remove “with CRC scrambled by UE-specific RNTI (e.g., C-RNTI)” in PTP transmission and PTM transmission scheme 2. |
| LG | RAN2 is currently discussing PTP/PTM bearer switching. In our view, PTP transmssion in P2-1 is different than PTP bearer in RAN2. Thus, it is better to further clarify that PTP transmission and PTM transmsisions schemes are used to transmit a MBS TB for PTM bearer, not for PTP bearer. In particular, concerning Proposal 2-3, PTP transmsision could support a HARQ retransmission of a TB over PTM bearer after PTM transmission of initial HARQ transmisson of the TB. However, it seems not so feasible to support a mixture of HARQ initial transmisison over PTM bearer and HARQ retransmsison over PTP bearer for a same TB. |
| vivo | We are fine with the clarification. |
| Huawei/HiSilicon | Support this clarification in RAN1 for facilitating discussion purpose. |
| CMCC | Support the clarification. |
| Chengdu TD Tech, TD Tech | We support the clarification as described with Proposal 2-1. |
| Moderator | Firstly, for the same reason in proposal 1-1 we’d better first focus on multicast for RRC_CNNECTED UEs. Secondly, most companies think such kind of clarification is helpful for the discussion later. Samsung and OPPO raised that UE-specific PDCCH does not have to be scrambled with UE-specific RNTI, it could also be scrambled with group-common RNTI but transmitted in USS. In order to keep everyone on the same page when we discuss the potential PTP/PTM transmission schemes, I added ‘PTP transmission scheme 2’ and ‘PTM transmission scheme 3’ to avoid mixing things up. Additionally, it is still open that the group-common PDCCH can be based on CSS or USS, so I didn’t limit the group-common PDCCH in PTM scheme 1 to CSS only in the current clarification, but I added two notes to further clarify the terminology ‘UE-specific’ and ‘group-common’ to avoid people misunderstanding that they are only characterized by RNTI and SS type. Thirdly, for sub-group-common PDCCH based group scheduling, based on companies’ feedback on Proposal 2-3, I think it would be better to not mix it in proposal 2-1 and leave it for further study in Proposal 2-3. @Nokia, Regarding the note raised by Nokia, for the “feedback mechanisms (PTM and/or PTP)”, do you mean HARQ-ACK feedback using PTP and/or PTM manner or something else? @Samsung, Regarding to Samsung’s comment on PTP, many companies propose to support PTP transmission for retransmission of multicast, and also in order differenciate it with PTM scheme 2 (both use UE-specific PDCCH), so it would be better to have a common understanding when we discuss these transmission schemes. @LG, regarding your comments, I understand your point, you actually mean we are dicussing all the transmission schemes for MRBs (MBS bear) not for DRBs. I agree with this, I think it should be a common understanding that for DRBs there is no difference from current Rel-15/Rel-16 from RAN1 perspective. I also think that a mixture of HARQ initial transmisison over MRB and HARQ retransmsison over DRB for a same TB is not in the scope of our discussion. I’m a little relunctant to mention MRBs/DRBs in RAN1 proposals since we are not so professional on usage of these terminologies. If more companies prefer to have such kind of clarifications, I will try my best to do it. The Proposal 2-1 was updated as following: Updated Proposal 2-1: For discussion purpose, consider the following clarification as RAN1 common understanding. - PTP transmission scheme 1: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI. - PTP transmission scheme 2: For RRC_CONNECTED UEs, use UE-specific PDCCH in USS with CRC scrambled by group-common RNTI to schedule UE-specific PDSCH which is scrambled with the same group-common RNTI or UE-specific RNTI (e.g., C-RNTI). - PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. - PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. - PTM transmission scheme 3: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH in USS with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. - Note: The ‘UE-specific PDCCH / PDSCH’ here means the PDCCH / PDSCH can only be identified by the target UE but cannot be identified by the other UEs in the same MBS group with the target UE. - Note: The ‘group-common PDCCH / PDSCH’ here means the PDCCH / PDSCH are transmitted in the same time/frequency resources and can be identified by all the UEs in the same MBS. |

    - Company views on Proposal 2-2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | We support proposal 2-2a. It makes clear the different RNTIs being used for this new PTP style Mcast. We can support proposal 2-2b as it is only study at the time being. What we would like clarification/discussion on, are the: - Characteristics of the PDSCH or PDSCHs, these different sub-groups are using. - The pros and cons of defining sub-groups – does it actually improve overall SE? |
| MTK | We don’t support Proposal 2-2a. If there are less UE in one MBS group, we can use legacy unicast for initial transmission. Compared with proposal 2-2a, it doesn’t have any spec impact. If they are more UEs in one MBS group, it is more reasonable to use PTM scheme 1. Thus, we don’t suggest to support PTM scheme 2 for MBS transmission. |
| CATT | We can support both proposal 2-2a and 2-2b for initial transmission based on different use cases/scenarios, since 2-2b can be applied when different small groups of UEs have different PUCCH coverage requirements as well as low PDCCH signaling overhead. |
| Ericsson | We disagree with Proposal 2.2a: Since PTM transmission scheme 1 is anyway agreed, the argument of limiting spec impact is no more valid for PTM transmission scheme 2. With this, we see no clear benefit of supporting PTM transmission scheme 2. We support Proposal 2.2b (i.e. FFS): However, there is a need to clarify whether the purpose is multiple services per UE and, if so, RAN1 needs to ask SA2 for an LS about what the associated requirements and use cases are. If the purpose is to have different G-RNTI for different groups of UEs (e.g. for coverage reasons) there is no RAN1 impact since from the UE perspective only one G-RNTI is monitored. |
| Qualcomm | We think for a large group of UEs, PTM using GC-PDCCH/GC-PDSCH is beneficial. While for a small group of UEs, PTP using unicast PDCCH/PDSCH may be flexible with bearable overhead. For Proposal 2-2a, to our understanding, the GC-PDSCH needs to use common GC-PDSCH configuration (not just scrambling) and scheduled by unicast PDCCH within the common frequency resource. The DCI format of unicast PDCCH may be more complicated. It is less flexible than PTP using unicast PDCCH/PDSCH not limited in the common frequency resource. We suggest further study whether to support PTM scheme 2. For Proposal 2-2b, we have concern on complicity since the UE needs to monitor C-RNTI, G-RNTI(s) and sub-G-RNTI(s). Also, if PTM scheme 2 is supported, the benefits of configuring sub-G-RNTI(s) become less. We prefer to delay the Proposal 2-2b after we decide whether to support PTM scheme 2. |
| ZTE | For proposal 2-2a, we prefer to discuss it together with Issue#6 (SPS for NR MBS). Our understanding is that the PDCCH overhead for PTM transmission scheme 2 is much higher than PTM transmission scheme 1. To address the PDCCH overhead concern, SPS for NR MBS is a good alternative. Dynamic update of SPS configuration is also possible. However, regarding the PDCCH overhead, the worst case of SPS for NR MBS is the same as PTM transmission scheme 2. Thus, we prefer to update the proposal as below. Proposal 2-2a Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs - FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation - FFS: Support PTM transmission scheme 2 for initial transmission of MBS service in RRC_CONNECTED state. For proposal 2-2b, it seems like a mixed solution of PTM transmission scheme 1 and PTM transmission scheme 2. Our preference is to address Proposal 2-2a first and then we can come back to see whether proposal 2-2b is needed or not. |
| Samsung | As commented for proposal 2-1, PTP can be considered with lower priority. As a side note, proposal 2-2 can probably be generalized for any transmission, not only initial. |
| Convida | We support proposal 2-2a. For proposal 2-2b, we are not sure if there is true gain on supporting it. But we are OK to further study it. |
| Lenovo, Motorola Mobility | Proposal 2-2a is not fine with us since it needs huge PDCCH capacity especially when the numbe r of Ues in the group is large. Proposal 2-2b may lead to extra UE blind decoding effort as UE needs to monitor DCI with CRC scrambled by C-RNTI, G-RNTI(s) and sub-G-RNTI(s). |
| OPPO | Proposal 2-2a is supportable for us, to scheduling group-common PDSCH with UE specific PDCCH is beneficial when HARQ feedback is needed, when the PDCCH needs to indicate some UE specific parameters, such as PUCCH resource indicator and even DAI if HARQ feedback for MBS is multiplexed with HARQ feedback for unicast. The motivation of Proposal 2-2b is not clear for us. |
| LG | We support PTM transmission scheme 2 for initial HARQ transmission of a MBS TB. We are skeptical about need for sub-group-common PDCCH based group scheduling for initial transmission. gNB may be able to configure different common resources for different UEs interested in the MBMS service. However, any special mechanism for sub-grouping of UEs receiving a same MBS service seems not so necessary. |
| vivo | Support proposal 2-2a. For NR MBS, ACK/NACK feedback is needed considering some services with very high Qos requirement. One problem for group common PDCCH is how to indicate UE dedicated PUCCH resource for ACK/NACK feedback in this case, which can be easily solved by UE-specific PDCCH. Proposal 2-2b seems to be a trade-off between PTM scheme 2 and PTM scheme 1 in terms of PDCCH overhead. We have the same concern as Convida. We don’t object to further study it. |
| Huawei/HiSilicon | Suggestion to FL that we need to be very careful when summarizing people’s proposals. For example, some companies listed as Object for PTM scheme 2. I doubt whether there is proposal of objecting this scheme from these companies’ paper. From our perspective, since PTM scheme 1 has been agreed, on top of this, PTM scheme 2 for initial transmission is not necessary because the specification effort for PTM scheme 1 is there anyway. We wonder the benefits to support PTM scheme 2 additionally. For proposal 2-2b, is the FFS in the sub-bullet necessary since the main bullet is FFS? Also, since both proposal 2-2a and 2-2b are related to initial transmission. Is it better to merge them into one proposal for initial transmission as the main bullet and list in sub-bullet which scheme is supported for initial transmission and FFS others if no consensus in the next updated proposal version? |
| CMCC | We support proposal 2-2a. The most advantage of PTM scheme 2 (proposal 2-2a) is that the HARQ-ACK feedback related design for unicast in Rel-15/16 can be maximally reused and some other aspects can follow the same design as for unicast, so that less spec effort is needed. As for proposal 2-2b, it is some extension of proposal 2-2a, but the PDCCH monitoring efforts may be an issue, we need FFS. |
| Chengdu TD Tech, TD Tech | We think no need for Poposals 2-2a and 2-2b. The SS/PBCH BLOCK is sent with M beams: beam 1~beam M. We use the beam coverage area “m” to denote the area where the beam “m” covers. For Propsal 2-2a, if the UE specific PDCCH is needed for the scenario where there’s only one UE in the beam coverage area “m”, the UE specific beam can be used to send the group-common PDCCH and the group-common PDSCH. If the UE specific PDCCH is needed when the group-common PDCCH can NOT be received by some UEs, more CCE resource can be used to send the group-common PDCCH in some beam coverage area needed. We find no need for the UE specific PDCCH to schedule the group common PDSCH. |
| Moderator | I have updated the proposal based on companies’ feedback. Regarding Proposal 2-3, let me have some clarification according to my understanding, in this scheme, each UE needs to be configured with one Sub-G-RNTI and one G-RNTI if we only consider one MBS service, UE needs to monitor Sub-G-RNTI for PDCCH monitoring and UE also needs to use G-RNTI for PDSCH scrambling. @Huawei, regarding your comments “Suggestion to FL that we need to be very careful when summarizing people’s proposals. For example, some companies listed as Object for PTM scheme 2. I doubt whether there is proposal of objecting this scheme from these companies’ paper”, can you please point out which company was listed on the objection side regarding PTM scheme 2 without such a proposal in their paper directly so that I can fix it? I do not think such a comment is helpful for me without telling me which company I captured incorrectly. For your other comments, I have reflected them in the updated proposal. |

    - Company views on Proposal 2-3

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | Given our initial analysis of PTP and PTM schemes, we see higher spectral efficiencies and greater PUCCH resource savings with PTM based retransmission schemes, hence we’d We prefer the wording of proposal 2-3 suggested below, where the support of PTP retransmission is FFS. Proposal 2-3a: For RRC_CONNECTED UEs, if initial transmission of MBS service is based on PTM scheme 1, then retransmission can at least use PTM scheme 1. • FFS: whether to support PTP for retransmissions • FFS: whether to support PTM scheme 2 for retransmission. • FFS: How to configure the retransmission scheme. • FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different subgroups of UEs? |
| MTK | Support this proposal. This issue is mainly focusing on how to retransmit the MBS packet. If the feedback mode is NACK only or there are more NACK UE in ACK/NACK feedback mode, it is better to use PTM transmission. If they are less NACK UE, PTP retransmission is more prefer. We also don’t support PTM scheme 2 for retransmission as commented in above issue (3.3.2). In order to soft combining b/w initial transmission (PTM scheme 1) and retransmission (PTP), the PTM scheme1 DCI and PTP DCI should be configured the same HP ID. |
| CATT | We are generally okay with the direction of proposal 2-3, but we think it would be better to address the potential combinations which can make it clearer to us. For the initial transmission and re-transmissions, there are several different combinations. The total number of re-transmission can also be considered like case 4 below. Case 1: PTM scheme1 (initial) + PTM scheme1 (Re-Tx) Case 2: PTM scheme1 (initial) + PTM scheme2 (Re-Tx) Case 3: PTM scheme1 (initial) + PTP (Re-Tx) Case 4: PTM scheme1 (initial) + PTM scheme1 (Re-Tx) + PTP (Re-Tx) There is also another combination when initial transmission is PTP: Case 5: PTP (initial) + PTM scheme1/2 (Re-Tx) Above all, we would like to suggest to first discuss/define proposal 2-3, and then to discussion proposal 2-2. After defining the possible / potential initial transmission and re-transmission cases in proposal 2-3, the detailed scheduling scheme in proposal 2-2 can be defined based on different cases/combinations. |
| Ericsson | We support Proposal 2-3. Regarding the FFSs we disagree with the first FFS, i.e. to support PTM transmission scheme 2 for retransmission. We support to continue study the other two FFSs. |
| Qualcomm | We support the Proposal 2-3 after deleting ‘FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different subgroups of UEs in the same MBS group?’ The reason is as explained for Proposal 2-2b. |
| ZTE | We are generally fine with this proposal. However, we are not sure whether we need the first FFS point considering that we haven’t decided whether to introduce PTM transmission scheme 2 for initial transmission yet. |
| Samsung | Support the proposal in principle. As commented for proposal 2-1, the description of PTM scheme 1 needs to be revised. |
| Convida | We are OK with the proposal |
| Lenovo, Motorola Mobility | We are Ok with the main bullet. However, we have concern on the first FFS and the third FFS. |
| OPPO | PTP transmission may not be used for re-transmission in PTM scheme 1. The prerequisite of using PTP for re-transmission is that gNB can distinguish HARQ feedback from each UE in PTM scheme 1, otherwise it is not reasonable to re-transmit with PTP. However, as group-common PDCCH is used in PTM scheme 1, we are not sure if UE specific PUCCH resource can be allocated for each UE for HARQ feedback. |
| LG | We support Proposal 2-3 with the following clarification as we mentioned for Proposal 2-1: PTP transmission and PTM transmsisions schemes are used to transmit a MBS TB for PTM bearer, not for PTP bearer. |
| vivo | We are OK with the proposal |
| Huawei/HiSilicon | We are ok with the proposal and also prefer the simplified version as suggested by Nokia in principle, though not sure I understand some FFS point. For example, why do we even consider different retransmission schemes be supported simultaneously for different subgroups of UEs in the same MBS group? |
| CMCC | We are OK with the proposal, and we also support the first FFS, which PTM scheme 2 can also be used as re-transmission scheme. The different re-transmission scheme can be used in different scenarios: • PTM scheme 1 used for re-transmission: If large number of UEs in the group feedback NACK or if NACK-only based HARQ-ACK feedback scheme is used. • PTP or PTM scheme 2 used for re-transmission: If only small number of UEs feedback NACK. PTM scheme 2 in this case can provide additional benefits over PTP for re-transmission. For example, if several cell edge UEs in the same beam direction feedback NACK, one alternative is to use PTP for re-transmission to improve the reliability, another better alternative is to use PTM scheme 2 for re-transmission to improve the reliability and at the same time improve the transmission efficiency since these UEs can share the same group-common PDSCH. |
| Chengdu TD Tech, TD Tech | We support the PTM scheme 1 is used for the retransmission. The retransmission with PTP needs further discussion to show the specific method and its benefits. |
| Moderator | Oppo raised that the prerequisite of using PTP for re-transmission is ACK/NACK feedback is agreed for multicast, which is based on the discussion in AI 8.12.2. Nokia also had similar suggestion. Based on this consideration, I updated PTP for retransmission as FFS for now. Regarding the last FFS point, maybe companies have misunderstanding on it. I further clarify it here. The last FFS basically means, e.g., if both PTP and PTM scheme 1 are agreed for retransmission, do we allow using PTP retransmission for some of the UEs and using PTM scheme 1 retransmission for other UEs in the same MBS group. I still keep this FFS for now, if more companies raise concern, I will remove it. For other potential cases, I think we can at least agree to further study these aspect and keep them as FFS to guide the discussion for next meeting. |

Updated Proposals (1st round of email discussion)

    - [High] Updated Proposal 2-1

[High] Updated Proposal 2-1: For discussion purpose, consider the following clarification as RAN1 common understanding.

- PTP transmission scheme 1: For RRC_CONNECTED UEs, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI.

- PTP transmission scheme 2: For RRC_CONNECTED UEs, use UE-specific PDCCH in USS with CRC scrambled by group-common RNTI to schedule UE-specific PDSCH which is scrambled with the same group-common RNTI or UE-specific RNTI (e.g., C-RNTI).

- PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme.

- PTM transmission scheme 2: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme.

- PTM transmission scheme 3: For RRC_CONNECTED UEs in the same MBS group, use UE-specific PDCCH in USS with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI.

- Note: The ‘UE-specific PDCCH / PDSCH’ here means the PDCCH / PDSCH can only be identified by the target UE but cannot be identified by the other UEs in the same MBS group with the target UE.

- Note: The ‘group-common PDCCH / PDSCH’ here means the PDCCH / PDSCH are transmitted in the same time/frequency resources and can be identified by all the UEs in the same MBS.

    - [Medium] Updated Proposal 2-2

[Medium] Updated Proposal 2-2: Further study whether to support following schemes for initial transmission for multicast in RRC_CONNECTED state.

- PTM transmission scheme 2

- sub-group-common PDCCH based group scheduling

    - [Medium] Updated Proposal 2-3

[Medium] Updated Proposal 2-3: For RRC_CONNECTED UEs, if initial transmission for multicast is based on PTM transmission scheme 1, at least support PTM scheme 1 for retransmission. Further following cases:

- FFS Case 1: PTM scheme 1 (initial) + PTP scheme 1/2(Re-Tx)

- FFS Case 2: PTM scheme 1 (initial) + PTM scheme 2 (Re-Tx)

- FFS Case 3: PTM scheme 1 (initial) + PTM scheme 1 (Re-Tx) + PTP scheme 1/2(Re-Tx)

- FFS: How to indicate the association between different transmission schemes transmitting the same TB.

- FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different subgroups of UEs in the same MBS group?

Company Views (2nd round of email discussion)

    - Company Views on Updated Proposal 2-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

    - Company Views on Updated Proposal 2-2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

    - Company Views on Updated Proposal 2-3

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

Updated Proposals (2nd round of email discussion)

To be added…

Issue #3: CORESET and Search Space configuration

Background and summary

Regarding the CORESET configuration for PTM transmission scheme 1 (group-common PDCCH based group scheduling), 6 companies [Huawei, Lenovo, Intel, Qualcomm, CMCC, Nokia] propose that the CORESET should be configured within the common frequency resource for group-common PDSCH. The concrete definition and clarification of common frequency resource is up to the discussion for issue#1 in section 2, but regardless which definition is adopted, it is natural to have this proposal.

| Proposals for CORESET configuration | Companies |
| --- | --- |
| The CORESET for group-common PDCCH should be configured within the common frequency resource for group-common PDSCH | Huawei, Lenovo, Intel, Qualcomm, CMCC, Nokia |

Regarding the search space set type for PTM transmission scheme 1, companies’ views are diverged. Two companies [ZTE, Intel] think both existing CSS type(s) and a new defined CSS type can be considered, one company [CMCC] proposes to use USS, one company [Qualcomm] considers CSS and/or USS can be used, and one company [Nokia] proposes to discuss first on whether a new SS set type is required to be introduced specifically for multicast or not, and how to handle the SS monitoring prioritization between multicast and unicast.

| Options for search space set type | Companies |
| --- | --- |
| Option 1: define a new search space set type specific for MBS | ZTE, Intel |
| Option 2: reuse the existing CSS type(s) in Rel-15/16 with possible modifications | ZTE, Intel, Qualcomm, Nokia |
| Option 3: reuse the existing USS in Rel-15/16 in Rel-15/16 with possible modifications | CMCC, Qualcomm |

Initial Proposals based on contributions

    - Proposal 3-1

[Moderator’s recommendation]

Proposal 3-1: For PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the common frequency resource for group-common PDSCH.

- FFS: number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH

    - Proposal 3-2

[Moderator’s recommendation]

Proposal 3-2a: For search space set of group-common PDCCH of multicast in RRC_CONNECTED state, the CCE indexes are common for different UEs in the same MBS group.

Proposal 3-2b: For search space set of group-common PDCCH of multicast in RRC_CONNECTED state, down select from the following options for the monitoring priority of search space set

- Option 1: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS

- Option 2: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS

- Option 3: The monitoring priority of search space set for multicast can be between the existing Rel-15/16 CSS and existing Rel-15/16 USS

Proposal 3-2c: For search space set of group-common PDCCH of multicast in RRC_CONNECTED state, down select from the following options.

- Option 1: Define a new search space type specific for multicast

- Option 2: Reuse the existing CSS type(s) in Rel-15/16

  - FFS: whether modifications are needed for multicast

- Option 3: Reuse the existing USS in Rel-15/16 with necessary modifications for MBS

  - FFS: detailed modifications

Company Views (1st round of email discussion)

    - Company views on Proposal 3-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | Proposal 3-2a: We support this SS “characteristic” definition approach to defining the SS. |
| MTK | Support this proposal. In unicast, there are up to 12 CORESETs per BWP. If the MBS frequency resource is within one unicast BWP, the CORESETs per BWP can be split into two parts based on services category, e.g., one part for MBS and others for unicast. |
| CATT | We are okay with proposal 3-1. |
| Ericsson | We support Proposal 3-1. We agree with the FFS. |
| Qualcomm | Support Proposal 3-1. If multicast specific BWP is supported, it is straightforward to support Proposal 3-1. |
| ZTE | Wait for the outcome of Proposal 1-1 first. From our perspective, we prefer to discuss proposal 1-1 first. Once proposal 1-1 is finalized, we can come back to proposal 3-1. Different outcome of proposal 1-1 may lead to different understandings of Proposal 3-1. |
| Samsung | Agree. |
| Convida | We are OK with the proposal |
| Lenovo, Motorola Mobility | We are OK with the proposal |
| OPPO | Agree |
| LG | We are OK with the proposal. |
| vivo | We are OK with the proposal |
| Huawei/HiSilicon | Support. |
| CMCC | Agree |
| Chengdu TD Tech, TD Tech | We support Proposal 3-1. |
| Moderator | Proposal 3-1 seems agreeable for all companies except ZTE. |

    - Company views on Proposal 3-2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | Proposal 3-2b: We definitely do not want option 1. We support either option 2 or 3, with a slight preference to option 2. But we would like to understand if there are arguments/use-cases to justify option 3 over option 2. Proposal 3-2c: We wonder if this can be postponed to the appropriate specifications author, after we have agreement on the CCE function (CSS based or USS derived) and the priority indexing needs for this SS. |
| CATT | One of our contributions in AI 8.12.4 discusses the search space type definition for group scheduling. A MBS-specific search space can be defined within CSS, e.g. CSS Type4. The candidate PDCCHs for MBS search space is determined based on legacy CSS method. When determining the PDCCH blind decoding order, MBS CSS has the lower priority than all the other CSS and according to the search space ID together with USS. Our proposal is as follows: An MBS-specific search space is supported, e.g. CSS Type4, which is the lowest order among CSS and making sure the group of UEs deriving a common CCE candidate location. |
| Ericsson | We agree with Proposal 3.2a: We disagree with Proposals 3.2b and 3.2c: In our view, there is a dependency on whether or not PTM scheme 2 is supported. We think it is premature to agree on Proposals 3-2b and 3-2c and propose to wait until the discussion on PTM scheme 2 support is concluded. |
| Qualcomm | We are generally fine with Proposal 3-2a, 3-2b and 3-2c. |
| ZTE | The discussion of proposal 3-2a, 3-2b highly depends on proposal 3-2c. We propose to discuss proposal 3-2c and postpone the discussion of proposal 3-2a, 3-2b. Regarding proposal 3-2c, we are fine with it. Regarding all the three options, our preference is Option1. On one hand, MBS targets for a group of UE, it should be a kind of CSS. However, fallback DCI in CSS is not allowed to schedule data for SCell. Then we may need to change the current CSS rule. From this perspective, it is preferred to define a new search space type specific for multicast. |
| Samsung | There are two aspects. The first is the configuration of search space sets. That can follow Rel-16. The second is whether the search space is a CSS or a USS. That needs to be further discussed. |
| Convida | We are OK with the direction. We suggest to discuss the details in the next meeting and focus more on the more fundamental issue, e.g., issue #1 and #2 in this meeting. |
| Lenovo, Motorola Mobility | We are generally fine with Proposal 3-2a, 3-2b and 3-2c. |
| OPPO | 3-2A: Agree 3-2b: generally fine, but in our understanding only one option will be down selected. 3-2c: generally fine, but in our understanding only one option will be down selected. |
| LG | We are OK with the proposals |
| vivo | For Proposal 3-2a, agree For Proposal 3-2b, does it mean that the monitoring priority of search space set for multicast is configurable for Option 3? One general question, for the proposals with “down select from the following options”, is it suggested to down select in this meeting or in next meeting? We hope it is the latter one here. |
| Huawei/HiSilicon | Proposal 3-2a is ok. Proposal 3-2b and 3-2c, we also see it is premature to discuss such proposals. We need further study on such issues. |
| CMCC | 3-2a: Agree 3-2b: Fine with proposal, and we prefer Option 2, which multicast has the same priority with unicast service 3-2c: Fine with proposal, and we prefer Option 3, the only enhancement is using group-common RNTI value as for CCE indexes calculation to guarantee UEs in the same MBS group receiving the same PDCCH, which is accord with Proposal 3-2a. |
| Chengdu TD Tech, TD Tech | 3-2a: We think the further clarification on Proposal 3-2a is needed. We don’t know what Proposal 3-2a means。 3-2b: We support option 1: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS. 3-2c: We support option 2: Reuse the existing CSS type(s) in Rel-15/16 |
| Moderator | For Proposal 3-2a, most companies are OK. For Proposal 3-2b and 3-2c, most companies are fine with the direction. Ericsson raised they are dependent on whether or not PTM scheme 2 is supported, so I limit the Proposal 3-2a/b/c to PTM scheme 1 only. Some companies raised that 3-2a/b and 3-2c are dependent, I agree, I think if there is no strong concern, we can at least agree to further study these aspects and keep them to guide the discussion in next meeting. @vivo, regarding your comment on 3-2b, it does not mean monitoring priority of search space set for multicast is configurable for Option 3, it can also be a predefined rule. Regarding your general question, the intention is to have a guidance for discussion in next meeting, In the updated proposal it was updated as ‘further study’. |

Updated Proposals (1st round of email discussion)

    - [Medium] Updated Proposal 3-1

[Medium] Proposal 3-1 (No update): For PTM transmission scheme 1, the CORESET for group-common PDCCH is configured within the common frequency resource for group-common PDSCH.

- FFS: number of CORESET(s) for group-common PDCCH within the common frequency resource for group-common PDSCH

    - [Medium] Updated Proposal 3-2

[Medium] Updated Proposal 3-2a: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, the CCE indexes are common for different UEs in the same MBS group.

[Medium] Updated Proposal 3-2b: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options for the monitoring priority of search space set

- Option 1: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 CSS

- Option 2: The monitoring priority of search space set for multicast is the same as existing Rel-15/16 USS

- Option 3: The monitoring priority of search space set for multicast can be between the existing Rel-15/16 CSS and existing Rel-15/16 USS

[Low] Updated Proposal 3-2c: For search space set of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, further study the following options.

- Option 1: Define a new search space type specific for multicast

- Option 2: Reuse the existing CSS type(s) in Rel-15/16

  - FFS: whether modifications are needed for multicast

- Option 3: Reuse the existing USS in Rel-15/16 with necessary modifications for MBS

  - FFS: detailed modifications

Company Views (2nd round of email discussion)

    - Company Views on Updated Proposal 3-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

    - Company Views on Updated Proposal 3-2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

Updated Proposals (2nd round of email discussion)

To be added…

Issue #4: DCI and Blind Decoding related issues

Background and summary

Regarding the number of BDs/CCEs, 3 companies [CATT, Nokia, CMCC] propose to keep the same maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell as in Rel-15 when Rel-17 MBS functionality is enabled. One company [Qualcomm] proposes to consider the group-common PDCCH monitoring for multicast as a virtual CC to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

| Options for number of BDs/CCEs | Company |
| --- | --- |
| Option 1: Keep the BDs/CCEs limits per slot per serving cell defined in Rel-15 for Rel-17 MBS | CATT, Nokia, CMCC |
| Option 2: Consider the group-common PDCCH monitoring for multicast as a virtual CC to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16 | Qualcomm |

Regarding the DCI size budget, two companies [Nokia, CMCC] propose to maintain the “3+1” DCI size budget defined in Rel-15 when Rel-17 MBS functionality is enabled. When considering the “3+1” DCI size budget rule for group-common PDCCH based group scheduling, one company [Nokia] proposes to count G-RNTI as “other RNTI” and another company [Intel] proposes to count G-RNTI as “C-RNTI”.

| Options for DCI size budge | Company |
| --- | --- |
| Option 1: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS | Nokia, CMCC |
| Option 2: The DCI size budget defined in Rel-15 can be increased for Rel-17 MBS |  |

Regarding the DCI format, companies’ views are also diverged. Basically there are two options in high level, that is, whether to reuse the existing DCI format(s) with possible modifications or to define a new DCI format for MBS.

| Options for DCI format | Company |
| --- | --- |
| Option 1: reuse the existing DCI format(s) in Rel-15/16 with possible modifications for group-common PDCCH for Rel-17 MBS | Huawei (format 1_0/1_1/1_2), ZTE (format 1_0), Intel (format 1_0), CMCC (format 1_0/1_1/1_2), Qualcomm (format 1_0/1_1, FFS 1_2) |
| Option 2: define a new DCI format for group-common PDCCH for Rel-17 | Oppo, Intel |

Regarding the FDRA field in the DCI format for group-common PDCCH, 4 companies [Huawei, Lenovo, Intel, CMCC] propose that, for PTM transmission scheme 1, the FDRA field of group-common PDCCH is interpreted based on the common frequency resource for group-common PDSCH.

| Options for FDRA field | Company |
| --- | --- |
| For PTM transmission scheme 1, the FDRA field of group-common PDCCH is interpreted based on the common frequency resource for group-common PDSCH. | Huawei, Lenovo, Intel, CMCC, Nokia |

Initial Proposals based on contributions

    - Proposal 4-1

[Moderator’s recommendation]

Proposal 4-1: Down select from the two options for BDs/CCEs limit for Rel-17 MBS

- Option 1: Keep the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 for Rel-17 MBS.

- Option 2: Consider the group-common PDCCH monitoring for multicast as a virtual CC to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

    - Proposal 4-2

[Moderator’s recommendation]

Proposal 4-2: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH based group scheduling.

- FFS: DCI size alignment procedure for group-common PDCCH

    - Proposal 4-3

[Moderator’s recommendation]

Proposal 4-3: For PTM transmission scheme 1, if Option 2A or Option 2B for common frequency resource for group-common PDSCH is agreed, the FDRA field of group-common PDCCH is interpreted based on the common frequency resource.

Company Views (1st round of email discussion)

    - Company views on Proposal 4-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | We would like more clarification of “virtual CC” … what does that actually mean? Does it mean that the device monitors group-common CCEs on a “best effort” effort basis? (this could make priority control very problematic) Without clarification of “virtual CC”, we initially support Option 1. We should strive to minimize any hardware changes and, in our view, this should make “priority indexing” of the CCEs monitored easier and more deterministic. |
| MTK | Option 1. From UE implementation perspective, there is no need to change the BDs/CCEs limit. |
| CATT | We support option 1 in proposal 4-1. |
| Ericsson | We prefer Option 1 in Proposal 4-1. |
| Qualcomm | Reply to Nokia’s question, the intention of ‘virtual CC’ is to reuse the multi-TRP way, where the CCEs for GC-PDCCH are not counted together with the unicast PDCCH CCEs in the same CC. We have agreed the simultaneous reception of unicast and multicast based on UE capability. - If the UE supports CA, it is possible to use the budget of PDCCH CCEs in an unused CC for the possible CCEs for GC-PDCCH. For UE not supporting CA, Opt2 is equivalent to Opt1, since only one CC is available. |
| ZTE | We prefer to postpone the discussion of proposal 4-1. The discussion here seems highly depends on the specific design of PDCCH for MBS. We prefer to come back to this proposal once RAN1 has some detailed design of PDCCH for MBS. |
| Samsung | Agree. |
| Convida | We are OK with the direction, but we also agree with Nokia that some clarifications are needed. |
| Lenovo, Motorola Mobility | We support option 1 in proposal 4-1. |
| OPPO | We prefer to keep this issue open until the size and the CORESET/search space configuration for MBS DCI is clear. |
| LG | We are OK with the proposals |
| vivo | We also prefer to postpone the discussion on this issue. For these two options, we are fine to be considered. |
| Huawei/HiSilicon | We’d like to FFS option2. Also, I guess option 1 intends to say “the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is unchanged”. The current wording in option 1 sounds like additional maximum… is kept for MBS. |
| CMCC | Support option 1 in proposal 4-1. |
| Chengdu TD Tech, TD Tech | We support option 1: Keep the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 for Rel-17 MBS. |
| Moderator | Regarding the concern raised by Nokia, some clarification and update are made for Proposal 4-1. ZTE and vivo prefer to postpone the discussion. I hope we can at least agree to consider these two options and have some guidance for discussion in next meeting. If more companies prefer to postpone it, I will remove it later. |

    - Company views on Proposal 4-2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | We support the proposal. For FFS bullet one – we see this as “other RNTI” |
| MTK | Support. |
| CATT | We support proposal 4-2. |
| Ericsson | We agree with Proposal 4-2, including FFSs |
| Qualcomm | We prefer to postpone Proposal 4-2 after discussing Proposal 4-1, since ‘3+1’ limitation is per CC. |
| ZTE | We prefer to postpone the discussion of proposal 4-2. The discussion here seems highly depends on the specific design of PDCCH for MBS. We prefer to come back to this proposal once RAN1 has some detailed design of PDCCH for MBS. |
| Samsung | Agree. As a side note, we think DCI format 1_2 should be baseline for MBS PDSCH. In addition to design simplicity and flexibility, size alignment is then not an issue and is left to gNB implementation. |
| Convida | We are OK with the proposal. |
| Lenovo, Motorola Mobility | We are OK with the proposal. |
| OPPO | Same comments as that for Proposal 4-1. |
| LG | We are OK with the proposal |
| vivo | Agree with QC’s view, it may depend on the discussion for Proposal 4-1 |
| Huawei/HiSilicon | Similar comment to proposal 4-1 if we are going to discuss it, i.e., whether the existing “3+1” is kept unchanged or additional “3+1” for MBS. We also see it is not urgent to make conclusion on this proposal at this stage. |
| CMCC | We are OK with the proposal. |
| Chengdu TD Tech, TD Tech | We think the detailed definition should be provided for the following concept: “3+1” DCI size budget defined in Rel-15 |
| Moderator | Although majority is OK with the proposal, Qualcomm and vivo raised Proposal 4-2 has dependency with Proposal 4-1. I marked it as low priority for this meeting. |

    - Company views on Proposal 4-3

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | Support the intention of the proposal but suggest waiting for the conclusion of the “if Option 2A or Option 2B for common frequency resource” proposals. For example, we may decide to support both options. |
| MTK | We also suggest waiting for the conclusion of MBS common frequency resource. |
| CATT | We support proposal 4-3. |
| Ericsson | We disagree with Proposal 4-3 now. Since this is currently a conditional proposal, subject to agreement on Option 2A/2B, we propose to postpone this until the Option 2A/2B is resolved. |
| Qualcomm | Do we have to discuss the common configuration parameters for GC-PDSCH one by one, e.g., FDRA, TDRA, MCS table, MIMO, DMRS/PTRS, rate matching patterns…? We support Proposal 4-3. But if multicast specific BWP is supported, it is straightforward to support this. |
| ZTE | Wait for the outcome of Proposal 1-1 first. This proposal is a very detailed design of FRDA for MBS scheduling. We prefer to come back to it once RAN1 finalizes proposal 1-1. |
| Samsung | Agree. |
| Convida | We also suggest to postpone this discuss until issue #1 is addressed. |
| Lenovo, Motorola Mobility | We are OK with the proposal. |
| OPPO | This proposal is based on an assumption that one of option 2A or option 2B is supported, we prefer to discuss this proposal after proposal 1-1 is confirmed. |
| LG | We suggest to wait for conclusion on Option 2A/2B |
| vivo | We are OK with the proposal. |
| Huawei/HiSilicon | We support this proposal. |
| CMCC | We are OK with the proposal. |
| Chengdu TD Tech, TD Tech | The further consideration is needed for the conclusion on this topic. |
| Moderator | Based on companies’ feedback, Proposal 4-3 is marked as low priority for this meeting. |

Updated Proposals (1st round of email discussion)

    - [Medium] Updated Proposal 4-1

[Medium] Updated Proposal 4-1: Down select from the two options for BDs/CCEs limit for Rel-17 MBS

- Option 1: the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS.

- Option 2: For UEs supporting CA capability, the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

    - [Low] Updated Proposal 4-2

[Low] Proposal 4-2 (No change): The “3+1” DCI size budget defined in Rel-15 is kept unchanged for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH based group scheduling.

- FFS: DCI size alignment procedure for group-common PDCCH

    - [Low] Updated Proposal 4-3

[Low] Proposal 4-3 (No change): For PTM transmission scheme 1, if Option 2A or Option 2B for common frequency resource for group-common PDSCH is agreed, the FDRA field of group-common PDCCH is interpreted based on the common frequency resource.

Company Views (2nd round of email discussion)

    - Company Views on Updated Proposal 4-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

    - Company Views on Updated Proposal 4-2

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

    - Company Views on Updated Proposal 4-3

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

Updated Proposals (2nd round of email discussion)

To be added…

Issue #5: Simultaneous operation with unicast reception

Background and summary

In RAN1#102-e, it was agreed at least to support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. There is still a FFS for TDM and SDM. In addition, there may be some ambiguities need to be further clarified, e.g., whether to support FDM between multiple group-common PDSCHs in a slot, whether to support FDM between one or multiple TDMed unicast PDSCHs and multiple group-common PDSCHs, etc.

In RAN1#103-e, 7 companies [Huawei, vivo, Nokia, Intel, Qualcomm, Ericsson, CMCC] propose to also support TDM between at least one unicast PDSCH and at least one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs,  but one company [Apple] proposes to support it without capability signaling.

One company proposes to support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot based on UE capability. Although other companies did not clearly mention this, moderator thinks it may be also the thinking of some of other companies.

One company proposes to support TDM among multiple group-common PDSCHs in a slot based on UE capability, and further discuss whether to support FDM among multiple group-common PDSCHs in a slot.

One company proposes to support both FDM and TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot based on UE capability.

One company proposes to support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots as a core MBS functionality (mandatory).

Two companies [Huawei, Intel] propose to support SDM between at least one unicast PDSCH and at least one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs, but two companies [Ericsson, CMCC] propose not to support this.

| Proposals for simultaneous operation with unicast reception | Company |
| --- | --- |
| TDM between one unicast PDSCH and one group-common PDSCH in a slot | Huawei, vivo, Nokia, Intel, Qualcomm, Ericsson, CMCC, Apple |
| TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot | CMCC |
| TDM among multiple group-common PDSCHs in a slot | CMCC |
| TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs | CMCC |
| FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs | CMCC |
| Inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots as a core MBS functionality (mandatory) | Ericsson |
| SDM between at least one unicast PDSCH and at least one group-common PDSCH in a slot | Support: Huawei, [Intel] Object: Ericsson, CMCC, Qualcomm |

One company [BBC] proposes that due consideration should be given to the potential support of Broadcast/Multicast and Unicast Superposition Transmission in NR based on UE capability to improve the system’s spectral efficiency.

One company [Oppo] proposes to define some dropping rule for the case that UE is scheduled with unicast PDSCH and group-common PDSCH in one slot but UE has no capability to receive them simultaneously.

One company [Intel] proposes to at least support PDSCH mapping type A for group-common PDSCH, and FFS for PDSCH mapping type B.

Initial Proposals based on contributions

    - Proposal 5-1

[Moderator’s recommendation]

Proposal 5-1a: Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

Proposal 5-1b: Down select from the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

- Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

- Case 2: support TDM among multiple group-common PDSCHs in a slot

- Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 5: support FDM among multiple group-common PDSCHs in a slot

- FFS: maximum number of PDSCHs in a slot simultaneous received per UE

Proposal 5-1c: For RRC_CONNECTED UEs, support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots as a core MBS functionality (mandatory).

Proposal 5-1d: Not to support SDM between unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED.

Company Views (1st round of email discussion)

    - Company views on Proposal 5-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | Proposal 5-1a: we support this. Proposal 5-1b: Case 1: We support Case 2,3 and 5: We support but this is pending decision (multiple group-common PDSCH) until agreement on Proposal 1-2? Case 4: Make FFS – needs further study (with diagrams) |
| MTK | Proposal 5-1a: Support. Proposal 5-1d: Not to support SDM. |
| CATT | We are generally fine with the proposal 5-1a. |
| Ericsson | We agree with Proposals 5-1a, 5-1c and 5-1d (i.e. not to support SDM). Regarding Proposal 5-1b we agree with Case 1. Regarding 5-1b Cases 2, 3, 4 and 5, we think this requires further discussion to agree or not on multiple G-RNTIs per UE, so this should be FFS. |
| Qualcomm | We support Proposal 5-1a. For Proposal 5-2b, we are fine to consider different cases. But how to down select is more dependent on how many PDSCHs in a slot the UE can support. For Proposal 5-2c, we want to clarify here ‘mandatory’ is ‘for the UE supporting MBS’. Proposal 5-1c: For RRC_CONNECTED UEs, support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots (mandatory for the UE supporting MBS). We are ok with Proposal 5-2d. |
| ZTE | We are supportive of Proposal 5-1a, 5-1b and 5-2c. We are not sure whether we need to have proposal 5-1d. If network can implement unicast PDSCH and group-common PDSCH in a slot via SDM by implementation, RAN1 shall not preclude the possibility. |
| Samsung | Support TDM in principle. Specific combinations in a slot can be discussed as part of UE capability. |
| Lenovo, Motorola Mobility | Proposal 5-1a: Support. Proposal 5-1b: Support. Proposal 5-1c: Support. Proposal 5-1d: Support. |
| OPPO | Agree with the proposals |
| LG | We are OK with the proposals |
| vivo | Fine with the proposals. For Proposal 5-1b, we think more than one of listed cases or all listed cases can be supported or further study. |
| Huawei/HiSilicon | As to multiplexing between unicast and MBS, one unicast and one MBS multiplexing seems more straightforward, so proposal 5-1a and 1c seems in principle ok. QC has a point for clarifying the mandatory for 1c. Proposal 1b, down-select means selecting only one case? For this proposal 1b, it would be good progress for this meeting to list all the cases in people’s mind for further study instead of “down-selection” purpose. Proposal 1d, no strong view to support it for MBS but can be supported without additional specification efforts envisioned. |
| CMCC | Agree with the proposals |
| Chengdu TD Tech, TD Tech | We think the further discussion is needed to clarify the above proposals. The necessity and the complexity of each proposal needs to be clarified firstly and then the selection of the proposals can be made. |
| Moderator | For Proposal 5-1a, all companies are supportive. For Proposal 5-1b, it seems most companies are OK with Case 1, some companies have concern with other cases. In the updated proposal, the main bullet is updated for further study. For Proposal 5-1c, seems all the companies are OK. For Proposal 5-1d, majority are OK with it except ZTE raised that SDM may be implemented based network implementation and whether we need to preclude this possibility in RAN1. The proposal was updated to address this concern. |

Updated Proposals (1st round of email discussion)

    - [High] Updated Proposal 5-1

[High] Proposal 5-1a (No change, stable): Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

[Medium] Updated Proposal 5-1b: Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs.

- Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot

- Case 2: support TDM among multiple group-common PDSCHs in a slot

- Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot

- Case 5: support FDM among multiple group-common PDSCHs in a slot

- FFS: maximum number of PDSCHs in a slot simultaneous received per UE

[Medium] Updated Proposal 5-1c: For RRC_CONNECTED UEs supporting Rel-17 MBS, support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots as a core MBS functionality (mandatory).

[Low] Updated Proposal 5-1d: No specification enhancement in Rel-17 to support SDM between unicast PDSCH and group-common PDSCH in a slot for RRC_CONNECTED UEs.

Company Views (2nd round of email discussion)

    - Company Views on Updated Proposal 5-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

Updated Proposals (2nd round of email discussion)

To be added…

Issue #6: SPS for NR MBS

Background and summary

7 companies [vivo, CATT, Samsung, ZTE, Intel, Qualcomm, Chengdu TD Tech] propose to support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs. One company [ZTE] proposes to use UE-specific PDCCH for SPS group-common PDSCH activation/deactivation, one company [Qualcomm] proposes to use group-common PDCCH for SPS group-common PDSCH activation/deactivation.

| Proposal for SPS group-common PDSCH | Company |
| --- | --- |
| Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs | vivo, CATT, Samsung, ZTE, Intel, Qualcomm, Chengdu TD Tech, Nokia |

Initial Proposals based on contributions

    - Proposal 6-1

[Moderator’s recommendation]

Proposal 6-1: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs

- FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation

- FFS: whether to support more than one SPS group-common PDSCH configuration per UE

- FFS: whether and how uplink feedback could be configured

Company Views (1st round of email discussion)

    - Company views on Proposal 6-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | We support the proposal |
| MTK | Support. |
| CATT | We are generally fine with the proposal 6-1. For the first FFS: 3 combinations of activation and deactivation can be listed as follows that can be supported by the proposal. Case 1: Group-common PDCCH (activation) + Group-common PDCCH (deactivation) Case 2: UE-specific PDCCH (activation) + UE-specific PDCCH (deactivation) Case 3: UE-specific PDCCH (activation) + Group-common PDCCH (deactivation) |
| Ericsson | We agree with Proposal 6-1, including FFSs. |
| Qualcomm | We support it. |
| ZTE | We propose to discuss this issue together with Issue2. The motivation of supporting SPS for MBS is to reduce the PDCCH overhead of UE-specific PDCCH based MBS scheduling. Thus, we prefer to discuss them together. |
| Samsung | Agree. |
| Convida | We are OK with the proposal |
| Lenovo, Motorola Mobility | We are OK with the proposal |
| OPPO | Agree with the proposal |
| LG | We are OK with the proposal and propose to add ‘FFS for UEs in RRC_IDLE/INACTIVE’, considering that UEs may receive same MBS resources regardless of RRC states. |
| vivo | Agree with the proposal |
| Huawei/HiSilicon | We prefer to keep the main bullet FFS and need more time for discussion since SPS also affects RAN2. |
| CMCC | We are OK with the proposal |
| Chengdu TD Tech, TD Tech | Support proposal 6-1. |
| Moderator | Seem all companies except Huawei are fine with Proposal 6-1. Regarding CATT’s comment, perhaps there are more combinations, we can leave details for companies’ input in next meeting. Regarding ZTE’s comment, I think the first FFS has already include the UE-specific PDCCH case you mentioned. Regarding LG’s comment, can we first focus on the RRC_CONNECTED state? |

Updated Proposals (1st round of email discussion)

    - [Medium] Updated Proposal 6-1

[Medium] Proposal 6-1 (No change): Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs

- FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation

- FFS: whether to support more than one SPS group-common PDSCH configuration per UE

- FFS: whether and how uplink feedback could be configured

Company Views (2nd round of email discussion)

    - Company Views on Updated Proposal 6-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

Updated Proposals (2nd round of email discussion)

To be added…

Issue #7: Multi-beam operation

Background and summary

5 companies [CATT, LG, Sony, ZTE, Chengdu TD Tech] mentioned the multi-beam operation for MBS in RRC_CONNECTED state, basically, there are two high-level options. One option is beam sweeping operation with all beams in the cell which is suitable for the case that network is not aware of the preferred beams of the MBS UEs, e.g., when the group-common PDCCH / group-common PDSCH are targeted to be received by both RRC_IDLE/INACTVE UEs and RRC_CONNECTED UEs. The other option is beam sweeping with partial beams in the cell which is suitable for the case that network is aware of the preferred beams of the MBS UEs, e.g., when the group-common PDCCH / group-common PDSCH are targeted to be received by only RRC_CONNECTED UEs.

| Options for multi-beam operation | Company |
| --- | --- |
| Option 1: Beam sweeping operation with all beams in the cell for RRC_CONNECTED UEs | CATT, LG, Sony, ZTE, Chengdu TD Tech |
| Option 2: Beam sweeping operation with partial beams in the cell for RRC_CONNECTED UEs | LG, Sony, ZTE |

There are some other detailed proposals regarding multi-beam operation which are only raised by one company as listed below.

| Detailed proposals for multi-beam operation | Company |
| --- | --- |
| Regarding Rel-17 NR MBS, define association between PDCCH MOs and SSBs or CSI-RSs for group common PDCCH transmission. | ZTE |
| Consider beam sweeping mechanism for NR Rel-15 SIBx transmission as the starting point for Rel-17 broadcast. | ZTE |
| Support configuration of multiple CORESETs in search space set for group scheduling of a MBS control TB on PDSCH with multiple beams/TRPs e.g. via system information or UE dedicated signaling, if MCCH-like logical channel is supported by RAN2 | LG |
| Support configuration of multiple CORESETs in search space set for group scheduling of a MBS data TB on PDSCH with multiple beams/TRPs e.g. via MCCH signaling or UE dedicated signaling, if MTCH-like logical channel is supported by RAN2. | LG |
| The UE shall report preference of beam(s) in which its interested MBS services can be provided. | Sony |

Initial Proposals based on contributions

    - Proposal 7-1

[Moderator’s recommendation]

Proposal 7-1: Consider whether to support the following multi-beam operations for group-common PDCCH / group-common PDSCH of MBS for RRC_CONNECTED UEs.

- Beam sweeping with all SSB beams in the cell

- Beam sweeping with partial SSB/CSI-RS beams in the cell

Company Views (1st round of email discussion)

    - Company views on Proposal 7-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | We support further study into multi-beam operation however we feel the bullets can/should be clarified. For example: - Beam sweeping with all SSB beams in the cell - Beam sweeping with partial SSB/CSI-RS beams in the cell Do you really mean: - Beam sweeping with all SSB in all or a subset beams in the cell - Beam sweeping with partial SSB/CSI-RS used in in all or a subset beams in the cell |
| CATT | The two sub-bullets in proposal 7-1 can be replaced by gNB configuration. So we would like to suggest to update the proposal 7-1 as: Proposal 7-1: To support multi-beam operations for group-common PDCCH / group-common PDSCH of MBS for RRC_CONNECTED UEs is configured by gNB. |
| Ericsson | We agree with Proposal 7-1 |
| Qualcomm | It is not clear whether the proposal here is applied to broadcast or multicast transmission. If it is for broadcast, it is overlapped with the beam sweeping discussion in 8.12.3, where broadcast beam sweeping is applied to RRC_IDLE/INACTIVE/CONNECTED UEs. We prefer to defer the proposal. |
| ZTE | We are supportive of the above proposal. Our understanding is that this proposal is applied to multicast transmission, thus it should be ok to discuss it here. |
| Samsung | Discuss further whether there is a need for either option. Rel-16 GC-PDCCH monitoring is default. |
| Convida | We think that beam sweeping could be considered for MBS, but further study is needed. |
| Lenovo, Motorola Mobility | We support FFS multi-beam operation. |
| OPPO | Agree with the proposal |
| LG | We are OK with the proposals |
| vivo | We are OK with the direction. |
| Huawei/HiSilicon | Similar comment to QC. We prefer to discuss this issue firstly for IDLE/INACTIVE UEs in AI 8.12.3 so suggest deferring this discussion for CONNECTED UE. |
| CMCC | We are OK with the proposal |
| Chengdu TD Tech, TD Tech | We clarify our views as below on the transmission of an MBS with PTM to a group of RRC_CONNECTED UEs. - Beam sweeping operation with partial beams in the cell for RRC_CONNECTED UEs is used if there’s no UE in one or several beam coverage area. In the above method, M beams are used to transmit the SS/PBCH BLOCK. The beam coverage area “m” is used to denote the area covered by the m-th beam of the M beams for the SS/PBCH BLOCK. If there’s no UE in the beam coverage area “m”, no need to transmit the MBS in this beam coverage area. - For the MBS transmitted to the UEs in all states, the beam sweeping operation with all beams in the cell is used to transmit the MBS with the group common PDSCH and the group common PDCCH. |
| Moderator | One company [Samsung] proposes to further discuss whether we need multi-beam operation. Two companies [Qualcomm, Huawei] propose to postpone this discussion considering it is correlated to beam sweeping in IDLE/INACTIVE. We can first limit the discussion for multicast. @Nokia, sorry, I really did not catch your point. @CATT, it is not clear what ’is configured by gNB’ means. I think we can keep it for further study to guide the discussion in next meeting. |

Updated Proposals (1st round of email discussion)

    - [Low] Updated Proposal 7-1

[Low] Updated Proposal 7-1: Further study the following multi-beam operations for group-common PDCCH / group-common PDSCH of multicast in RRC_CONNECTED state.

- Beam sweeping with all SSB beams in the cell

- Beam sweeping with partial SSB/CSI-RS beams in the cell

Company Views (2nd round of email discussion)

    - Company Views on Updated Proposal 7-1

Companies are encouraged to provide comments in the table below.

| Company | Comment |
| --- | --- |

Updated Proposals (2nd round of email discussion)

To be added…

Issue #8: Other issues (Low priority)

Background and summary

There are some other issues and proposals which are raised by one or two companies as listed below. These proposals can be low priority and can be discussed based on more progress of previous issues.

| Proposals | Company |
| --- | --- |
| Proposal 8-1: A UE can be configured with multiple group common RNTIs for PDSCH scrambling for different Broadcast/Multicast services. - FFS: whether UE is expected to receive all the PDSCHs scrambled with these group common RNTIs | vivo |
| Proposal 8-2: The configurable number of maximum HARQ process number is kept unchanged for UE supporting MBS reception, and - the total number of HARQ processes for initial transmissions are shared and split between unicast and MBS; - the HARQ process number for retransmission is kept the same as for initial transmission. | Huawei |
| Proposal 8-3: Consider supporting DRX/WUS for NR MBS for power saving. The detailed design is FFS. | CATT |
| Proposal 8-4: Further discuss whether to support CA for Rel-17 MBS, e.g., UE receive MBS services on SCell. | CMCC, LG |
| Proposal 8-5: Study whether the same basic broadcast mechanism is applied for RRC_CONNECTED UEs and RRC_IDLE/RRC_INACTIVE UEs. | ZTE |

Company Views (1st round of email discussion)

Companies can provide comments in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | Of these proposals, we could support Proposal 8-2. The other proposals need more investigations. |
| CATT | As it is discussed in our contribution, supporting proposal 8-3 that DRX/WUS for NR MBS can help to save power for UE. |
| Ericsson | Proposal 8-1: We agree with this proposal, provided the proposal only refers to the configuration of the UEs and not to the multiplexing for parallel reception. Proposal 8-2: We agree with this proposal. Proposal 8-3: We agree with this proposal. Proposal 8-4: We agree with this proposal. Proposal 8-5: We agree with this proposal. |
| Qualcomm | We prefer to have more time to consider proposal 8-1, 8-2, 8-3, 8-4, 8-5. |
| Samsung | Can deprioritize for now. Some of the proposals require RAN2 coordination. For others, whether there needs to be specification impact is unclear. |
| LG | We are OK with Proposal 8-1, 8-4, and 8-5. We can further study whether Proposal 8-2 is supported or not. We do not support Proposal 8-3. |
| vivo | For Proposal 8-1, we think it has some relations with Proposal 1-2. From our understanding, these multiple common frequency resources are configured for multiple MBS services. |
| Huawei/HiSilicon | We support proposal 8-1 and 8-2, others need more discussion. |
| CMCC | Support proposal 8-1, 8-4, others can be discussed later. |
| Chengdu TD Tech, TD Tech | The further clarification and discussion on the proposals in this section are needed to make each proposal much clearer. |
| Moderator | Based on companies’ feedback, these proposals can be low priority for this meeting |

Updated Proposals (1rd round of email discussion)

To be added…

Proposals for 1st GTW session

To be added…

- References

- RP-193248	New WID proposal: NR Multicast and Broadcast Services

- RP-201038	Revised WID: Core part: NR multicast and broadcast services

- R1-2007556	Group scheduling for MC/BC services	FUTUREWEI

- R1-2007562	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2007637	Group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

- R1-2007691	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

- R1-2007835	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2008034	Discussion on group scheduling mechanisms	CMCC

- R1-2008064	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

- R1-2008192	On mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

- R1-2008242	Group scheduling for NR Multicast and Broadcast Services	OPPO

- R1-2008375	Considerations on MBMS group scheduling for RRC_CONNECTED UEs	Sony

- R1-2008449	Discussion on group scheduling mechanism for RRC_connected UEs	Apple

- R1-2008826	Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

- R1-2008833	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ETRI

- R1-2008882	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

- R1-2008926	Discussion on group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

- R1-2008961	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

- R1-2009000	Group Scheduling for NR-MBS	Intel Corporation

- R1-2009055	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	Asia Pacific Telecom co. Ltd

- R1-2009165	On group scheduling mechanism for NR multicast and broadcast	Convida Wireless

- R1-2009238	On Optimal Multiplexing for Simultaneous Operation of Broadcast/Multicast and Unicast Services		BBC

- R1-2009274	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2009305	Mechanisms to support group scheduling for RRC_CONNECTED Ues	Ericsson

- Appendix 1: Agreements in previous meetings

RAN1#102-e

Agreements:

For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

o   FFS: whether to support UE-specific PDCCH to schedule a PDSCH for MBS.

Agreements:

- For RRC_CONNECTED UEs, define/configure common frequency resource for group-common PDSCH.

  - FFS: whether to reuse the BWP framework or not

  - FFS: the relation between the common frequency resource and UE dedicated BWP, e.g., the common frequency resource is a MBS specific BWP, or the common frequency resource is confined within UE’s dedicated BWP, etc.

  - FFS: whether more than one common frequency resource can be configured per UE

Agreements:

- For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability.

  - FFS: TDM or SDM in a slot.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

- Appendix 2: Summary of proposals

| Tdoc | Source | Proposals |
| --- | --- | --- |
| [3] R1-2007556 | FUTUREWEI | Proposal 1: Support common frequency resource for MBS to be confined within UE’s dedicated BWP. Proposal 2: Support simultaneous reception of NR unicast TB and MBS TBs in the UE dedicated BWP. Proposal 3: New DCI fields are needed for group scheduling MBS. FFS if a new DCI format is needed or if existing DCI format(s) can be modified Proposal 4: Both downlink resource allocation schemes type 0 and type 1 are supported for MBS. Proposal 5: - Define common PUCCH resource for MBS HARQ feedback. - This common PUCCH resource is in addition to any PUCCH resource that NR unicast may have. - The common PUCCH resource is indexed based on the HARQ process number. - The common-RNTI scrambled DCI PRI field indicate orthogonal PUCCH resources per HARQ process ID to transmit UL feedback. Observation 1: The DCI field PDSCH-to-HARQ_feedback timing indicator can reuse the DCI format 1_0 or DCI format 1_1 method of indicating as long as it enables indicating multiple PDSCHs that may be scheduled in the slot |
| [4] R1-2007562 | Huawei, HiSilicon | Proposal 1：UE-specific PDCCH scheduling group-common PDSCH for MBS is not necessary. Proposal 2: Group-common PDCCH and UE-specific PDCCH can both be supported for scheduling retransmission of MBS. Proposal 3: A common sub-band for MBS configured within dedicated unicast BWPs of UEs of the group is more proper in terms of less restriction and without BWP switching for receiving both unicast and MBS. Proposal 4: More than one common frequency resources can be configured per UE with each confined within dedicated unicast BWP. Proposal 5: For a common sub-band for MBS configured within dedicated unicast BWP and a group-common PDCCH based scheduling: - The CORESET and search space is configured within the common sub-band; - The configurations for the CORESET and search space are kept the same for all UEs of the group. Proposal 6: DCI formats 1_0, 1_1 and 1_2 can be used for scheduling MBS with necessary modifications, and new DCI format is not needed: - For a common sub-band for MBS configured within dedicated unicast BWP and a group-common PDCCH based scheduling, the FDRA field in DCI is dimensioned per the common sub-band. Proposal 7: DCI size alignment for monitoring DCI for MBS scheduling needs to be determined including whether the DCI size budget is kept or can be extended. Proposal 8: The configurable number of maximum HARQ process number is kept unchanged for UE supporting MBS reception, and - the total number of HARQ processes for initial transmissions are shared and split between unicast and MBS; - the HARQ process number for retransmission is kept the same as for initial transmission. Proposal 9: For RRC_CONNECTED UEs, support TDM and SDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. |
| [5] R1-2007637 | CHENGDU TD TECH LTD | Proposal 1: Pre-configure at least one MBS dedicated BWP with each MBS dedicated BWP providing the radio resource for the MBS. Proposal 2: Preconfigure at least one general BWP for each MBS dedicated BWP with each general BWP containing the MBS dedicated BWP Proposal3: gNB should support to pre-configure the same MBS dedicated BWPs and the same general BWPs for each MBS dedicated BWP for all NR cells controlled by the gNB. Proposal 4: gNB pre-configures at least one CORESET and SS group on each MBS dedicated BWP Proposal 5: gNB should support to pre-configure the same CORESET and SS groups on each MBS dedicated BWP for all NR cells Proposal 6: Support more than one SC-MTCHs for the PTM bearer of an MBS Proposal 7: G-RNTI and SPS G-RNTI are configured for an MBS. Proposal 8: For a P-RB of the PTM bearer, the following items need to be supported. - Configure the SPS PDSCH resource to transmit the data of a P-RB of the PTM bearer periodically - PDCCH with CRC scrambled with SPS G-RNTI is used to activate/de-activate the SPS PDSCH resource. - SPS G-RNTI is used in the bit scrambling of the PDSCH carrying a P-RB of the PTM bearer. Proposal 9: For all the NP-RBs of the PTM bearer, the following items need to be supported. - The dynamic scheduling is used to transmit the data of all the NP-RBs of the PTM bearer with the SC-MTCHs of all the NP-RBs multiplexed onto one DL-SCH. - Each time the PTM bearer is scheduled, the PDCCH and PDSCH resource is allocated. The PDCCH with CRC scrambled with G-RNTI is transmitted with the allocated PDCCH resource. The PDSCH with G-RNTI used in the bit scrambling is transmitted with the allocated PDSCH resource - Each time the PTM bearer is scheduled, one TB is generated and repeatedly transmitted N1 times in each beam coverage area where at least one RRC_CONNECTED UE is located. Proposal 10: Support the group scheduling method for B2=B and/or B2=B1. Proposal 11: For a RRC_CONNECTED UE, support the DTCHs of the UE’s PTP bearer and the DTCHs of the UE’s unicast services are multiplexed onto one DL-SCH. Proposal 12: For a RRC_CONNECTED UE, support the DTCHs of the UE’s PTP bearer are multiplexed onto one DL-SCH which is independent from the DL-SCH of the UE’s unicast services. |
| [6] R1-2007691 | vivo | Observation 1: The retransmission scheme with dynamically selected C-RNTI/g-RNTI brings about 6.23% and 1.11% gain in term of RU compared to the g-RNTI only and C-RNTI retransmission scheme respectively. Observation 2: For the cell spectral efficiency, the performances of the three kinds of MBS HARQ retransmission schemes are similar. Proposal 1: For RRC_CONNECTED UEs, when defining/configuring common frequency resource for group-common PDSCH, it is suggested to define an MBS common frequency resource confined within UE’s active BWP. Proposal 2: For RRC_CONNECTED UEs, support TDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. Proposal 3: A UE can be configured with multiple common RNTIs for PDSCH scrambling for different Broadcast/Multicast services. Proposal 4: For RRC_CONNECTED UEs, support UE-specific PDCCH with CRC scrambled by a UE-specific RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is scrambled by a common RNTI. Proposal 5: The following two modes are supported for MBS service scheduling. - Dynamic scheduling PDSCH for Broadcast/Multicast service - Semi-persistent scheduling PDSCH for Broadcast/Multicast service. Proposal 6: For the retransmission of group common PDSCH for MBS service, either UE-specific PDSCH or group common PDSCH can be used. |
| [7] R1-2007835 | CATT | Proposal 1: UE-specific PDCCH and sub-group-common PDCCH group scheduling is supported in NR MBS. Proposal 2: Reusing current BWP framework for MBS common frequency resource definition/configuration. Proposal 3: The common frequency resource is confined with UEs’ dedicated BWPs. Proposal 4: SPS can be supported for NR MBS by reusing the Rel-16 SPS mechanism. Proposal 5: In NR MBS system, both options of PDCCH monitoring occasion method can be considered. - Option 1: PDCCH MOs in one period are for different SSBs for scheduling the same PDSCH. - Option 2: PDCCH MOs in one period are for the same SSB for scheduling different PDSCHs. Proposal 6: The maximum number of monitored PDCCH candidates per slot is not increased based on Rel-15 when PDCCH candidates for MBS are considered in the monitoring. Proposal 7: Rel-15 mechanism on how to allocate the PDCCH candidates in CSS/USS can be a baseline to determine the monitoring priority, when PDCCHs for MBS are added for blind decoding. Proposal 8: DRX/WUS can be supported for NR MBS for power saving. The detailed design is FFS. |
| [8] R1-2008034 | CMCC | Proposal 1. For RRC_CONNECTED UEs, define following two PTM schemes only for discussion purpose. - PTM scheme 1: For PTM transmission for UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. This scheme can also be called group-common PDCCH based group scheduling scheme. - PTM scheme 2: For PTM transmission for UEs in the same MBS group, use UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule group-common PDSCH which is scrambled with group-common RNTI. This scheme can also be called UE-specific PDCCH based group scheduling scheme. Proposal 2. For RRC_CONNECTED UEs, the configured common frequency resource for group-common PDSCH is confined within UE’s dedicated BWP, and the common frequency resource is configured per DL BWP. Proposal 3. For RRC_CONNECTED UEs and PTM scheme 1, if the common frequency resource is configured for the group-common PDSCH, the CORESET for the group-common PDCCH should be configured in the common frequency resource, and the FRDA field of group-common PDCCH is determined based on the common frequency resource instead of UE’s active DL BWP. Proposal 4. For PTM scheme 1, dedicated physical layer parameters for group-common PDSCH e.g., TDRA table, DMRS configuration, etc., can be configured under the configuration of common frequency resource. Proposal 5. Further discuss whether more than one common frequency resource can be configured per DL BWP. Proposal 6. For PTM scheme 1, USS is preferred for group-common PDCCH monitoring, but group-common RNTI value can be used in for CCE indexes calculation to guarantee UEs in the same MBS group receiving the same PDCCH. Proposal 7. For PTM scheme 1, both fallback DCI format 1_0 and non-fallback DCI format 1_1/1_2 could be considered with new interpretations. Proposal 8. Keep the “3+1” DCI size budget as in Rel-15/16 when PTM transmission is enabled. Proposal 9. For PTM scheme 1, decide whether the DCI size associated with group-common RNTI (G-RNTI) should be counted in the DCI size budget associated with C-RNTI or counted in the DCI size budget associated with all RNTIs. Proposal 10. For PTM scheme 1, keep the same maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell as in Rel-15 when R17 NR MBS is enabled. Proposal 11. For RRC_CONNECTED UEs, support PTM scheme 2 for NR MBS, i.e., UE-specific PDCCH with CRC scrambled by UE-specific RNTI to schedule group-common PDSCH scrambled with group-common RNTI. Proposal 12. The common frequency resource for group-common PDSCH can be optionally configured for PTM scheme 2. If type 0 frequency domain resource allocation is used, the RBG size and RBG numbering for FDRA indication in the UE-specific DCI are determined based on the size of common frequency resource instead of UE’s active BWP. Proposal 13. For PTM scheme 2, dedicated physical layer parameters for group-common PDSCH e.g., TDRA table, DMRS configuration, etc., can be configured under the configuration of common frequency resource. Proposal 14. For PTM scheme 2, non-fallback DCI format 1_1/1_2 could be considered, and one additional DCI field is defined to differentiate that the scheduled PDSCH’s scrambling initialization is based on UE-specific RNTI or group-common RNTI. Proposal 15. For PTM scheme 2, keep the same maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell as in Rel-15 when R17 NR MBS is enabled. Proposal 16. For NR MBS, if the initial transmission is based on PTM scheme 1, support that the re-transmission can be based on PTM scheme 1, PTM scheme 2 or PTP. Proposal 17. For NR MBS, if the initial transmission is based on PTM scheme 2, support that the re-transmission can be based on PTM scheme 2 or PTP. Proposal 18. For RRC_CONNECTED UEs, support TDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. Proposal 19. For RRC_CONNECTED UEs, support TDM between multiple group-common PDSCHs in a slot based on UE capability. Proposal 20. For RRC_CONNECTED UEs, support TDM or FDM between unicast PDSCH(s) and multiple TDMed group-common PDSCHs in a slot based on UE capability. Proposal 21. Further discuss whether to support FDM between multiple group-common PDSCHs in a slot for RRC_CONNECTED UEs. Proposal 22. Further discuss the PDSCH prioritization rule when PTM PDSCH is partially or fully overlapped in time in non-overlapping PRBs with another SI-RNTI PDSCH in one slot. Proposal 23. Further discuss whether to consider the two typical CA cases in section 4.1 for R17 NR MBS. |
| [9] R1-2008064 | LG Electronics | Proposal 1: Support all the following scenarios for UE specific scheduling as well as group scheduling in reception of a MBS TB in RRC_CONNECTED UEs. - Scenario 1: Group scheduling of a MBS TB with Group RNTI - Scenario 1A: Group scheduling of a MBS TB within UE’s active BWP - Scenario 1B: Group scheduling of a MBS TB in a BWP not overlapped with UE’s active BWP within UE’s serving cell - Scenario 1C: Group scheduling of a MBS TB in a cell other than UE’s serving cell NOTE: Group scheduling of a MBS TB with Group RNTI in Scenario 1A/B/C is defined as follows: - DCI(s) with Group RNTI schedules PDSCH transmission(s) of the TB with one or more beams. - Each of UEs in the group selects a beam and receives at least one PDCCH/PDSCH transmission possibly received by other UE(s) in the group. - Scenario 2: Unicast scheduling of a MBS TB with UE’s C-RNTI in UE’s active BWP - DCI(s) with UE’s C-RNTI schedules UE’s own PDSCH transmission(s) of the TB with at least a beam selected for the UE. - Each of UEs in the group receives UE’s own PDCCH/PDSCH transmission of the TB. Proposal 2: PDCCH/PDSCH transmissions of a MBS TB can be repeated in time and/or frequency resources with all beams/TRPs, e.g. if idle/inactive UEs as well as connected UEs receive the TB. Proposal 3: Support configuration of multiple CORESETs in search space set for group scheduling of a MBS control TB on PDSCH with multiple beams/TRPs e.g. via system information or UE dedicated signaling, if MCCH-like logical channel is supported by RAN2. Proposal 4: Support configuration of multiple CORESETs in search space set for group scheduling of a MBS data TB on PDSCH with multiple beams/TRPs e.g. via MCCH signaling or UE dedicated signaling, if MTCH-like logical channel is supported by RAN2. Proposal 5: PDCCH/PDSCH transmissions of a MBS TB can be repeated in time and/or frequency resources only with multiple selective beams/TRPs (i.e. not all beams/TRPs), e.g. if only connected UEs receive the TB. |
| [10] R1-2008192 | Samsung | Proposal 1: Configure MBS PDSCH reception parameters for UE-specific BWPs. Proposal 2: Supports SPS for multicast. Proposal 3: Consider whether a same BWP or different BWPs are used for unicast and MBS PDSCH transmissions for determining support of FDM between multicast PDSCH and unicast PDSCH. Observation 1: Whether a UE monitors PDCCH for scheduling MBS PDSCH receptions according to CSS or according to USS can be determined after a decision whether the UE also monitors PDCCH for scheduling unicast PDSCH/PUSCH in the same DL BWP or whether MBS PDSCH receptions from multiple TRPs are supported. |
| [11] R1-2008242 | OPPO | Proposal 1: Support to use overlapping part of active BWP of multiple UEs as the common frequency resource for group-common PDSCH. Proposal 2: Support to use dedicated MBS BWP as the common frequency resource for group-common PDSCH. Proposal 3: Support to use UE specific PDCCH to schedule group common PDSCH. Proposal 4: A new DL DCI format should be defined for the scheduling of group-common PDSCH. Proposal 5: Group scheduling design for MBS should take Case 1 and Case 2 above into account. Proposal 6: In case of gNB schedule unicast PDSCH and group-common PDSCH in one slot but UE has no capability to receive them simultaneously, some dropping rule should be defined. |
| [12] R1-2008375 | Sony | Proposal 1: Support the use of beam(s) for NR_MBS contents delivery. Proposal 2: The network shall configure beam location and periodicity for NR_MBS contents delivery. Proposal 3: The UE shall report preference of NR_MBS content and beam(s) in which the NR_MBS content can be provided. Proposal 4: Configure multiple beam sweeping resources for same NR_MBS session(s) delivery. |
| [13] R1-2008449 | Apple | Observation 1: For Rel-15 paging mechanism, different UEs are separated into different POs. While for Rel-15 SIBx mechanism, different SI messages are separated into different SI-windows. Proposal 1: MBS specific BWP is configured for common frequency resource for group-common PDSCH. Observation: The UE behavior should be defined if it doesn’t receive the group common PDSCH correctly, in other words, UE receives the group common PDSCH re-transmission or receives the re-transmission via the unicast PDSCH. Proposal 2: Support dynamic indication of MBS PDSCH re-transmission via PTM or PTP. Proposal 3: Support joint indication of MBS PDSCH re-transmission and PTM and PTP switching. Proposal 4: Support UE-specific PDCCH to schedule a PDSCH for MBS. Proposal 5: TDM reception of unicast PDSCH and group-common PDSCH in a slot is supported without capability signalling. |
| [14] R1-2008826 | ZTE | Proposal 1: Rel-17 NR MBS can consider partial beam sweep in order to reduce the resource overhead. Proposal 2: Rel-17 NR MBS can consider introducing a MBS specific BWP for better flexibility. - The MBS specific BWP is confined within UE’s unicast BWP, and the SCS and CP are the same as unicast BWP. Proposal 3: Regarding Rel-17 NR MBS - Define a Type x-PDCCH CSS set for the group common PDCCH. - Define association between PDCCH MOs and SSBs or CSI-RSs for group common PDCCH transmission. Proposal 4: DCI format 1_0 scrambled with the G-RNTI can be used for MBS group scheduling under group common PDCCH. Proposal 5: Current mechanism can be reused for determining the size of DCI format 1_0 for MBS group scheduling under group common PDCCH. - DCI size is determined according to CORESET0 if CORESET0 is configured; otherwise, the DCI size will be determined according to initial DL BWP. Proposal 6: UE-specific PDCCH based group scheduling is NOT supported for MBS transmission. Proposal 7: SPS that is activated/deactivated by UE-specific PDCCH can be considered for multicast service transmission. Proposal 8: The same basic broadcast mechanism is applied for RRC_CONNECTED UEs and RRC_IDLE/RRC_INACTIVE UEs. Proposal 9: Consider beam sweeping mechanism for NR Rel-15 SIBx transmission as the starting point for Rel-17 broadcast. Proposal 10: NR MBS UEs support reporting its interested broadcast service under RRC_CONNECTED state. Proposal 11: RAN1 further studies whether to support HARQ-ACK for broadcast service for UEs under RRC_CONNECTED state. |
| [15] R1-2008833 | ETRI | Proposal1: UE-specific PDCCH is not considered in Rel-17. Proposal2: BWP framework is required to be reused for MBS. Proposal3: Trade-off between two alternatives: the common frequency resource is a MBS specific BWP or the common frequency resource is confined within UE’s dedicated BWP should be considered together with other related issues. |
| [16] R1-2008882 | Nokia, Nokia Shanghai Bell | Observation-1: Having a UE specific PDCCH that can schedule UEs to use a group common PDSCH is desirable for the following reasons: 1. In scenarios where there is a low density of users receiving multicast traffic with high data rates and requiring uplink feedback, gNB will have the flexibility to choose the appropriate control channel signalling mechanism 2. Enables the support of seamless mobility and switching from multicast-to-unicast 3. Enables simultaneous BWP switching and scheduling of MBS PDSCH resources using the same DCI Observation-2: In order to support both signalling options to access the same group common PDSCH, new signalling mechanisms will be required to allow the network to configure and modify on a dynamic basis these connections. Observation-3: For Option-1, it is not clear whether the unicast traffic can be scheduled in the MBS BWP or not. Observation-4: For Option-1, if the unicast is considered not part of MBS BWP, there is the drawback of limiting the overall capacity that is available for MBS and unicast. Observation-5: For Option-1, if the unicast is considered not part of MBS BWP, it does not support multiplexing of unicast / multicast reception. Observation-6: Option-2 does not fully avoid the BWP switching for multicast reception, but UEs would not be required to switch BWPs for receiving unicast and multicast traffic simultaneously. Observation-7: Option-1 can be seen as a special case of configuration when Option-2 is supported. Observation-8: From UE power savings perspective, it is not beneficial to configure a large dedicated BWP to a UE in the MBS group. Observation-9: Multiple common frequency resources can be configured per UE based on gNB implementation – even though the motivations for doing so are not clear, with the maximum limit dependent on UE capabilities and available system resources. Observation-10: For slot based FDM multiplexing of unicast and multicast traffic, the network will need to optimize the active BWP bandwidth to account for the expected combined unicast and multicast traffic capacity and to minimize power consumed when there is no/less traffic. Observation-11: Slot based FDM multiplexing of unicast and multicast traffic, may need to be suspended temporarily if the active BWP needs to be reconfigured/changed. Proposal-1: For RRC_CONNECTED UEs, support UE-specific PDCCH with CRC scrambled by a C-RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on a common RNTI. Proposal-2: The same group-common PDSCH for PTM transmission can be accessed either by: • A set of UEs using the same group-common PDCCH with CRC scrambled by a common RNTI • A set of UEs, where each UE uses a UE-specific PDCCH with CRC scrambled by a C-RNTI • A mix of the UEs, where some of them use UE-specific PDCCH and others use group-common PDCCH Proposal-3: The network can dynamically modify the signalling used to configure a UE to access a group-common PDSCH. Proposal-4: BWP framework should be re-used at Rel-17 MBS. Proposal-5: For RRC_CONNECTED UEs Option-2 shall be supported • Option-1 is a special case of Option-2 and thus implicitly also supported. • BWP where MBS data is provided must support multiplexing with unicast. Proposal-6: Need to discuss on how to handle the SS monitoring prioritization between multicast and unicast. Proposal-7: RAN1 should discuss whether a new SS set type specific for MBS is needed or not. Proposal-8: Propose to have the SS configuration and UE monitoring for 5G NR multicast to depend on the multicast service types, i.e. high-priority multicast services are configured in CSS and low-priority multicast services are configured in USS if no new SS set type introduced specifically for MBS. Proposal-9: Propose to maintain the maximum supported CORESET/SS sets numbers and BD/CCE limits defined in Rel-15 for Rel-17 MBS. Proposal-10: Propose to count G-RNTI as “other RNTI” when considering the (3+1) DCI size budget rule defined in Rel-15 NR. Proposal-11: Propose to consider the RNTI prioritization between C-RNTI and G-RNTI in a configured SS set. Proposal-12: Further discuss whether a New DCI format with G-RNTI is needed or not when the issue of common frequency resource with BWP operation is clarified. Proposal-13: For RRC_CONNECTED UEs, support Type-B based TDM between unicast PDSCH and group-common PDSCH in a slot subject to UE capability. |
| [17] R1-2008926 | Lenovo, Motorola Mobility | Proposal 1: Using a UE-specific DCI to schedule a group-common PDSCH for MBS is not supported. Proposal 2: Using a UE-specific DCI to schedule a UE-specific PDSCH for MBS is supported for MBS retransmission. Proposal 3: A common frequency region confined with each UE’s dedicated BWP is defined/configured for MBS. Proposal 4: The group-common DCI and scheduled group-common PDSCH are transmitted within the common frequency region. Proposal 5: RB numbering within the common frequency region is with reference to the lowest RB of the common frequency region. Proposal 6: The number of bits for frequency domain resource assignment indicator in DCI is determined based on the bandwidth of the common frequency region. |
| [18] R1-2008961 | MediaTek Inc. | Observation 1: UE-specific PDCCH based group scheduling can be configured when the number of UEs in one MBS group are relatively small. Observation 2: UE-specific PDCCH based group scheduling can reuse R15/R16 unicast as much as possible. Observation 3: UE-specific PDSCH associated with a G-RNTI in UE-specific PDCCH based group scheduling can increase the system time delay for latency sensitive services and need more PUCCH resources in one feedback window to feed back timely. Observation 4: gNB doesn’t need to know which UE report NACK when both the initial transmission and retransmission adopt the group common scheduling. Observation 5: From gNB’s perspective, initial transmission (PTM) and retransmission (PTP) have different HARQ process. Observation 6: The common NACK feedback mode is not suitable for case 2’s group scheduling. Observation 7: case 3’s group scheduling mechanism is similar to unicast. Proposal 1: The MBS common frequency resource is confined with UE’s active BWP for group-common PDCCH based group scheduling. Proposal 2: The group-common PDCCH based group scheduling can be used for initial transmission and retransmission. Proposal 3: UE-specific PDSCH associated with a C-RNTI should be supported in UE-specific PDCCH based group scheduling. Proposal 4: Case 1 with common NACK mode should be supported when there are many UEs in one MBS group. Proposal 5: The initial transmission (PTM) DCI and retransmission (PTP) DCI should be configured the same HP ID. Proposal 6: case 2 with ACK/NACK mode should be supported when the UEs in the same MBS group are relatively small. Proposal 7: case 2 with separate NACK mode should be supported for reducing UE’s power consumption. Proposal 8: The UE-specific PDCCH based group scheduling should be supported for initial (re)transmission when the UEs in one MBS group is smaller (e.g., N<3). |
| [19] R1-2009000 | Intel Corporation | - The common frequency resource for receiving group common PDSCH should be contained within the active BWPs of the UEs in the group. The CORESET for the group-common PDCCH carrying the scheduling DCI should be contained within the defined MBS frequency resource. - BWP framework need not be fully re-used and special frequency resource or a special BWP can be configured which includes at least a configuration of subcarrier spacing (SCS), cyclic prefix (CP), starting PRB index via an offset to the common resource block (CRB) #0, and a number of PRBs on the CRB grid. - The FDRA field of the DCI should be dimensioned on the basis of the common frequency resource for the MBS group-common PDSCH reception. - Define a new RNTI, namely SC-RNTI for scrambling the CRC of DCI scheduling a PDSCH mapped to the MCCH containing multicast configuration information. Define a new RNTI, namely the SC-N-RNTI for scrambling the CRC of the DCI notifying a change in the multicast configuration. Define a new group RNTI, namely G-RNTI for scrambling the CRC of DCI scheduling a PDSCH carrying the multicast data corresponding to MTCH. - For groupcast, search space configuration for monitoring DCI scheduling multicast PDSCH can have the following options: - Re-use NR Type 3 CSS configuration while additionally supporting monitoring of DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI - Alternately, define new NR CSS Type 4 for monitoring multicast DCI with CRC scrambled by SC-RNTI, SC-N-RNTI and G-RNTI - For RRC_CONNECTED UEs groupcast PDCCH can also be monitored in USS - NR MBS uses PDSCH Mapping Type A with DM-RS Type 1 as a baseline. PDSCH Mapping Type B and use of Type 2 DM-RS are not precluded. - RRC_CONNECTED UEs may be able to receive both multicast and unicast transmissions within a slot in a TDM manner. Simultaneous reception of unicast and multicast transmissions by a UE on orthogonal DM-RS ports is not precluded. - DCI Format for scheduling NR MBS transmissions: - As a baseline DCI format 1_0 (fallback DCI) can be re-used - If needed, a compact DCI format for multicast scheduling can be defined - The group-common DCI format for MBS transmission is included in the scheduling DCI size budget of 3 for UEs and UEs can perform size alignment for other DCI formats if MBS DCI size exceeds other scheduling DCI in its active BWP. - DL semi-persistent scheduling can be used for NR multicast PDSCH transmission - For NR MBS support of multi-layer MIMO transmission with rank adaptation is not precluded - For groupcast transmission, all UEs within the group share the same DM-RS port(s). Additionally, multiplexing UEs receiving unicast transmission on remaining orthogonal DM-RS ports is not precluded - Advanced transmission schemes like multiuser superposition transmission (MUST) for improving group spectral efficiency are not precluded |
| [20] R1-2009055 | Asia Pacific Telecom co. Ltd | Observation 1: UE-specific feedback mechanism cannot be easily supported if a group-common PDCCH is used to schedule a group-common PDSCH. Nevertheless, having UE-specific feedback mechanism is beneficial because the network can fully understand the channel condition of each UE. Observation 2: In the case where UE-specific PDCCH schedules a group-common PDSCH, the feedback mechanism of PTP transmission, i.e., UE specific feedback, could be easily adopted. Proposal 1: Using UE-specific PDCCH to schedule group-common PDSCH could be supported by NR MBS for RRC_CONNECTED UEs. |
| [21] R1-2009165 | Convida Wireless | Proposal 1: UE-specific PDCCH can be supported to schedule the PDSCH for MBS in addition to the group-common PDCCH for RRC_CONNECTED UEs in NR MBS. Proposal 2: Dedicated MBS BWP should be supported for RRC_CONNECTED UEs in NR MBS. |
| [22] R1-2009238 | BBC | Observation 1: Orthogonal TDM and/or FDM multiplexing in the time-frequency OFDMA grid for simultaneous operation of broadcast/multicast and unicast services, as considered for NR MBS, is a suboptimal transmission approach in terms of maximum sum-rate. Observation 2: A two-layered broadcast/multicast and unicast superposition transmission (BMUST) is optimal in terms of the maximum sum-rate supported. Hence, BMUST can be more spectral efficient than orthogonal TDM/FDM multiplexing. Observation 3: System level simulations confirm that BMUST provides the best average spectral efficiency performance against other orthogonal multiplexing schemes such as FDM, unicast-only and multicast-only for any number of UEs. Observation 4: With 20 UEs, the average spectral efficiency gain of BMUST against FDM is 40% and 45% in the simulated rural and urban environments, respectively. Observation 5: While multicast-only provides the best 5th percentile UE spectral efficiency in the considered scenarios, BMUST provides similar performance to multicast-only, especially as the number of UEs increases. Observation 6: In the case of simultaneous operation of broadcast/multicast and unicast services in a slot, both information streams contribute to the UE’s data rate and both information streams would need to be decoded regardless of the multiplexing scheme, i.e., FDM, TDM or BMUST. Observation 7: For BMUST, the receiver has the additional complexity for the operations required to reencode and subtract the broadcast/multicast stream from the main received signal. Observation 8: The link level results with realistic channel estimation and imperfect interference cancellation show that BMUST can provide significant gains in terms of increased spectral efficiency against orthogonal multiplexing. Recommendation: Due consideration should be given to the potential support of Broadcast/Multicast and Unicast Superposition Transmission in NR based on UE capability to improve the system’s spectral efficiency. |
| [23] R1-2009274 | Qualcomm Incorporated | Observation 1: Most of the parameters related to PDCCH/PDSCH reception are configured per BWP. Reusing the BPW signalling to define the common frequency resource for MBS allows for flexible configuration for GC-PDCCH and GC-PDSCH. Proposal 1: For RRC_CONNECTED UEs, one or more Multicast BWPs can be configured per UE for GC-PDCCH/GC-PDSCH reception. - A Multicast BWP is configured to be associated with a dedicated DL BWP if it is fully contained in the dedicated DL BWP and using the same numerology. - UE can monitor a Multicast BWP if its associated dedicated DL BWP is active. Proposal 3: For RRC_CONNECTED UEs, CSS and/or USS for GC-PDCCH can be configured per Multicast BWP. - Reuse legacy priority rules for mapping CSS and USS sets for GC-PDCCH. Proposal 4: For RRC_CONNECTED UEs, at least DCI format 1_0 and 1_1 can be used for GC-PDCCH. - FFS DCI size alignment for GC-PDCCH and unicast PDCCH with same DCI format. Proposal 5: For RRC_CONNECTED UEs, consider the GC-PDCCH monitoring for multicast as a virtual CC to count the number of BDs/CCEs. Proposal 6: For RRC_CONNECTED UEs, common parameters for GC-PDSCH are configured per Multicast BWP. Proposal 7: For RRC_CONNECTED UEs, support dynamic GC-PDSCH and SPS GC-PDSCH configuration per Multicast BWP. - GC-PDCCH is used for SPS GC-PDSCH activation/deactivation. - FFS: more than one SPS GC-PDSCH. Proposal 8: UE-specific PDCCH can be used to schedule PDSCH for multicast retransmission. - UE-specific PDCCH schedules multicast retransmission with HARQ process ID associated with that of GC-PDSCH scheduled by GC-PDCCH. Proposal 9: Consider separate UE capabilities for FDM only, TDM only, or FDM+TDM for unicast PDSCH and GC-PDSCH in a slot. Proposal 10: Further discuss the potential RAN1 impact related with the configuration of G-RNTI(s) and the interaction between G-RNTI and C-RNTI for PDSCH reception, including: - Aspects related to simultaneous reception of G-RNTI(s) and C-RNTI - Aspects related to simultaneous reception of multiple G-RNTIs. - Aspects related to retransmission of packets between G-RNTI(s) and C-RNTI. Proposal 11: For RRC_CONNECTED UEs, the configuration for NR multicast reception is received by unicast RRC signalling. |
| [24] R1-2009305 | Ericsson | Observation 1 The existing BWP framework can be used, i.e. with no specification impact, for the reception of MBS and unicast. Observation 2 BWP switching for MBS UEs while in RRC Connected will require RRC reconfiguration of all MBS UEs at the same time, which is costly from the point of view of radio resources and latency. Observation 3 Fall back to default/initial BWP mechanisms can be used to switch to the MBS bandwidth, at the condition on the no activity on all the active BWPs for the MBS UEs. - Limit scheduling of G-RNTI based PDCCH to G-RNTI based PDSCH, i.e., i.e. not support C-RNTI-based PDCCH for such scheduling in Rel.17. - The existing framework for BWP management is sufficient for ensuring all UEs in MBS share the same BWP during common PDSCH transmission. The common BWP is either an MBS specific bandwidth part or the initial bandwidth part. Transmission over a common frequency resource within each UE dedicated BWP is not supported. - For RRC_CONNECTED UEs, support inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots as a core MBS functionality. - For RRC_CONNECTED UEs, at least support intra-slot TDM of unicast PDSCH and group-common PDSCH in a slot subject to UE capability. - For RRC_CONNECTED UEs, SDM of unicast PDSCH and group-common PDSCH is not pursued in rel17. |