3GPP TSG RAN WG1 #103-e			R1-2008941

e-Meeting, October 26th – November 13th, 2020

Source:	NEC

Title:	UE feedback enhancements for HARQ-ACK

Agenda Item:	8.3.1.1

Document for: 	Discussion and Decision

- Introduction

In RAN1 #102- e meeting, following agreements related to HARQ-ACK enhancement for Rel-17 URLLC/IIoT were achieved [1]:

Agreements: Support Rel-17 enhancements to avoid SPS HARQ-ACK dropping for TDD due to PUCCH collision with at least one DL or flexible symbol. - This topic is to be considered as high priority - FFS detailed solution(s) Agreements: - Simultaneous PUSCH / PUCCH within a cell group (of Sec. 6.13 of R1-2007216) and enhanced (sub-slot) HARQ-ACK multiplexing on PUSCH (of Sec. 4.3 of R1-2007216) can be further discussed as part of AI 8.3.3 in this WI (but not as part of AI 8.3.1.1). Agreements: Study further at least the following schemes: - SPS HARQ skipping for ‘skipped’ SPS PDSCH - PUCCH repetition enhancements (at least for HARQ-ACK), e.g., sub-slot based, etc. - Retransmission of cancelled HARQ - SPS HARQ payload size reduction and / or skipping for ‘non-skipped’SPS PDSCH - Type 1 HARQ codebook based on sub-slot PUCCH config - PUCCH carrier switching for HARQ feedback Companies are encouraged to provide detailed analysis and comparison accordingly.

In this contribution, we firstly discuss the enhancement for Type-1 HARQ codebook based on sub-slot PUCCH configuration. Then we discuss the enhancement for HARQ-ACK feedback for SPS PDSCH.

- Enhancement on Type-1 HARQ-ACK codebook based on sub-slot PUCCH configuration

For a Rel.16 URLLC UE, sub-slot based HARQ-ACK feedback is supported to reduce the latency. Regarding the HARQ-ACK codebook construction, Type-2 HARQ-ACK codebook for sub-slot-based HARQ-ACK feedback procedure has been supported In Rel-16. For Type-1 HARQ-ACK codebook, it was agreed in last meeting to discuss whether and how to support Type-1 HARQ-ACK codebook based on sub-slot PUCCH configuration in Rel-17. In our view, compared with Type-2 HARQ-ACK codebook, Type-1 HARQ-ACK codebook can provide robustness against missed DCI transmissions, which is benefit for high reliability URLLC services. Some companies may have the concerns on the redundancy of the semi-static HARQ-ACK codebook, which will lead to high UCI overhead. However, the redundancy of URLLC Type-1 HARQ-ACK codebook can be controlled by network with suitable configuration for TDRA table, HARQ-ACK timing K1 set. Therefore, Tyep-1 HARQ-ACK codebook based on sub-slot PUCCH configuration should be supported in Rel-17.

In order to further reduce the HARQ-ACK codebook size, small enhancements on Type-1 HARQ-ACK codebook construction based on sub-slot PUCCH configuration can be considered. One simple option for sub-slot based Tyep-1 HARQ-ACK codebook determination is as following:

- Step 1: Determine the HARQ-ACK multiplexing window based on the HARQ-ACK timing set and sub-slot length.

Fig.1 Example for step 1 with K1={1,2,3} and 7-symbol sub-slot length

- Step 2: Split the TDRA table into N sub-tables based on the sub-slot length and PDSCH-to-UL sub-slot association. N is the number of sub-slot within a slot. Which means that the SLIV in the TDRA table is associated with which sub-table depends on the ending symbol of the SLIV is located in which sub-slot. E.g., the first sub-TDRA table is corresponding to the first sub-slot within a slot, the second sub-TDRA table is corresponding to the second sub-slot within a slot.

Fig.2 Example for TDRA table splitting for 7-symbol sub-slot

- Step 3: Do pruning based on TDD configuration and sub-table per sub-slot similar as Rel-15.  E.g., if the sub-slot for HARQ-ACK timing set K1= {1, 2, 3} in the above given example are all DL symbols, the set of PDSCH occasions M is {0, 1, 2, 3, 4}.

Proposal 1:

- Semi-static HARQ-ACK codebook for sub-slot based HARQ-ACK feedback procedure should be supported.

- The sub-slot based semi-static HARQ-ACK codebook can be determined based on following three-steps:

  - Step 1: Determine the HARQ-ACK multiplexing window based on the HARQ-ACK timing set and sub-slot length.

  - Step 2: Split the TDRA table into N sub-tables based on the sub-slot length and PDSCH-to UL sub-slot association. N is the number of sub-slot within a slot.

  - Step 3: Do pruning based on TDD configuration and sub-table per sub-slot similar as Rel-15.

- Enhancement on HARQ-ACK feedback for SPS PDSCH

3.1 Support avoiding SPS HARQ-ACK dropping for TDD

In Rel-16, the HARQ-ACK timing value k for all of the SPS PDSCHs of a SPS configuration are same and indicated by the activation DCI. When a SPS PDSCH is received in slot n, the PUCCH resource for HARQ-ACK for the SPS PDSCH will be transmitted in slot n+k. If the PUCCH resource in slot n+k collides with DL symbol or flexible symbol for a given TDD configuration, UE will cancel the PUCCH transmission for HARQ-ACK for SPS PDSCH. Since short SPS periodicity for SPS configuration was introduced in Rel-16 for URLLC services, the collision may happen frequently, which will lead to unnecessary SPS PDSCH retransmission and degrade the performance of SPS PDSCH transmission. In last meeting, it was agreed to support avoiding the SPS HARQ-ACK dropping for unavailable PUCCH resource, while the detail solutions need to be further studied [2]. In our view, it can be down-selected from following three options:

- Option 1: Deferring HARQ-ACK until the first available valid PUCCH resource. The first available PUCCH resource refers to the earliest available PUCCH resource among PUCCH resources indicated for transmission of HARQ-ACK for a DG PDSCH and configured for transmission of HARQ-ACK for a SPS PDSCH, as shown in Fig.1.

- When the delayed HARQ-ACK is multiplexed with other HARQ-ACK indicated in the first available PUCCH resource, the HARQ-ACK codebook construction need to be studied.

- In addition, some companies may have the concern that option 1 may incur imbalanced HARQ-ACK feedback. There are some simple solutions. For example, configuring a value M for UE controls the maximum number of HARQ-ACK bits for a PUCCH resource.  If the number of HARQ-ACK bits carried in a PUCCH resource exceeds the value M, then the PUCCH resource becomes invalid for the delayed HARQ-ACK, UE will find next available PUCCH resource. An example is given in the Fig.2 below, the k1 value for the SPS PDSCH is 2, the M value for the PUCCH is 4, then the first 4-bits HARQ-ACK and the second 4-bits HARQ-ACK are separately transmitted in the PUCCH resource in slot n+8 and the PUCCH resource in slot n+9.

Fig.1 Example for option 1, SPS periodicity=1 slot

Fig.2 Example for option 1 configured with M

- Option 2: A set of HARQ-ACK timing values are configured for a UE by RRC. For each SPS PDSCH reception, UE selects the first applicable k1 value from the set to find an available PUCCH resource for HARQ-ACK for the SPS PDSCH.

- Option 3:  gNB dynamically indicates the retransmission of the delayed HARQ-ACK for SPS PDSCH e.g., enhanced one-shot HARQ-ACK feedback.

- In Rel-16, when one-shot HARQ-ACK request is received by UE, a Type-3 HARQ-ACK codebook containing the all DL HARQ processes for all CCs configured for a UE in the PUCCH group will be generated. If directly reuse the Rel-16 one-shot HARQ-ACK feedback mechanism for SPS PDSCH, it will cause large redundancy. Some simple enhancements can be considered, e.g., gNB can trigger/request the HARQ-ACK feedback for the SPS PDSCH only in an available PUCCH resource indicated by a DL DCI format with CRC scrambled by CS-RNTI. The requested HARQ-ACK codebook containing the number of DL HARQ processes for the indicated SPS configuration(s) in the serving CC(s). Which SPS configuration is indicated by reusing HPN field in the request DCI.

All of the above options can work. Option 1 is simple and does not need additional signaling. Option 2 can provide more flexibility at the cost of RRC overhead. For Option 3, it increases the DCI overhead but provides flexibility for gNB to control whether to transmit the delayed SPS HARQ-ACK or not. Since it is not clear whether such flexibility is needed, the simple option 1 is slightly preferred.

Proposal 2:

- In case PUCCH resource for SPS HARQ-ACK is not available due to collision with DL symbol or flexible symbol, the HARQ-ACK for SPS PDSCH will be delayed to next available PUCCH resource.

  - If the number of HARQ-ACK bits carried in a PUCCH resource exceeds the value M, then the PUCCH resource is not valid for the delayed HARQ-ACK.

  - FFS the HARQ-ACK codebook construction including the delayed HARQ-ACK.

3.2 SPS HARQ skipping

NACK skipping for skipped SPS PDSCH

In Rel-16, multiple SPS configurations with short SPS periodicity in a cell is supported to reduce the latency. For TSC/TSN traffic with non-integer periodicities, multiple SPS configurations maybe configured for one service, there are some SPS PDSCH occasions may be skipped and not used for DL data transmission. For such case, in order to avoid the unnecessary PUCCH transmission for skipped SPS PDSCH, skipping NACK for skipped SPS PDSCH was agreed to be further studied in RAN1 #102-e meeting. There are two solution can be considered as following:

- Option 1: UE does not need to identify whether the SPS PDSCH is skipped SPS PDSCH (without DL data) or failed to be decoded. UE only skips PUCCH transmission when all HARQ-ACK bits multiplexed in the PUCCH for SPS PDSCH only are NACK.

- Option 2: UE is able to identify whether the SPS PDSCH is skipped SPS PDSCH (without DL data) or failed to be decoded. UE skips NACK for skipped SPS PDSCH in the PUCCH resource regardless of the HARQ-ACK codebook containing HARQ-ACK for SPS PDSCH only or multiplexed HARQ-ACK for skipped SPS PDSCH and DG PDSCH.

Regarding which option should be applied, there are three cases need to be discussed:

- Case 1: Only HARQ-ACK for skipped SPS PDSCH(s) are mapped on the PUCCH resource.

- Option 1 is sufficient, it can reduce the PUCCH transmission and save UE power, and gNB will not receive the corresponding PUCCH.

- Case 2: HARQ-ACK for skipped SPS PDSCH(s) are multiplexed with HARQ-ACK for the non-skipped SPS PDSCH(s) only on the PUCCH resource.

- For option 1, since gNB knows whether DL data is transmitted in the SPS PDSCH occasion or not, so UE can skip PUCCH transmission when all HARQ-ACK bits multiplexed in the PUCCH are NACK, it is simple and does not need much specification work.

- For option 2, UE can skip the NACK for skipped SPS PDSCH and transmit PUCCH for HARQ-ACK for non-skipped PDSCH only, it can reduce the UCI payload.

- Case 3: The HARQ-ACK for skipped SPS PDSCH(s) are multiplexed with HARQ-ACK for the dynamic scheduled PDSCH on the PUCCH resource.

- For option 2, when Tyepe-2 HARQ-ACK codebook is configured, it may can reduce the UCI payload to support NACK skipping for skipped SPS PDSCH, but it relies on the reliability of skipped SPS PDSCH detection to ensure the same understanding between gNB and UE on the HARQ-ACK codebook size.

Fig.3 Example for case 1

Fig.4 Example for case 2

Fig.5 Example for case 3

Based on above analysis, the benefits of option 2 for NACK skipping for case 2 and case 3 relies on the reliable skipped SPS PDSCH detection. The URLLC performance will be degraded if the high accuracy of skipped SPS PDSCH detection cannot be guaranteed.  Therefore, for simplicity and less specification impact, option 1 is slightly preferred. If the evaluation results provided for the skipped SPS PDSCH detection, e.g., based on DMRS presence detection, can prove its high accuracy, the option 2 is acceptable.

Proposal 3:

- Support skipping PUCCH transmission if all HARQ-ACK bits in the PUCCH resource for SPS PDSCH only are NACK.

ACK skipping for non-skipped SPS PDSCH

In last meeting, it was agreed to further study whether support HARQ skipping for non-skipped SPS PDSCH. In order to reduce the latency, a URLLC UE may be configured with one or more SPS configuration with short periodicity. Some companies think that the reliability for the SPS PDSCH transmission is very high, so that ACK for SPS PDSCH is always need to be transmitted, which is inefficient. Then ACK skipping for SPS PDSCH should be considered. However, it can only be used for the scenario that PUCCH carried HARQ-ACK for SPS PDSCH only and the PUCCH resource is always available. When the HARQ-ACK for SPS PDSCH is multiplexed with HARQ-ACK for dynamic scheduled PDSCH in a PUCCH, the ACK skipping is not applicable. In addition, it may impact the intra-UE multiplexing behaviour. For example, when a PUCCH for high priority HARQ-ACK for SPS PDSCH and another PUCCH for low priority SR are overlapped in time domain, if UE successfully decodes the SPS PDSCH, it is not clear whether UE only transmits PUCCH for ACK for SPS PDSCH and drops PUCCH for SR, or skips PUCCH transmission for ACK and transmits PUCCH for SR, or neither transmits PUCCH for ACK nor transmits PUCCH for SR. Therefore, it is not necessary to support ACK skipping for SPS PDSCH since the benefits is limited.

Proposal 4:

- Skipping PUCCH transmission for ACK for SPS PDSCH is not supported for URLLC in Rel-17.

- Enhancement on sub-slot based Type-1 HARQ-ACK codebook by the addition of DAI counters

An enhancement to reduce the size of Type-1 HARQ-ACK codebook is proposed as following:

The sub-slot based Type-1 HARQ-ACK codebook ACK/NACK bits will only be present if the corresponding slot or sub-slot has at least one PDCCH transmission, and the reliability of Type-1 HARQ-ACK codebook for URLLC service can be enhanced by the addition of DAI counters.

In the following example, a Type-1 HARQ-ACK codebook has a window size of 4 slots in total. It is also configured to have 7 sub-slots per slot, with 1 PDCCH monitoring occasion for each sub-slot.

The Rel-16 Type-1 HARQ-ACK codebook will be composed of these bits:

ACK/NACK bits = 1000100 0000000 0000000 1000000

With the proposed enhancement, the type 1 slot-based HARQ-ACK codebook will feedback as following:

ACK/NACK bits = 1000100 1000000

The number of bits for type 1 sub-slot based HARQ-ACK codebook can be further reduced:

ACK/NACK bits = 11 1

For the addition of C-DAI counters, the existing C-DAI field can be simply reused:

slot 0	(C-DAI=0, C-DAI=1)

slot 3	(C-DAI=2)

To further improve reliability against missed detection, the existing T-DAI field, which is not currently present for URLLC cases, may be redefined:

New Definition of T-DAI for URLLC

The value of the T-DAI, when configured as ‘present’, in DCI format 1_1 (or DCI format 1_2) denotes the total number of slots in which PDCCH/PDSCH transmission(s) associated with DCI format 1_1 (or DCI format 1_2) occur(s), up to the current slot, in increasing order of slot index.

Let x be the number of PDCCH monitoring occasions per slot or per sub-slot. x bits feedback is always present for each value pair of C-DAI/T-DAI (for URLLC) in the enhanced type 1 codebook.

With the proposed enhancement, the following C-DAI/T-DAI settings will be applied in DCI with corresponding HARQ-ACK feedback (x=7 in this case):

ACK/NACK bits = 1000100 1000000

slot 0	(C-DAI=0, T-DAI=0; C-DAI=1, T-DAI=0)

slot 3	(C-DAI=2, T-DAI=1)

| URLLC DCI for DL assignment | Rel-15/Rel-16 DCI | Rel-17 DCI | Comment |
| --- | --- | --- | --- |
| Total Downlink Assignment Index | 0 or 2 | 0 or 2 | 0 or 2 bits can be configured for this field. In Rel-17, 0 bit if it is codebook-less; in Rel-15/Rel-16, 0 bit if there is no CA. |

Table 1: T-DAI field is enhanced for URLLC

Proposal 5:

- The Type-1 HARQ-ACK codebook ACK/NACK bits will only be present if the corresponding slot or sub-slot has at least one PDCCH transmission, and the reliability of Type-1 HARQ-ACK codebook for URLLC service can be enhanced by the addition of DAI counters.

- Conclusion

In this contribution, we discussed the possible enhancements for HARQ-ACK feedback for URLLC. Proposals are summarized as following:

Proposal 1:

- Semi-static HARQ-ACK codebook for sub-slot based HARQ-ACK feedback procedure should be supported.

- The sub-slot based semi-static HARQ-ACK codebook can be determined based on following three-steps:

  - Step 1: Determine the HARQ-ACK multiplexing window based on the HARQ-ACK timing set and sub-slot length.

  - Step 2: Split the TDRA table into N sub-tables based on the sub-slot length and PDSCH-to UL sub-slot association. N is the number of sub-slot within a slot.

  - Step 3: Do pruning based on TDD configuration and sub-table per sub-slot similar as Rel-15.

Proposal 2:

- In case PUCCH resource for SPS HARQ-ACK is not available due to collision with DL symbol or flexible symbol, the HARQ-ACK for SPS PDSCH will be delayed to next available PUCCH resource.

  - If the number of HARQ-ACK bits carried in a PUCCH resource exceeds the value M, then the PUCCH resource is not valid for the delayed HARQ-ACK.

  - FFS the HARQ-ACK codebook construction including the delayed HARQ-ACK.

Proposal 3:

- Support skipping PUCCH transmission if all HARQ-ACK bits in the PUCCH resource for SPS PDSCH only are NACK.

Proposal 4:

- Skipping PUCCH transmission for ACK for SPS PDSCH is not supported for URLLC in Rel-17.

Proposal 5:

- The Type-1 HARQ-ACK codebook ACK/NACK bits will only be present if the corresponding slot or sub-slot has at least one PDCCH transmission, and the reliability of Type-1 HARQ-ACK codebook for URLLC service can be enhanced by the addition of DAI counters.

References

- 3GPP RAN1 NR #103-e meeting, chairman’s notes.

- R1-2007354, ‘Feature lead summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT (AI 8.3.1.1),’ Nokia, Nokia Shanghai Bell.