3GPP TSG RAN WG1 #103-e												  R1-2008945

e-Meeting, October 26th – November 13th, 2020

Agenda item:	8.1.2.2

Source:	NEC

Title:               Discussion on multi-TRP inter-cell operation

Document for:	Discussion and Decision

- Introduction

In RAN1#102e meeting [1], agreements on multi-TRP inter-cell operation was achieved as:

Agreement

Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation.

- Details on configuration of non-serving cell RS;

- Allowed source and target RS types for RS transmitted from the non-serving cell TRP ;

- Allowed QCL types for RS transmitted from the non-serving cell TRP ;

- Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any;

- Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels;

- Other details not precluded.

In this contribution, we provided our views on inter-cell operation for multi-TRP.

- Discussion

To enable inter-cell multi-DCI based multi-TRP operation, source RS transmitted from non-serving cell TRP should be allowed, and SSB is one of key source RS, which should also be supported in case of inter-cell multi-TRP operation.

The SSB from non-serving cell is not needed to be identified by initial access procedure. The configuration of SSB from non-serving cell can be directly indicated to the UE, for example, at least the PCI of the SSB.

In additional, for different cells, to achieve more flexibility, the time and frequency resource for SSBs may be different, which may also be needed to be indicated to the UE.

And multiple SSBs from non-serving cell may be configured for UE to measure, and select one or more SSBs as source RS for QCL. So the SSB related measurement should be enhanced, for example, to include a sequence of non-serving cell SSB resource lists in the CSI-SSB-ResourceSet.

Proposal: SSB from non-serving cell should be supported for source RS, and PCI, time/frequency resource of the SSB should be configured to UE.

- Conclusion

In this contribution, we provided our views on inter-cell operation for multi-TRP, and we proposed that:

Proposal: SSB from non-serving cell should be supported for source RS, and PCI, time/frequency resource of the SSB should be configured to UE.

- References

- Chairman’s notes, RAN1#102-e, August 17th – 28th, 2020.