3GPP TSG RAN WG1 #103-e		R1-2008954

e-Meeting, October 26th – November 13th, 2020

Source:	Panasonic

Title: 	Enhancements for unlicensed band URLLC/IIoT

Agenda Item:		8.3.2

Document for:	Discussion

Introduction

A work item on enhanced IIoT and URLLC was approved [1]. One of objectives of this work item is uplink enhancements for URLLC in unlicensed controlled environments such as

- Specify support for UE-initiated COT for FBE with minimum specification effort

- Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel.16 to be applicable for unlicensed spectrum

This document provides our view on potential enhancements related to unlicensed band URLLC/IIoT.

Support of UE-initiated COT for FBE

Starting offsets for UL configured grant

In Rel.16, for operation with unlicensed band, when a UE is performing uplink transmission with CGs in contiguous OFDM symbols on all resource blocks of an RB set, for the first such UL transmission, the UE determines a duration of a cyclic prefix extension where the value of cyclic prefix extension (i.e., starting offset) is chosen randomly from a set of values configure by higher layers. On the other hand, when a UE is performing uplink transmission with CGs in contiguous OFDM symbols on fewer than all resource blocks of an RB set, for the first such transmission, the UE determines a duration of a cyclic prefix extension to fixed value indicated by higher layers. In RAN#102e, it was agreed that for operation with semi-static channel access, the Rel.16 random starting offsets for UL CG with full bandwidth allocation when UE initiates a COT, is not supported. The remaining discussion point would be whether/how to support the Rel.16 fixed (configured) starting offsets for UL CG with resource allocation fewer than full bandwidth when UE initiates a COT.

In our view to reuse the multiple starting time offset for CG would be useful for the prioritization among devices especially between gNB and UEs. Figure 1 shows an example to reuse multiple starting offset for CG in FBE. FFP as OFDM symbol level is common among gNB and UEs. The location of CP extension can be immediately before the beginning of FFP (or immediately after the beginning of FFP). The amount of CP extension for each UE which can initiate COT is configured by CG configuration. Anyone (gNB or UEs) who transmits faster can initiate COT based on the amount of CP extension. Since LBT is necessary to initiate COT, collision can be avoided depending on the different amounts of CP extension for each UE.

According to the above mechanism, the prioritization among UEs can be configured by the amount of CP extension. If a high priority UE has data, the high priority UE can initiate COT. If a high priority UE has no data and a low priority UE has data, the low priority UE can initiate COT. gNB can also initiate COT if gNB transmit faster than UEs. Even if no UE initiates COT due to no data, if at least gNB initiates COT, the FFP can be utilized. Therefore, by reusing the configured starting time offset for CG, UE-initiated COT can be supported.

Observation 1: Multiple starting time offset for configured grant, which is configured as the amount of CP extension, can be reused to support UE-initiated COT.

Figure 1: Reusing multiple starting time offset for CG

If the multiple starting time offset for configured grant is reused for UE-initiated COT, whether/how to support UE-initiated COT by DG PUSCH needs to be considered. For the case where gNB knows that UE still has UL data at the end of the FFP and the UL data should be transmitted as soon as possible, UE-initiated COT by DG PUSCH would also be useful to reduce delay across idle period. If UE-initiated COT by DG PUSCH is supported together with CG PUSCH, one potential issue would be that CP extension for multiple starting offset as in CG PUSCH is not supported for DG PUSCH (CP extension for DG PUSCH is used to adjust gap between transmissions for LBT). Then, even if gNB wants to prioritize DG PUSCH over CG PUSCH (of gNB wants to priority CG PUSCH over DG PUSCH), gNB might not control the prioritization of transmission because CP extension for DG PUSCH does not have sufficient granularity (i.e., 9 us granularity). Therefore, we think also to support multiple starting time offset as in CG PUSCH for DG PUSCH is beneficial.

Although we think to support multiple starting time offset as in CG PUSCH for DG PUSCH is beneficial, if the amount of CP is different between CG PUSCH and DG PUSCH, and/or the amount of CP is dynamically changed, it may not meet the requirement that “an equipment may change its Fixed Frame Period but it shall not do more than once every 200 ms”. Then, it should be clarified that whether the difference of CP extension is called as the change of FFP or not. If the change of CP extension is not called as the change of FFP, the amount of CP extension for a UE can be different between CG PUSCH and DG PUSCH and it could be dynamically changed by dynamic grant. On the other hand, if the change of CP extension is called as the change of FFP, the start of FFP might be always CG PUSCH if DG PUSCH does not have CP extension. If DG PUSCH has CP extension, the amount of CP extension for DG PUSCH would be same as that configured to CG PUSCH.

Observation 2: It should be clarified that whether the difference of CP extension is called as the change of FFP or not.

Observation 3: If DG PUSCH is used for UE-initiated COT together with CG PUSCH, to support CP extension for multiple starting time offset as in CG PUSCH for DG PUSCH could be considered.

Observation 4: If the difference of CP extension is called as the change of FFP, the start of FFP might be always CG PUSCH if DG PUSCH does not have CP extension. If DG PUSCH supports CP extension, the amount of CP extension for DG PUSCH should be same as that configured to CG PUSCH.

How to handle idle period when gNB/UE initiates COT or UE/gNB shares COT

In RAN1#102e, following agreement were made.

Agreement:

- For semi-static channel access mode

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT.

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT.

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB.

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that FFP in which the gNB shares the COT initiated by the UE.

  - FFS: Whether/how to support additional restrictions to the idle period

This agreement clarifies how to handle idle period when gNB or UE initiates COT or UE or gNB shares COT. Whether/how to support additional restrictions to the idle period is FFS. Figure 2 illustrates the possible DL/UL transmissions with gNB and UEs as COT initiator. In Fig.2, FFP associated with the gNB starts from 1st portion and its duration is 4 portions. FFP associated with the UE2 has offset from FFP associated with the gNB and its duration is same as FFP associated with the gNB. In our interpretation of the above agreement, FFS point would be whether UE can transmit the idle period of FFP associated with gNB even if gNB does not obtain the gNB initiated COT. For example, in Fig.2, the idle period of FFP associated with UE2 would be 2nd portion and 6th portion (i.e., the last portion of UE2’s FFP). If gNB does not obtain gNB-initiated COT in 1st FFP (i.e., from 1st to 4th portion), UE2 may obtain UE-initiated COT at 3rd portion. In this case, whether UE2 is allowed or not allowed to transmit at 4th portion, which corresponds to idle period of FFP associated with the gNB, is unclear. In our view, at least the restriction that UE should not transmit in the serving gNB’s idle period would be needed from the perspective of coexistence with other RAT.

Proposal 1: The UE is not allowed to transmit during the idle period of any FFP associated with the serving gNB (regardless whether the serving gNB initiates a COT in that FFP).

Figure 2: Possible DL/UL transmissions with gNB and UEs as COT initiator

FFP parameters for UE-initiated COT

In RAN1#102e, it was agreed that for semi-static channel access mode, start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT. The remaining discussion point is whether FFP periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT. Our preference is separate configuration of gNB’s and UE’s FFP is allowed to support more flexible use cases. The potential concern to support different periodicity between gNB and UE would be increase of scheduling complexity, but it could be handled by gNB implementation.

Proposal 2: FFP periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Indication of FFP parameters

In RAN1#102e, it was agreed that for semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC CONNECTED mode. On the other hand, FFS for the case when the UE is IDLE/INACTIVE mode. In our view, to support RACH transmission to initiate a COT by UE in RRC IDLE/INACTIVE mode is useful, however, it might not be essential enhancement since IDLE UE behaviour is not a URLLC-related enhancement. It can have some more discussion in RAN2 and then, the decision could be up to RAN2.

On the indication of FFP parameter, at least dedicated RRC signalling was agreed. Whether SIB indication is supported or not depends on the conclusion on whether UE-initiated COT is supported in IDLE/INACTIVE mode.

Observation 5: The decision on whether to support RACH transmission to initiate a COT by UE in RRC IDLE/INACTIVE mode could be up to RAN2.

Observation 6: If UE-initiated COT is supported in IDLE/INACTIVE mode, FFP parameters for UE-initiated COT should be provided to the UE by SIB.

To initiate a COT in a next FFP associated to the UE

In RAN1#102e, following was proposed [2].

Proposal:

- For semi-static channel access

  - FFS: A UE can be explicitly indicated by DCI whether or not to initiate a COT in a next FFP associated to the UE.

    - FFS details of signalling (e.g., interpretation of DCI fields)

    - FFS on common-DCI

In our view, explicit indication to UE to initiate a COT should be supported. Explicit indication gives more flexibility and more efficient for gNB to control the channel change.

Proposal 3: A UE can be explicitly indicated by DCI whether or not to initiate a COT in a next FFP associated to the UE.

To indicate UE-initiated COT information to gNB

In RAN1#102e, following was proposed [2].

Proposal:

- A UE can indicate to the gNB that it has initiated a COT in an FFP.

In our view, it is sufficient that the gNB just needs to detect the UL transmissions which are configured/scheduled by the gNB. Then, the above proposal is not necessary.

Proposal 4: No explicit indication to indicate to the gNB that it has initiated a COT in an FFP is necessary.

Harmonizing UL configured-grant enhancements in NR-U and URLLC

In RAN1#102e, following options were identified [2].

- Option 1: Operation based on only URLLC CG features

- Option 2: Operation based on either URLLC CG features or NR-U CG features (network configurability between two independent operational modes)

- Option 3: Operation based on combined URLLC and NR-U CG features.

In Rel.16 specification, cg-RetransmissionTimer is always configured for unlicensed band operation. In RAN1#102e, it was agreed that at least cg-RetransmissionTimer is not mandated in unlicensed band operation. Option 2 seems simpler. One of operations would be if the higher layer parameter cg-RetransmissionTimer is provided, NR-U CG is adopted, otherwise URLLC CG is used.

In our view, Option 3 should also be considered. One of motivation for harmonizing CG enhancement in NR-U and URLLC is to support the same CG operation as licensed band (e.g., CG-UCI/DFI less transmission). The reason why CG-UCI/DFI should be disabled is to reduce unnecessary signalling overhead and retransmissions. For the functions related to autonomous transmission on CG (e.g., the CG retransmission timer, DFI, indication of NDI and RV on CG-UCI), when miss-detection probability at gNB is low (e.g., by configuring low MCS and dense DMRS) and/or the environment where LBT failure probability is low, these functions should be disabled. For the function related to COT sharing (i.e., COT sharing information on CG-UCI, the merit is not related to miss-detection and LBT failure probability. COT sharing can be used to reduce LBT delay by using Cat.1 or Cat.2 LBT instead of Cat.4 LBT. If whole CG-UCI field is disabled (CG-UCI is not transmitted on CG PUSCH), delay can be increased because of the lack of COT sharing information. Then, at least COT sharing information should be indicated by CG-UCI in some cases.

Proposal 5: When configured grant Type 1 or Type 2 are configured on unlicensed spectrum, it would be useful that at least functions related to autonomous retransmission on CG and COT sharing are separately enabled/disabled by the configuration.

Conclusion

In this contribution, we provide our view on potential enhancements related to unlicensed band URLLC/IIoT. We made following observations and proposals.

Observation 1: Multiple starting time offset for configured grant, which is configured as the amount of CP extension, can be reused to support UE-initiated COT.

Observation 2: It should be clarified that whether the difference of CP extension is called as the change of FFP or not.

Observation 3: If DG PUSCH is used for UE-initiated COT together with CG PUSCH, to support CP extension for multiple starting time offset as in CG PUSCH for DG PUSCH could be considered.

Observation 4: If the difference of CP extension is called as the change of FFP, the start of FFP might be always CG PUSCH if DG PUSCH does not have CP extension. If DG PUSCH supports CP extension, the amount of CP extension for DG PUSCH should be same as that configured to CG PUSCH.

Proposal 1: The UE is not allowed to transmit during the idle period of any FFP associated with the serving gNB (regardless whether the serving gNB initiates a COT in that FFP).

Proposal 2: FFP periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Observation 5: The decision on whether to support RACH transmission to initiate a COT by UE in RRC IDLE/INACTIVE mode could be up to RAN2.

Observation 6: If UE-initiated COT is supported in IDLE/INACTIVE mode, FFP parameters for UE-initiated COT should be provided to the UE by SIB.

Proposal 3: A UE can be explicitly indicated by DCI whether or not to initiate a COT in a next FFP associated to the UE.

Proposal 4: No explicit indication to indicate to the gNB that it has initiated a COT in an FFP is necessary.

Proposal 5: When configured grant Type 1 or Type 2 are configured on unlicensed spectrum, it would be useful that at least functions related to autonomous retransmission on CG and COT sharing are separately enabled/disabled by the configuration.

Reference

[1] 	RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR,” Nokia, Nokia Shanghai Bell, RAN#88e.

[2]	R1-2007391, “Summary#5 on enhancements for unlicensed band URLLC/IIoT for R17,” Moderator (Ericsson), RAN1#102e.

[3]	R1-2005376, “Enhancements for unlicensed band URLLC/IIoT,” vivo, RAN1#102e.

- Appendix: Agreements in the previous meetings

RAN1#102e

Agreement:

- For semi-static channel access mode

  - If sensing is needed, it is performed immediately before the configured / scheduled transmission opportunity.

  - For operation with semi-static channel access, the Rel.16 random starting offsets for UL configured grants with full bandwidth allocation when UE initiates a COT, is not supported.

Agreement:

- For semi-static channel access mode

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initiates a COT.

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initiates a COT.

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB.

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that FFP in which the gNB shares the COT initiated by the UE.

  - FFS: Whether/how to support additional restrictions to the idle period

Agreement:

- For semi-static channel access mode, support using the transmission of any scheduled / configured UL channel / signal to initiate a COT by a UE in RRC CONNECTED mode.

  - FFS: The case when the UE is IDLE / INACTIVE mode

Agreement:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9 us immediately before the UL transmission burst.

Agreement:

- Conditions on the channel access procedures with respect to sensing during and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel.16 by exchanging UE and gNB roles.

Agreement:

- UE-to-gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UE transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS: Details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS: Whether/how UE to gNB COT sharing when the gap is >16 us.

Agreement:

- For semi-static channel access mode.

  - Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

  - FFS: FFP periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT

Agreement:

- For semi-static channel access mode

  - FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

    - FFS on to be provided by SIB1

  - FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

Agreement:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

- Further study and decide how to harmonize the CG features for Rel.16 URLLC and Rel.16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.