3GPP TSG RAN WG1 Meeting #103-e	R1-2008969

E-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.9.1

Source:	Mediatek

Title:	Further considerations on support of 16QAM for NB-IOT

Document for:	Discussion and Decision

Introduction

In the RAN1 102-e meeting, following agreements have been captured in [1].

Agreement

Further study on TBS/MCS table design, resource assignment and TBS allocation to support 16QAM in DL considering at least:

- MCS field size

- Achievable code rates

- Avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows)

- The break point between different modulation schemes

- Impacts of deployment modes

- Indication of modulation scheme for retransmissions

- Applicability of repetitions

- UE data rate

In this contribution, based on the approved simulation consumption, we investigate the MCS table design and applicability of repetitions.

Discussions

Investigation on MCS table design

As discussed last meeting, support of 16QAM necessarily introduces MCS table which may be designed as LTE. In [2], we put forward an initial TBS table for NPDSCH as follow.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Table 1: Transport block size (TBS) table to support 16QAM for NPDSCH

Based on Table1, we investigate avoidance of link-adaptation issues (i.e., large SINR differences between different entries within one TBS row or between different entries in adjacent TBS rows). Because code rate is much smooth along the TBS row in Table1, we focus on column situation. Due to support of 16QAM, we can find the code rate curve along column encounters a break point between (=14) and (=13), so we select 6 items near the break point to do simulation. Based on approved Simulation assumptions for DL, the simulation result is illustrated in Figure 1.

| Parameter | Value/Description |
| --- | --- |
| Operation mode for DL | Stand-alone |
| Number of antennas | 1T, 1R |
| Channel model | AWGN |
| Time Resource | 10 sub-frames(Isf=7) |
| Frequency Resource | 1 PRB |
| Number of repetitions | Baseline number of repetitions = 1/2 |
| Modulation Order | QPSK, 16-QAM |
| Noise Estimation | Ideal |
| Channel Estimation | Realistic |
| Frequency Offset | 0 |
| Time Offset | 0 |

Figure1: BLER on 10ms RU

We can find that there is a 2.1dB SNR interval between 16QAM (=14) and QPSK (=13. The left 3 curves (QPSK) are loose (0.8 dB interval) than right 3 curves (0.35dB interval) due to the different step of code rate along last column in Table 1. For the sake of simplicity, it’s unnecessary to insert a new TBS between 2536 and 2856, thus there are 2 options to narrow down the 2.1dB SNR interval, option1 is setting =14 (2856 bit) for QPSK, and option2 is setting =13 (2536 bits) for 16QAM. Option1 will get 0.939 code rate however option2 gets 0.42 code rate. Thus option2 is preferred and the corresponding MCS table should be like following considering 5-bit MCS is adopted:

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12 | 2 | 12 |
| 13 | 2 | 13 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 4 | 16 |
| 18 | 4 | 17 |
| 19 | 4 | 18 |
| 20 | 4 | 19 |
| 21 | 4 | 20 |
| 22 | 4 | 21 |

Table 2: DL MCS table for NB-IOT 16QAM

Proposal:  5-bit MCS should be adopted and Table 2 should be DL MCS table

Investigation on applicability of repetitions

As we know, besides the time diversity gain, the repetition method essentially is an exchange between power/time resource and SNR. Here we consider two possible scenarios for repetition, one is using repetition-16QAM to replace normal 16QAM, and the other one is using repetition-16QAM to replace normal QPSK. Considering most of NB-IOT application scenarios are low speed and there is no fading channel suggestion until last meeting, we preliminarily do simulation based on approved simulation consumption i.e. AWGN. We select 3 typical cases to compare and Table 3 is the configuration detail, Figure 2/3/4 are the results. Here are 3 temporary definitions for convenience:

- Repetition-16QAM: with repetition

- Normal 16QAM: without repetition

- Normal QPSK: without repetition

|  | With 1 Repetition | without repetition | without repetition |
| --- | --- | --- | --- |
| Figure2 | 16QAM TBS=488 bits RU length=1 sub-frame Total time resource=2 ms | 16QAM TBS=552 bits RU length=2 sub-frames Total time resource=2 ms | QPSK TBS=488 RU length=2 sub-frames Total time resource=2 ms |
| Figure3 | 16QAM TBS=1000 RU length=2 sub-frames Total time resource=4 ms | 16QAM TBS=1128 RU length=4 sub-frames Total time resource=4 ms | QPSK TBS=1000 RU length=4 sub-frames Total time resource=4 ms |
| Figure4 | 16QAM TBS=2472 RU length=5 sub-frames Total time resource=10 ms | 16QAM TBS=2856 RU length=10 sub-frames Total time resource=10 ms | QPSK TBS=2536 RU length=10 sub-frames Total time resource=10 ms |

Table 3: Typical cases for repetition investigation

Figure2: BLER on 1ms and 2ms RU

Figure3: BLER on 2ms and 4ms RU

Figure4: BLER on 5ms and 10ms RU

From above simulation, we could observe that the repetition-16QAM is worse more than normal QPSK. Even comparing to normal 16QAM which has larger TBS (means higher code rate), the repetition-16QAM still be worse more than 1 db.

Observation: Repetition for 16QAM has much worse performance under AWGN.

Conclusions

In this contribution we have investigated the MCS table design and applicability of repetitions. Based on the observation, it seems that repetition for 16QAM doesn’t work well. And for MCS table design we have one proposal:

Observation: Repetition for 16QAM has much worse performance under AWGN.

Proposal: 5-bit MCS should be adopted and Table 2 should be DL MCS table i.e.

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12 | 2 | 12 |
| 13 | 2 | 13 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 4 | 16 |
| 18 | 4 | 17 |
| 19 | 4 | 18 |
| 20 | 4 | 19 |
| 21 | 4 | 20 |
| 22 | 4 | 21 |

- References

- “https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_102-e/Report/Draft_Minutes_report_RAN1%23102-e_v020.zip.

- RP-2005648, “Considerations on support of 16QAM in NB-IOT”, Mediatek