3GPP TSG RAN WG1 #103-e                                                              	       R1-2008980

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.1.2.3

Source: 	Intel Corporation

Title:	Multi-TRP enhancements for beam management

Document for: 	Discussion/Decision

# Introduction

According to NR MIMO WID RP-193133, the following is the scope of work in this AI.

2. Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In this contribution we provide our views on the features that should be considered for beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception.

# Enhancements for mTRP beam-management

## Indicating mTRP beam grouping from NW

Agreement For L1-RSRP, consider measurement / reporting enhancement to facilitate inter-TRP beam pairing - Option-1: Group-based reporting, - e.g., beam restriction to facilitate inter-TRP pairing. - Option-2: Non-group-based reporting

The motivation for this agreement is to allow a UE to be aware that certain transmit beams are associated with the same panel/TRP and is not suitable for multi-TRP operation. This can be achieved by grouping SSB indices into two mutually exclusive groups which can be sufficient to also partition the associated QCL-ed CSI-RS resources. As an example, when groupBasedBeamReporting is enabled in a CSI-ReportConfig, the associated CSI-ResourceConfig may comprise of two CSI-SSB-ResourceSets, each representing a TRP/panel. Note that a CSI-SSB-ResourceSet representing TRP-1 also acts as a resource for NZP-IM for the CSI-SSB-ResourceSet representing TRP-2.

Proposal-1: When groupBasedBeamReporting is enabled in a CSI-ReportConfig, the associated CSI-ResourceConfig may comprise of two CSI-SSB-ResourceSets, each representing a TRP/panel

## Enabling mTRP beam grouping at UE

Agreement Evaluate and study at least but not limited to the following issues for multi-beam enhancement - Issue 1: Consideration of inter-beam interference - Issue 2: For group-based reporting, increased number of groups and/or beams per group - Issue 3: UE Rx panel related beam measurement/report - NOTE: “UE panel” is used for discussion purpose only

Issue 1 (inter-beam interference):

If groupBasedBeamReporting is enabled, UE should report CRI-1 and CRI-2 in a single reporting instance for both the cases of single Rx beam or 2 Rx beams as shown in  Fig 1 and Fig 2 respectively. In both cases it is possible that high value of L1-RSRP is observed for both CRI-1 and CRI-2. However, in Fig 2, significant cross-beam interference is expected compared to Fig.1 and it may not be suitable for mTRP reception. Due to the lack of consideration of cross-beam interference, current groupBasedBeamReporting is not sufficient for the gNB to make determination of beam-pairs for mTRP scheduling.

| Figure 1: 2 spatial domain Rx filter used, small cross-beam interference | Figure 2: 1 spatial domain Rx filter used, large cross-beam interference |
| --- | --- |

As observed above, a UE needs to consider both L1-RSRP and inter-beam interference for determining the simultaneous reception criteria and ranking the beam pairs for groupBasedBeamReporting. The exact details can be left to UE implementation but it should be clarified that enabling groupBasedBeamReporting implies mTRP reception hypothesis - in  particular this impacts the following UE behaviors – L1-RSRP reporting is based on reception from the selected best UE Rx panel (and not based on reception due to multiple panels), L1-SINR reported includes interference due to the other reported beam-pair.

Proposal-2: Specify that enabling groupBasedBeamReporting implies mTRP (simultaneous) reception hypothesis. This means a) L1-RSRP reported is based on reception from the selected best UE Rx panel (and not based on reception due to multiple panels) and b) L1-SINR reported includes interference due to the other reported beam-pair.

Similarly, it should be clarified that disabling groupBasedBeamReporting implies sTRP reception hypothesis - in  particular this impacts the following UE behaviors – L1-RSRP reported may be based on reception from one or more Rx panels, L1-SINR reported includes interference due to other cells.

Proposal-3: Specify that disabling groupBasedBeamReporting implies sTRP reception hypothesis. This means a) L1-RSRP reported may be based on reception from one or more Rx panels and b) L1-SINR reported includes interference due to other cells.

mTRP and sTRP hypotheses is illustrated in the figures below:

| Figure 5: L1-RSRP measurement based on mTRP reception hypothesis | Figure 6: L1-RSRP measurement based on sTRP reception hypothesis |
| --- | --- |

Issue 2 (increased number of groups and/or beams per group):

In order to increase the efficiency of CSI reporting, reporting multiple beam-pairs in a single instance of groupBasedBeamReporting  may be supported.

Proposal-4: In order to increase the efficiency of CSI reporting, reporting multiple beam-pairs in a single instance of groupBasedBeamReporting  may be supported.

## Beam failure recovery for mTRP

Agreement - Evaluate enhancement to enable per-TRP based beam failure recovery starting with Rel-15/16 BFR as the baseline. - Consider following potential enhancement aspects to enable per-TRP based beam failure recovery - Issue 1: TRP-specific BFD - Issue 2: TRP-specific new candidate beam identification - Issue 3: TRP-specific BFRQ - Issue 4: gNB response enhancement - Issue 5: UE behavior on QCL/spatial relation assumption/UL power control for DL and UL channels/RSs after receiving gNB response

In terms of scope we propose to focus on single cell mTRP case

Issue 1: TRP-specific BFD

In order to enable TRP-specific BFD, we can consider identification of two sets of BFD resources representing TRP-1 and TRP-2 (in failureDetectionResources). If failureDetectionResources is not configured, the current default cell-specific BFD procedure may be used. Whether the maximum number of BFD -RS per BWP should be increased could be further discussed.

Issue 2: TRP-specific new CBD

In order to enable TRP-specific CBD, we can consider identification of two sets of CBD resources representing TRP-1 and TRP-2 (candidateBeamRSSCellList-r16) as used for SCell BFR. It can be further studied whether CBD resources associated with dedicated PRACH need to be TRP-specific.

Issue 3: TRP-specific BFRQ

In a given serving cell, if beam failure is observed in both TRP-1 and TRP-2 BFD resources, BFRQ can generally follow Rel-15/16 BFR procedure. If beam failure is observed on any one of TRP-1 or TRP-2, BFRQ can generally follow Rel-16 SCell BFR procedure comprising of the following steps:

- link recovery request (LRR): LRR (reused from SCell BFR) can be transmitted over dedicated SR-like PUCCH resource (PUCCH-BFR). If a UE is able to identify an existing uplink grant for the non-failed TRP, this step can be skipped.

- MAC-CE to report details: If at least one new beam is identified for the failed TRP, the UE reports only 1 new beam with corresponding beam index, otherwise UE reports no new beam identified and failed TRP index

Issue 4: gNB response

This can also follow SCell BFR (Rel-16). BFRR to step 2 MAC-CE of the BFRQ can be a normal uplink grant to schedule a new transmission for the same HARQ process ID as the PUSCH carrying the step 2 MAC-CE. When UE receives this uplink grant, the BFR procedure can be considered to be completed.

Proposal-5: For TRP specific BFD consider the following:

- focus on the single cell mTRP case

- consider two sets of BFD resources representing TRP-1 and TRP-2

- consider two sets of SCell CBD resources representing TRP-1 and TRP-2

- if beam failure is observed in both TRP-1 and TRP-2 BFD resources, BFRQ can generally follow Rel-15/16 BFR procedure.

- If beam failure is observed on any one of TRP-1 or TRP-2, BFRQ can generally follow Rel-16 SCell BFR procedure

- Beam failure recovery response (BFRR) can also follow SCell BFR (Rel-16) procedure

# Conclusions

Proposal-1: When groupBasedBeamReporting is enabled in a CSI-ReportConfig, the associated CSI-ResourceConfig may comprise of two CSI-SSB-ResourceSets, each representing a TRP/panel

Proposal-2: Specify that enabling groupBasedBeamReporting implies mTRP (simultaneous) reception hypothesis. This means a) L1-RSRP reported is based on reception from the selected best UE Rx panel (and not based on reception due to multiple panels) and b) L1-SINR reported includes interference due to the other reported beam-pair.

Proposal-3: Specify that disabling groupBasedBeamReporting implies sTRP reception hypothesis. This means a) L1-RSRP reported may be based on reception from one or more Rx panels and b) L1-SINR reported includes interference due to other cells.

Proposal-4: In order to increase the efficiency of CSI reporting, reporting multiple beam-pairs in a single instance of groupBasedBeamReporting  may be supported.

Proposal-5: For TRP specific BFD consider the following:

- focus on the single cell mTRP case

- consider two sets of BFD resources representing TRP-1 and TRP-2

- consider two sets of SCell CBD resources representing TRP-1 and TRP-2

- if beam failure is observed in both TRP-1 and TRP-2 BFD resources, BFRQ can generally follow Rel-15/16 BFR procedure.

- If beam failure is observed on any one of TRP-1 or TRP-2, BFRQ can generally follow Rel-16 SCell BFR procedure

- Beam failure recovery response (BFRR) can also follow SCell BFR (Rel-16) procedure

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86