3GPP TSG RAN WG1 #103-e			    R1-2009001

e-Meeting, October 26th – November 13th, 2020

Source:	Intel Corporation

Title:	Mechanisms to Improve Reliability of NR MBS

Agenda item:	8.12.2

Document for:	Discussion and Decision

# Introduction

In this paper, reliability aspects of NR multicast and broadcast services (MBS) for RRC_CONECTED UEs are discussed. The new work item on NR Support of Multicast and Broadcast Services [1] has the following objectives for normative work on NR MBS related to improving the reliability of multicast/broadcast service:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]: - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [ RAN1, RAN2]

Additionally, the following agreements were made in

Agreement: For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this. - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based. - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled. Agreements: For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH. - FFS: whether enhancement is needed Agreements: For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission. - FFS: whether enhancement is needed

Based on initial set of agreements, in this paper, we discuss mechanisms for improving the reliability of NR based multicast and broadcast services within a single cell for RRC_CONNECTED UEs

# Discussion

In this section, design considerations for reliability mechanisms for NR MBS focused on RRC_CONNECTED UEs is provided along with considerations for support of RRC_IDLE or RRC_INACTIVE UEs.

## 2.1. Improving Reliability based on UL Feedback

### HARQ/ACK Procedure

For HARQ/ACK feedback for RRC_CONNECTED UEs, some functionality from NR Sidelink HARQ procedure can be re-used [5]. For NR-MBS, the presence of HARQ feedback should be configurable.  The utility of this mechanism is mainly for the cases when connected and idle/inactive UEs are grouped together and when broadcast like transmissions are done for a large number of UEs. For RRC_CONNECTED UEs the configuration of HARQ on/off can be handled semi-statically via dedicated RRC signaling or dynamically via a bit in the DCI. Similar functionality can also be supported for RRC_IDLE/INACTIVE mode UEs as discussed in our companion contribution [3].

Proposal 1: For NR MBS, HARQ feedback should be configurable i.e., it can be enabled or disabled. The following options can be considered for such configuration

    - Semi-static configuration through RRC signaling

    - Dynamic indication using a single bit in the scheduling DCI for the groupcast transmission

When HARQ feedback is enabled, ACK/NACK based and NACK only feedback mode can be considered for connected mode UEs.

#### ACK/NACK Based HARQ

When connected mode UEs have PUCCH resource sets configured by dedicated RRC signaling, the UE can support ACK/NACK based feedback on dedicated PUCCH resources. The existing signaling mechanism can be used wherein the PUCCH resource for ACK/NACK feedback based on the value of the 3-bit PUCCH resource indicator and starting CCE index for the reception of the PDCCH reception which scheduled the MBS PDSCH.

Since all UEs in the group receive the same PRI and starting CCE index values, it can be up to the network to configure UE specific PUCCH resources such that collision can be avoided between different UEs in the group.

#### NACK Only HARQ

- NACK only feedback mode can be enabled in an SFN like manner wherein multiple UEs transmit NACK only signal on a shared PUCCH resource. The gNB can configure a common PUCCH resource to the group of UEs. The common resource needs to be indicated by the same PRI and hence should be map to the same index for all UEs receiving the groupcast transmission. Additionally, when connected mode UEs have no dedicated PUCCH resource configuration, the UEs can use common cell specific PUCCH resource to transmit NACK only feedback. In Rel-15 a common PUCCH resource set is defined which includes 256 PUCCH resources using PUCCH Formats 0 and 1, and a 4-bit indication in RMSI selects 16 cell-specific resources. The selection of the particular PUCCH resource is again based on PRI and starting CCE index used for reception of the scheduling PDCCH. In this case, the PUCCH resource will be shared by UEs in the group and UEs are expected to only transmit only a NACK

- The use of ACK/NACK based or NACK only feedback can be configured either semi-statically through RRC or can be informed as part of the PUCCH resource configuration which can additionally include the information whether the UE is expected to transmit only NACK on the indicated resource.

- Proposal 2: For RRC_CONNECTED UEs, NR MBS supports both ACK/NACK based and NACK-only HARQ feedback. The configuration of ACK/NACK and NACK only mode can be done using the following options

- Option 1: Semi-static RRC configuration of ACK/NACK or NACK only mode

- Option 2: The configured PUCCH resource can contain additional indication that the UE is expected to transmit only NACK on the configured resource

- Option 3: If UE has no dedicated PUCCH resource configuration, the UE uses cell-specific PUCCH resource and is expected to only transmit NACK

- Note that the three options in the Proposal 2 are not mutually exclusive and can co-exist.

#### HARQ Process and Codebook Design

- For supporting HARQ with ACK/NACK based feedback in NR MBS, we also need to consider the operation MBS HARQ protocol co-existing with unicast HARQ. For the HARQ process operation, there are two different design options which have possible indication on UE implementation – (i) MBS shares HARQ process with unicast or (ii) MBS has independent HARQ processes which operate on top of current unicast HARQ capabilities. There may be a total of 16+X HARQ processes wherein X HARQ processes are reserved for MBS and the number of processes X can be further studied.

- In the case that additional MBS HARQ processes are considered, UE implementation might need to account for additional buffer capacity depending on payload size. Furthermore, explicit signaling may be needed in DCI indicating if the re-transmission is for an MBS or unicast especially if UE specific DCI is supported.

- Proposal 3: For NR MBS, no additional HARQ processes are defined and MBS shares HARQ process ID with unicast i.e., the total of 16 HARQ processes is unchanged.

- In case MBS shares HARQ process IDs with unicast, consideration needs to be given to HARQ codebook design. Two main options are to have joint HARQ codebook design with unicast transmission or to have separate HARQ codebook for MBS. If joint codebook is chosen, then HARQ codebook redesign is required to account for MBS transmission. Additionally, dynamic switching of HARQ on or off via DCI signaling may not be compatible with joint codebook especially if UE misses the DCI since it would impact HARQ indication for unicast as well. From this perspective separate HARQ codebook might have lesser impact to current NR design. However, currently NR UEs can only transmit single PUCCH with HARQ ACK feedback within a slot. Thus, gNB has to configure PUCCH resources such that collisions are avoided with unicast HARQ feedback otherwise specification of collision avoidance rules and prioritization may become important.

- Proposal 4: Further study the following options for MBS HARQ codebook design

- Joint codebook with unicast

- Separate HARQ codebook for NR MBS

#### HARQ Retransmission Schemes

- One major feature in NR is the introduction of CBG based retransmissions wherein UEs can signal which CBs failed and only those CBs need to be retransmitted instead of the entire TB. However, in the case of groupcast, each UE in the group might face independent CB failures and the entire TB will need to be retransmitted. Furthermore, retransmissions are also limited by the worst UEs in the group. To counter this, in case of ACK/NACK based HARQ, UE specific retransmission may be supported if PDCCH monitoring on USS is supported for NR MBS. In this case, gNB can retransmit failed CBGs to a single or smaller sub-group of UEs instead of retransmitting the entire TB.

- Proposal 5: For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

### CSI Feedback

- In the previous meeting, it was agreed that current CSI feedback is supported and to further study if enhancements are required. Given the groupcast transmissions are limited by the channel conditions of the worst UE in the group, it is enough to support current NR CSI feedback mechanism to leverage some channel information for efficient scheduling. For NR MBS, Type I CSI feedback should be enough.

- Proposal 6: No further enhancements to NR CSI feedback mechanism is needed for NR MBS

## 2.2. Improving Reliability based on Repetition

Slot level repetition has been agreed to be supported for NR MBS. In terms of configuration of repetition, in Rel-16, dynamic configuration of repetition was specified wherein the TDRA field in DCI points to a codepoint which also contains the repetition number as follows

PDSCH-TimeDomainResourceAllocation-r16 ::=  SEQUENCE {

k0-r16                                     INTEGER(0..32)                                              OPTIONAL,   -- Need S

mappingType-r16                            ENUMERATED {typeA, typeB},

startSymbolAndLength-r16                   INTEGER (0..127),

repetitionNumber-r16                       ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n16}                OPTIONAL,   -- Cond Formats1-0and1-1

...

}

For NR MBS the dynamic repetition indication from Rel-16 can be re-used. Additionally, semi-static repetition configuration can also be supported.

Proposal 7: NR MBS supports dynamic indication of number of repetitions for PDSCH transmissions with slot-based repetitions.

# Conclusions

In this paper, design options and related evaluations were proposed for reliability improvements for NR MBS. To this end, the following proposals were made:

- Proposal 1: For NR MBS, HARQ feedback should be configurable i.e., it can be enabled or disabled. The following options can be considered for such configuration

  - Semi-static configuration through RRC signaling

  - Dynamic indication using a single bit in the scheduling DCI for the groupcast transmission

- Proposal 2: For RRC_CONNECTED UEs, NR MBS supports both ACK/NACK based and NACK-only HARQ feedback. The configuration of ACK/NACK and NACK only mode can be done using the following options

  - Option 1: Semi-static RRC configuration of ACK/NACK or NACK only mode

  - Option 2: The configured PUCCH resource can contain additional indication that the UE is expected to transmit only NACK on the configured resource

  - Option 3: If UE has no dedicated PUCCH resource configuration, the UE uses cell-specific PUCCH resource and is expected to only transmit NACK

- Proposal 3: For NR MBS, no additional HARQ processes are defined and MBS shares HARQ process ID with unicast i.e., the total of 16 HARQ processes is unchanged.

- Proposal 4: Further study the following options for MBS HARQ codebook design

  - Joint codebook with unicast

  - Separate HARQ codebook for NR MBS

- Proposal 5: For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

- Proposal 6: No further enhancements to NR CSI feedback mechanism is needed for NR MBS

- Proposal 7: NR MBS supports dynamic indication of number of repetitions for PDSCH transmissions with slot-based repetitions

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- Chairman’s Notes, 3GPP RAN1 102-e Meeting, August 2020

- R1-2009002, NR MBS for RRC_IDLE and INACTIVE UEs, Intel Corporation, RAN1#103-e Meeting, October 2020.

- 3GPP TR 36.890 v13.0.0, Study on Single Cell Point-to-Multipoint Transmission (Release 13)

- 3GPP TS 38.211 v16.2.0, NR Physical Channels and Modulation (Release 16)

- 3GPP TS 38.212 v16.2.0, NR Multiplexing and Channel Coding (Release 16)

- 3GPP TS 38.213 v16.2.0, NR Physical Layer Procedures for Control (Release 16)

- 3GPP TS 38.214 v16.2.0, NR Physical Layer Procedures for Data (Release 16)