3GPP TSG RAN WG1 #103-e																			R1-2009009

e-Meeting, October 26th – November 13th, 2020

Source:	Intel Corporation

Title:	Support of 1024QAM

Agenda item:	8.16

Document for:	Discussion and Decision

# Introduction

In RAN#89e meeting the work item on downlink 1024QAM support in FR1 was agreed for Rel-17 [1]. The objectives of WID include specification of MCS and CQI tables supporting 1024QAM.

The objective is to specify downlink 1024QAM for NR PDSCH operation in FR1, together with related procedures, signalling and necessary RF requirements. The main objectives are: - Specify high order modulation for PDSCH [RAN1] - Specify 1024QAM constellation as specified in E-UTRA for DL PDSCH - Specify corresponding MCS table with 1024QAM entries as defined in E-UTRA - Note: DCI overhead for MCS indication should be the same as in Rel-15 - Specify corresponding CQI feedback with 1024QAM entries as defined in E-UTRA

In this contribution, we propose details of the corresponding enhancements for Rel-17.

# CQI table design for 1024QAM

According to 1024QAM WID objective 1024QAM CQI table for NR should support the same 1024QAM entries as in LTE. It is, therefore, proposed to adopt the corresponding table without modifications as shown in Table 1.

Table 1 CQI table for 1024QAM

| CQI index | modulation | code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3321 |
| 15 | 1024QAM | 948 | 9.2578 |

# MCS table design for 1024QAM

LTE supports two MCS tables for 1024QAM – with 5 bits and 6-bits overhead. The 5-bits MCS table is designed using MCS sub-sampling principles, while 6-bits on the MCS table aggregation. In this section we provide similar design options with 1024QAM for Rel-17 NR.

## Discussion on 5-bit MCS table

For 5-bit MCS table design with 1024QAM, we propose to use the following principles:

- 256QAM MCS table is used as a starting point for introduction of 1024QAM

- Remove five MCS entries from 256QAM MCS table

  - Remove every other MCS entry with lowest spectral efficiency

  - Consider MCS entries which were not sub-sampled for 256QAM

  - MCS #5, #7, #9, #12, #14 can be considered as candidate for removal from 256QAM table

- Introduce 5 additional MCS entries supporting 1024 QAM

  - 4 MCS with TBS indication

  - 1 for adaptive HARQ without TBS indication

- Define spectral efficiency for 1024QAM entries as follows

  - Two MCS entries (out of 4) have the same spectral efficiency as in CQI table

  - Two additional MCS entries (out of 4) have spectral efficiency linearly interpolated from spectral efficiencies of the nearest MCS entries

  - Quantize the spectral efficiency and coding rates

  - Order according to modulation order and spectra efficiency

Using the above principles, the following MCS table supporting 1024QAM is proposed as show in Table 2, where 1024QAM entries are highlighted in yellow:

Table 2 5-bit MCS table with 1024QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 434 | 1.6953 |
| 6 | 4 | 553 | 2.1602 |
| 7 | 4 | 658 | 2.5703 |
| 8 | 6 | 466 | 2.7305 |
| 9 | 6 | 567 | 3.3223 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.3320 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 806 | 7.8711 |
| 24 | 10 | 853 | 8.3321 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |
| 28 | 4 | reserved |  |
| 29 | 6 | reserved |  |
| 30 | 8 | reserved |  |
| 31 | 10 | reserved |  |

## Discussion on 6-bit MCS table

In Rel-15 LTE it was observed that sub-sampling of “legacy” MCS tables (to accommodate 256QAM or 1024QAM entries) leads to performance degradation for some SNR region. In addition, TBS selection procedure may be highly sub-optimal as it does not account potential difference in overhead from the reference signal (e.g. from CSI-RS) and different DL subframes. As the result, PDSCH in the DL subframes containing CSI-RS may have substantially higher coding rates for all MCS entries in many cases exceeding maximum coding rate threshold. To solve the above issues, 6-bits MCS table was introduced for LTE, where MCS for all modulation schemes are kept. In addition, some MCS values are replicated but with different overhead to support dynamic adaptation of TBS.

Although Rel-16 NR allows dynamic adaptation between 64QAM and 256QAM MCS table by using MCS-RNTI, the issue of varying overhead due to CSI-RS or rate matching resources is not yet addressed in NR systems. As the results, it is expected that similar performance degradation may be observed in NR systems as in LTE.

To overcome the above issues, we propose 6-bits MCS table, which is constructed based on a similar principle as 6-bit MCS table in LTE. More specifically, the following design rules can be formulated:

- 6-bit MCS table support all modulation schemes, i.e. QPSK, 16QAM, 64QAM, 256QAM and 1024QAM

  - The unsupported modulations are not expected to be indicated for the UE in DCI

- MCS entries for all modulation schemes are included without MCS sub-sampling

- Several MCS entries support the same spectral efficiencies, but associated with different overhead parameter for TBS selection

  - Additional overhead parameter is configured by RRC

The proposed MCS tables based on the above principles is shown in Table 3

Table 3 6-bit MCS table with 1024QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency | Index i for overhead parameter NohPRB,i |
| --- | --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 | 0 |
| 1 | 2 | 157 | 0.3066 | 0 |
| 2 | 2 | 193 | 0.3770 | 0 |
| 3 | 2 | 251 | 0.4902 | 0 |
| 4 | 2 | 308 | 0.6016 | 0 |
| 5 | 2 | 379 | 0.7402 | 0 |
| 6 | 2 | 449 | 0.8770 | 0 |
| 7 | 2 | 526 | 1.0273 | 0 |
| 8 | 2 | 602 | 1.1758 | 0 |
| 9 | 2 | 679 | 1.3262 | 0 |
| 10 | 4 | 340 | 1.3281 | 0 |
| 11 | 4 | 378 | 1.4766 | 0 |
| 12 | 4 | 434 | 1.6953 | 0 |
| 13 | 4 | 490 | 1.9141 | 0 |
| 14 | 4 | 553 | 2.1602 | 0 |
| 15 | 4 | 616 | 2.4063 | 0 |
| 16 | 4 | 658 | 2.5703 | 0 |
| 17 | 6 | 438 | 2.5664 | 0 |
| 18 | 6 | 466 | 2.7305 | 0 |
| 19 | 6 | 517 | 3.0293 | 0 |
| 20 | 6 | 567 | 3.3223 | 0 |
| 21 | 6 | 616 | 3.6094 | 0 |
| 22 | 6 | 666 | 3.9023 | 0 |
| 23 | 6 | 719 | 4.2129 | 0 |
| 24 | 6 | 772 | 4.5234 | 0 |
| 25 | 6 | 822 | 4.8164 | 0 |
| 26 | 6 | 873 | 5.1152 | 0 |
| 27 | 6 | 910 | 5.3320 | 0 |
| 28 | 6 | 948 | 5.5547 | 0 |
| 29 | 8 | 682.5 | 5.3320 | 0 |
| 30 | 8 | 711 | 5.5547 | 0 |
| 31 | 8 | 754 | 5.8906 | 0 |
| 33 | 8 | 797 | 6.2266 | 0 |
| 33 | 8 | 841 | 6.5703 | 0 |
| 34 | 8 | 885 | 6.9141 | 0 |
| 35 | 8 | 916.5 | 7.1602 | 0 |
| 36 | 8 | 948 | 7.4063 | 0 |
| 37 | 10 | 797 | 7.7833 | 0 |
| 38 | 10 | 841 | 8.2129 | 0 |
| 39 | 10 | 885 | 8.6426 | 0 |
| 40 | 10 | 916.5 | 8.9503 | 0 |
| 41 | 10 | 948 | 9.2578 | 0 |
| 42 | 4 | 616 | 2.4063 | 1 |
| 43 | 4 | 658 | 2.5703 | 1 |
| 44 | 6 | 772 | 4.5234 | 1 |
| 45 | 6 | 822 | 4.8164 | 1 |
| 46 | 6 | 873 | 5.1152 | 1 |
| 47 | 6 | 910 | 5.3320 | 1 |
| 48 | 6 | 948 | 5.5547 | 1 |
| 49 | 8 | 797 | 6.2266 | 1 |
| 50 | 8 | 841 | 6.5703 | 1 |
| 51 | 8 | 885 | 6.9141 | 1 |
| 52 | 8 | 916.5 | 7.1602 | 1 |
| 53 | 8 | 948 | 7.4063 | 1 |
| 54 | 10 | 797 | 7.7833 | 1 |
| 55 | 10 | 841 | 8.2129 | 1 |
| 56 | 10 | 885 | 8.6426 | 1 |
| 57 | 10 | 916.5 | 8.9503 | 1 |
| 58 | 10 | 948 | 9.2578 | 1 |
| 59 | 2 | reserved |  | - |
| 60 | 4 | reserved |  | - |
| 61 | 6 | reserved |  | - |
| 62 | 8 | reserved |  | - |
| 63 | 10 | reserved |  | - |

.

It can be seen from the Table 3 that different entries in MCS table may be associated with different number of resource elements NohPRB,i which is used for calculation of the available resource elements in the resource block. The value NohPRB,i, where i=0,1 configured by higher layers (e.g. in RRC). The parameter defining second overhead, may also support larger overhead values comparing to Rel-15 to accommodate scenarios with extreme large CSI-RS overhead.

# Conclusion

In this contribution, we have provided our views on the details of 1024QAM support in NR. The following proposal were made:

- For CQI design with 1024QAM, adopt the Table 1 (same as LTE CQI table)

- For MCS design adopt both 5-bits and 6-bits MCS tables

  - 5-bits MCS table is constructed based on sub-sampling principle (provided in Table 2)

  - 6-bit MCS table is constructed based on aggregation principles and (provided in Table 3)

    - Some of MCS entries may has the same spectral efficiency, but associated with different overhead parameter configured by RRC

    - The second value for overhead support extended the range of candidate values

# References

[1] RP-202044, New WID on Introduction of DL 1024QAM for NR FR1, Ericsson, 3GPP TSG RAN Meeting #89e, Electronic Meeting, September 14 - 18, 2020.