# 3GPP TSG RAN WG1 #103-e								R1-2009020

# e-Meeting, October 26th – November 13th, 2020

# Source:	ETRI

# Title:		Other enhancements to IAB

# Agenda item:	8.10.3

# Document for:	Discussion/Decision

Introduction

The Rel-17 IAB enhancement work item was approved in RAN#86 meeting and it contains several specification aspects regarding the simultaneous operation in its scope [1]. as follows:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]: - Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including: - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx). - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing. - Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In this contribution, we discuss some forward compatibility issues when designing Rel-17 IAB simultaneous operation enhancements such as timing alignment and power control.

Discussions

As described in the WID [1], in Rel-17 IAB specification, several enhancements on the simultaneous operation are expected to be introduced including IAB-node timing, DL/UL power control, CLI/interference measurements, and so on.

Regarding the relaying scenario for the IAB, although the need for mobile relaying-based IAB such as buses and trains was identified, only fixed relaying was included in the scope of Rel-17 IAB WID. However, deploying IAB through mobile relaying has several advantages over direct access of each passenger to the gNodeB, as follows:

  - By relaying signals through the mobile relay, it is possible to avoid severe signal penetration loss due to a metallic vehicle body.

  - The relaying architecture is also advantageous to avoid the problem of so-called signaling storm due to group handover. The number of passengers can be 1000+ for a train.

Another possible scenario for IAB is the Non-Terrestrial Network (NTN) scenario where a satellite can be used to provide 5G service. In this scenario, IAB nodes connected to the satellite through the NTN backhaul link can be used to provide access to the UEs.

In order to ensure forward compatibility with these potential future IAB scenarios, Rel-17 IAB enhancements specification needs to consider the following aspects:

  - IAB mobility: When the IAB nodes are mobile, timing alignment and power control need to be more dynamic and flexible. In addition, these increased dynamicness and flexibility are also beneficial to fixed IAB nodes since the backhaul and access links can be time-varying since surrounding objects can be mobile.

  - Backhaul link with long delay: For the IAB nodes with NTN-type backhaul links, latency can be much longer. In this case, the dynamic range of timing alignment needs to be much larger.

Proposal 1: Rel-17 IAB enhancements specification need to provide some flexibility for forward compatibility.

Conclusion

In this contribution, we discussed NR V2X sidelink synchronization mechanism. Our proposals are reproduced below:

Proposal 1: Rel-17 IAB enhancements specification need to provide some flexibility for forward compatibility.

References

- RP-193251, “New WID on Enhancements to Integrated Access and Backhaul,” Dec. 2019.