3GPP TSG-RAN WG1 Meeting #103-e	R1- 2009071

Online, October 26th – November 13th, 2020

Agenda Item:	8.11.1

Source:	Ericsson

Title:	Remaining evaluation assumptions and methodology for power saving

Document for:	Discussion, Decision

# 1	Introduction

The Rel-17 Work item on NR sidelink enhancement [1] targets specifying radio solutions that can enhance NR sidelink for the V2X, public safety (PS) and commercial use cases. One of the items stated in the WID is the following:

1. Sidelink evaluation methodology update: Define evaluation assumption and performance metric for power saving by reusing TR 36.843 and/or TR 38.840 (to be completed by RAN#89) [RAN1] - Note: TR 37.885 is reused for the other evaluation assumption and performance metric. Vehicle dropping model B and antenna option 2 shall be a more realistic baseline for highway and urban grid scenarios.

Agreements:

- For evaluation, the followings are baseline

- 2 RX APs

- 1 TX AP

- 40 MHz for SL BWP size

- Note that parameters or cases other than baseline is not precluded for evaluation, and companies are encouraged to provide the assumptions in details.

Agreements:

- For evaluation metric, the followings are considered

- PRR

- PIR

- Power consumption reduction ratio = (power consumption for baseline scheme with Rel-16 Mode 2 resource allocation (i.e. full sensing) - power consumption for proposed scheme)/power consumption for baseline scheme with Rel-16 Mode 2 resource allocation (i.e. full sensing)

- Note that power consumption for baseline scheme with Rel-16 Mode 2 resource allocation (i.e. full sensing) and the power consumption for the proposed scheme are evaluated under the same evaluation assumptions.

During last meeting RAN1#102-e, several agreements regarding the reference configuration for the power consumption model and the baseline parameters were reached. Moreover, the different performance metric such as packet reception ratio (PRR), Packet Inter-Reception Ratio (PIR) and power reduction ratio were also agreed as follows:

Therefore, in our contribution, we focus on the remaining details for the reference configuration for power consumption model and remaining evaluation assumptions (scenarios, traffic models) with special attention to the public safety use case.

# 2	Reference power consumption model for public safety use cases

Agreements:

- For reference configuration for power consumption model,

- 14 SL symbols in a slot (including AGC and TX-RX switching period)

- SL sub-carrier spacing (SCS)

  - 30 kHz SCS for FR1

- SL BWP size

  - 100 MHz for FR1

- 2 OFDM symbols for PSCCH (excluding AGC symbol)

- TX antenna port (AP)

  - 1 TX AP for FR1

- RX AP

  - 4 RX APs for FR1

- TX power of {0 dBm, 23 dBm} for FR1

- Note that FR2 is not precluded as an optional/additional reference configuration, and companies are encouraged to provide power consumption model for FR2.

- Note that 15 kHz SCS is not precluded as an optional/additional reference configuration, and companies are encouraged to provide power consumption model for 15 kHz SCS.

During last meeting in RAN1#102-e, it was agreed to have a reference configuration for the power consumption model in SL Rel-17 as follows:

In the previous agreement, the reference value for SL sub-carrier spacing (SCS) and SL BWP size is 30 kHz and 100 MHz, respectively.

However, as indicated in the WID, RAN1 is targeting solutions for different use cases. In our view, the agreed parameters for the reference power consumption model (SCS = 30 kHz and SL BWP = 100 MHz) are not suitable ones to estimate the power consumption of public safety use cases – one of the main targeted use cases – where typically a lower SCS and SL BWP is used, and therefore, an additional reference power consumption model is needed. Moreover, if only the SCS = 30 kHz is used as the reference configuration, it is not possible to obtain a power consumption model for SCS = 15 kHz by means of power scaling for adaptation to this parameter, i.e., a new power consumption model is needed for SCS = 15 kHz.

The current agreement allows SCS = 15 kHz which is important for public safety cases, but it is not possible to obtain a power consumption model for SCS = 15 kHz by using scaling for adaptation from the reference power consumption model for SCS = 30 kHz.

Therefore, we have included a power consumption model for the SL FR1 UEs operating at SCS = 15 kHz and SL BWP = 20 MHz. We assume the same UE HW configuration as the reference in TR 38.840, i.e., all the sleep related power numbers are the same as for the FR1 reference. In the Table 1 below are the detailed values.

Table 1: Models for UE consumption in FR1

| Parameter | FR1 (Reference) | Sidelink FR1 |
| --- | --- | --- |
| SCS | 30 kHz | 15 kHz |
| BW | 100MHz | 20MHz |
| Rx branch | 4Rx | 4Rx |
| Tx branch | 1Tx | 1Tx |
| Light sleep | 20 | 20 |
| Micro sleep | 45 | 45 |
| Power consumption | PDCCH only 100 | PSCCH 75 |
|  | PDCCH+PDSCH 300 | PSCCH+PSSCH 100 |
| UL | 250 @0dBm 700 @23dBm | 80 @0dBm 215 @23dBm |

We propose to capture this new configuration (corresponding to SCS = 15 kHz in Table 1) as one reference configuration for evaluation.

Use the configuration corresponding to SCS = 15 kHz and SL BWP = 20 MHz specified in Table 1 as one reference configuration for power consumption model.

Additionally, we also propose to use the reference model with SCS = 15 kHz and SL BWP = 20 kHz as a baseline for evaluation.

Use the configuration corresponding to SCS = 15 kHz and SL BWP = 20 MHz specified in Table 1 as a baseline for evaluation.

Agreements:

- For power consumption scaling for adaptation,

- (Working assumption) Scaling of SL BWP size adaptation in RX perspective

  - X MHz is (0.4 +0.6*(X-20)/80)*100 MHz

Additionally, the following working assumption regarding the scaling for adaptation of the SL BWP using as reference the SL BWP = 100 MHz was proposed during last meeting:

In our understanding, the formula for the SL BWP size adaptation should give as a result using the scaling equals to one when using the reference value, i.e., SL BWP = 100 MHz. However, in our view the formula in the working assumption does not work in that direction, i.e., for a X = 100 MHz the value is 100. Therefore, we propose to modify the formula to achieve scaling equal to one which is also based on the formula used in TR 38.840.

Modify the scaling of SL BWP size adaptation to: X MHz is (0.4 +0.6*(X-20)/80)

On a high level, we believe that the evaluation methodology specified in TR 37.885 for V2X should be updated with various aspects related to characteristics and behaviors of UEs in public safety (PS) use cases. In the following we examine every section in the methodology of TR 37.885.

# 3 	Remaining evaluation assumptions

## 3.1	Evaluation scenarios

It is important to update system deployment with PS scenarios. In our view, the scenario for PS and UE dropping models in A2.1.1 of TR 36.843 can be reused with minor modifications. Specifically, Public Safety Scenarios in Table A.2.1.1-1: Details of Deployment Scenarios in TR 36.843 is reused wherein:

- Layout Option 5 is mandatory, Option 3 is optional. No need for Option 1.

- Include a suitable SL BWP for public safety use cases

- eNB is replaced by gNB

- UE RF parameters are taken from Table 6.1.1-1 in TR 37.885.

Update Section 6.1.1 - Evaluation scenarios in TR 37.885 with the Public Safety scenarios specified in A2.1.1 of TR 36.843, including some changes:

- Only Option 5 and Option 3 are considered.

- Modify the system bandwidth to 20 MHz.

- eNB is replaced by gNB.

- UE RF parameters are taken from Table 6.1.1-1 in TR 37.885.

## 3.2	UE drop and mobility modelling

Just as with the evaluation scenarios, we believe that the UE dropping and association models in TR 36.843 can be reused.

Update Section 6.1.2 - UE drop and mobility modeling in TR 37.885 with the dropping and association models specified in A2.1.1.1—A2.1.1.3 of TR 36.843.

## 3.3	Antenna model

In our view the antenna model for pedestrian UE and cellular UE in TR 37.885 can be used for PS UE.

The antenna model for pedestrian UE and cellular UE in TR 37.885 can be used for PS UE.

## 3.4	Traffic Model

Currently TR 37.885 includes models for periodic and aperiodic traffic. Additionally, FTP model 3 traffic is used for evaluations of UE-to-network relaying or coexistence of SL and Uu. To capture the traffic models for PS use cases in the sidelink interface, it is necessary to include traffic models for Mission critical Push-to-talk (MC PTT), MC Video, and MC Data services. Table 2 summarizes the KPIs of these MC services [2,3]:

Table 2: KPIs for Mission-critical services

|  | Latency | Reliability (PER) | Data Rate | Traffic characteristic |
| --- | --- | --- | --- | --- |
| MCPTT | 75ms | 10-2 | 10-32Kbps | Periodic @ 20ms interval |
| MC data | 200 ms | 10-6 | 10Kbps –1Mbps | MBB like |
| MC video | 30ms-100ms | 10-3 | 0.15Mbps-5Mbps | Periodic @ 30-60 fps |

We can observe the following by comparing the types of traffic listed in Table 2 and the traffic models captured in TR 37.885:

- Traffic models for MCPTT and MC Data are missing in TR 37.885.

- Periodic traffic model 2 and model 3 in TR 37.885 are suitable for MC Video service.

Therefore, we believe following traffic models should be added to TR 37.885:

- VoIP model for MCPTT: For this type of traffic, we propose to reuse “Table A.2.1.3-1: Parameters for VoIP model” in TR 36.843: Evaluation methodology for D2D.

- FTP and instant message for MC Data: For this type of traffic, we propose to reuse FTP model 3 traffic with parameters as defined in TR 38.840, Section 8.2: Traffic model used for the UE power saving scheme evaluation.

Update traffic models in TR 37.885 with the following:

- VoIP traffic model for MCPTT service: reuse Table A.2.1.3-1: Parameters for VoIP model in TR 36.843,

- FTP and instant message traffic model for MC Data service: reuse FTP model 3 traffic with parameters as defined in TR 38.840, Section 8.2.

- Use periodic traffic model 2 and 3 in TR 37.885 for MC Video services.

## 3.5	Channel model

As for channel models, currently TR 37.885 captures only V2V channel models. Therefore, it is necessary to include channel models for D2D communication as specified in A2.1.2 of TR 36.843 in order to include PS use cases which have a lower mobility as compared to V2V scenarios. In particular, for channel models for PS use cases:

- PL and Shadowing models in TR 36.843 can be reused.

- Fast fading model in TR 37.885 can be reused.

- For LLS, the CDL models for Urban LOS, Urban NLOS in TR 37.885 can be reused.

D2D channel models are more suitable for public safety scenarios due to the lower mobility of the users as compared to V2V scenarios.

Update Section 6.2 – Channel model in TR37.885 with PL and Shadowing models specified in Appendix A of TR 36.843 and reuse fast fading model in TR 37.885 for PS use cases.

For LLS of PS use cases, reuse CDL models for Urban LOS and Urban NLOS in TR 37.885.

## 3.6 	Simulation profiles

Based on the previous section and due to the modification and additions proposed, we believe that it makes sense to have a specific simulation profile for PS use cases.

Create a simulation profile for PS use cases based on the above updates and power consumption models.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The current agreement allows SCS = 15 kHz which is important for public safety cases, but it is not possible to obtain a power consumption model for SCS = 15 kHz by using scaling for adaptation from the reference power consumption model for SCS = 30 kHz.

Observation 2	D2D channel models are more suitable for public safety scenarios due to the lower mobility of the users as compared to V2V scenarios.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use the configuration corresponding to SCS = 15 kHz and SL BWP = 20 MHz specified in Table 1 as one reference configuration for power consumption model.

Proposal 2	Use the configuration corresponding to SCS = 15 kHz and SL BWP = 20 MHz specified in Table 1 as a baseline for evaluation.

Proposal 3	Modify the scaling of SL BWP size adaptation to: X MHz is (0.4 +0.6*(X-20)/80)

Proposal 4	Update Section 6.1.1 - Evaluation scenarios in TR 37.885 with the Public Safety scenarios specified in A2.1.1 of TR 36.843, including some changes:

	Only Option 5 and Option 3 are considered.

	Modify the system bandwidth to 20 MHz.

	eNB is replaced by gNB.

	UE RF parameters are taken from Table 6.1.1-1 in TR 37.885.

Proposal 5	Update Section 6.1.2 - UE drop and mobility modeling in TR 37.885 with the dropping and association models specified in A2.1.1.1—A2.1.1.3 of TR 36.843.

Proposal 6	The antenna model for pedestrian UE and cellular UE in TR 37.885 can be used for PS UE.

Proposal 7	Update traffic models in TR 37.885 with the following:

	VoIP traffic model for MCPTT service: reuse Table A.2.1.3-1: Parameters for VoIP model in TR 36.843,

	FTP and instant message traffic model for MC Data service: reuse FTP model 3 traffic with parameters as defined in TR 38.840, Section 8.2.

	Use periodic traffic model 2 and 3 in TR 37.885 for MC Video services.

Proposal 8	Update Section 6.2 – Channel model in TR37.885 with PL and Shadowing models specified in Appendix A of TR 36.843 and reuse fast fading model in TR 37.885 for PS use cases.

Proposal 9	For LLS of PS use cases, reuse CDL models for Urban LOS and Urban NLOS in TR 37.885.

Proposal 10	Create a simulation profile for PS use cases based on the above updates and power consumption models.

# 5	References

RP-201385, WID revision: NR sidelink enhancement, LG Electronics, RAN Meeting #88e, July 2020

3GPP TS 23.203, Policy and charging control architecture, version 16.2.0

3GPP TR 22.281, Mission Critical (MC) video, version 16.0.0